3GPP TSG RAN WG1 #109-e	R1-2203129

e-Meeting, May 9th - 20th, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	Potential solutions for SL positioning

Agenda item:	9.5.1.3

Document for:	Discussion and Decision

# Introduction

At RAN#94, a new study item “Study on expanded and improved NR positioning” (FS_NR_pos_enh2) was approved [1].

In this contribution, we outline key design aspects of sidelink (SL) positioning (including ranging), indicate potential key problems, and discuss possible approaches to tackle these problems.

# Discussion

## Terminology

Before discussing potential solutions for SL positioning, we see it beneficial to align on using the same terminology related to SL positioning, at least during this study item. We propose to use the following terms with the respective definitions:

- Target UE: UE to be positioned (in this context, using SL, i.e. PC5 interface).

Note: We prefer to use a simpler definition than the one provided in 3GPP TR 23.700-86 [2].

- Anchor UE (or Supporting UE): UE supporting positioning of target UE, e.g., by transmitting and/or receiving reference signals for positioning, providing positioning-related information, etc., over the SL interface.

Note: Any UE can be both target UE and anchor UE at the same time, or switch roles during a positioning session.

Note: In [2], the terms called “Reference UE” and “Assistant UE” are defined. From the RAN perspective, we see that the naming “Reference UE” might create confusion with the Positioning Reference Unit (PRU) introduced in Rel. 17. Second, we see that the defined roles for “Reference UE” and “Assistant UE” in [2] are interchangeable, and see no need to define such types of UEs separately.

- Sidelink positioning: Positioning UE using reference signals transmitted over SL, i.e., PC5 interface, to obtain absolute position, relative position, or ranging information.

Note: In [2], the definition of “sidelink positioning” is not logically connected to absolute/relative positioning or ranging over SL. We propose to have “sidelink positioning” as a collective term that covers all, so as not to list them every time separately and explicitly.

- Ranging: determination of the distance and/or the direction between a UE and another entity, e.g., anchor UE.

Note: While relative positioning information refers to relative coordinates of a UE with respect to another entity, ranging information refers to only the distance and/or angle between them.

Note: In [2], ranging definition also covers relative positioning. However, we see these terms differently as captured in the Note above.

- Sidelink positioning reference signal (SL PRS): reference signal transmitted over SL for positioning purposes.

- SL PRS (pre-)configuration: (pre-)configured parameters of SL PRS such as time-frequency resources including its bandwidth and periodicity. The (pre-)configuration information provided to UEs can be part of an “assistance data for SL positioning”.

Proposal 1: Align on the above-provided terminology on SL positioning, to be used (at least) during the study.

## Role of SL in Positioning

Figure 1 – Role of sidelink in positioning

SL interface can undertake various roles when positioning a target UE. Specifically, we distinguish four such roles, which correspond to a) offloading Uu control signalling; b) offloading Uu positioning; c) enhancing Uu positioning; d) SL positioning as standalone. All such cases are illustrated in Figure 1 and described below.

SL communications to offload Uu control signalling: First, UEs can exploit SL communications in order to offload control signalling on the Uu link for positioning purposes, under in-coverage and partial coverage scenarios. In this case, a UE can act as a relay between the other UEs and the network (as well as between different UEs), by forwarding data related to positioning procedures, such as assistance data, positioning measurements, location information requests, capability information, location information, etc. In this way, 1-to-1 signalling between each UE and the network as in legacy positioning procedures can be avoided, while requiring only one of the UEs to be in coverage.

SL positioning to offload Uu positioning: Second, UEs can do positioning using the SL interface, which can be again used to offload positioning procedures using the Uu interface, under in-coverage and partial coverage scenarios. In this case, one UE can be positioned via Uu positioning, while other UEs in proximity can be positioned by combining SL positioning results on top of the positioned UE. This approach enables multiple UEs to be positioned, while requiring only one of the UEs to be positioned via the Uu link.

SL positioning to enhance Uu positioning: Third, SL positioning can be utilized to enhance the performance of Uu-based positioning under in-coverage and partial coverage scenarios. Under limited network deployment or harsh NLOS conditions, UEs can make use of SL anchors with known/fixed locations (e.g., RSUs) in addition (or alternative) to the gNBs. Such approach would enable increased positioning accuracy.

Standalone SL positioning: Finally, SL positioning can take place as a standalone positioning solution, especially outside the network or GNSS coverage. In this case, UEs can obtain relative location information with respect to each other, or can obtain absolute positioning information by taking UEs with known locations into account, via positioning measurements over SL.

We here further note that, multiple such roles can take place within a given positioning session.

Proposal 2: Besides offering a standalone positioning solution, SL can be also utilized to enhance or offload Uu-based positioning, or simply offload Uu control signaling for positioning purposes via SL communications.

## Procedures for SL Positioning

SL positioning necessitates several procedures including:

- Discovery and selection of positioning anchors

- Configuration of SL PRS and positioning measurements

- Exchange of assistance data, measurement reports, location estimates, etc.

The above procedure can be effectively realized by a central coordinating entity. The LMF can take this role as in the case of Uu-based positioning if UEs are under in- (or partial-) coverage. Alternatively, network can also delegate a UE to help coordinating and/or configuring SL positioning. Similarly, UEs can configure themselves autonomously for SL positioning, which can be coordinated by e.g., the target UE, when considering out-of-coverage scenarios. However, such roles and the coordination would depend on capability, eligibility, and suitability of the UEs.

Proposal 3: UEs may perform functionalities related to coordination and/or configuration of a SL positioning session, especially in out-of-coverage scenarios. Such role may depend on the capability, eligibility, and suitability of UEs that might be potentially involved in SL positioning.

SL positioning procedures are expected to take place simultaneously for multiple SL positioning sessions in a local area. This implies that such procedures should be coordinated with each other, in order to ensure maximum efficiency as well as minimum resource conflicts. For example, when a new UE is appointed with the role of anchor UE (also known as supporting UE) for the SL positioning session of a target UE, this anchor UE should coordinate with other anchor UEs in the same area (serving other target UEs) on which SL resources will be used for each positioning session. To allow for such optimal resource allocation, UEs should be able to exchange information on the resources allocated to each SL positioning session.

Proposal 4: Study mechanisms to allow UEs to exchange information on resources used for SL positioning to coordinate between concurrent SL positioning sessions.

## Measurement Techniques for SL Positioning and SL PRS Design

SL positioning could make use of the well-known techniques for estimating the location of the target UE.

Among them, UEs could use timing-based methods such as TDOA or Multi-RTT over the SL interface. While TDOA would require synchronization among the anchors, Multi-RTT method does not require it. However, with Multi-RTT, the positioning procedure may take longer due to additional signalling required. As a drawback, accuracy of timing-based methods is impacted by the bandwidth of the reference signal used for positioning.

Under limited availability of bandwidth, positioning methods based on angular measurements (e.g., AoA, AoD) become useful. Moreover, for various use cases involving ranging where angle information is required, angular solutions are a must. On the other hand, these solutions require multiple antennas, involve complex procedures, which might further have specification impact on the use of the FR2 spectrum, which is not yet available for the SL interface. In addition, angular techniques require at least the anchor or the target UE to be static. Nevertheless, in V2X use cases, UEs with known orientation could be exploited for this purpose.

Proposal 5: Different positioning techniques based on time and angle-based measurements such as TDOA, Multi-RTT, AoA, AoD, etc. should be studied for sidelink positioning, to better evaluate their advantages and drawbacks.

To conduct measurements related to positioning techniques, transmission of a reference signal over the SL is necessary. Currently, the NR SL supports the following reference signals:

- Demodulation reference signal (DMRS) transmitted with associated SL physical channels PSCCH, PSSCH, and PSBCH

- SL primary synchronization signal (S-PSS) and SL secondary synchronization signal (S-SSS) transmitted in SL synchronization signal block S-SSB

- SL channel state information reference signal (SL CSI-RS) transmitted within PSSCH

- SL phase-tracking reference signal (SL PT-RS) transmitted within PSSCH

These, however, may not be suitable for positioning purposes. First, these signals are typically of small bandwidth. DRMS can occupy at most the size of the associated PSSCH. This is also the case for CSI-RS and SL PT-RS. Similarly, S-SSB occupies 11 RBs in frequency. Second, these signals cannot be transmitted as a stand-alone. DMRS is always associated with a PSCCH, PSSCH or PSBCH transmission, and SL CSI-RS and SL PT-RS are transmitted within a PSSCH. For synchronization signals, UEs transmit the same information using the same resources.

Observation 1: Currently defined reference signals over the NR SL are not suitable for positioning purposes.

We therefore propose to define a new reference signal over SL for positioning purposes, which can be transmitted in a standalone manner, possibly with a large bandwidth. Further, parameters of this signal should be configurable, including time-frequency resources such as defined in terms of bandwidth, periodicity, bitmap, combing, etc. We refer to this signal as SL PRS, and the related configuration as SL PRS configuration.

With regards to design of SL PRS, one option could be to re-use existing structure of the reference signals used for positioning on the Uu interface. For this purpose, we see re-using the UL SRS as beneficial, so that the same signal can be also received by gNB when the SL PRS transmission takes place on the SL resources sharing the UL bands.

Proposal 6: A new reference signal should be defined over SL interface for positioning purposes. For this, the structure of the UL SRS can be re-used, with any necessary modifications to be studied.

## Resource Allocation for SL Positioning

Similar to the on-demand PRS concept introduced in Rel. 17 for Uu-based positioning, SL PRS needs to be transmitted only when required, in order not to waste SL resources. This necessitates mechanisms for allocation of required resources for the transmission/reception of SL PRS. Related design questions would be:

- which spectrum is allocated for SL PRS transmission,

- whether SL PRS co-exists with other SL transmissions,

- where exactly is SL PRS transmitted (in which SL resources),

- whether resource allocation modes 1 and 2 in NR SL communications can be reused for scheduling SL PRS.

Spectrum for SL positioning plays an important role, since when time-based measurement techniques are utilized, accuracy of the positioning would depend on the bandwidth of the utilized SL PRS. According to the current regulations [3]:

- In Europe, 40 MHz (5875-5915 MHz) is designated for safety related ITS, prioritized for road.

- In the US, 30 MHz (5895-5925 MHz) dedicated for C-V2X from the ITS band at 5.9 GHz.

- In China, 10 MHz (5905-5915 MHz) from the ITS band is dedicated for the cellular V2V communications.

Further, in 3GPP Rel. 17, maximum allowed NR SL Resource pool configuration is 275 PRBs, which amounts to a maximum of 49.5 MHz [1].

Such sizes of available bandwidth would be a limitation considering the high accuracy performance required for the SL positioning use cases (V2V, public safety, IoT, commercial). Therefore, besides the ITS band, allocation of licensed, and/or if necessary, unlicensed bands, for SL positioning, as well as required modification to the configuration and utilization of SL resource pools should be studied to enable transmission of wide-band SL PRS. In order to determine the frequency bands to be allocated for SL positioning purposes, the bandwidth requirements based on the performance evaluations should be considered first.

Proposal 7: Allocation of frequency bands for SL positioning purposes, and any required modifications to the configuration and use of SL resource pools should consider the bandwidth requirements of SL positioning based on the performance evaluations to be conducted.

Given the scarcity of spectrum, SL resources can be more efficiently utilized if SL PRS transmissions can co-exist with other SL transmissions. Otherwise, reserving a certain band or sub-band for SL PRS transmission might waste these resources when they are not utilized. Co-existence, on the other hand, would require additional mechanisms, e.g., in order not to harm legacy SL communication transmissions. This would include multiplexing of SL PRS with other SL transmissions, as well as multiplexing of SL PRS belonging to different users, such as in time, frequency, or code. It is noted that resulting SINR (or decoding) performance of SL-PRS may be degraded at the respective receiver UE, e.g., target UEs when multiple, e.g., anchor UEs select the same time-frequency resources for their SL-PRS sequences transmission that may be non-orthogonal at the target UEs. Such degradation would in turn significantly affect the quality of positioning measurements (based on time, power, etc. over SL-PRS), hence the accuracy of location information to be estimated. Hence, coordination with the target UE may be needed for multiplexing of SL PRS.

Proposal 8: Study mechanisms to support co-existence of SL PRS transmissions with other SL transmissions.

Proposal 9: Study mechanisms to support multiplexing of SL PRS belonging to different users in time, frequency or code domain, such as in coordination with the target UEs.

Further, it needs to be specified, using which SL resources is SL PRS transmitted, such as PSCCH, PSSCH, PSFCH, etc., within SL resource pools, or S-SSB-like transmissions occupying independent time/frequency resources, and needs to be understood whether any changes to the SL frame structure is required.

Proposal 10: Study which SL resources (e.g., PSCCH, PSSCH, etc. within SL resource pools or independent time/frequency resources as in the case of S-SSB) can be utilized to transmit/receive SL PRS, and any required changes to the SL frame structure.

Lastly, efficient mechanisms for scheduling of SL PRS are required. While SL PRS transmissions could be scheduled by the network when UEs are in (or partially) coverage, UEs should be able to schedule their own SL PRS transmissions when outside the network coverage. For these purposes, NR SL resource allocation modes 1 and 2 seem a suitable fit. However, any necessary modifications for SL positioning should be studied.

Proposal 11: Prioritize re-using existing resource allocation modes 1 and 2 from NR SL communications for scheduling SL PRS transmissions, and study any necessary modifications.

# Conclusions

In this contribution, we have made the following observations and proposals related to potential solutions for SL positioning:

Proposal 1: Align on the above-provided terminology on SL positioning, to be used (at least) during the study.

Proposal 2: Besides offering a standalone positioning solution, SL can be also utilized to enhance or offload Uu-based positioning, or simply offload Uu control signaling for positioning purposes via SL communications.

Proposal 3: UEs may perform functionalities related to coordination and/or configuration of a SL positioning session, especially in out-of-coverage scenarios. Such role may depend on the capability, eligibility, and suitability of UEs that might be potentially involved in SL positioning.

Proposal 4: Study mechanisms to allow UEs to exchange information on resources used for SL positioning to coordinate between concurrent SL positioning sessions.

Proposal 5: Different positioning techniques based on time and angle-based measurements such as TDOA, Multi-RTT, AoA, AoD, etc. should be studied for sidelink positioning, to better evaluate their advantages and drawbacks.

Observation 1: Currently defined reference signals over the NR SL are not suitable for positioning purposes.

Proposal 6: A new reference signal should be defined over SL interface for positioning purposes. For this, the structure of the UL SRS can be re-used, with any necessary modifications to be studied.

Proposal 7: Allocation of frequency bands for SL positioning purposes, and any required modifications to the configuration and use of SL resource pools should consider the bandwidth requirements of SL positioning based on the performance evaluations to be conducted.

Proposal 8: Study mechanisms to support co-existence of SL PRS transmissions with other SL transmissions.

Proposal 9: Study mechanisms to support multiplexing of SL PRS belonging to different users in time, frequency or code domain, such as in coordination with the target UEs.

Proposal 10: Study which SL resources (e.g., PSCCH, PSSCH, etc. within SL resource pools or independent time/frequency resources as in the case of S-SSB) can be utilized to transmit/receive SL PRS, and any required changes to the SL frame structure.

Proposal 11: Prioritize re-using existing resource allocation modes 1 and 2 from NR SL communications for scheduling SL PRS transmissions, and study any necessary modifications.

# References

- RP-213588, “Study on expanded and improved NR positioning”

- 3GPP TR 23.700-86, “Technical Specification Group Services and System Aspects; Study on Architecture Enhancement to support Ranging based services and sidelink positioning; (Release 18)”, V0.2.0 (2022-04)

- 5GAA, “Deployment band configuration for C-V2X at 5.9 GHz in Europe”, 5GAA Automotive Association Position Paper, June 2021.

- 3GPP TS 38.331, “NR; Radio Resource Control (RRC); Protocol specification”, v 17.0.0