3GPP TSG-RAN WG1 Meeting #109-e	R1-2203131

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.11.1

Source:	Huawei, HiSilicon

Title:	Discussion on XR-specific power saving techniques

Document for:	Discussion and Decision

Introduction

In the RAN1 Rel-17 study item “Study on XR Evaluations for NR”, progress has been made on many aspects, e.g. applications, KPIs, evaluation assumptions/methodologies, and performance evaluations. The results have been summarized in TR 38.838 [1]. For RAN Rel-18, a new SID “Study on XR Enhancements for NR” has been approved in RAN#94e with the following areas: XR-awareness in RAN, XR-specific power saving, and XR-specific capacity improvements [2]. The details are as follows:

(Copied from RP-213587 [2]) - 4 Objective - 4.1 Objective of SI or Core part WI or Testing part WI The study is to be based on Release 17 TR 38.838, on corresponding Release 17 work from SA4 (as per SP-210043) and on Release 18 work from SA2 (as per SP-211166). Objectives on XR-awareness in RAN (RAN2): - Study and identify the XR traffic (both UL and DL) characteristics, QoS metrics, and application layer attributes beneficial for the gNB to be aware of. - Study how the above information aids XR-specific traffic handling. Objectives on XR-specific Power Saving (RAN1, RAN2): - Study XR specific power saving techniques to accommodate XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc...). Focus is on the following techniques: - C-DRX enhancement. - PDCCH monitoring enhancement. Objectives on XR-specific capacity improvements (RAN1, RAN2): - Study mechanisms that provide more efficient resource allocation and scheduling for XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc…). Focus is on the following mechanisms: - SPS and CG enhancements; - Dynamic scheduling/grant enhancements.

In this contribution, we mainly provide our views on XR-specific power saving techniques.

Discussion

XR-specific power saving issues

Many XR and Cloud Gaming devices are expected to be mobile and lightweight, thus having limited battery power resources. It is important to study the potential enhancements for XR power saving. According to the objectives, XR-specific power saving techniques should take XR traffic characteristics into account, e.g. periodicity, multiple flows, jitter, latency, reliability, etc., which has been identified in R17 XR study [1]. We analyze these XR traffic characteristics in detail as below:

- Non-integer periodicity: For a typical video, the frames arrive periodically every 1/F second, where F is the frame rate in FPS of the video. Typical frame rates of XR video are 30FPS, 60FPS, 90FPS, and 120FPS, resulting in the non-integer periodicity of XR traffic, e.g., 1/30s, 1/60s, 1/90s, and 1/120s.  However, the existing power saving techniques for periodic traffic arrival, e.g., C-DRX and PDCCH monitoring, only support integer cycle values, e.g., integer multiples of 1 ms or 
1 slot, which will lead to mismatch between XR traffic and DRX OnDuration or Search Space duration over time, as illustrated in Figure 1.

Figure 1. Mismatch between XR traffic and DRX OnDuration or Search Space duration

- Jitter: Since each XR video frame may experience different encoding/rendering delay and network transmission delay, the frame arrival at gNB is not exactly periodic. The XR traffic may arrive early or late, resulting jitter at gNB, e.g. jitter range of [-4ms, 4ms] is assumed in R17 XR study [1]. As a result, to meet the low-latency requirement of XR services, the UE may need to keep monitoring the PDCCH during the jitter range in order to be scheduled in time, which causes extra power consumption.

- Multiple flows: As identified in RAN1 Rel-17 study, there can be multiple data streams with different traffic characteristics and QoS requirements in both DL and UL for a given XR application, for example, I-frame and P-frame, FOV stream and omnidirectional stream, video/depth and pose/control, video stream and audio stream [1]. Similar to single-flow case, the non-integer periodicity issue and the jitter issue may also exist for multiple flows. At the current stage, we do not see any specific power saving issue for multiple flows from RAN1’s perspective. We can focus on handling the power saving issues for single-flow case, and assume the enhancements can also apply for multiple flows.

- Low latency and high reliability requirements: Compared with eMBB, XR services have even more stringent requirements in aspects including data rate, latency, and reliability. For example, according to RAN1 Rel-17 study [1], to provide an acceptable user experience, a spatial resolution of 2K per eye and a frame rate of 60FPS is required for the XR video, the data rate of which can be up to 30~60Mbps. For strong-interaction XR applications and split rendering, the frame delay budget of 10ms for DL XR video transmission is required. For reliability, frame error rate less than 10-2 for UL/DL transmission is required to guarantee the satisfactory quality-of-experience (QoE). These characteristics should be considered when studying the XR UE power saving, since there might be some trade-off between power consumption and capacity/latency/reliability. Higher power saving gain will typically result in poorer capacity/latency/reliability performance. If the non-integer periodicity issue and the jitter issue discussed above are addressed, then it has already considered the latency and reliability requirements. For example, if there is no latency or reliability requirement for XR service or if such requirement is very loose, there seems no need to address non-integer periodicity and jitter issue, since the XR UE can always wait a while for scheduling and retransmission can be performed in case of transmission/reception failure.

Based on the above discussion, we think that the non-integer periodicity and the jitter are the key issues that need to be addressed when considering XR-specific power saving techniques.

Proposal 1: Further study power saving techniques to address the following XR-specific issues:

- Non-integer periodicity

- Jitter

XR-specific power saving techniques

There are lots of power saving techniques for NR, including Rel-15 power saving techniques, e.g. DRX and PDCCH monitoring (e.g., search space configuration, including periodicity, duration, and offset), Rel-16 power saving techniques, e.g., PDCCH-based wake-up signaling, cross-slot scheduling, MIMO layer adaptation, etc., and Rel-17 power saving techniques, e.g., the PDCCH skipping and SSSG switching. According to the SID scope, we focus on C-DRX enhancement and PDCCH monitoring enhancement to address the above issues. We would like to note that the PDCCH skipping and SSSG switching identified in Rel-17 are PDCCH monitoring adaptations, which belong to the scope of PDCCH monitoring enhancement and can also be considered. In the following, we mainly discuss the C-DRX enhancement, PDCCH monitoring enhancement (including search space periodicity/duration/offset configuration, PDCCH skipping, and SSSG switching), and the combination of them to address the above XR-specific power saving issues.

When studying XR-specific power saving techniques, some general principles can be considered. For example, the solutions should be forward compatible. Although typical frame rates of XR video are 30FPS, 60FPS, 90FPS, and 120FPS, there may be other new frame rates in the future. The potential solutions for addressing the non-integer periodicity issue should be applicable to handle potential new frame rate in the future. Furthermore, since the identified XR-specific issues (e.g. non-integer periodicity, jitter) exist simultaneously in real XR applications, solution(s) that can address multiple or all the identified issues are more preferred.

Proposal 2: When studying the potential enhancements to address XR-specific issues, the following general principles are considered

- Principle 1: The solutions should be forward compatible, e.g., applicable to potential new frame rate in the future.

- Principle 2: Solution(s) that can address multiple or all the identified issues are more preferred.

Potential enhancements to address non-integer periodicity issue

To address the non-integer periodicity issue, enhanced C-DRX or enhanced PDCCH monitoring (search space configuration) mechanisms, which aims to align the start of ON duration and the XR frame arrival, can be further studied.

Both C-DRX and PDCCH monitoring (search space configuration) mechanisms allow UE to periodically monitor PDCCH for some time, which is realized by configuring a cycle/periodicity, an offset, and an ON duration. Generally, some directions for C-DRX and PDCCH monitoring enhancements can be considered to address the non-integer periodicity issue, for example, 1) adjusting the start offset of duration, 2) adjusting the cycle/periodicity, e.g. configuring a cycle pattern, 3) multiple C-DRX configurations with different offsets. The details of these mechanisms can be further studied.

According to the Rel-17 performance evaluation in clause 8.3.3.1 and clause 8.3.3.8 of TR 38.838 [1], both enhanced C-DRX and enhanced PDCCH monitoring mechanisms can achieve certain power saving gain compared with baseline scheme (i.e. AlwaysOn) with acceptable reduced number of satisfied UEs.

Proposal 3: Further study the following directions for C-DRX and PDCCH monitoring enhancements to address the non-integer periodicity issue:

- Adjusting the start offset of DRX OnDuration and/or the start offset of PDCCH monitoring

- Adjusting the periodicity, e.g. configuring a cycle pattern

- Multiple C-DRX configurations

Potential enhancements to address jitter issue

Due to jitter, the XR traffic may arrive at gNB early or late. Generally, to meet the low-latency requirement, the UE may need to keep monitoring the PDCCH during the jitter range in order to be scheduled in time. For example, the length of C-DRX OnDuration or the length of search space Duration needs to be large enough to cover the whole or most parts of the jitter range, as illustrated in Figure 2, resulting in extra power consumption.

Observation 1: C-DRX OnDuration or the length of search space Duration needs to be large enough to cover the whole or most parts of the jitter range, resulting in extra power consumption.

Figure 2. Power saving gain may be sacrificed by monitoring the PDCCH during the jitter range

To avoid increasing the UE power consumption too much, mechanisms to allow skipping unnecessary PDCCH monitoring within jitter range can be further studied. Generally, there is no need to monitor every slot during the jitter range.

As shown in Figure 3 below, according to Rel-17 XR study [1], jitter follows truncated Gaussian distribution with the range of [-4, 4] ms. Such information can be taken into account to reduce the monitoring occasions, e.g., the monitoring occasions could match the jitter distribution.

Figure 3. Jitter distribution

For the case of early arrival, the UE can skip unnecessary PDCCH monitoring once the XR frame is received correctly. For the case of late arrival, the UE can monitor PDCCH sparsely before the data arrives and then monitors PDCCH densely once the data arrives, where the SSSG switching can be implicit based on data arrival.

Proposal 4: Further study the following mechanisms to allow skipping unnecessary PDCCH monitoring caused by jitter:

- Configuring a PDCCH monitoring pattern within DRX OnDuration or Search Space duration to match jitter distribution;

- Skipping unnecessary PDCCH monitoring once the XR frame is received correctly;

- UE monitors PDCCH sparsely before the data arrival and monitors PDCCH densely once the data arrives, e.g., implicit SSSG switching based on data arrival.

XR-awareness in RAN for power saving

According to the above discussion, some of the potential enhancements of C-DRX and PDCCH monitoring may require assistance information from the core network (CN) or application to configure their parameters, e.g. periodicity, the length of duration, etc. SA2 sent an LS to RAN1 in R1-2203219 [5] and asked RAN1 and RAN2 to clarify which type of information should be provided to the RAN for power saving enhancements for XR applications. We have a companion paper [6] to discuss this LS. According to our companion paper [6], the following information can be beneficial to the RAN for power saving enhancements for XR applications:

  - Frame rate or traffic arrival interval/periodicity

  - Jitter distribution related parameters, including range, STD (standard deviation)

  - PDU set related information, including the relationship between packets belonging to the same PDU set and relationship between packets and the PDU set

Conclusions

In this contribution, we identify that the non-integer periodicity and the jitter of XR traffic are two key issues that need to be further studied when considering XR-specific power saving techniques. Some potential enhancements of C-DRX and PDCCH monitoring are discussed and can be further studied to address these issues. In summary, we have the following observations and proposals:

Observation 1: C-DRX OnDuration or the length of search space Duration needs to be large enough to cover the whole or most parts of the jitter range, resulting in extra power consumption.

Proposal 1: Further study power saving techniques to address the following XR-specific issues:

- Non-integer periodicity

- Jitter

Proposal 2: When studying the potential enhancements to address XR-specific issues, the following general principles are considered

- Principle 1: The solutions should be forward compatible, e.g., applicable to potential new frame rate in the future.

- Principle 2: Solution(s) that can address multiple or all the identified issues are more preferred.

Proposal 3: Further study the following directions for C-DRX and PDCCH monitoring enhancements to address the non-integer periodicity issue:

- Adjusting the start offset of DRX OnDuration and/or the start offset of PDCCH monitoring

- Adjusting the periodicity, e.g. configuring a cycle pattern

- Multiple C-DRX configurations

Proposal 4: Further study the following mechanisms to allow skipping unnecessary PDCCH monitoring caused by jitter:

- Configuring a PDCCH monitoring pattern within DRX OnDuration or Search Space duration to match jitter distribution;

- Skipping unnecessary PDCCH monitoring once the XR frame is received correctly;

- UE monitors PDCCH sparsely before the data arrival and monitors PDCCH densely once the data arrives, e.g., implicit SSSG switching based on data arrival.

- References

3GPP TR 38.838, “Study on XR (Extended Reality) Evaluations for NR,” Dec. 2021.

RP-213587, “Study on XR Enhancements for NR”, RAN#94e, Dec. 6 - 17, 2021.

R1-2102308, “Reply LS on New Standardized 5QIs for 5G-AIS (Advanced Interactive Services)”, 3GPP TSG SA WG4#112-e, February 01-10, 2021.

3GPP TR 23.700-60 V0.1.0, “Study on XR (Extended Reality) and media services”, Feb. 2022.

R1-2203219, “LS on UE Power Saving for XR and Media Services”, RAN1#109-e, May 9 – 20, 2021.

R1-2204926, “Discussion on LS from SA2 on UE Power Saving for XR and Media Services”, RAN1#109-e, May 9 – 20, 2021.