3GPP TSG-RAN WG1 Meeting #109-e	R1-2203133

e-Meeting, May 9 – 20, 2022

Agenda Item:	9.8.1

Source:	Huawei, HiSilicon

Title:	Discussions on side control information to enable NR network-controlled repeaters

Document for:	Discussion and Decision

# Introduction

The study item of NR network-controlled repeaters (NCR) was approved at RAN#94e [1]. The detailed objectives related to RAN1 include:

- Scenarios and assumptions: NCRs are inband RF repeaters used for extension of network coverage on FR1 and FR2 bands, while during the study FR2 deployments may be prioritized for both outdoor and O2I scenarios.

- Side control information: study and identify which of the below is necessary for NCR including assumption of max transmission power.

  - Beamforming information

  - Timing information to align transmission / reception boundaries of network-controlled repeater

  - Information on UL-DL TDD configuration

  - ON-OFF information for efficient interference management and improved energy efficiency

  - Power control information for efficient interference management (as the 2nd priority)

- L1/L2 signaling: study and identify L1/L2 signaling (including its configuration) to carry the side control information

In this contribution, we discuss the NCR and side control information to enable NCR.

# Discussions on NCR

## NCR modules

As shown in Figure 1, an NCR is an enhanced RF repeater with a control unit (MT) and a radio unit (RU). The NCR MT performs as a full/reduced function UE, i.e., NCR accesses the gNB, then it is controlled by the gNB to amplify-and-forward the signals using the NCR RU. The NCR RU is composed of a downlink branch and an uplink branch, and MT and RU share the antennas and part of the RF chain.

Figure 1 NCR block diagram

## NCR working procedures

An NCR mainly works in two phases, as shown in Figure 2 and Figure 3 respectively.

In the first phase, the NCR MT accesses the network via the gNB-side antenna, as a UE using NR defined initial access and beam management procedures. The NCR node is identified by the network and a control link between NCR MT and gNB is established, the capability of NCR is reported to the gNB, and a set of suitable backhaul (BH) beams are also identified for downlink and uplink.

Figure 2 NCR access to gNB

In the second phase, the NCR RU performs signal reception, amplification, and then forwarding for both uplink and downlink. The BH beams used for forwarding the uplink and downlink signals are determined in the first phase. For forwarding the uplink signal, the UE-side antenna receives the signal from the UE, then the uplink RF branch processes, amplifies, and finally forwards the signal to the gNB by the gNB-side antenna. Forwarding the downlink signal is performed in a similar way: the gNB-side antenna receives the signal, the downlink RF branch processes, amplifies, and finally forwards the signal to the UE by the UE-side antenna. The access (AC) beam, backhaul (BH) beam, and the amplifying gain are controlled by gNB.

Figure 3 NCR amplify-and-forward signal

Based on the above discussions, we have the following observation:

Observation 1: An NCR can access the network and identify the backhaul beam following the existing NR procedures, and then amplify-and-forwards the signals under network control.

# NR coverage performance

The coverage performance for both FR1 and FR2 has been evaluated extensively in the Rel-17 coverage enhancement SI. The performance of the UL and DL channels are summarized in 5.1.2 for FR1 and 5.2.2 for FR2 [2], and the observations for Urban FR1 O2I and FR2 O2I are quoted below:

From the representative values for Urban 4GHz TDD scenario, the following observation is made: - PUSCH for eMBB with frame structure DDDSU is the worst channel in terms of MIL, and can be the coverage bottleneck for the given scenario. - In order to achieve the target MPL calculated from 400m ISD, the following channel(s) needs to be enhanced: - PUSCH for eMBB with frame structure DDDSU - PUSCH for eMBB with frame structure DDDSUDDSUU - In order to achieve the target MPL calculated from 500m ISD, the following channel(s) needs to be enhanced: - PUSCH for eMBB with frame structure DDDSU - PUSCH for eMBB with frame structure DDDSUDDSUU - PUSCH for VoIP with frame structure DDDSU - However as shown by standard deviation in the table, this additional gain may be achieved by the existing technologies or parameter optimization, which means that new a functionality in Rel-17 is not always required. - PUSCH for VoIP with frame structure DDDSUDDSUU - However as shown by standard deviation in the table, this additional gain may be achieved by the existing technologies or parameter optimization, which means that a new functionality in Rel-17 is not always required. - PUCCH format 3 with 22 bit payload - PRACH Format B4 - However as shown by standard deviation in the table, this additional gain may be achieved by the existing technologies or parameter optimization, which means that a new functionality in Rel-17 is not always required. From the representative values for Urban 28GHz TDD NLOS O2I scenario, the following observations can be made: For the 1st priority channels (with more than 2 available samples): - With absolute MPL target corresponding to ISD target 200m, only PDCCH can meet the absolute MPL target. 1st priority channels which do not meet the absolute MPL target are: - PUSCH eMBB DDDSU - PUSCH eMBB DDSU - PUSCH VoIP DDDSU - PDSCH eMBB DDSU - PUCCH Format 1 - PUCCH Format 3 11bits - PUCCH Format 3 22bits - SSB - PRACH Format B4: 20.30dB - PDCCH of Msg2 - PUSCH of Msg3 - PDSCH of Msg4 - PDSCH eMBB DDDSU - PUSCH VoIP DDSU

The general observation from the Rel-17 coverage enhancement SI is that for FR1, the coverage bottleneck is uplink channels and the coverage gap is only up to several dBs, while for FR2, there is coverage bottleneck for both downlink and uplink and the performance gap can be up to more than 10 dB. In Rel-17, joint channel estimation, TB processing over multiple slot, and enhanced repetitions have been specified to compensate the coverage gap of PUSCH, PUCCH, and Msg3. The uplink channels are to be further enhanced in Rel-18. The target coverage for FR1 is expected to be fulfilled by Rel-17 and Rel-18 coverage enhancements. In contrast, it is anticipated the coverage for FR2 will be still challenging considering the larger performance gap in both UL and DL. From this perspective, it can be expected that NCR is more beneficial to improve the coverage performance for FR2 deployment.

Observation 2: The target coverage for FR1 is expected to be achieved by the Rel-17/Rel-18 coverage enhancements while the target coverage for FR2 is challenging considering the larger performance gap in both UL and DL.

Observation 3: NCR is expected to be more beneficial to improve the coverage performance for FR2 deployment.

## Evaluation assumptions

To further investigate the coverage performance of NCR, we provide some evaluations in this section. The basic assumptions are listed in Table 1 and mostly same as ones in the Rel-17 coverage enhancement study in TR 38.830 [2].

Table 1 Assumptions for performance analysis

| Parameters | FR1 | FR2 |
| --- | --- | --- |
| Frequency (GHz) | 4 | 28 |
| Bandwidth (MHz) | DL 100 MHz UL 10.8 MHz (30 RB) | DL 100 MHz UL 43.2 MHz (30 RB) |
| gNB EIRP (dBm) | 70 | 65 |
| Channel model for NCR-UE and gNB-UE | NLOS | NLOS |
| Channel model and distance for gNB to NCR | LOS, 200 m | LOS, 100m |
| Pathloss model | UMa | UMa |
| NCR gain (dB) | 65 | 90 |
| Required RSRP (dBm) at target data rate | DL -125 dBm @ 10 Mbps UL -126 dBm @ 1 Mbps | DL -113 dBm @ 25 Mbps UL -114 dBm @ 5 Mbps |
| Shadowing and Penetration margin (dB) | 13.85 dB for NCR 24.39 dB for gNB | 13.85 dB for NCR 26.41 dB for gNB |

## Evaluation results

In Figure 4, the performance of gNB-NCR-UE link (gNB to UE via NCR only) and gNB-UE link (gNB to UE directly) are compared. For the case without NCR, the overall distance from gNB to UE is the sum of gNB-to-NCR distance and NCR-to-UE distance.

For FR1 as shown in Figure 4 (a), when the NCR-to-UE distance ranges from 20m to 200m, the gNB-to-UE distance ranges from 220m to 400m, which corresponds to an inter-site distances (ISD) from 330m to 600m. For a target data rate of 10 Mbps, it can be observed that a UE is better served by the gNB directly rather than via NCR for a NCR-to-UE distance larger than ~50m. For FR2 as shown in Figure 4 (b), when the NCR-to-UE distance ranges from 20m to 100m, the gNB-to-UE distance ranges from 120m to 200m, which corresponds to an ISD from 180m to 300m. For a target data rate of 25 Mbps, it can be observed that a UE is better served via the NCR rather than by gNB directly. Similar observations can be also made for UL.

The evaluation results further verify Observation 3 that NCR provides much more coverage extension benefits for FR2 than FR1 since the target coverage for FR1 can be achieved even without NCR while the target coverage performance can only be achieved with NCR for FR2.

Proposal 1: Capture the above evaluation results and the following observations into TR 38.867

- According to the evaluation results, the target coverage for FR1 can be achieved without NCR while the target coverage performance can only be achieved with NCR for FR2.

- NCR provides much more coverage extension benefits for FR2 than FR1

(a) FR1

(b) FR2

Figure 4 Performance of the gNB-UE link and gNB-NCR-UE link

# Discussion on NCR control information

## AC beam management

In NR, multi-beam transmission is utilized to provide coverage with high efficiency especially in FR2. Beam management procedures have been specified based on SSB/PRACH and CSI-RS/SRS. Similarly, an NCR can cover a larger area with multiple beams and higher beamforming gain for FR2. Nevertheless, if multiple beams are assumed at the NCR, beam management involving NCR and the impacts on the gNB performance should be considered.

As the example in Figure 3 shows, the gNB serves the UEs in the green area directly with M beams, and (M is the total number of gNB backhaul beams), and serves the UEs via NCR in the gray area indirectly with N beams (N is the total number of access beams). To enable beam sweeping at the NCR, the gNB needs to allocate some additional resources, e.g. SSBs, to the NCR using the same backhaul beam so that each time NCR can forward the signal using a dedicated access beam. In this way, beam sweeping can be done for the access link between NCR and UE.

The procedure is shown by Figure 5(a), gNB transmits M+N-1 signals (e.g., SSB) at t0, t1, …, tM+N-2, of which N signals are forwarded by the NCR. Here, it is assumed that one of the M beams (e.g., gNB beam B0 at t0) is forwarded by the NCR after being identified during the NCR access procedure, to reduce the number of additional required beams. The UEs in the coverage of gNB/NCR searches the SSB signals. For the SSBs at t0, tM, …, tM+N-2, the gNB transmits with the same beam B0, the NCR receives with the same BH beam b0 and forwards for the UEs with N AC beams. The NCR forwarding beams (or beam indices), and their forwarding time t0, tM, …, tM+N-2 are configured/indicated by the gNB. The gNB aligns its signal transmission and NCR forwarding.

Proposal 2: The beamforming information for an NCR including AC beam index and forwarding time is beneficial for NCR operation.

However, it can be observed there is an overhead increase at the gNB to enable the multi-beam operation for NCR. To be more specific, N-1 additional beams are required in the example. Based on the above analysis, the total number of required SSB/CSI-RS beams for different number of NCRs can be derived. Assuming M=32 and N=16, the total number of required gNB SSBs and the overhead are shown in Figure 5(b) for FR2. In the figure, only the overhead of SSB is shown, and the actual overhead for gNB is much larger considering the associated PRACH and SIB1. When there are more than two NCRs attached to the gNB, the total number of required SSBs exceeds 64 which cannot be supported by current specification.

In FR1, at most 4 or 8 SSBs are allowed for a gNB, and typically all these beams will be used by the gNB to ensure a good coverage for broadcast channels. Hence there is little room to allocate any additional SSB for an NCR to enable beam sweeping in FR1. Given that an NCR can be assumed to be deployed with good backhaul quality, e.g., LOS to the gNB. Then, the NCR can forward its best received SSB for the AC link and no additional SSB is required.

(a) SSB beam sweeping for a gNB with an NCR

(b)  Required number of gNB SSB beams with different number of NCRs

Figure 5 SSB sweeping with NCR

In NR, wide beams can be used for SSB and broadcast channels while narrower beams can be used for CSI-RS and unicast channels. The same coverage can be achieved with a smaller number of required SSBs of larger beamwidth. To reduce the number of required SSBs by NCR, the same design principle should be supported. For an NCR with different AC beams shown in Figure 6, the SSB can be forwarded by a single wide beam C0, and the CSI-RSs can forwarded by 4 medium width or 16 narrow beams.

Figure 6 NCR AC beams with different beamwidth

From another perspective, an NCR may be designed to support a certain number of access beams. However, since the overhead will increase with the number of access beams employed at the NCR, it is better that the gNB can have the flexibility to configure a smaller number of access beams used by the NCR, not simply according to the maximum number of the beams that can be supported by the NCR.

From the above analysis, we have the following observations and proposals:

Observation 4: To enable beam sweeping for NCR access link, the gNB has to allocate more backhaul beams for the NCR, leading to an increase of overhead. And the overhead increases with number of access beams at the NCR and the number of associated NCRs.

Observation 5: For FR1, there is little room for the gNB to allocate any additional SSB for an NCR to enable beam sweeping for broadcast channels.

Proposal 3: The number of SSB/CSI-RS access beams that can be used by the NCR can be configured by the gNB.

Proposal 4: For NCR in FR1, the maximum number of SSB beams should be limited to at most one.

Proposal 5: For NCR in FR2, the maximum number of SSB beams and CSI-RS beams should be limited, e.g., {1, 2} SSB beams, {4, 8, 16} CSI-RS beams.

## Timing

NCR timing includes the timing for both NCR MT and NCR RU, where NCR MT timing is the same as normal UE. The NCR RU timing is shown in Figure 7, which consist of uplink and downlink the forwarding windows. The RU AC reception and RU BH transmission are for uplink signal forwarding, and RU BH reception and RU AC transmission are for downlink signal forwarding.

Figure 7 Timing for gNB, NCR, and UE

If an NCR performs downlink and uplink forwarding using multiple beams, an error in the forwarding timing may cause signal lost and interference. An example is shown in Figure 8, three users UE0, UE1, UE2 are in the coverage of three NCR AC beams #0, #1, and #2, respectively. The gNB schedules the uplink transmissions in time slots 0~2, and one slot for each UE. As shown in the right sub-figure, an error in NCR AC beam timing leads to partial signal loss for all the three users, and the uplink transmission will be impacted. Similar problems exist for other operations such as ON-OFF, UL/DL conversion, and power control.

Moreover, the NCR RU forwarding window boundaries can be derived based on the NCR MT timing. The NCR RU BH and NCR MT transmission (or reception) boundaries are expected to be the same since the uplink signal departure time (or downlink signal arrival) should be the same, due to that they share the gNB-side antenna. The RU AC transmission and reception boundaries can also be derived based on NCR MT timing, due to the fact that the delay between gNB-side and UE-side antennas can be fixed and known by the NCR.

Observation 6: The NCR RU BH/AC transmission and reception boundaries can be determined based the NCT MT timing for both uplink and downlink.

Proposal 6: The timing information for NCR RU BH/AC transmission and reception boundaries does not needs to be indicated explicitly from the gNB to NCR.

Figure 8 Impact of error in NCR AC timing

## UL-DL TDD configuration

NR defines a flexible UL-DL TDD configuration framework, and time resources can be configured either semi-statically or dynamically. The semi-static UL-DL TDD configuration can be conveyed to a UE via SIB1 or dedicated RRC configuration, while dynamic UL-DL TDD configuration is indicated by scheduling or slot-format indication. In practical deployment, only semi-static UL-DL TDD configuration are used to avoid unnecessary cross-link interference among gNBs as well as the UEs.

For NCR, the coverage area is much smaller and hence a smaller number of covered UEs. Therefore, only a small portion of the overall time resources are expected to be required by the UEs connected via NCR. Moreover, an NCR has higher energy efficiency and lower overhead if it is only ON in the semi-static UL and DL time, due to less dynamic UL-DL indication.

Observation 7: In practical deployment, only semi-static UL-DL TDD configuration are used.

Proposal 7: Only semi-static UL-DL configuration is considered for NCR, e.g., obtained by SIB1 or dedicated RRC configuration.

## ON-OFF

The signal flow of an NCR is shown in Figure 9. The NCR receives a signal with power PNCR,Rx, then amplifies the signal together with the thermal noise and interference PNCR, Noise by a gain GNCR, and forwards to the destination (e.g., gNB for uplink, UE for downlink). Denote the overall loss between the NCR and the destination as Loss, then the total noise at the destination is:

PE2E, Noise=PDest, Noise+10log1010PNCR, Noise-GNCR-Loss-PDest, Noise/10 +1 amplified noise.

The amplified noise of uplink is illustrated by Figure 10(a), it is shown that the NCR may lead to significant interference (e.g., > 10 dB) for gNB when the amplifying gain is large. To avoid unnecessary noise rise, the most efficient way is to turn OFF the NCR when there is no traffic. However, the coverage of gNB broadcast channels could be not be impacted by NCR RU OFF, such as SSB, PRACH, and SIB1. For example, if an NCR is used to extend the coverage of a SSB, then NCR RU OFF over the SSB transmission time should be avoided. For the remaining channels/signals forwarded by NCR, turning ON can be in an on-demand manner under network control. OFF can be set as a default state for NCR RU if not configured or indicated to be ON. Moreover, the ON state is accompanied by beam/power indication. Overall, there seems no need to have a dedicated control information for ON-OFF information.

Observation 8: If an NCR is used to extend the coverage of a broadcast channel (e.g., SSB, PRACH, SIB1), NCR RU OFF over the broadcast channel time will lead to coverage hole.

Observation 9: OFF can be a default state for NCR RU if not configured or indicated to be ON while ON indication is accompanied by beam/power indication.

Proposal 8: The coverage of broadcast channels should not be impacted by NCR RU OFF.

Proposal 9: There is no need to have a dedicated control information for ON-OFF information for NCR RU.

Figure 9 NCR signal flow

## Power control

The thermal noise and signal distortion due to NCR lead to interference for the receiver, which degrades the network performance. For NCR, a dynamic power control can reduce the interference. If dynamic power control is not supported, a fixed amplifying gain has to be applied at the NCR. As analyzed in Figure 10, there are major problems due to fixed gain for uplink:

- Amplified noise leads to interference for UL multi-user multiplexing (e.g., FDM).

- Uplink transmission failed for a UE far away from the NCR if the fixed gain is low.

- Even worse, an over-amplified signal may block the gNB reception.

For downlink, more severe problems exist if a fixed gain is adopted:

- Signal distortion makes the forwarding useless if the fixed gain make the NCR saturated (due to a large NCR received power).

- Reduced NCR coverage leads to coverage hole at a lower NCR received power (as a result of a lower NCR DL transmission power).

- Unstable coverage performance due to the time changing NCR received power. The received power depends on backhaul link condition, such as channel fading or blockage.

Observation 10: For the uplink transmission via NCR, a fixed NCR amplifying gain may lead to interference to the gNB or NCR UL coverage loss.

Observation 11: For the downlink transmission via NCR, a fixed NCR amplifying gain may lead to NCR RU saturation or NCR DL coverage loss.

(a) NCR UL performance               (b) NCR DL output EIRP

Figure 10 Impact of fixed gain for NCR UL and DL

In light of above discussions, we have the following proposal:

Proposal 10: Dynamic control of amplifying gain in both UL and DL is beneficial for NCR operation.

# Conclusions

The contribution provides our considerations on NCR side control information, and the observations are listed as following:

Observation 1: An NCR can access the network and identify the backhaul beam following the existing NR procedures, and then amplify-and-forwards the signals under network control.

Observation 2: The target coverage for FR1 is expected to be achieved by the Rel-17/Rel-18 coverage enhancements while the target coverage for FR2 is challenging considering the larger performance gap in both UL and DL.

Observation 3: NCR is expected to be more beneficial to improve the coverage performance for FR2 deployment.

Observation 4: To enable beam sweeping for NCR access link, the gNB has to allocate more backhaul beams for the NCR, leading to an increase of overhead. And the overhead increases with number of access beams at the NCR and the number of associated NCRs.

Observation 5: For FR1, there is little room for the gNB to allocate any additional SSB for an NCR to enable beam sweeping for broadcast channels.

Observation 6: The NCR RU BH/AC transmission and reception boundaries can be determined based the NCT MT timing for both uplink and downlink.

Observation 7: In practical deployment, only semi-static UL-DL TDD configuration are used.

Observation 8: If an NCR is used to extend the coverage of a broadcast channel (e.g., SSB, PRACH, SIB1), NCR RU OFF over the broadcast channel time will lead to coverage hole.

Observation 9: OFF can be a default state for NCR RU if not configured or indicated to be ON while ON indication is accompanied by beam/power indication.

Observation 10: For the uplink transmission via NCR, a fixed NCR amplifying gain may lead to interference to the gNB or NCR UL coverage loss.

Observation 11: For the downlink transmission via NCR, a fixed NCR amplifying gain may lead to NCR RU saturation or NCR DL coverage loss.

The proposals are:

Proposal 1: Capture the above evaluation results and the following observations into TR 38.867

- According to the evaluation results, the target coverage for FR1 can be achieved without NCR while the target coverage performance can only be achieved with NCR for FR2.

- NCR provides much more coverage extension benefits for FR2 than FR1

Proposal 2: The beamforming information for an NCR including AC beam index and forwarding time is beneficial for NCR operation.

Proposal 3: The number of SSB/CSI-RS access beams that can be used by the NCR can be configured by the gNB.

Proposal 4: For NCR in FR1, the maximum number of SSB beams should be limited to at most one.

Proposal 5: For NCR in FR2, the maximum number of SSB beams and CSI-RS beams should be limited, e.g., {1, 2} SSB beams, {4, 8, 16} CSI-RS beams.

Proposal 6: The timing information for NCR RU BH/AC transmission and reception boundaries does not needs to be indicated explicitly from the gNB to NCR.

Proposal 7: Only semi-static UL-DL configuration is considered for NCR, e.g., obtained by SIB1 or dedicated RRC configuration.

Proposal 8: The coverage of broadcast channels should not be impacted by NCR RU OFF.

Proposal 9: There is no need to have a dedicated control information for ON-OFF information for NCR RU.

Proposal 10: Dynamic control of amplifying gain in both UL and DL is beneficial for NCR operation.

# References

- RP-213700, “New SI: Study on NR Network-controlled Repeaters”, ZTE Corporation, RAN#94e, December 9 – 17, 2021.

- TR 38.830, “Study on NR coverage enhancements”.