3GPP TSG-RAN WG1 Meeting #109-e	R1-2203134

e-Meeting, May 9 – 20, 2022

Agenda Item:	9.8.2

Source:	Huawei, HiSilicon

Title:	Discussion on L1/L2 signaling for side control information

Document for:	Discussion and Decision

Introduction

The study item of NR network-controlled repeaters (NCR) was approved at RAN#94e[1]. The detailed objectives related to RAN1 include:

- Side control information: study and identify which of the below is necessary for NCR including assumption of max transmission power.

  - Beamforming information

  - Timing information to align transmission / reception boundaries of network-controlled repeater

  - Information on UL-DL TDD configuration

  - ON-OFF information for efficient interference management and improved energy efficiency

  - Power control information for efficient interference management (as the 2nd priority)

- L1/L2 signaling: study and identify L1/L2 signaling (including its configuration) to carry the side control information

In this contribution, we discuss the L1/L2 signaling for the side control information.

Discussions

In general, all the three types of signaling can be considered for the side control information: radio resource control (RRC), medium access control - control elements (MAC-CE), and downlink control information (DCI). Typically, RRC is used to configure and re-configure the basic settings, such as the resources in time, frequency, or spatial (e.g., antenna ports, TCI state) domain. Then MAC-CE and DCI are used to convey semi-static and dynamic control signaling. The typical use cases, pros and cons of the signaling are summarized in Table 1.

Table 1 RRC, MAC-CE, DCI based control signaling

| Signaling | Typical usage | Pros | Cons |
| --- | --- | --- | --- |
| RRC | (Semi-)static configuration and scheduling | Low overhead | 10ms-level delay |
| MAC-CE | Dynamic configuration | Low overhead | ms-level delay |
| DCI | Dynamic scheduling | Symbol/slot level delay | High overhead |

Table 2 Control information for NCR

| Candidate control information | Contents | Comments |
| --- | --- | --- |
| Beamforming | Beam index for each NCR forwarding slot/symbol | New signaling from gNB to NCR |
| Timing | RU AC beam transmit/receive time RU BH beam transmit/receive time | No explicitly signaling from gNB to NCR |
| UL-DL TDD configuration | Static UL-DL TDD configuration | Legacy UL-DL TDD configuration |
| ON-OFF | ON slot/symbol | No dedicated signaling from gNB to NCR |
| Power control | Amplifying gain for each slot/symbol | New signaling from gNB to NCR |

For NCR, the candidate control information are summarized in Table 2 as discussed in [2]. Therein, the timing can be based NCR MT while the UL-DL TDD configuration can be obtained from SIB1 or NCR MT dedicated configuration. The ON-OFF information can be implicitly indicated by other control information. Thus, we have the following observation:

Observation 1: For the NCR RU, only AC beamforming and power control are to be controlled via NCR specific signaling.

Since an NCR is transparent to the UE, the NR defined DL/UL timeline should be maintained. For example, the downlink transmission timeline between gNB and UE is shown in Figure 1. The NCR should be controlled to forward the signal at the same OFDM symbols of gNB transmission and UE reception, with the corresponding AC beam. To guarantee the existing timeline of beam switching and data transmission, the NCR specific signaling processing delay and the application time should be considered.

Proposal 1: The processing delay and the application time of NCR specific signaling should be considered.

Figure 1 DL transmission timeline with NCR

Configuration

To reduce the control overhead, the RRC based signaling can be used to carry the semi-static configurations, such as the number of AC beams, the candidate AC beams, the AC beam for periodic signals/channels, and amplifying gain for the periodic signals/channels.

Proposal 2: If beamforming is supported for NCR, RRC-based NCR AC beam configuration should be studied.

Proposal 3: If power control is supported for NCR, RRC-based NCR amplifying gain configuration should be studied.

L2 signaling

To enable dynamic change of AC beam, MAC-CE based activation and deactivation for NCR AC beam can be considered.

Proposal 4: If beamforming is supported for NCR, MAC-CE based NCR AC beam activation/deactivation should be studied.

L1 signaling

RRC/MAC-CE based signaling are relatively time consuming compared to L1 signaling. To support dynamic scheduling, L1 signaling based NCR control is more efficient. DCI based control signaling would allow the NCR to adjust its beam and amplifying gain in the current gNB-UE timeline.

Proposal 5: If beamforming is supported for NCR, DCI based NCR AC beam indication and the timeline should be studied.

Proposal 6: If power control is supported for NCR, DCI based NCR amplifying gain indication and the timeline should be studied.

Conclusions

The contribution provides our considerations on L1/L2 signaling and configuration, and the observation is listed as following:

Observation 1: For the NCR RU, only AC beamforming and power control are to be controlled via NCR specific signaling.

The proposals are:

Proposal 1: The processing delay and the application time of NCR specific signaling should be considered.

Proposal 2: If beamforming is supported for NCR, RRC-based NCR AC beam configuration should be studied.

Proposal 3: If power control is supported for NCR, RRC-based NCR amplifying gain configuration should be studied.

Proposal 4: If beamforming is supported for NCR, MAC-CE based NCR AC beam activation/deactivation should be studied.

Proposal 5: If beamforming is supported for NCR, DCI based NCR AC beam indication and the timeline should be studied.

Proposal 6: If power control is supported for NCR, DCI based NCR amplifying gain indication and the timeline should be studied.

References

- RP-213700, “New SI: Study on NR Network-controlled Repeaters”, ZTE Corporation, RAN#94e, December 9 – 17, 2021.

- R1-2203133, “Discussions on side control information to enable NR network-controlled repeaters”, Huawei, HiSilicon, RAN1#109e, May 9 – 20, 2022.