3GPP TSG-RAN WG1 Meeting #109-e	 R1-2203136

e-Meeting, May 9th - 20th, 2022

Agenda Item:	9.10.2

Source:	Huawei, HiSilicon

Title:	Discussion on multi-carrier UL Tx switching

Document for:	Discussion and Decision

Introduction

In RAN#94e, multi-carrier enhancements for NR were approved [1]. The following enhancements for multi-carrier UL operation were agreed to be studied.

Study and if necessary specify following enhancements for multi-carrier UL operation [RAN1, RAN2, RAN4] - UL Tx switching schemes across up to 3 or 4 bands with restriction of up to 2 Tx simultaneous transmission for FR1 UEs, including mechanisms to enable more configured UL bands than its simultaneous transmission capability and to support dynamic Tx carrier switching across the configured bands for both single TAG and multiple TAGs configurations (RAN1, RAN4) - UE capability and RRC configuration related signalling (RAN2) - Note: strive for RAN1/2 design agnostic with the number of bands, i.e., common design between 3 and 4 bands - Note: no additional TAG is introduced for UL transmission on a carrier without corresponding DL carrier - Note: this objective does not target to extend the SUL framework to support more than 1 SUL for 1 NUL - Switching time and other RF aspects, and RRM requirements for above UL Tx switching schemes across up to 3 or 4 bands (RAN4) - Note: Prioritize UL Tx switching across up to 3 bands is to be addressed first and then that for up to 4 bands can also be addressed

In this contribution, firstly, simulation results are presented to evaluate the performance of UL Tx switching among 3 and 4 bands for the single TAG case. Secondly, Rel-18 UL Tx switching scenarios are discussed. Finally, UL Tx switching in the mTAG case is analyzed.

Performance evaluation for UL 2Tx switching in the 4-bands scenario

Many operators own three or four NR frequency bands, with different bandwidths, TDD/FDD duplex and DL/UL configurations, for example, 3.5GHz (TDD), 2.6GHz (TDD), 1.8GHz/2.1GHz (FDD/SUL) and 900MHz/800MHz/700MHz (FDD/SUL). The current main stream UEs are only capable of 2 Tx chains which cannot support concurrent transmission among all those bands. To improve user experience data rate, UL Tx switching among three bands or four bands, which enables sharing limited number of UE Tx chains among bands dynamically, is necessary. Its performance in the 4-bands scenario is evaluated in the following.

From the perspective of system, four bands are deployed for uplink access, i.e., 4.9G, 2.6G, 2.1G, and 700M. We assume that each cell has multiple 2Tx UEs. As shown in Fig. 1, three schemes are considered.

- The first scheme is Rel-17 UL 2Tx switching among 2 bands, where each Rel-17 2Tx UE can only be configured with 2 uplink bands. All UEs in the system are randomly and semi-statically divided into two groups. More specifically, some UEs are grouped as group I that have access to 4.9G and 700M, while the rest of UEs are grouped as group II that have access to 2.6G and 2.1G.

- The second scheme is Rel-18 UL 2Tx switching among 3 bands, where each Rel-18 2Tx UE can be configured with 3 uplink bands. All UEs in the system are randomly and semi-statically divided into two groups. More specifically, some UEs are grouped as group I that have access to 4.9G, 700M, and 2.6G, while the rest of UEs are grouped as group II that have access to 700M, 2.6G, and 2.1G. Each UE can dynamically select the best carrier(s) from the configured 3 bands per slot.

- The third scheme is Rel-18 UL 2Tx switching among 4 bands, where each Rel-18 2Tx UE are configured with 4 uplink bands. Each UE can dynamically select the best carrier(s) from the configured 4 bands per slot.

For a fair comparison, in the first and second scheme, the proportion of UEs in one group to the total UEs equals the proportion of available uplink resources for the group to the total available uplink resources. Single TAG is assumed in the simulation, and more detailed evaluation assumptions are provided in Appendix.

Fig. 1. Band configuration of UEs for 4-bands scenario

The uplink average user-perceived throughput (UPT) at different packet arrival rate is plotted for the considered three schemes, as shown in Fig. 2. Compared with the Rel-17 uplink 2Tx switching among 2 bands, the Rel-18 uplink 2Tx switching among 4 bands obtains 20.3%, 32.5%, and 38.4% uplink average UPT gain at the packet arrival rates of 2 packet/s, 4 packet/s and 6 packet/s, respectively. Moreover, compared with the Rel-18 uplink 2Tx switching among 3 bands, the Rel-18 uplink 2Tx switching among 4 bands still obtains 12%, 19%, and 19% uplink average UPT gain at the packet arrival rates of 2 packet/s, 4 packet/s and 6 packet/s, respectively. The reason is as follows. With more configured bands, a UE can be dynamically scheduled on the bands with more unoccupied resources. Moreover, more configured bands are also beneficial for a better channel adaptation.

Fig. 2. Simulation results for 4-bands scenario

Observation 1:

- 2Tx switching among 3 bands can bring average UPT gain up to ~16% compared with 2Tx switching among 2 bands.

- 2Tx switching among 4 bands can bring average UPT gain up to ~38% compared with 2Tx switching among 2 bands.

- 2Tx switching among 4 bands can bring average UPT gain up to ~20% compared with 2Tx switching among 3 bands.

In Rel-16 and Rel-17, UL 2Tx switching between 2 bands has already been specified.  Therefore, in Rel-18, some of the parameters, signalling procedures, and mechanisms in Rel-16 and Rel-17 could be reused to alleviate the specification workload of UL 2Tx switching among 3 or 4 bands.

Based on the above discussions, we propose to enable both UL Tx switching among 3 bands and 4 bands in Rel-18.

Proposal 1: the scenarios of UL Tx switching among 3 bands and 4 bands should be both supported in Rel-18.

Scenarios

In the scope of Rel-18 UL Tx switching enhancement, UL Tx switching schemes across up to 3 or 4 bands will be studied. Thus, there are two scenarios, i.e., the 3-bands scenario and the 4-bands scenario. We will discuss each scenario, respectively.

3-bands scenario

In Rel-16 & 17, no concurrent transmission is allowed between SUL and NUL within a serving cell while whether concurrent transmission is allowed between uplink carriers of different serving cells refers to UE capability Option 1 (switchedUL) and Option 2 (dualUL).

- In option 1, there is no concurrent transmission on the two uplink carriers of two serving cells

- In option 2, the concurrent UL transmissions can be scheduled on the two uplink carriers

- The uplink carrier in any serving cell above can be a SUL carrier but only up to two uplink carriers within a serving cell are configured.

In Rel-18, following the same rule as Rel-16 and Rel-17, no concurrent transmission is allowed between SUL and NUL within a serving cell. Two options can be considered for UL Tx switching among uplink carriers of different serving cells.

Considering a 3-bands scenario, each band contains an uplink carrier, where an uplink carrier can be a SUL carrier.

For uplink CA option 1 and SUL, as shown in Table 1, in Case 1, UE supports 1-port or 2-port UL transmissions on carrier 1; in Case 2, UE supports transmitting 1-port or 2-port UL transmissions on carrier 2; in Case 3, UE supports transmitting 1-port or 2-port UL transmissions on carrier 3.

Table 1. Tx switching for uplink CA option 1 and SUL in the 3-bands scenario

|  | Number of Tx chains Carrier 1+Carrier 2+Carrier 3 | Number of antenna ports for UL transmission Carrier 1+Carrier 2+Carrier 3 |
| --- | --- | --- |
| Case 1 | 2T/1T+0T+0TNote 1 | 1P+0P+0P, 2P+0P+0PNote 1 |
| Case 2 | 0T+2T/1T+0T Note 1 | 0P+1P+0P, 0P+2P+0P Note 1 |
| Case 3 | 0T+0T+2T/1T Note 1 | 0P+0P+1P, 0P+0P+2P Note 1 |

For uplink CA option 2, the possible RF chain to antenna ports mapping is listed in Table 2.

Table 2. Tx switching for uplink CA option 2 in the 3-bands scenario

|  | Number of Tx chains Carrier 1+Carrier 2+Carrier 3 | Number of antenna ports for UL transmission Carrier 1+Carrier 2+Carrier 3 |
| --- | --- | --- |
| Case 1 | 2T/1T+0T+0T Note 1 | 1P+0P+0P, 2P+0P+0P Note 1 |
| Case 2 | 0T+2T/1T+0T Note 1 | 0P+1P+0P, 0P+2P+0P Note 1 |
| Case 3 | 0T+0T+2T/1T Note 1 | 0P+0P+1P, 0P+0P+2P Note 1 |
| Case 4 | 0T+1T+1T | 0P+1P+1P, 0P+0P+1P, 0P+1P+0P |
| Case 5 | 1T+0T+1T | 1P+0P+1P, 0P+0P+1P, 1P+0P+0P |
| Case 6 | 1T+1T+0T | 1P+1P+0P Note 2, 0P+1P+0P, 1P+0P+0P |

Note 1: A UE may only support 1 Tx in any one of three bands so that there is only 1 antenna in the band. For example, if carrier 3 on band 3 only supports 1 Tx, then 0P+0P+2P is not supported.

Note 2: Carrier 1 and carrier 2 can belong to the same serving cell. For example, carrier 1 is a SUL carrier and carrier 2 is an NUL carrier. In this case, 1P+1P+0P is not supported since no concurrent transmission is allowed between SUL and NUL within a serving cell.

From Table 1 and Table 2, we have the following two observations for the 3-bands scenario:

- For uplink CA option 1 and SUL, transmission switching time is triggered whenever current UL transmission carrier is different from the preceding carrier regardless the number of ports used for the transmission. Thus, the R17 triggering mechanism of UL Tx switching specified in S6.1.6.2 and S6.1.6.3 of TS 38.214 for UL CA Option 1 and SUL can be reused.

- For uplink CA option 2, the current UL transmission carrier(s) and the preceding carrier(s) can involve 3 carriers (e.g., from case 1 to case 4, or from case 5 to case 4, in Table 2), which is not covered by Rel-16 & Rel-17 specifications.  For such cases, the specification impact can be further discussed.

In the scope of Rel-17 UL Tx switching enhancement, there are two parts. One is the UL Tx switching between two inter-band carriers and the other is between two bands and band B contains two aggregated contiguous carriers.

Similarly, in the Rel-18 3-bands scenario, one band may contain two aggregated contiguous uplink carriers. In such cases, the Tx switching is similar with the Tx switching between bands containing only a single uplink carrier. The RF chain to antenna ports mapping is similar as in the above tables. As a result, UL Tx switching is only applied between different bands. For one band, the same state of UE Tx chains is applied to all intra-band uplink carriers.

4-bands scenario

Follow the same rule in the 3-bands scenario, we provide the following tables for the 4-bands scenario.

For uplink CA option 1 and SUL, the possible RF chain to antenna ports mapping is listed in Table 3.

Table 3. Tx switching for uplink CA option 1 and SUL in the 4-bands scenario

|  | Number of Tx chains Carrier 1+Carrier 2+Carrier 3+Carrier 4 | Number of antenna ports for UL transmission Carrier 1+Carrier 2+Carrier 3+Carrier 4 |
| --- | --- | --- |
| Case 1 | 2T/1T+0T+0T+0T Note 3 | 1P+0P+0P+0P, 2P+0P+0P+0P Note 3 |
| Case 2 | 0T+2T/1T+0T+0T Note 3 | 0P+1P+0P+0P, 0P+2P+0P+0P Note 3 |
| Case 3 | 0T+0T+2T/1T+0T Note 3 | 0P+0P+1P+0P, 0P+0P+2P+0P Note 3 |
| Case 4 | 0T+0T+0T+2T/1T Note 3 | 0P+0P+0P+1P, 0P+0P+0P+2P Note 3 |

For uplink CA option 2, the possible RF chain to antenna ports mapping is listed in Table 4.

Table 4. Tx switching for uplink CA option 2 in the 4-bands scenario

|  | Number of Tx chains Carrier 1+Carrier 2+Carrier 3+Carrier 4 | Number of antenna ports for UL transmission Carrier 1+Carrier 2+Carrier 3+Carrier 4 |
| --- | --- | --- |
| Case 1 | 2T+0T+0T+0T Note 3 | 1P+0P+0P+0P, 2P+0P+0P+0P Note 3 |
| Case 2 | 0T+2T+0T+0T Note 3 | 0P+1P+0P+0P, 0P+2P+0P+0P Note 3 |
| Case 3 | 0T+0T+2T+0T Note 3 | 0P+0P+1P+0P, 0P+0P+2P+0P Note 3 |
| Case 4 | 0T+0T+0T+2T Note 3 | 0P+0P+0P+1P, 0P+0P+0P+2P Note 3 |
| Case 5 | 1T+1T+0T+0T | 1P+1P+0P+0P Note 4, 1P+0P+0P+0P, 0P+1P+0P+0P |
| Case 6 | 1T+0T+1T+0T | 1P+0P+1P+0P, 1P+0P+0P+0P, 0P+0P+1P+0P |
| Case 7 | 1T+0T+0T+1T | 1P+0P+0P+1P, 1P+0P+0P+0P, 0P+0P+0P+1P |
| Case 8 | 0T+1T+1T+0T | 0P+1P+1P+0P, 0P+1P+0P+0P, 0P+0P+1P+0P |
| Case 9 | 0T+1T+0T+1T | 0P+1P+0P+1P, 0P+1P+0P+0P, 0P+0P+0P+1P |
| Case 10 | 0T+0T+1T+1T | 0P+0P+1P+1P Note 4, 0P+0P+1P+0P, 0P+0P+0P+1P |

Note 3: A UE may only support 1 Tx in any one of four bands so that there is only 1 antenna in the band. For example, if carrier 3 on band 3 can only support 1 Tx, then 0P+0P+2P is not supported. For example, if carrier 4 on band 4 can only support 1 Tx, then 0P+0P+0P+2P is not supported.

Note 4: Carrier A and carrier B can belong to the same serving cell. For example, carrier 1 is a SUL carrier and carrier 2 is an NUL carrier. In this case, 1P+1P+0P+0P is not supported since no concurrent transmission is allowed between SUL and NUL within a serving cell.

From Table 3 and Table 4, we have the following two observations for the 4-bands scenario:

- For uplink CA option 1 and SUL, transmission switching time is triggered whenever current UL transmission carrier is different from the preceding carrier regardless the number of ports used for the transmission. Thus, the R17 triggering mechanism of UL Tx switching specified in S6.1.6.2 and S6.1.6.3 of TS 38.214 for UL CA Option 1 and SUL can be reused.

- For uplink CA option 2, the current UL transmission carrier(s) and the preceding carrier(s) can involve 4 carriers (e.g., from case 5 to case 10 in Table 4), which is not covered by Rel-16 & Rel-17 specifications.  For such cases, the specification impact can be further discussed.

Therefore, the observations obtained from the 3-bands scenario and the 4-bands scenario can be summarized as follows.

Observation 2:

- For uplink CA option 1 and SUL, transmission switching time is triggered whenever current UL transmission carrier is different from the preceding carrier regardless the number of ports used for the transmission. Thus, the R17 triggering mechanism of UL Tx switching specified in S6.1.6.2 and S6.1.6.3 of TS 38.214 for UL CA Option 1 and SUL can be reused for both 3-band and 4-band scenarios.

- For uplink CA option 2, the current UL transmission carrier(s) and the preceding carrier(s) can involve 3 or 4 carriers, which is not covered by Rel-16 & Rel-17 specifications. Its specification impact can be further discussed for both 3-band and 4-band scenarios.

Similarly, in the Rel-18 4-bands scenario, some bands may contain two aggregated contiguous uplink carriers. In such cases, the Tx switching is similar with the Tx switching between bands containing only a single uplink carrier. The RF chain to antenna ports mapping is similar as in the above tables.

However, as shown in Table 4, for UL CA Option 2 in the 4-bands scenario, there are 6 possible concurrent UL cases (i.e., Case 5-Case 10). Supporting all the concurrent UL cases will increase UE implementation complexity. Thus, a UE may support only a subset of the concurrent UL cases by UE capability reporting.

Proposal 2: for both 3-bands and 4-bands scenarios, both switched mode (UL CA option 1) and dual mode (UL CA option 2) should be supported.

- UL carrier of any serving cell can be a SUL carrier

- Reuse the R17 triggering mechanism of UL Tx switching specified in S6.1.6.2 of TS 38.214 for UL CA Option 1.

- Note: no more than one SUL carrier configured in one serving cell.

Proposal 3: for UL CA Option 2, considering UE implementation complexity, a UE can report supports of only some of concurrent UL cases at least for 4-bands scenario.

Analysis for UL Tx switching in mTAG case

Regarding the dynamic switching between two uplinks belonging to different TAG, there has been some proposals (referring to [2]) for regulating the interruption time in either carrier. However, there are some shortcomings with multiple TAGs introduced in the dynamic switching, as shown in Fig.3.

Compared with dynamic switching between two carriers belonging to the same TAG, it calls for more complicated system requirement.

- Firstly, when the switched-to carrier (i.e., CC2) is of larger TA than the switching-from carrier (i.e., CC1), the interruption time has to be set as the sum of switching period and TA difference, which results in a larger UL interruption time. The interruption time may further aggravate as TA difference is not limited to 35us and may increase with the random small cell deployment. In some extreme case, the TA difference may even exceed the duration of the switching period. This leads to the unexpected system performance degradation with respect to the spectrum utilization.

- Additionally, with such a larger switching time and two sets of independent TA management, the base stations have to be imposed with a larger scheduling delay to ensure sufficient switch gap scheduled for the UE0999.

- Thirdly, only FR1-FR1 UL Tx switching is in scope. The necessity of multiple TAGs is not justified for FR1-FR1 non-collocated scenario yet, where the difference of propagation delay is limited and single TAG is sufficient similar to UL CoMP deployment.

- Finally, in the latest SID, the specification impact has been limited to RAN4 for two band scenarios.

In summary, the above issues for multiple TAG are expected to be discussed in RAN4 first. Given the limited TU, the scenario of single TAG is prioritized in RAN1.

Fig. 3. Shortcomings caused by multiple TAGs to dynamic Tx switching

Proposal 4: Whether mTAG is necessary and its potential impact on the switching gap should be discussed in RAN4 first.

Conclusion

According to the above discussions, we have the following observations and proposals:

Observation 1:

- 2Tx switching among 3 bands can bring average UPT gain up to ~16% compared with 2Tx switching among 2 bands.

- 2Tx switching among 4 bands can bring average UPT gain up to ~38% compared with 2Tx switching among 2 bands.

- 2Tx switching among 4 bands can bring average UPT gain up to ~20% compared with 2Tx switching among 3 bands.

Observation 2:

- For uplink CA option 1 and SUL, transmission switching time is triggered whenever current UL transmission carrier is different from the preceding carrier regardless the number of ports used for the transmission. Thus, the R17 triggering mechanism of UL Tx switching specified in S6.1.6.2 and S6.1.6.3 of TS 38.214 for UL CA Option 1 and SUL can be reused for both 3-band and 4-band scenarios.

- For uplink CA option 2, the current UL transmission carrier(s) and the preceding carrier(s) can involve 3 or 4 carriers, which is not covered by Rel-16 & Rel-17 specifications. Its specification impact can be further discussed for both 3-band and 4-band scenarios.

Proposal 1: the scenarios of UL Tx switching among 3 bands and 4 bands should be both supported in Rel-18.

Proposal 2: for both 3-bands and 4-bands scenarios, both switched mode (UL CA option 1) and dual mode (UL CA option 2) should be supported.

- UL carrier of any serving cell can be a SUL carrier

- Reuse the R17 triggering mechanism of UL Tx switching specified in S6.1.6.2 of TS 38.214 for UL CA Option 1.

- Note: no more than one SUL carrier configured in one serving cell.

Proposal 3: for UL CA Option 2, considering UE implementation complexity, a UE can report supports of only some of concurrent UL cases at least for 4-bands scenario.

Proposal 4: Whether mTAG is necessary and its potential impact on the switching gap should be discussed in RAN4 first.

- References

RP-213577, “New WID on Multi-carrier enhancements”, TSG RAN Meeting #94e, Dec. 6 - 17, 2021.

R4-2112825, “DRAFT LS on TX switching with multiple TAG”, TSG-RAN WG4 Meeting #100-e, Aug. 16 - 27, 2021.

- Appendix

| Simulation assumptions for the 4-bands scenario |  |  |  |  |
| --- | --- | --- | --- | --- |
| Frequency | 4.9G | 2.6G | 2.1G | 700M |
| Bandwidth | 100MHz | 160MHz | 50MHz | 30MHz |
| DL:UL | 7:3 | 8:2 | / | / |
| BS antenna | 64T64R | 64T64R | 32R | 4T4R |
| Subcarrier spacing | 30KHz | 30KHz | 30KHz | 15kHz |
| Deployment | 3GPP Urban Macro, 21cells |  |  |  |
| UE number | 20 UE per cell |  |  |  |
| ISD | 500 m |  |  |  |
| UE power | 23 dBm |  |  |  |
| Traffic model | FTP model 3, 1 Mbyte, 2, 4 or 6 packet/s |  |  |  |