3GPP TSG-RAN WG1 Meeting #109-e 	R1-2203137

e-Meeting, May 9 – 20, 2022

Agenda Item:	9.9.1

Source:	Huawei, HiSilicon

Title:	Discussion on NR PDCCH reception in symbols with LTE CRS REs

Document for:	Discussion and Decision

Introduction

In RAN #94 plenary meeting, a new WID on enhancement of NR dynamic spectrum sharing was approved for Release 18 [1], where RAN1 agreed to study solutions to improve the NR spectrum efficiency on the shared carriers:

The following objectives shall be included for improvement of NR spectrum efficiency for LTE-NR co-existence (RAN1): - Study and if needed specify NR PDCCH reception in symbols with LTE CRS REs. [RAN1] - Investigate enabling LTE CRS to puncture NR PDCCH, including the impact to NR PDCCH DMRS if there is the performance gain from the additional PDCCH resources. - Allow a UE to support, and be configured with, two overlapping CRS rate matching patterns regardless of support or configuration of multi-TRP [RAN1, RAN2]

In this contribution, objective 1, i.e., NR PDCCH reception in symbols with LTE CRS REs is discussed.

Discussion on the motivation of Objective 1

In Rel-17, if at least one RE of a PDCCH candidate for a UE on the serving cell overlaps with at least one RE of LTE CRS configured by gNB, the UE is not required to monitor the PDCCH candidate. Therefore, in DSS, NR PDCCH candidates are generally configured on symbols that do not overlap with LTE CRS.

In commercial LTE networks, the most typical CRS configurations are  2-port CRS occupying the first and fifth symbol of a LTE slot, and 4-port CRS occupying the first, second and fifth symbol of a LTE slot.

For a Rel-15 UE supporting PDCCH reception capability FG 3-1 in TR38.822, the monitoring occasion shall be within the first 3 OFDM symbols of a slot. Besides FG 3-1, other PDCCH monitoring capabilities are introduced in Rel-15, including FG 3-2 which can support PDCCH monitoring on any span of 3 consecutive OFDM symbols in the slot, and FG 3-5 which can support PDCCH monitoring on any OFDM symbol(s). In addition, in RAN1 #108e meeting [2], a Rel-16 UE capability has been introduced for PDCCH monitoring with a single span of three contiguous OFDM symbols that is within the first four OFDM symbols in a slot for Rel-16 as shown in below.

Agreement Introduce a new UE capability reporting for PDCCH monitoring with a single span of three contiguous OFDM symbols that is within the first four OFDM symbols in a slot for Rel-16 as below 22. NR Others; [22-x]; PDCCH monitoring with a single span of three contiguous OFDM symbols that is within the first four OFDM symbols in a slot; Indicates whether the UE supports receiving PDCCH in a search space configured to be monitored within a single span of any three contiguous OFDM symbols that are within the first four OFDM symbols in a slot with the capability of supporting at least 44 blind decodes in a slot for 15 kHz subcarrier spacing.; Yes; N/A; Per UE; No; FR1 only; No; Optional with capability signalling.

In this contribution, we will analyze the PDCCH performance based on the above two CRS port configurations and three UE PDCCH monitoring capabilities (i.e. FG 3-1, FG 3-2, and the Rel-16 UE capability). Table 1 shows the 3 possible CORESET configuration cases in legacy DSS for different combinations between CRS and PDCCH monitoring capabilities, where the number of symbols of CORESET can be 1, 2 and 3 symbols. FG 3-5 is not discussed in the following, since it can be configured at any non-conflicted symbol(s) and therefore has no PDCCH capacity issue.

- Case 1: 1-symbol CORESET

- Case 2: 2-symbol CORESET

- Case 3: 3-symbol CORESET

Table 1 legacy CORESET configuration cases

| PDCCH monitoring capability | 4-port CRS | 2-port CRS |
| --- | --- | --- |
| FG 3-1 (Fig.1) (monitoring occasion within the first 3 OFDM symbols) | Case 1 | Case 2 |
| FG 3-2 (Fig.2) (monitoring occasion within 3 consecutive OFDM symbols) | Case 2 | Case 2 or Case 3 |
| Rel-16 UE capability (Fig.3) (monitoring occasion within the first 4 OFDM symbols) | Case 2 | Case 3 |

Fig. 1 CORESET configuration cases for FG 3-1.

Fig. 2 CORESET configuration cases (examples) for FG 3-2.

Fig. 3 CORESET configuration cases for the Rel-16 UE capability.

Among the 3 cases, Case 1 is most likely to have the PDCCH capacity pressure, because only a single-symbol CORESET can be configured on the third symbol of a slot in this case. In contrast, for Case 2/3, the 2-symbol/3-symbol CORESET can be configured, so that the PDCCH capacity pressure has been largely relieved. As shown in Table 1 and analyzed above, FG 3-2, FG 3-5, and the Rel-16 UE capability supporting the monitoring of the first 4 symbols can all support Case 2/3, while FG 3-1 can also support Case 2 under the 2 ports CRS scenario.

Observation 1: NR PDCCH capacity pressure may not be a big problem if the UE supports FG 3-2, FG 3-5, or the newly introduced Rel-16 UE capability to support monitoring PDCCH within the first 4 symbols.

In addition, as scheduling PDSCH and PUSCH on PCell with PDCCH on SCell is introduced in Rel-17 DSS, the PDCCH capacity pressure of a PCell operating in DSS mode can also be relieved even for Case 1 under FG 3-1.

Observation 2: There are other alternatives in legacy system to improve PDCCH capacity even if the PDCCH capacity is the bottleneck for UEs which can only monitor PDCCH within the first 3 symbols, such as Rel-17 SCell PDCCH scheduling PCell PDSCH/PUSCH.

Discussion and evaluation on Objective 1

Potential cases with increased number of PDCCH symbols under Objective 1

Case 1 and Case 2 are considered as baseline cases in the following for potential PDCCH symbol increasing in Rel-18. As the number of CORESET symbols is the same (i.e. 2 symbols) in Case 2 for FG 3-1 with 2 ports CRS, for FG 3-2 and for the Rel-16 UE capability with 4 ports CRS, we take Case 2 for FG 3-1 as an example for description.

According to Objective 1 of Rel-18 DSS, on top of the legacy Case 1 and Case 2, three Objective 1 cases (Case 1_enh1, Case 1_enh2 and Case 2_enh) can be considered for potential PDCCH symbol increasing, where the PDCCH and PDCCH DM-RS overlapping with the LTE CRS need to be punctured.

- For legacy DSS Case 1, Rel-18 PDCCH symbols can be increased to:

  - Case 1_enh1: CORESET is configured on 2 symbols as shown in Fig. 4, where one symbol does not overlap with the CRS (the 3rd symbol of a slot), and the other symbol overlaps with the CRS (the 2nd symbol of a slot).

  - Case 1_enh2: CORESET is configured on 3 symbols as shown in Fig. 5, where one symbol does not overlap with the CRS (the 3rd symbol of a slot), and the other two symbols overlaps with the CRS (the 1st and 2nd symbol of a slot).

- For legacy DSS Case 2, Rel-18 PDCCH symbols can be increased to:

- Case 2_enh: CORESET is configured on 3 symbols as shown in Fig. 6, where two symbols do not overlap with the CRS (the 2rd and 3rd symbol of a slot), and the other one symbol overlaps with the CRS (the 1rd symbol of a slot).

Fig. 4 Case 1 vs Case 1_enh1

Fig. 5 Case 1 vs Case 1_enh2

Fig. 6 Case 2 vs Case 2_enh

Restriction of Objective 1 from channel estimation perspective

In PDCCH reception, UE first performs the channel estimation on the DM-RS REs, and then uses the channel estimation filter to obtain the channel estimation on the neighboring PDCCH REs. The channel estimation filter coefficients may be calculated in real time (namely real-time-based) or stored in advance based on the DM-RS pattern, channel statistics and SINR (namely storage-based). Channel estimation complexity is reflected by the computational complexity for real-time-based method and reflected by the memory size for storage-based method. UE implementation complexity would be critically increased in case of additional changes on computational complexity or storage on top of the legacy methods.

Within the legacy CORESET, there is only one unified DM-RS pattern in a REG, i.e., subcarriers 1, 5 and 9. The channel estimation filtering is performed based on this regular DM-RS pattern and by using a unified algorithm, under either the real-time-based method or storage-based method.

For Objective 1, however, the DM-RS pattern in a CORESET would be irregular over symbols due to collision with LTE CRS symbols. E.g., if the CORESET overlaps with one LTE CRS pattern (i.e., frequency overlap between 1 NR carrier and 1 LTE carrier), two DM-RS patterns will occur in a REG, one is the legacy regular pattern (‘Pattern 1’ as shown in Fig. 1), and the other is a new pattern with a DM-RS RE punctured (e.g., ‘Pattern 2’ as shown in Fig. 1).

Moreover, if the CORESET overlaps with more than one LTE CRS patterns (i.e., frequency overlap between 1 NR carrier and multiple LTE carriers), more than two DM-RS patterns will occur. As shown in Fig. 7 where the NR carrier is overlapped with 3 LTE CRS patterns configured with different v-shifts, there would be 4 DM-RS patterns as a result, including

- Legacy DM-RS pattern (‘Pattern 1’) in one REG including the subcarriers 1, 5 and 9

- New DM-RS pattern (‘Pattern 2’) in one REG including the subcarriers 1 and 5

- New DM-RS pattern (‘Pattern 3’) in one REG including the subcarriers 5 and 9

- New DM-RS pattern (‘Pattern 4’) in one REG including the subcarriers 1 and 9

Fig. 7 One NR carrier overlapping with one or more LTE carriers.

Observation 3: Irregular DM-RS patterns in PDCCH will occur due to the puncture from LTE CRS to DM-RS when the NR PDCCH is transmitted on the collided OFDM symbols.

Since the punctured DM-RSs should not be used for channel estimation, such irregular DM-RS pattern would significantly impact the complexity if requiring the channel estimation to be performed based on the irregular DM-RS pattern. E.g., for the real-time-based method, the channel estimation filter coefficients need to be calculated separately over symbols with respect to varying DM-RS patterns, which causes multiple times increase on computational complexity. For storage-based method, the UE needs to store different channel estimation filter coefficients for different DM-RS patterns, which requires multiple times increase on the size of the storage. Either way is quite challenging for the UE with the legacy channel estimation implementation.

Due to the complexity issues above, Rel-18 DSS should not rule out the legacy UE channel estimation implementations based on regular DM-RS patterns. To guarantee the channel estimation based on regular DM-RS pattern, a restriction is needed to ensure that the CORESET is configured to include at least one symbol that does not overlap with LTE CRS if in the end NR PDCCH reception in symbols with LTE CRS REs will be supported, thus the DM-RS pattern on this symbol is the legacy regular DM-RS pattern. The UE can use this regular DM-RS pattern on this symbol to perform channel estimation, and applies it to other symbols of the CORESET.

In the later part of this contribution, we analyze the impact on the PDCCH performance based on this basic channel estimation method for a Rel-18 DSS UE.

Proposal 1: Rel-18 DSS should not rule out the basic UE channel estimation implementations based on regular DM-RS patterns.

Evaluation of Objective 1 on PDCCH performance

Evaluation Methodology

In order to evaluate the PDCCH performance of Objective 1, the link simulation is performed to obtain the required SINR at the target BLER (1%) for various ALs in different cases. In addition, the SINR CDF of the UEs in a DSS cell is also performed based on a system level simulation.

According to the required SINR for different ALs and the SINR CDF of a DSS cell, the UEs in this DSS cell can be divided into several groups：

- AL1 is applied for the group of UEs with the SINR higher than the required SINR of AL1.

- AL2 is applied for the group of UEs with the SINR lower than the required SINR of AL1 but higher than the required SINR of AL2.

- AL4 is applied for the group of UEs with the SINR lower than the required SINR of AL2 but higher than the required SINR of AL4.

- AL8 is applied for the group of UEs with the SINR lower than the required SINR of AL4 but higher than the required SINR of AL8.

- AL16 is applied for the group of UEs with the SINR lower than the required SINR of AL8 but higher than the required SINR of AL16.

- For UEs with the SINR lower than the required SINR of AL16, this group of UEs cannot satisfy the PDCCH target BLER 1% even if they are scheduled by PDCCH with the largest AL16. This group of UEs is named as outage UEs.

Evaluation Metrics

Two metrics can be used to compare the PDCCH performance of Objective 1 cases and legacy DSS case:

- PDCCH capacity: The PDCCH capacity is represented by the average number of PDCCHs scheduled in an NR slot, which is derived by dividing the total number of CCEs of the slot with the average number of CCEs per UE, while the average number of CCEs per UE in this cell can be calculated according to the percentage of UEs in each group and the corresponding ALs.

- Outage UE percentage: The percentage of UEs that cannot satisfy the PDCCH target BLER 1% under AL16. Higher percentage of outage UEs means worse PDCCH coverage performance.

Evaluation results

The evaluation results for Case 1 (baseline) and Case 1_enh2 (Objective 1) are shown in Fig.8, which shows the PDCCH symbol increasing from 1 symbol to 3 symbols as given in Fig.5. The blue curves denote Case 1, and the red curves denote Case 1_enh2. The UE speed is assumed as 30km/h, and other simulation assumptions are given in the Appendix.

- PDCCH capacity

According to the aforementioned PDCCH capacity analysis, the average number of CCEs of the PDCCHs for Case 1 is 4.1, and the average number of CCEs of the PDCCHs for Case 1_enh2 is 7.9. Therefore, the PDCCH capacity gain of Case 1_enh2 over Case 1 is  37.914.1-1=55.7%.

- Outage UE percentage

For Case 1, it is shown in Fig.8 that the required SINR for AL16 PDCCH is -6.2dB, and it can be derived from the SINR CDF that about 1% of the UEs in this cell have SINR less than -6.2dB. That is, only 1% outage UEs in this cell cannot reach the PDCCH target BLER 1%.

For Case 1_enh2, the required SINR for AL16 PDCCH is -2.5dB, and it can be derived from the SINR CDF that about 9% of the UEs in this cell have SINR less than -2.5dB. That is, if NR PDCCH reception in symbols with LTE CRS REs is used, the outage would significantly increase to 9% which cannot reach the PDCCH target BLER 1%. Note that such outage cannot be resolved by assigning even more resources as it has reached the largest PDCCH aggregation level AL16.

In order to solve the PDCCH coverage problem of outage UEs, an alternative is to configure Objective 1 CORESET with CRS collision to non-outage UEs (i.e., the UEs with required SINR >= -2.5dB), and configure legacy CORESET without CRS collision (i.e., Case 1) for outage UEs (i.e., the UEs with required SINR < -2.5dB). However, in a NR slot, if the PDCCH of AL16 in Case 1 CORESET is assigned to outage UE which consumes 96 RBs already, there is hardly any spare bandwidth on the non-collision symbol to schedule other non-outage UEs with respect to the restriction of Objective 1 on channel estimation.

Fig. 8 Case 1 vs Case 1_enh2

Based on the similar approach, the PDCCH performance for other cases, including Case 1 vs Case 1_enh1 and Case 2 vs Case 2_enh are also provided and summarized in Table 2. The corresponding SINR-BLER curves are provided in the Appendix.

Table 2 PDCCH performances for Case 1 vs Case 1_enh1, Case 1 vs Case 1_enh2, and Case 2 vs Case 2_enh

|  | Occupied PDCCH symbols (consumed resources) | Capacity gain over baseline | Outage UE percentage |
| --- | --- | --- | --- |
| Case 1 (baseline) | 1-symbol | -- | 1% |
| Case 1_enh1 (taking Case 1 as baseline) | 2-symbol w/ 1 collision symbol (+100% resources) | 36.2% | 4% |
| Case 1_enh2 (taking Case 1 as baseline) | 3-symbol w/ 2 collision symbols (+200% resources) | 55.7% | 9% |
| Case 2 (baseline) | 2-symbol CORESET | -- | 1% |
| Case 2_enh (taking Case 2 as baseline) | 3-symbol w/ 1 collision symbol (+50% resources) | 19.1% | 4% |

Compared to the baseline cases, 100%, 200% and 50% additional PDCCH resources on the symbol(s) configured with LTE CRS are consumed by Case 1_enh1, Case 1_enh2 and Case 2_enh, respectively, achieving only 36.2%, 55.7% and 19.1% PDCCH capacity gain, respectively. That means the PDCCH capacity gain is achieved with the sacrifice of resource utilization efficiency. Assuming the PDCCH capacity issue is relieved by other means, these resources can be used to transmit PDSCH with higher efficiency.

In addition, from our companion contribution [3], the PDCCH performances are evaluated under high speed scenario (350km/h), where it can be observed that the PDCCH capacity gains for Case 1 vs Case 1_enh1, Case 1 vs Case 1_enh2, and Case 2 vs Case 2_enh are 29.6%, 38.2% and 15.7%, respectively. That means the PDCCH capacity gain is limited under high speed scenario.

Observation 4: Enabling NR PDCCH reception in symbols with LTE CRS REs will cause deteriorated PDCCH coverage, as the percentage of outage UEs which cannot reach the 1% PDCCH target BLER with AL16 would increase from 1% for legacy UEs to 4%~9% for Objective 1 UEs.

Observation 5: Enabling NR PDCCH reception in symbols with LTE CRS REs brings additional symbols which contributes to PDCCH capacity gain but with the sacrifice of resource utilization efficiency.

It should be also noted that the above analysis especially Case 1_enh2 is considered with the ideal assumption where there is no LTE PDCCH existence, i.e., all symbols in the first 3 symbols are available for NR. When considering the co-existence of LTE PDCCH, the NR PDCCH capacity gain may hardly reach as high as 55.7% as shown for Case 1_enh2. E.g., assuming LTE PDCCH load to NR PDCCH load is 1:2, the first 3 symbols may be assigned as 1 symbol for LTE and 2 symbols for NR. Moreover, even under a smaller LTE PDCCH payload, achieving Case 1_enh2 may be challenging. Fig. 9 shows an example of LTE PDCCH transmitted in a LTE subframe (NR slot). In this example, a LTE UE is scheduled with an AL4 LTE PDCCH on the first symbol of the LTE subframe. Considering LTE PCFICH, PHICH and PDCCH are sent on the first symbol in a distributed manner after interleaving, it is difficult for NR PDCCH to be transmitted on the 1st symbol without collision with LTE channels. That means, as long as there is any LTE PDCCH occupying the slot, Case 1_enh2 can be hardly achieved, and Case 1_enh1 may be a reasonable case where up to 36.2%/29.6% capacity gain can be achieved for low speed/high speed respectively.

Fig. 9 Co-existing with LTE PDCCH/PCFICH/PHICH

Observation 6: The co-existence of LTE PDCCH/PCFICH/PHICH will limit the PDCCH capacity gain that can be achieved by enabling NR PDCCH reception in symbols with LTE CRS REs.

From the above analysis from the perspectives of necessity, restriction on CORESET configuration with respect to channel estimation, and the performances of PDCCH capacity as well as outage, it is observed with no strong motivation to support Objective 1.

Proposal 2: No strong motivation is observed to support Objective 1.

Conclusion

According to the above discussions, we have the following observations and proposals:

Observation 1: NR PDCCH capacity pressure may not be a big problem if the UE supports FG 3-2, FG 3-5, or the newly introduced Rel-16 UE capability to support monitoring PDCCH within the first 4 symbols.

Observation 2: There are other alternatives in legacy system to improve PDCCH capacity even if the PDCCH capacity is the bottleneck for UEs which can only monitor PDCCH within the first 3 symbols, such as Rel-17 SCell PDCCH scheduling PCell PDSCH/PUSCH.

Observation 3: Irregular DM-RS patterns in PDCCH will occur due to the puncture from LTE CRS to DM-RS when the NR PDCCH is transmitted on the collided OFDM symbols.

Proposal 1: Rel-18 DSS should not rule out the basic UE channel estimation implementations based on regular DM-RS patterns.

Observation 4: Enabling NR PDCCH reception in symbols with LTE CRS REs will cause deteriorated PDCCH coverage, as the percentage of outage UEs which cannot reach the 1% PDCCH target BLER with AL16 would increase from 1% for legacy UEs to 4%~9% for Objective 1 UEs.

Observation 5: Enabling NR PDCCH reception in symbols with LTE CRS REs brings additional symbols which contributes to PDCCH capacity gain but with the sacrifice of resource utilization efficiency.

Observation 6: The co-existence of LTE PDCCH/PCFICH/PHICH will limit the PDCCH capacity gain that can be achieved by enabling NR PDCCH reception in symbols with LTE CRS REs.

Proposal 2: No strong motivation is observed to support Objective 1.

- References

- RP-213575, “New WI: Enhancement of NR Dynamic spectrum sharing (DSS)”, TSG RAN Meeting #94e, Dec. 6 - 17, 2021

- R1-2202643, Summary on [108-e-R16-UE-features-Others-02], Moderator (NTT DOCOMO, INC.), March, RAN1#108-e.

- R1-2204910, Discussion on additional evaluation results for NR PDCCH in collision with LTE CRS, May, RAN1#109-e.

- Appendix

Table 3 Link level simulation assumptions

| Parameters | Values |
| --- | --- |
| Carrier frequency | 2.1 GHz |
| SCS | 15 kHz |
| Bandwidth | 20 MHz |
| Channel model | TDL-A |
| Delay spread | 100 ns |
| UE speed | 30 km/h |
| Number of BS antennas | 4 Tx (Two cross-polarized antenna pairs) |
| Number of UE antennas | 4 Rx (Two cross-polarized antenna pair) |
| DCI payload (excluding CRC) | 60 bits |
| Interleaving | Non-Interleaved |
| Precoding | Precoder cycling per REG bundle |
| REG bundle size | 6 PRBs |

Table 4 System level simulation assumptions

| Parameters | Values |
| --- | --- |
| Carrier frequency | 2.1 GHz |
| SCS | 15 kHz |
| Simulation bandwidth | 20 MHz |
| BS antenna height | 25 m |
| UE height | 1.5m |
| TRP transmit power | 49 dBm |
| Scenario | Urban Macro |
| ISD | 500m |
| Device deployment | 80% indoor, 20% outdoor |
| UE speeds | Indoor users: 3km/h |
|  | Outdoor users (in-car): 30 km/h |
| BS noise figure | 5 dB |
| BS antenna element gain | 8 dBi |
| UE noise figure | 9 dB |
| Thermal noise level | -174 dBm/Hz |
| Traffic | Full Buffer |
| Macro sites | 19 |
| Downtilt | 102° |
| Minimum BS to UE distance | 35m |

Fig. 10 PDCCH BLER performance of Case 1 vs Case 1_enh1.

Fig. 11 PDCCH BLER performance of Case 2 vs Case 2_enh.