3GPP TSG-RAN WG1 Meeting #109-e	R1-2203139

e-Meeting, May 9 – 20, 2022

Agenda Item:	9.2.1

Source:	Huawei, HiSilicon

Title:	Discussion on general aspects of AI/ML framework

Document for:	Discussion and Decision

Introduction

In RAN plenary meeting [1], a new SID on AI/ML for air-interface was approved for Release 18.

Study the 3GPP framework for AI/ML for air-interface corresponding to each target use case regarding aspects such as performance, complexity, and potential specification impact. Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels Note: the selection of use cases for this study solely targets the formulation of a framework to apply AI/ML to the air-interface for these and other use cases. The selection itself does not intend to provide any indication of the prospects of any future normative project. AI/ML model, terminology and description to identify common and specific characteristics for framework investigations: - Characterize the defining stages of AI/ML related algorithms and associated complexity: - Model generation, e.g., model training (including input/output, pre-/post-process, online/offline as applicable), model validation, model testing, as applicable - Inference operation, e.g., input/output, pre-/post-process, as applicable - Identify various levels of collaboration between UE and gNB pertinent to the selected use cases, e.g., - No collaboration: implementation-based only AI/ML algorithms without information exchange [for comparison purposes] - Various levels of UE/gNB collaboration targeting at separate or joint ML operation. - Characterize lifecycle management of AI/ML model: e.g., model training, model deployment, model inference, model monitoring, model updating - Dataset(s) for training, validation, testing, and inference - Identify common notation and terminology for AI/ML related functions, procedures and interfaces - Note: Consider the work done for FS_NR_ENDC_data_collect when appropriate … Note 1: specific AI/ML models are not expected to be specified and are left to implementation. User data privacy needs to be preserved. Note 2: The study on AI/ML for air interface is based on the current RAN architecture and new interfaces shall not be introduced.

This contribution provides our views on the general aspects of the AI/ML framework, including defining stages of AI/ML algorithms, UE-gNB collaboration levels, lifecycle management, and construction of dataset.

Defining stages of AI/ML algorithms and associated complexity

The defining stages of AI/ML algorithms includes AI/ML model generation and AI/ML model inference, which are discussed in the following.

2.1 Candidate classes of AI/ML algorithms

The research on AI/ML is a rapidly developing and various AI/ML algorithms have been proposed in both academia and industry. In general, AI/ML model can be trained to the optimized direction of outputs based on the dataset of training-inputs. After an AI/ML model is well trained, it can be used for inference where the model can derive the inference result based on a given input. Based on the public research literature, the representative AI/ML algorithms used for wireless systems include following, which may have different mechanisms:

Supervised learning

Supervised learning is one of the most mature AI/ML algorithms where an AI/ML model is trained to learn the mapping between inputs and labels, where such labels do not need to be derived based on real-time AI/ML output. Under this mechanism, the node needs to collect the labels from the network, e.g., based on the measurement and feedback of the other node(s). For the use cases in SID scope, the label is relatively easy to obtain, which facilitates supervised-learning-based solutions. For example, the original CSI, optimal beam ID, real UE LOS/NLOS tag or coordinates can be regarded as the labels for AI/ML-based CSI feedback, beam management, and positioning, respectively.

Reinforcement learning

Reinforcement learning obtains the real-time feedback (rather than labels) from the environment as the inputs for training, where the feedback is derived based on the real-time outputs of the AI/ML model. Under this mechanism, real-time interaction of the inputs and outputs exchanges are needed between NW and UE. Reinforcement learning is suitable for tasks where labels are difficult to obtain, such as application for higher layer (e.g., RRM, where the optimal RRM policy as a label is normally difficult to obtain). Besides, reinforcement learning works in an online learning manner where continuous model-training and interaction with the environment is required for maintaining the model performance, which is inefficient in realistic networks. E.g., the network performance may not be guaranteed during this online training phase.

Federated learning

Federated learning is a distributed architecture to train a unified model among NW and multiple UEs. Under this mechanism, frequent exchange of AI/ML models may be needed between NW and UE in an online manner. Thus, federated learning involves heavy overheads for exchanging AI/ML models over air-interface.

Based on the spec impacts analysis and the potential applicable use cases, supervised learning is a good starting point for studying the potential benefits of AI/ML to air-interface enhancement.

Observation 1: Different mechanisms are observed for the following representative AI/ML algorithms:

- For supervised learning, labels need to be collected from the network as inputs for model training.

- For reinforcement learning, AI/ML model needs to be trained based on feedback from the network, where the feedback is derived based on the real-time outputs of the AI/ML model. Such continuous model-training and interaction procedure harms the realistic network efficiency.

- For federated learning, frequent AI/ML model exchange may be needed between NW and UE, which consumes heavy air-interface overheads for model exchange.

Proposal 1: Supervised learning should be considered as a starting point for studying the potential benefits of AI/ML to air-interface enhancement.

2.2 AI/ML model generation

- 2.2.1 Categorization of AI/ML operation modes

AI/ML operation mode can be categorized into three operation modes depending on which side (NW or UE) is to perform the AI/ML model training. In addition, each operation mode can be further subdivided according to the side to perform AI/ML model inference.

NW-oriented operation mode

NW-oriented operation mode refers to cases that AI/ML model is trained at NW. Normally, NW is efficient in collecting a large amount and diversified training-inputs through UEs distributed over the cell. Besides, NW has powerful computing and storage capabilities, which could facilitate large-scale model training/fine-tuning. In addition, since NW could collect extensive training-inputs over the cell, it could train scenario-specific models and performs model-selection adaptively.

NW-oriented AI/ML model training could be further subdivided into following three sub-modes as also illustrated in Figure 1.

Training @ NW, inference @ NW:  a single-sided AI/ML model is both trained and inferred at NW side. For this case, NW may require UE to assist the collection of training-inputs over the air-interface, for example, feedback ground-truth labels. Yet, the training-inputs collection is not always-on, but rather only being performed in an event-driven or periodical manner (e.g., when AI/ML model needs to be updated), so the ground-truth feedback overhead may be negligible on average. This mode is applicable to use cases including AI/ML-based beam management, AI/ML-based positioning, as well as CSI prediction, as analyzed in our companion contributions [2]-[4].

Training @ NW, inference @ UE: a single-sided AI/ML model is trained at NW and inferred at UE. For this case, NW also has to be aware of the UE’s computational capability and storage capability before training as the model will be inferred at UE. NW would deliver the AI/ML model to UE after the model is well trained. However, this would face the model delivering overhead issue as well as the model representation format issue as will be analyzed in Section 3. Substantial discussions and spec efforts may be needed accordingly. This mode might be applicable to AI/ML-based beam management, AI/ML-based positioning, as well as CSI prediction.

Training @ NW, inference @ NW&UE: a two-sided AI/ML model is trained at NW and inferred both at NW and UE. For this case, NW may require UE to assist the collection of training-inputs for training a pair of dual models, for example, an auto-encoder. Then, the well-trained model is delivered to UE for inference. Similarly, UE’s computational capability and storage capability should be also known by the NW, and the model delivery faces issues of overhead and model representation format. This mode might be applicable to the sub use cases of CSI compression [2].

| (1) Training @ NW, inference @ NW | (2) Training @ NW, inference @ UE | (3) Training @ NW, inference @ NW&UE |
| --- | --- | --- |
| Figure 1 NW-oriented operation mode |  |  |

Observation 2: For NW-oriented operation mode where AI/ML model is trained at NW, the sub mode of training @ NW, inference @ NW is beneficial, including simple from spec perspective, no need of UE capability awareness at NW, and free from the issue of model exchange via air-interface.

UE-oriented operation mode

UE-oriented operation mode refers to cases that AI/ML model is trained at UE. For the scenarios that UE can obtain training labels and performs model inference, this operation mode can avoid the overhead and delay of delivering training-inputs and models over the air-interface. However, this is at the expense of a power consumption and implementation complexity at UE for supporting model training.

UE-oriented AI/ML operation mode could be further subdivided into following three classes as also illustrated in Figure 2:

Training @ UE, inference @ NW: a single-sided AI/ML model is trained at UE and inferred at NW. For this case, NW would be aware of the AI/ML training at UE to configure corresponding reference signal (RS) for UE to obtain ground-truth labels. UE would deliver the AI/ML model to NW after the model is well trained. NW might need to maintain (store, update, etc.) UE-specific AI/ML models from different UEs which would impose high implementation complexity on NW. In addition, this mode also faces the issues of overhead and model representation format. This mode might be applicable to AI/ML-based positioning.

Training @ UE, inference @ UE: a single-sided AI/ML model is both trained and inferred at UE. For this case, UE may require NW to assist the collection of training-inputs over the air-interface. This mode is applicable to AI/ML-based positioning, as well as CSI prediction [2][4].

Training @ UE, inference @ NW&UE: a two-sided AI/ML model is trained at UE and inferred both at NW and UE side. For this case, NW might need to assist UE to collect training-inputs for training a pair of dual models. Then, the well-trained model is delivered to NW for inference. Similarly, NW is required to maintain UE-specific models from different UEs. In addition, this mode also faces the issues of overhead and model representation format. This mode might be applicable to the sub use cases of CSI compression.

| (1) Training @ UE, inference @ NW | (2) Training @ UE, inference @ UE | (3) Training @ UE, inference @ NW&UE |
| --- | --- | --- |
| Figure 2 – UE-oriented operation mode |  |  |

Observation 3: For UE-oriented operation mode where AI/ML model is trained at UE, the sub mode of training @ UE, inference @ UE is beneficial, including simple from spec perspective, no need of maintaining UE-specific models at the NW side, and free from the issue of model exchange via air- interface.

Separate NW&UE operation mode

For two-sided AI/ML models as mentioned previously, the model exchange via air-interface would be an issue. To avoid this issue, separated NW&UE operation mode can be considered, as illustrated in Figure 3.

Separate training @ NW&UE, inference @ NW&UE: a two-sided AI/ML model is separately trained at NW and UE, and inferred by both NW and UE, so that AI/ML model exchange over air-interface is not needed. In particular, NW and UE train their own AI/ML model independently and perform the inference based on their own part. Considering the two-sided models need to work in a pair, how to ensure that the separately trained models are matched with each other can be studied as analyzed in our companion contribution [2]. This mode is applicable to the sub use cases of CSI compression.

Figure 3  Separated NW&UE operation mode

Observation 4: For two-sided AI/ML models, separate NW&UE operation mode can support separated training at NW and UE, which is free from the exchange of the AI/ML model via air-interface.

Based on the analysis above, the recommended operation modes are given in the following proposal.

Proposal 2: Study the following AI/ML operation modes for SI of AI/ML-based air-interface enhancement:

- NW-oriented operation mode: training @ NW, inference @ NW.

- UE-oriented operation mode: training @ UE, inference @ UE.

- Separate NW&UE operation mode: separate training @ NW&UE, inference @ NW&UE.

- 2.2.2 Procedure of AI/ML model generation

A typical procedure of AI/ML model generation is shown in Figure 4. Pre-processing of training-inputs can be applied before AI/ML model training in order to obtain high quality training-inputs and accelerate the training.

Figure 4  Procedure of AI/ML model generation

Pre-processing may include removing invalid training-inputs, and training-inputs transformation for accelerating AI/ML model training. Post-processing might include bit quantization, etc. For example, for CSI-compression-based CSI feedback, the pre-processing can be generating eigenvectors from full-channel-matrix, adaptation of inputs for varying system settings; the post-processing can be the quantization of outputs with certain bit width. For LOS/NLOS identification-based positioning, the pre-processing includes transforming the channel frequency response to power delay profile (PDP), and post-processing includes output quantization.  For beam management, adaptation of inputs for varying system settings (e.g., bandwidth) may be needed for pre-processing.

Proposal 3: Study pre/post-processing over use cases for inputs generation (e.g., generating eigenvector, inputs adaptation, generating PDP) and outputs generation (e.g., quantization).

AI/ML model validation and AI/ML model testing are parts of AI/ML model generation. In particular, model training is to optimize model parameters, model validation aims to optimize model hyper-parameters (e.g., model structure), and model testing is to test the performance of converged models. Model verification and model testing are generally considered to be left for implementation. In the context of AI/ML-based air-interface, the understanding of model validation and testing might need to be further clarified in RAN1 and other groups (e.g., RAN4). For example, model validation and testing may be subject to implementation.

Observation 5：Depending on the further clarification in RAN1 and other groups for model validation and testing whether it can be left for implementation.

- 2.2.3 Scenario-specific AI/ML model training

The propagation environment of wireless signals varies greatly, such as LOS and NLOS scenarios, indoors and outdoors scenarios, and slow-speed and fastmoving scenarios, etc. In different scenarios, characteristics of wireless channels are also diverse, for example, the multipath distribution characteristics and the channel sparsity could be very different. AI/ML is good at learning channel characteristics in a data-driven manner and performing a variety of signal processing tasks more efficiently. However, it is normally difficult to train a model so that it could always perform optimally in a variety of scenarios. Therefore, in order to fully exploit the potential gain of AI/ML, constructing scenario-based training datasets is beneficial, based on which a series of scenario-specific AI/ML models could be trained, and a suitable model could be selected for inference so as to adapt to various scenarios.

Proposal 4: Study scenario-specific AI/ML model training for AI/ML for air-interface.

2.3 AI/ML Model inference

In the inference stage, the input/output and pre/post-processing should be aligned with the training stage.  Otherwise it may lead to inaccurate interference results.

UE-gNB collaboration levels

The classification of UE-gNB collaboration levels is expected to follow the principle that there is no overlap between different collaboration levels. Depending on the content of information exchanged between gNB and UE, three classes of UE-gNB collaboration levels as below are recommended for further study and illustrated in Figure 5:

- Level 0: Implementation-based only AI/ML algorithms. AI/ML algorithms is purely implementation-based and is transparent to spec.

- Level 1: Signaling enhancement for AI/ML without model exchange. Necessary assistance signaling between UE and gNB is required for facilitating AI/ML operation, but AI-model exchange is not involved over the air-interface between gNB and UE.

- Level 2: Inter-node assistance with model exchange via air-interface. Explicit model exchange is needed between gNB and UE via air-interface, while necessary assistance signaling could be involved to support the model delivery.

Figure 5  UE-gNB collaboration levels

Proposal 5: The UE-gNB collaborations are classified into three levels

- Level 0: Implementation-based only AI/ML algorithms.

- Level 1: Signaling enhancement for AI/ML without model exchange.

- Level 2: Inter-node assistance with model exchange via air-interface.

For collaboration level 1, signaling enhancements includes, for example, feedback of training-inputs/labels. Besides, use cases in SID that may be realized in collaboration level 1 include CSI compression [2], beam prediction [3], LOS/NLOS identification, and heavy NLOS positioning [4].

Collaboration level 2 faces two major issues as follows:

First, since the size of an AI/ML model could be ranged from tens of KB to hundreds of MB, collaboration level 2 leads to air-interface overhead for exchanges of AI/ML model package, especially for the mobility and handover cases.

Second, the AI/ML model representation format (MRF) of NW’s and UE’s models might be incompatible since NW and UE may utilize different platforms for generating AI/ML models, as shown in Figure 6. As a consequence, the NW and UE may have problems in compiling the AI/ML model package sent from the peer node. As analyzed below, among the three potential candidates to the MRF issue, defining 3GPP AI/ML MRF is the most promising candidate for this issue, however it is challenging in Rel-18:

Potential candidate 1 - NW converts the AI/ML model to MRF what UE supports. But, it might cause heavy efforts on NW product for supporting a large amount of MRFs generated by legacy and new versions of AI/ML platforms.

Potential candidate 2 - Reuse existing MRF. NW and UE could convert their AI/ML models to an intermedia common MRF and only exchange models in the common MRF, e.g., Open Neural Network Exchange (ONNX) is an open-source AI/ML format and has been discussed in AI/ML society for model translation. However, ONNX was originally developed by IT companies and is not optimized for wireless network. Besides, ONNX is maintained by open-source community and beyond the 3GPP capability.

Potential candidate 3 - Define 3GPP AI/ML MRF. 3GPP could define its own AI/ML MRF which is deeply customized for wireless systems. The 3GPP-specific MRF could be maintained by 3GPP and support wireless AI/ML continuous evolution. Nevertheless, this may be challenging for RAN with heavy work load, and may involve joint WG work.

Figure 6  AI/ML model exchange across platforms

Observation 6: Collaboration level 1 with signaling enhancement for AI/ML without model exchange is simple for specification and implementation.

Observation 7: Collaboration level 2 faces the issue of AI/ML model representation format across platforms, which may be challenging to be addressed in Rel-18 since it would involve heavy burden of joint WG work.

Proposal 6: Prioritize the study of AI/ML for air-interface enhancement assuming collaboration level 1.

As analyzed in Section 2.2.1, collaboration level 1 can be applied to the following AI/ML operation modes:

- NW-oriented operation: training @ NW, inference @ NW

- UE-oriented operation: training @ UE, inference @ UE

- Separate NW&UE operation: separate training @ NW&UE, inference @ NW&UE

Lifecycle management

Due to the randomness of wireless channels and the mobility of UEs, the propagation environment of wireless signals changes frequently. Nevertheless, it is difficult for an AI/ML model to maintain optimal performance in all scenarios all the time, and the performance may even deteriorate sharply in some scenarios. Therefore, the lifecycle management (LCM) of AI/ML models is essential for sustainable operation of AI/ML in NR air-interface. LCM involves five aspects, including model training, model deployment, model inference, model monitoring, and model updating.

A basic functional framework of AI/ML has been proposed in TR 37.817 [5] which could be considered as a starting point for the study of LCM. On top of that, the terminology should be modified to avoid misleading in RAN1 discussion, as shown in Figure 7. For example, ‘data collection’ can be changed to ‘model-inputs collection’, ‘training data’ can be changed to ‘training-inputs’, and ‘inference data’ can be changed to ‘inference-inputs’, since the term of ‘data’ might be misinterpreted as user data which involves privacy issues and are out of the SID scope [1].

Figure 7  AI/ML functional framework in 37.817

Proposal 7: The functional framework of AI/ML given in TR 37.817 is considered as a starting point for the study of LCM.

4.1 AI/ML Model monitoring

Model monitoring can operate in event-driven or periodic manner. Model monitoring identifies the mismatch between AI/ML model and environment, which can avoid model failure. Model monitoring requires to collect information that reflects the model status. Yet, the process of model monitoring does not need to be always-on, but rather be configured as a monitoring window. The period of the monitoring window could be in terms of, e.g., days or weeks, while the samples for monitoring (e.g., labels) collected within the monitoring window can be subject to hundreds or thousands of TTIs, thereby the resulting overhead of model monitoring can be negligible on average, taking a tiny portion of time during LCM, as shown in Figure 8.

Figure 8 Relationship between model monitoring and LCM

In general, there are two options for model monitoring in terms of different metrics:

Option 1 – Inference accuracy, which can be directly monitored. Such performance can be obtained by comparing the inference results with the ground-truth labels. The label during the inference stage needs to be collected for evaluating whether the performance of the model is degraded. Taking NW based monitoring for instance, as the NW could collect ground-truth labels from multiple UE at a time, constructing a large number of diversified labels efficiently, so the duration of monitoring window and the overhead of ground-truth labels may be relatively small.

Option 2 – Eventual system performance, e.g. system throughput. For example, if the throughput using AI/ML decreases or lower than the legacy non-AI system, it might indicate that the model is not suitable to the current environment.

Observation 8: Overhead of ground-truth labels between NW and UE via air-interface may not be a big issue with respect to relatively small monitoring window within long monitoring periodicity in lifecycle management.

Proposal 8: Study the following metrics for AI/ML model monitoring in life cycle management

- Inference accuracy.

- Eventual system performance.

4.2 AI/ML Model updating

Model updating occurs after the model performance is monitored as not optimized any longer. In particular, either model fine-tuning based on the current employed model or switch to another model could be studied.

Proposal 9: Study model fine-tuning and model-selection for AI/ML model updating.

Construction of dataset

The dataset construction includes the training-inputs collection from realistic networks and training-inputs generation for performance evaluation.

5.1 Training-inputs collection from realistic networks

The AI/ML model trained based on offline datasets may not completely match the channel environment in realistic networks because wireless channel varies dynamically, leading to mismatch between the deployed model and the realistic networks. A practical method is that NW deploys a model pre-trained offline as a basic and fine-tune the model based on training-inputs collected from realistic networks. To this end, the procedure for training-inputs collection from realistic network for fine-tuning NW’s AI/ML model needs to be studied. In addition, from the realistic network perspective, larger size of training-inputs would mean longer time for collection and heavier overhead, thus how large dataset can guarantee accurate and optimized AI/ML model should be studied, e.g., by evaluations.

Proposal 10: Study the training-inputs collection from realistic network for optimizing the performance of AI/ML model.

5.2 Dataset generation for evaluation purpose

In order to evaluate the performance of AI/ML models in a convincing way, the corresponding dataset should reflect various channel scenarios, have a large amount of data, and have high and stable data quality. Dataset construction could be based on simulated training-inputs generated according to the 3GPP channel model. The 3GPP channel model can reflect various typical channel scenarios, and can efficiently generate a large amount of training-inputs under various system configurations. The independence of the training set and test set could be guaranteed by configuring different random seeds.

On the contrary, it is difficult for the field data to reflect various channel scenarios since richness of the data features is limited to the field sites. In addition, the consistency of the measured data quality may be difficult to guarantee, because it is closely related to the hardware conditions and operations during collection.

Proposal 11: For 3GPP evaluation of AI/ML, dataset constructed from simulation platform should be a baseline.

Conclusions

According to the discussions, following observations and proposals are provided:

Observation 1: Different mechanisms are observed for the following representative AI/ML algorithms:

- For supervised learning, labels need to be collected from the network as inputs for model training.

- For reinforcement learning, AI/ML model needs to be trained based on feedback from the network, where the feedback is derived based on the real-time outputs of the AI/ML model. Such continuous model-training and interaction procedure harms the realistic network efficiency.

- For federated learning, frequent AI/ML model exchange may be needed between NW and UE, which consumes heavy air-interface overheads for model exchange.

Observation 2: For NW-oriented operation mode where AI/ML model is trained at NW, the sub mode of training @ NW, inference @ NW is beneficial, including simple from spec perspective, no need of UE capability awareness at NW, and free from the issue of model exchange via air-interface.

Observation 3: For UE-oriented operation mode where AI/ML model is trained at UE, the sub mode of training @ UE, inference @ UE is beneficial, including simple from spec perspective, no need of maintaining UE-specific models at the NW side, and free from the issue of model exchange via air-interface.

Observation 4: For two-sided AI/ML models, separate NW&UE operation mode can support separated training at NW and UE, which is free from the exchange of the AI/ML model via air-interface.

Observation 5：Depending on the further clarification in RAN1 and other groups for model validation and testing whether it can be left for implementation.

Observation 6: Collaboration level 1 with signaling enhancement for AI/ML without model exchange is simple for specification and implementation.

Observation 7: Collaboration level 2 faces the issue of AI/ML model representation format across platforms, which may be challenging to be addressed in Rel-18 since it would involve heavy burden of joint WG work.

Observation 8: Overhead of ground-truth labels between NW and UE via air-interface may not be a big issue with respect to relatively small monitoring window within long monitoring periodicity in lifecycle management.

Proposal 1: Supervised learning should be considered as a starting point for studying the potential benefits of AI/ML to air-interface enhancement.

Proposal 2: Study the following AI/ML operation modes for SI of AI/ML-based air-interface enhancement:

- NW-oriented operation mode: training @ NW, inference @ NW.

- UE-oriented operation mode: training @ UE, inference @ UE.

- Separate NW&UE operation mode: separate training @ NW&UE, inference @ NW&UE.

Proposal 3: Study pre/post-processing over use cases for inputs generation (e.g., generating eigenvector, inputs adaptation, generating PDP) and outputs generation (e.g., quantization).

Proposal 4: Study scenario-specific AI/ML model training for AI/ML for air-interface.

Proposal 5: The UE-gNB collaborations are classified into three levels

- Level 0: Implementation-based only AI/ML algorithms.

- Level 1: Signaling enhancement for AI/ML without model exchange.

- Level 2: Inter-node assistance with model exchange via air-interface.

Proposal 6: Prioritize the study of AI/ML for air-interface enhancement assuming collaboration level 1.

Proposal 7: The functional framework of AI/ML given in TR 37.817 is considered as a starting point for the study of LCM.

Proposal 8: Study the following metrics for AI/ML model monitoring in life cycle management

- Inference accuracy

- Eventual system performance

Proposal 9: Study model fine-tuning and model-selection for AI/ML model updating.

Proposal 10: Study the training-inputs collection from realistic network for optimizing the performance of AI/ML model.

Proposal 11: For 3GPP evaluation of AI/ML, dataset constructed from simulation platform should be a baseline.

- References

- RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, December 6 - 17, 2021.

- R1-2203141, “Discussions on AI/ML for CSI Feedback Enhancement”, RAN1#109e, Huawei, HiSilicon, May 9 - 20, 2022.

- R1-2203142, “Evaluation on AI/ML for Beam Management”, RAN1#109e, Huawei, HiSilicon, May 9 - 20, 2022.

- R1-2203144, “Evaluations on AI/ML for Positioning Accuracy Enhancement”,  RAN1#109e, Huawei, HiSilicon, May 9 - 20, 2022.

- 3GPP TR 37.817, “Study on Enhancement for Data Collection for NR and EN-DC”, April, 2022.