3GPP TSG-RAN WG1 Meeting #109-e	R1-2203140

e-Meeting, May 9 – 20, 2022

Agenda Item:	9.2.2.1

Source:	Huawei, HiSilicon

Title:	Evaluation on AI/ML for CSI feedback enhancement

Document for:	Discussion and Decision

- Introduction

In RAN plenary meeting [1], a new SID on AI/ML for air-interface was approved for Rel-18.

Study the 3GPP framework for AI/ML for air-interface corresponding to each target use case regarding aspects such as performance, complexity, and potential specification impact. Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels Note: the selection of use cases for this study solely targets the formulation of a framework to apply AI/ML to the air-interface for these and other use cases. The selection itself does not intend to provide any indication of the prospects of any future normative project. …… For the use cases under consideration: - Evaluate performance benefits of AI/ML based algorithms for the agreed use cases in the final representative set: - Methodology based on statistical models (from TR 38.901 and TR 38.857 [positioning]), for link and system level simulations. - Extensions of 3GPP evaluation methodology for better suitability to AI/ML based techniques should be considered as needed. - Whether field data are optionally needed to further assess the performance and robustness in real-world environments should be discussed as part of the study. - Need for common assumptions in dataset construction for training, validation and test for the selected use cases. - Consider adequate model training strategy, collaboration levels and associated implications - Consider agreed-upon base AI model(s) for calibration - AI model description and training methodology used for evaluation should be reported for information and cross-checking purposes - KPIs: Determine the common KPIs and corresponding requirements for the AI/ML operations. Determine the use-case specific KPIs and benchmarks of the selected use-cases. - Performance, inference latency and computational complexity of AI/ML based algorithms should be compared to that of a state-of-the-art baseline - Overhead, power consumption (including computational), memory storage, and hardware requirements (including for given processing delays) associated with enabling respective AI/ML scheme, as well as generalization capability should be considered. …… Note 1: specific AI/ML models are not expected to be specified and are left to implementation. User data privacy needs to be preserved. Note 2: The study on AI/ML for air interface is based on the current RAN architecture and new interfaces shall not be introduced.

In this contribution, we provide our views on the evaluations on AI/ML for Channel State Information (CSI) feedback enhancement, specifically on CSI compression on spatial/frequency domain and CSI compression involving time domain, including evaluation methodology and preliminary simulation results.

- Evaluation methodology

CSI feedback plays an important role in cellular communication systems. AI/ML-based CSI feedback is one of the most popular topics of AI/ML for air-interface, aiming to study if AI/ML can help attain significant performance gains by improving CSI accuracy and/or reducing the feedback overhead.

In this section, we will discuss the dataset construction, AI/ML-based procedure, metrics, and evaluation assumptions used for evaluating AI/ML-based CSI compression for overhead reduction or improving accuracy. The principles on dataset construction, metrics, and generic MIMO related evaluation assumptions may also be applicable to other CSI feedback enhancement methods such as CSI prediction.

2.1 Dataset construction

The training and inference inputs are wireless channel information for AI/ML-based CSI feedback enhancement, which are used to construct training and testing dataset, respectively. For evaluation purpose, the dataset can be generated by simulation platforms, e.g., system level simulation platform, based on channel models defined in TR 38.901. For dataset generated by simulation platforms, the size and the quality of dataset can be easily controlled. The diversity of dataset can also be achieved by adjusting parameters or channel models in the simulation platform. However, for dataset generated from field tests, the above mentioned characters may be hard to ensure since the field tests are usually deployed in some specific scenarios and specific configurations. Therefore, at least dataset constructed by simulation platforms should be the baseline for evaluation. Whether and how to use dataset from field tests can be optional and reported by companies. In addition, how to evaluate the generalization of the AI/ML model in the simulation can be studied, e.g., the dataset for training the AI/ML model can be generated based mixed scenarios, and the dataset for testing is generated based on one of the mixed scenarios or another scenario.

Proposal 1: For CSI feedback enhancement evaluation, generating training inputs of AI/ML models by simulation platform should be a baseline.

- Training inputs from field test is optional and can be reported by companies.

Proposal 2: For CSI feedback enhancement evaluation, study how to evaluate the generalization of the AI/ML model in simulation.

Supervised learning is considered in our evaluation, and the training inputs of the ground-truth information are channel information. Channel models defined in TR 38.901, including UMa/UMi/InH channel models, can be used in the simulation platform as a starting point to generate the training and testing dataset. To avoid overfitting, the training dataset should be large enough and orthogonal with the testing dataset. For instance., in our evaluation for spatial frequency domain CSI compression, the training dataset includes 4*10^5 samples of channel information, and a separate testing dataset includes 1*10^5 samples, i.e. size of training dataset to size of testing dataset is 4:1; as another example, in our evaluation for spatial-frequency-time domain CSI compression, the dataset includes 6*10^5 samples of channel information, and a separate testing dataset includes 4*10^5 samples. Companies can adopt a relatively large size of testing dataset to avoid the randomness of intermediate results.

To generate such large size of training dataset and testing dataset, a separate training phase can be considered in the simulation for dataset construction and model training before the deployment of the AI/ML model. Substantial number of drops and/or UEs are generated in the training phase to generate the training inputs; e.g., in our evaluation for spatial frequency domain CSI compression, 500 drops and 1000 UEs for per drop are generated to contribute the 4*10^5 samples in training dataset and a separate 1*10^5 samples in testing dataset (i.e., one sample is collected from per UE per drop), so that the number of samples is large enough to avoid overfitting of the dataset; as another example, in our evaluation for spatial-frequency-time domain CSI compression, 10 drops and 100 UEs for per drop are generated, where 1000 samples of channel information with the interval of 5 TTIs between neighboring samples are collected from per UE per drop to capture the time correlation in the training inputs for learning. After that, the trained AI/ML model is deployed, and normal MIMO simulation (e.g., 10/20UEs per cell) is performed to evaluate the throughput performance.

Proposal 3: To construct training/testing dataset in the evaluation which is large enough to avoid overfitting, substantial number of drops and/or UEs can be adopted in the training phase for collecting the training inputs.

The channel information per sample in the training inputs can be the channel matrix or the eigenvector. Since legacy CSI codebook are typically based on eigenvector, it is better to use eigenvector for AI/ML-based CSI compression to make the evaluation comparable with the legacy schemes. In addition, the overhead of eigenvector is usually less than the channel matrix. Therefore, eigenvector should be used as the original CSI that needs to be compressed for AI/ML model-based CSI compression.

Proposal 4: To align with legacy CSI codebook, eigenvector should be used as the original CSI applied for training/inference under AI/ML-based CSI feedback.

In order to calibrate the simulation results among companies, 4 potential methods for AI/ML model calibration are discussed in the following:

- Option 1: Align the setup of the dataset construction phase and parameters for channel generation in the simulation.

- Option 2: Generate a common testing dataset. Each company uses its own training dataset to train the AI/ML model, and uses the common testing dataset to assess the trained AI/ML model.

- Option 3: Generate common training dataset and testing dataset. Each company uses the common training dataset to train the AI/ML model, and uses the common testing dataset to assess the trained AI/ML model.

- Option 4: Generate common training dataset and testing dataset and align the AI/ML model. E.g., a calibration AI/ML model can be aligned. Each company uses the common training dataset to train the calibrating AI/ML model, and uses the common testing dataset to assess the trained AI/ML model.

Option 1 is the simplest way to avoid discussing and converging on how to build the common dataset and/or calibrating AI/ML model. The performance of the AI/ML model and the quantity/quality of the dataset can be reflected by some intermediate results, such as generalized cosine similarity (GCS) or NMSE as we will discuss later. Since such intermediate results can be calculated using the direct output of the AI/ML model, they are less impacted by the scheduling mechanisms such as MU-MIMO paring, and can directly reflect the performance of AI/ML model compared to the eventual KPI such as throughput. For Option 2 and Option 3, even if the dataset is aligned over companies, the AI/ML models may still be companies’ implementation, which may lead to misaligned evaluation results. For Option 4, it is even more challenging to align on the AI/ML model as the implementations of the AI/ML model are quite diverse.

Proposal 5: The calibration of the AI/ML model and dataset can be achieved by aligning the setup of the dataset construction phase and parameters for channel generation in the simulation, and calibrating the intermediate results such as generalized cosine similarity.

2.2 AI/ML-based CSI compression procedure

In this section, we discuss the procedure of the AI/ML-based CSI compression sub use cases, including CSI compression in spatial-frequency domain, and CSI compression involving time domain. Figure 1 illustrates the flowchart for AI/ML-based CSI compression, where the AI/ML model is based on a two-sided auto-encoder structure including a CSI compression part and a CSI decompression part. In the training phase, UEs performs channel estimation based on the received CSI-RS, and feeds back the corresponding channel information (i.e., eigenvectors) as training inputs to the gNB for model training. After the training, the AI/ML model is applied for inference; in particular, the UE sends the eigenvector into the encoder for CSI compression and the compressed CSI is then fed back to the gNB; the gNB uses the CSI decoder to decompress the CSI feedback to obtain the recovered eigenvector(s) which are then used for generating the precoder(s). For CSI compression involving time domain, the CSI compression and decompression additionally use historic channel information for training/inference.

In addition, how to apply the AI/ML model(s) for multiple ranks can be discussed for evaluation. In our evaluation, a unified AI/ML model is trained and applied for each rank to perform individual inference, while the calculation of the rank number is based on the legacy approach. Other options can be FFS, for example, separated AI/ML models are trained per rank and applied for corresponding ranks to perform individual inference, or a unified AI/ML model is trained and applied for adaptive ranks to perform inference.

Proposal 6: A two-sided AI/ML model including a CSI compression part and a CSI decompression part can be considered for CSI compression sub use cases.

Proposal 7: Further study how to apply the AI/ML model(s) for the scenario with multiple ranks in evaluation of CSI compression.

- Option1: Separated AI/ML models are trained per rank and applied for corresponding ranks to perform individual inference.

- Option2: A unified AI/ML model is trained and applied for each rank to perform individual inference.

- Option3: A unified AI/ML model is trained and applied for adaptive ranks to perform inference.

Figure 1  The flowchart of AI/ML-based CSI compression

As preliminary results, some ideal assumptions are considered for simplicity, including the following:

- Training is performed in an offline manner, where the network collects the training inputs and performs training at a separate training phase as described previously, and after the AI/ML model is deployed, it is not updated during the simulation. Evaluations on the AI/ML model updating or fine-tuning can be further studied.

- The encoder part and the decoder part of the auto-encoder model is assumed to be jointly trained at network, and model is aligned by gNB and UE with offline approach. For realistic network, whether/how the encoder (CSI compression part) and decoder (CSI decompression part) can be matched if the encoder part and the decoder part are separately trained/fine-tuned at UE and gNB, respectively, without model exchange as described in our companion contribution [2] can be further studied.

- The training inputs in the dataset are assumed as ideal eigenvectors. Considering the realistic network where AI/ML model is trained/fine-tuned at gNB, the training inputs of eigenvectors are estimated by UEs and fed back as quantized values. The impact of quantization for such ground-truth eigenvectors can be further studied.

- Proposal 8: Offline training and ideal ground-truth label can be taken as a starting point for evaluation of CSI compression. The following non-ideal aspects can be further studied:

- AI/ML model updating or fine-tuning after model deployment.

- Whether/how the CSI compression part and the CSI decompression part of a two-sided AI/ML model can be matched under separate training at gNB and UE without model exchange.

- Training inputs fed back from UEs are quantized.

2.2.1 AI/ML model for CSI compression at Spatial-frequency domain

An encoder and a quantizer are deployed at the UE side for CSI compression, while a decoder and a de-quantizer are used at the gNB side for CSI decompression. The quantizer is used to quantize the output of the encoder which is a floating-point vector to fit the bit width for CSI feedback, while the de-quantizer is used to recover the floating-point vector as the input to the decoder. The AI/ML-based CSI feedback taking into account both spatial and frequency domain channel correlation is named as Vector-Quantization-based Spatial-Frequency (VQ-SF) domain compression, which is depicted in Figure 2.

Figure 2  The structure of VQ-SF

- Encoder: The encoder takes channel information as the input, and outputs the compressed CSI with smaller size than the original channel information. Specifically, the input of the encoder includes eigenvectors for N subbands, and can be formulated as W=w1…wN, where wn denotes the eigenvector for the n-th subband. Then, the encoder can use multiple Transformer layers to process the eigenvector matrix W, and gets the compressed CSI as a floating-point vector as a result. The compressed CSI can be formulated as WEN=fencoder(W), where fencoder(∙) represents the function of the encoder.

- Quantizer: The quantizer at the UE side maps the compressed CSI of a floating-point vector to a quantized bit sequence to fit the bit width for CSI feedback. Various methods of quantization may be adopted, such as scalar quantization, vector quantization, and etc. The quantized CSI feedback can be formulated as WQ=fquantizer(WEN), where fquantizer(∙) represents the function of the quantizer.

- De-quantizer: The de-quantizer recovers the compressed CSI from the feedback CSI bit sequence and sends it as the input to the decoder. The de-quantized CSI can be formulated as W'EN=fdequantizer(WQ) where fdequantizer(∙)represents the function of the de-quantizer.

- Decoder：The decoder recovers the eigenvectors. Specifically, the decoder can use multiple Transformer layers for CSI decompression, which is in alignment with the structure of the encoder. The recovered eigenvectors can be formulated as W'=fdecoder(W'EN), where fdecoder(∙) represents the function of the decoder.

2.2.2 AI/ML model for CSI compression at Spatial-Frequency-Time domain

The AI/ML-based CSI compression can also learn the time domain correlation of channels on top of spatial-frequency domain compression, namely Vector-Quantization-based-Spatial-Frequency-Time domain compression (VQ-SFT), which is depicted in Figure 3.

Figure 3 The structure of VQ-SFT

- Encoder: Similar to VQ-SF, the input of the encoder includes eigenvectors for N subbands. Different from VQ-SF, the encoder for VQ-SFT can store and utilize historic eigenvectors related information for further CSI compression. Specifically, the compressed CSI can be formulated as WEN=fencoder(W|Wt-1,Wt-2,…), where fencoder(∙) represents the function of the encoder. Wt-1,Wt-2,… represent the previous CSI related information at time t-1, t-2… which are already stored by the encoder fencoder(∙).

- Quantizer: The quantizer at the UE side maps the compressed CSI of a floating-point vector to a quantized bit sequence. Scalar quantization, vector quantization, etc., can be adopted. The quantized CSI feedback can be formulated as  WQ=fquantizer(WEN).

- De-Quantizer: The de-quantizer recovers the compressed CSI from the feedback CSI bit sequence and sends it as the input to the decoder. The de-quantized CSI can be formulated as W'EN=fdequantizer(WQ).

- Decoder: The decoder recovers the eigenvectors. Different from VQ-SF, the decoder for VQ-SFT can store and utilize historic feedback related information for CSI recovery. Specifically, the recovered eigenvectors can be formulated as W'=fdecoder(W'EN|Wt-1,Wt-2,…), where fdecoder(∙) represents the function of the decoder. Wt-1,Wt-2,… denote the previous CSI related information at time t-1, t-2…, which are stored by the decoder fdecoder(∙).

2.3 Metrics

The following metrics can be considered for performance evaluation of AI/ML-based CSI feedback:

Intermediate results

As discussed previously, intermediate results derived by the output of the AI/ML model during the testing/inference phase can directly reflect the performance of the AI/ML model. For the CSI compression sub use cases, there are two candidate options.

- Generalized cosine similarity (GCS) can be used as an intermediate result for accuracy assessment in training and inference phases, where higher value of GCS means higher CSI recovery accuracy and leads to high throughput performance. Specifically, GCS is formulated as

GCS=E{||wHw'||||w|| ||w'||}

where w denotes the original eigenvector for a subband and w'denotes the corresponding recovered eigenvector. E{∙} denotes the average operation over multiple samples and sub bands.

- Normalized mean square error (NMSE) can also be used for the accuracy evaluation of channel recovery. Different from GCS, NMSE reflects the element-wisesimilarity for the vector. Specifically, NMSE is formulated as

NMSE=E{w-w'22w22}

Considering the eigenvector is adopted as the input of the AI/ML model, GCS can better reflect the impact of the recovered CSI since it is used to derive the precoder based on vector operation. Therefore, GCS can be a more effective KPI than NMSE for the accuracy evaluation.

Proposal 9: GCS between original channel information and recovered channel information can be used as an intermediate indicator to evaluate the accuracy of AI/ML-based CSI feedback.

Eventual KPI

Throughput should be used as the eventual KPI for evaluating system level performance, which can be obtained by performing system level simulation. E.g., the average cell throughput and cell edge throughput can be regarded as the metric of eventual KPI. For the baseline CSI feedback schemes, it is recommended to use the following baselines to assess the performance gain of AI-based CSI model.

- Rel-16 Type II codebook which is also used for spatial-frequency domain CSI compression but with a linear compression approach.

- Rel-17 Type II port selection codebook can be also considered as the state-of-the-art non-AI CSI feedback scheme for performance comparison.

Proposal 10: System level simulation should be adopted to evaluate throughput performance of AI/ML-based CSI feedback.

Proposal 11: Take Rel-16 Type II codebook and Rel-17 Type II codebook as baselines to AI/ML-based CSI compression solutions.

Complexity

The complexity of the AI model, including time complexity and space complexity, should be considered in the evaluation of AI/ML-based CSI feedback. Space complexity is the amount of memory/storage space that is required by the AI/ML model, which can be measured by the model size. Larger AI/ML model size means bigger challenge for the storage to save the AI/ML model especially for the UE side. Time complexity is the amount of time needed to execute the AI/ML model. Intuitively, Floating Point Operations (FLOPs) is a reference metric for time complexity. Bigger FLOPs generally means more steps of computation and longer time accordingly. In addition, the power consumption can also be reflected by FLOPs. As different companies may adopt different AI/ML models, each company can report the FLOPs and model size of its own AI/ML model in the evaluation phase, which can be taken as a reference for studying the complexity/latency/power consumption. In addition, because of the diversity of AI/ML model structure and software/hardware environment, in order to achieve more accurate estimation of the AI/ML complexity or executed latency/power the AI/ML model, other metrics can also be FFS.

Proposal 12: FLOPs and model size should be considered to evaluate the complexity of the AI/ML model. Other metrics for assessing the complexity/latency/power consumption can be FFS.

2.4 Generic MIMO related evaluation assumptions

The CSI feedback enhancement has been discussed and evaluated in Rel-17 with the evaluation assumptions given in [3], which is provided in Table 1 of the Appendix. For Rel-18 AI/ML-based CSI feedback, the evaluation assumptions for Rel-17 CSI enhancements can be taken as a baseline for general MIMO related parameters. Some selected parameters in our preliminary evaluations are highlighted as green in the table. In addition, in our preliminary evaluation, some parameters are considered differently with the Rel-17 assumptions, as highlighted in yellow; e.g., for the entry of ‘Traffic model’, full-buffer traffic is considered in our evaluation for simplicity; as another example, for the entries of ‘Evaluation Metric’ and ‘Baseline for performance evaluation’, some different assumptions in our evaluation are analyzed in previous sections.

Proposal 13: Take the evaluation assumptions of Rel-17 CSI enhancements as a baseline for generic MIMO related parameters.

- Evaluation Results

In the evaluation, we consider a dense urban scenario operating on 2GHz FDD spectrum, and Uma channel model is assumed. Other generic MIMO related parameters and AI/ML specific simulation assumptions can refer to the Appendix.

3.1 Vector-quantization-based spatial-frequency domain compression

This section provides the evaluation results of the VQ-SF compression scheme. Both rank=1 and rank=2 are considered, where for rank=2, the AI/ML model is trained by using the eigenvectors of both ranks, and applied for inference for each rank separately to derive the compressed CSI per rank.

Figure 4 illustrates the comparison of GCS between VQ-SF and Rel-16 Type II codebook under the rank=1 case and the rank=2 case. It can be seen that, VQ-SF outperforms Rel-16 Type II codebook in terms of GCS for each rank, indicating higher accuracy of CSI recovery by VQ-SF. In addition, the accuracy of the 1st rank outperforms the accuracy of the 2nd rank since the sparsity of the 1st rank is sparser than the 2nd rank, so the compression efficiency is higher.

Figure 4  GCS between AI/ML-based recovered CSI and original eigenvector

The system level simulation results of throughput for VQ-SF are illustrated in Figure 5 and Figure 6. It is illustrated that, with the same overhead of CSI feedback, VQ-SF has a performance gain of 8.74%-9.58% over Rel-16 Type II codebook in terms of the throughput under rank=1, while it has a performance gain of 10.38%-11.63% over Rel-16 Type II codebook under rank=2. On the other hand, Figure 6 shows that, for achieving the same throughput, VQ-SF requires less feedback overhead, with an overhead reduction of about 60%. It can be seen that the performance of VQ-SF is also superior to Rel-17 Type II codebook. In addition, it can be seen from Figure 4 and Figure 5 that the trend of GCS can well match the trend of throughput.

Figure 5  Throughput gain over Rel-16 Type II codebook

Figure 6  Overhead reduction over Rel-16/17 Type II codebook

Observation 1: The trend of the GCS for AI/ML-based CSI compression matches with the trend of the throughput performance.

Observation 2: From the preliminary results, with the same overhead of CSI feedback and in terms of average throughput, AI/ML-based CSI compression on spatial-frequency domain can provide better performance with around 8%-11% over Rel-16 Type II codebook and outperforms Rel-17 Type II codebook.

3.2 Vector-quantization-based spatial-frequency-time domain compression

This section provides the evaluation results of the VQ-SFT compression scheme, where rank=1 is considered. Figure 7 illustrates the GCS and throughput of VQ-SFT with feedback overhead of 60bits and 120bits. It can be seen that VQ-SFT has improved GCS over Rel-16 Type II codebook, and performance gain of 12.77%-14.22% over Rel-16 Type II codebook in terms of the throughput. The VQ-SFT outperforms VQ-SF under the same feedback overhead.

Figure 7   GCS and throughput gain of VQ-SFT over Rel-16 Type II codebook

Observation 3: From the preliminary results, with the same overhead of CSI feedback and in terms of average throughput, AI/ML-based CSI compression on spatial-frequency-time domain can provide around 12%-14% performance gain over Rel-16 Type II codebook and outperforms AI/ML-based CSI compression on spatial-frequency domain.

- Conclusions

According to the discussion, following proposals and observations are provided:

Proposal 1: For CSI feedback enhancement evaluation, generating training inputs of AI/ML models by simulation platform should be a baseline.

- Training inputs from field test is optional and can be reported by companies.

Proposal 2: For CSI feedback enhancement evaluation, study how to evaluate the generalization of the AI/ML model in simulation.

Proposal 3: To construct training/testing dataset in the evaluation which is large enough to avoid overfitting, substantial number of drops and/or UEs can be adopted in the training phase for collecting the training inputs.

Proposal 4: To align with legacy CSI codebook, eigenvector should be used as the original CSI applied for training/inference under AI/ML-based CSI feedback.

Proposal 5: The calibration of the AI/ML model and dataset can be achieved by aligning the setup of the dataset construction phase and parameters for channel generation in the simulation, and calibrating the intermediate results such as generalized cosine similarity.

Proposal 6: A two-sided AI/ML model including a CSI compression part and a CSI decompression part can be considered for CSI compression sub use cases.

Proposal 7: Further study how to apply the AI/ML model(s) for the scenario with multiple ranks in evaluation of CSI compression.

- Option1: Separated AI/ML models are trained per rank and applied for corresponding ranks to perform individual inference.

- Option2: A unified AI/ML model is trained and applied for each rank to perform individual inference.

- Option3: A unified AI/ML model is trained and applied for adaptive ranks to perform inference.

- Proposal 8: Offline training and ideal ground-truth label can be taken as a starting point for evaluation of CSI compression. The following non-ideal aspects can be further studied:

- AI/ML model updating or fine-tuning after model deployment.

- Whether/how the CSI compression part and the CSI decompression part of a two-sided AI/ML model can be matched under separate training at gNB and UE without model exchange.

- Training inputs fed back from UEs are quantized.

Proposal 9: GCS between original channel information and recovered channel information can be used as an intermediate indicator to evaluate the accuracy of AI/ML-based CSI feedback.

Proposal 10: System level simulation should be adopted to evaluate throughput performance of AI/ML-based CSI feedback.

Proposal 11: Take Rel-16 Type II codebook and Rel-17 Type II codebook as baselines to AI/ML-based CSI compression solutions.

Proposal 12: FLOPs and model size should be considered to evaluate the complexity of the AI/ML model. Other metrics for assessing the complexity/latency/power consumption can be FFS.

Proposal 13: Take the evaluation assumptions of Rel-17 CSI enhancements as a baseline for generic MIMO related parameters.

Observation 1: The trend of the GCS for AI/ML-based CSI compression matches with the trend of the throughput performance.

Observation 2: From the preliminary results, with the same overhead of CSI feedback and in terms of average throughput, AI/ML-based CSI compression on spatial-frequency domain can provide better performance with around 8%-11% over Rel-16 Type II codebook and outperforms Rel-17 Type II codebook.

Observation 3: From the preliminary results, with the same overhead of CSI feedback and in terms of average throughput, AI/ML-based CSI compression on spatial-frequency-time domain can provide around 12%-14% performance gain over Rel-16 Type II codebook and outperforms AI/ML-based CSI compression on spatial-frequency domain.

References

- RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, December 6-17, 2021.

- R1-2203139, “Discussion on general aspects of AI/ML framework”, RAN1#109-e, Huawei, HiSilicon, May 9-20, 2022.

- R1-2006973, “Discussion Summary for CSI enhancements MTRP and FR1 FDD reciprocity”, RAN1#102-e, August 17-28, 2020.

Appendix: Simulation Assumptions

Table 1 General simulation assumptions for system level simulation

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD (TDD is not precluded), OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) is a baseline. Other scenarios (e.g. UMi@4GHz 2GHz, Urban Macro) are not precluded. |
| Frequency Range |  | FR1 only, 2GHz with duplexing gap of 200MHz between DL and UL, optional for 4GHz |
| Inter-BS distance |  | 200m |
| Antenna setup and port layouts at gNB |  | Companies need to report which option(s) are used between - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ - 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ Other configurations are not precluded. |
| Antenna setup and port layouts at UE |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for rank > 2 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) Other configuration is not precluded. |
| BS Tx power |  | 41 dBm for 10MHz, 44dBm for 20MHz, 47dBm for 40MHz |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz |
| Simulation bandwidth |  | 20 MHz for 15kHz as a baseline (optional for 10 MHz with 15KHz), and configurations which emulate larger BW, e.g., same sub-band size as 40/100 MHz with 30kHz, may be optionally considered |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | For low RU, SU-MIMO with rank adaptation are assumed For medium/high RU, SU/MU-MIMO with rank adaptation is assumed |
| MIMO layers |  | For all evaluation, companies to provide the assumption on the maximum MU layers (e.g. 8 or 12) |
| CSI feedback |  | Feedback assumption at least for baseline scheme - CSI feedback periodicity (full CSI feedback) : 5 ms, - Scheduling delay (from CSI feedback to time to apply in scheduling) : 4 ms |
| Overhead |  | Companies shall provide the downlink overhead assumption |
| Traffic model |  | FTP model 1 with packet size 0.5 Mbytes Other FTP model is not precluded. |
| Traffic load (Resource utilization) |  | - 70% for SU/MU-MIMO with rank adaptation - 20% for SU-MIMO with rank adaptation Companies are encouraged to report the MU-MIMO utilization. |
| UE distribution |  | - 80% indoor (3km/h), 20% outdoor (30km/h) |
| UE receiver |  | MMSE-IRC as the baseline receiver |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Realistic |
| Evaluation Metric |  | Throughput and CSI feedback overhead as baseline metrics. Additional metrics, e.g., ratio between throughput and CSI feedback overhead, can be used. Maximum overhead (payload size for CSI feedback)for each rank at one feedback instance is the baseline metric for CSI feedback overhead, and companies can provide other metrics. |
| Baseline for performance evaluation |  | Rel-16 PS eTypeII Codebook is the baseline for performance and overhead evaluation. (Type I Codebook can be considered at least for performance evaluation) - Note that it is encouraged to disclose further details of beamforming mechanism/ordering over CSI-RS ports/resources. |

Table 2 Simulation assumptions for training inputs of VQ-SF

| Parameters | Value |
| --- | --- |
| Number of drops | 500 |
| UEs per drop | 1000 |
| TTI sample for per UE | 1 sample per UE |
| Training set size | 4*10^5 |
| Testing set size | 10^5 |
| Training input | Eigenvector(s) of the channel |

Table 3 Simulation assumptions for training inputs of VQ-SFT

| Parameters | Value |
| --- | --- |
| Number of drops | 10 |
| UEs per drop | 100 |
| TTI interval between neighboring samples | 5ms |
| TTI samples for per UE | 1000 samples per UE |
| Training set size | 6 *10^5 |
| Testing set size | 4* 10^5 |
| Training input | Eigenvector(s) of the channel |