3GPP TSG-RAN WG1 Meeting #109-e	R1-2203142

e-Meeting, May 9 – 20, 2022

Agenda Item:   9.2.3.1

Source:        Huawei, HiSilicon

Title:	       Evaluation on AI/ML for beam management

Document for:  Discussion and Decision

Introduction

In the RAN#94 plenary meeting [1], a new SID on artificial intelligence (AI)/machine learning (ML) for air-interface was approved for Rel-18, and the objectives of the SID are attached in the Appendix. Based on the objectives, three use cases require investigations with high priority. Among the use cases, beam management may include beam prediction in spatial domain and beam prediction in time domain which are analyzed in our companion contribution [2]. In this contribution, we focus on the evaluations of AI/ML-based beam management (BM), particularly the beam prediction in spatial domain.

Background

The BM procudure aims at finding the strongest Tx/Rx beam pair. The conventional BM includes two typical methods: exhaustive beam sweeping and sparse beam sweeping. Exhaustive beam sweeping sweeps all available transmit and receive beams to determine the optimal pair. The performance can be optimized with this method but at the cost of beam sweeping overhead and power consumption. The sparse beam sweeping avoids sweeping all available beams by introducing 3 stages as described in 3GPP TR 38.802 [3], where Stage-1 selects one or a subset of Tx beam(s) from a sparse/wide beam pattern, and Stage-2 performs Tx beam sweeping for the neighboring/narrow beams of this subset of sparse/wide beam(s) to find the best narrow Tx beam.

- Stage-1: Beam sweep to support selection for both gNB Tx beams/ and UE Rx beam(s)

- Stage-2: Measuring gNB Tx beam(s) with UE Rx beams fixed

- Stage-3: Measuring UE Rx beams with gNB Tx beams fixed

The sparse beam sweeping approach can reduce the overhead and power consumption but may face the risk of inaccuracy on finding the optimal beam pair since this traditional way to determine the neighboring/narrow beams is performed based on fixed rules.

Observation 1: Legacy beam management faces the challenge to achieve a high beam selection accuracy with low beam sweeping overhead.

Motivated by this, we will focus on discussing and evaluating the spatial domain beam prediction enhancements empowered by AI/ML methods to improve the accuracy while keeping the beam sweeping overhead low.

Evaluation methodology for spatial domain beam prediction

Procedure of AI/ML-based spatial domain beam prediction

For the AI/ML-based sparse beam prediction, the target is to increase the accuracy compared to the legacy method and to narrow the performance gap to optimal exhaustive sweeping. Figure 1 below provides a flow chart to illustrate how the AI/ML-based BM is operated, and NW-oriented operation mode is considered here where the AI/ML model is assumed to be trained and inferred at the gNB side. Supervised learning is considered in this evaluation, and the training inputs of the ground-truth information are fed back from the UE.

For the AI/ML training phase, the gNB will perform the sweeping of all narrow beams, and the UE feeds back training inputs including the RSRPs and the optimal beam ID to the gNB which then will be used for the training.

After the AI/ML model is deployed, the gNB sweeps sparse beams at Stage-1 (e.g., 16 beams), and based on the feedback information from the UE, i.e. the received RSRP reports for the sparse beams, the set of narrow beams (a subset from the full beam set, e.g., 64 beams), namely Top-K beams, is inferred by using the AI/ML model. These Top-K narrow beams are then swept in Stage-2/3 as in the legacy system, and the optimal beam will be fed back from the UE. It should be noted that in the step 4 of the flow chart, the sparse SSB beam sweeping in Stage-1 can be either wide beams or narrow beams. In our preliminary evaluation, narrow beams are considered, and in Stage-2/3, the baseline of the non-AI/ML scheme will select the strongest sparse SSB beam ID as well as its 4 neighboring beams for Tx beam sweeping.

Figure 1 Flow chart for AI/ML-based spatial domain beam management

Though the AI/ML model for beam prediction can be also trained and inferred at the UE side under UE-oriented operation mode, it faces the issue of accuracy/generalization of the AI/ML model when the UE may not obtain the beam pattern from the gNB in the realistic networks as analyzed in [2]. Therefore, the evaluation for beam prediction can take the case where AI/ML model is trained and inferred at gNB as a starting point.

Proposal 1: The evaluation for beam prediction can take the NW-oriented operation mode where the AI/ML model is trained and inferred at the gNB as a starting point.

Dataset construction

For spatial domain beam prediction, the training inputs can be the RSRPs and the optimal beam ID of the Stage-1 sparse beams as described in the procedure. For the purpose of evaluation, this can be directly generated from the simulation platform as a starting point.

Proposal 2: For AI/ML-based spatial domain beam prediction evaluation, adopt the RSRP for sparse beams and the optimal beam ID as the training inputs.

Proposal 3: For AI/ML-based beam management evaluation, training inputs generated from simulation platform should be a baseline.

The performance of the AI/ML model and the quantity/quality of the dataset can be reflected by some intermediate results, such as beam prediction accuracy. Such intermediate results can be considered for AI/ML model calibration across companies. Other calibration options, such as generating a common training/testing dataset and/or generating a calibration AI/ML model across companies would spend lots of time and discussions, and should be considered with lower priority.

Proposal 4: The calibration of the AI/ML model and dataset for beam management evaluations can be achieved by aligning simulation assumptions and calibrating intermediate results such as beam prediction accuracy.

The dataset needs to be sufficiently large to avoid the overfitting problem during AI/ML training and inclusive to guarantee the generalization. It needs to study how to evaluate the generalization of the dataset. E.g., the dataset is generated from one scenario or mixed scenario and the trained AI/ML model is applied for another scenario. An example to guarantee the generalization is to construct a mixed training dataset based on multiple channel models, which may include, e.g., CDLA, CDLB, CDLD with various delay-spreads. In our preliminary evaluations, at the training stage, the training dataset includes 5 channel models (CDLA30, CDLB30, CDLB300, CDLD30, CDLD300), each of which consists of 2×105 samples. At the testing stage, as only one test scenario needs to be considered, the CDLB30 with 1000 samples is selected.

Proposal 5: For AI/ML-based beam management evaluation, study how to evaluate the generalization of the AI/ML model in simulation.

Applicable cases for AI/ML-based beam prediction in spatial domain

In this part, we provide two applicable cases for AI/ML-based beam prediction. Case 1 is the sparse beam sweeping based on 64-DFT codebook, where the codebook is supposed to have the same size as the gNB antenna array. The motivation of this approach it to employ AI/ML to enable sparse beam sweeping with accurate beam selection to reduce overhead and power consumption at the Stage-2/3, which is beneficial from UE perspective.

For Case 2, the target is to improve the coverage/performance from the network perspective. To improve the network performance, a denser beam sweeping codebook with more than 64 Tx beams (e.g., 256 Tx beams) can be considered. In this case, we consider to apply an AI/ML inferred Top-K subset among 256 dense beams to perform beam sweeping at Stage-2/3, which is expected to achieve better coverage than the legacy exhaustive 64 Tx beam sweeping as long as the AI/ML inferred Top-K beams are accurate. On the other hand, when K is configured as relatively small value, the number of CSI-RS resources can still be configured as no more than 64, which does not need to extend the maximum number of CSI-RS resources supported by the legacy spec.

Narrow codebook of 64-DFT beams

This evaluation method assumes that the beams are generated from a DFT codebook. The size of the DFT codebook is K×K, where K denotes the number of gNB antennas elements. The mathematical expression of the DFT codebook can be given by

W=e-2jkπi/KKj,k=1…K

Where j and k are the indexes for gNB antenna and beam id, respectively. The design of the sparse beam pattern is illustrated in Figure. 2 below.

Figure 2 Pattern of 16 sparse beam within 64 full beam set

Figure 2 illustrates how a sparse beam pattern (i.e., the red grids) is selected based on DFT codebook. Accordingly, this pattern of 16 beams is sent into the AI/ML model in Stage-1, then Top-K beams are predicted from all 64 Tx beams for Stage-2/3 sweeping.

Proposal 6: Consider the evaluation of the AI/ML based spatial domain beam prediction with the target to obtain accurate narrow beams for CSI-RS beam sweeping to reduce the UE’s overhead and power consumption.

Super-narrow codebook of 256 Tx beams

The super-narrow codebook can achieve 4 times higher resolution than the DFT codebook. For illustration, compared to 64-DFT codebook, the beam angle granularity of the super-narrow codebook is four times finer, and the resolution of beam steering improves by four times. The mathematical expression to generate this codebook is given by

W=e-2jkπi/KKj=1…K, k=1…4K

Similar to Figure 2, 16 sparse beams are swept in Stage-1, so that the overhead will not change. For Stage-2/3, the super-narrow beam is inferred by the gNB from the full set of 256 Tx beams so that the performance is expected to be improved due to more accurate beam steering than 64-DFT codebook. Figure 3 shows how the sparse beam pattern (i.e., the red grids) is selected for the super-narrow codebook.

Figure 3 Pattern of 16 sparse beam within 256 full beam set

Proposal 7: Consider a super-narrow codebook (e.g., with 256 Tx beams) for the evaluation of the AI/ML-based spatial domain beam prediction to improve the system performance (e.g., coverage, throughput) from the network perspective without increasing the maximum number of CSI-RS resources.

Metrics

The metrics can include the performance and complexity.

- Intermediate results

The intermediate results derived by the output of the AI/ML model during the testing/inference phase can directly reflect the performance of the AI/ML model. For the spatial domain beam prediction sub use case, the beam prediction accuracy is considered as the intermediate results.

- Eventual performance

Regarding the eventual performance, it is supposed to illustrate the network-level performance. Therefore, the RSRP of the selected beam after Stage-2/3 is considered as the eventual KPI. The RSRP of the AI/ML-based beam prediction can be compared with the baseline of traditional sparse beam sweeping (with fixed rule on Stage-2 Tx beam determination) by aligning the overhead. In addition, the exhaustive beam sweeping can be used to provide the upper bound of the RSRP performance, and the margin between the AI/ML-based beam prediction and this upper bound can be observed.

In our preliminary results, the RSRP results are provided, while other eventual performance metrics such as throughput can also be considered in future.

- Complexity

Regarding the AI/ML related contributions, the complexity is normally considered from two aspects: time complexity and space complexity. Space complexity is the amount of storage space required by the AI/ML model. Time complexity is the amount of time (or number of operations) needed to operate the AI/ML model, where Floating Point Operations (FLOPs) are normally regarded as the typical metric. In addition, the power consumption can also be reflected by FLOPs. Each company can report the FLOPs and model size of its own AI/ML model in the evaluation phase.

Simulation results for beam prediction in spatial domain

Link level simulation (LLS) is performed in our preliminary evaluation, where a cell and a single UE is considered. The simulation assumptions are listed in Appendix.

Sparse beam sweeping based on 64-DFT codebook

According to Table 1, there are two non-AI/ML schemes. The first scheme is the exhaustive beam sweeping over all 64 Tx beams described in Section 3.3.1, which can be regarded as the upper bound. The second one is the baseline of traditional sparse beam sweeping, where 16 sparse beams are swept at Stage-1, and after the UE feeds back the optimal Stage-1 beam ID, the gNB will determine the Tx beams for Stage-2 to include this optimal Stage-1 beam as well as its 4 neighboring beams, which corresponds to a fixed determination rule. For the AI/ML-based scheme, the values of K are assumed as 2, 4, 5, and 8 for the inference of Top-K beams for Stage-2, where K=5 is used to align the overhead with the baseline of the traditional sparse beam sweeping. It is noted that ‘BS X * UE Y’ denotes the X Tx beams are swept and Y Rx beams are swept during the corresponding stage.

Table 1 Schemes for evaluating the DFT codebook case

| Schemes | Stage-1 SSB | Stage-2 CSI-RS |  |
| --- | --- | --- | --- |
| Exhaustive 64 | BS 64 * UE 4 Exhaustive sweep | Optimal beam |  |
| Baseline | BS 16 * UE 4 Sparse beam sweep | One best measured beam and 4 neighbors |  |
| AI/ML | BS 16 * UE 4 Sparse beam sweep | Predicted Top-K beams from 64 narrow beams |  |

The evaluation result is provided in Figure 4 where the RSRP is provided as the relative gain over the baseline of traditional sparse beam sweeping. It can be found in that, with the increase of the value of K, the performance also improves. Taking K=5 for AI/ML-based scheme as an example which has the same overhead as the baseline, the RSRP is 1.55dB higher than the baseline. When compared with the upper bound performance of the exhaustive 64 Tx beam sweeping, the RSRP margin is 0.35dB, but the AI/ML-based scheme only needs 21 Tx beams in total which consists of 16 sparse Tx beams at Stage-1 and K=5 beams at Stage-2, thus the sweeping overhead is reduced by 67%. The power consumption will be reduced accordingly.

Figure 4 RSRP gain over baseline of traditional 16 sparse sweeping

Observation 2: In our preliminary evaluation results for AI/ML-based spatial domain beam prediction, the AI/ML-based scheme under 64-DFT codebook can provide RSRP gain over the traditional sparse beam sweeping under the same beam sweeping overhead.

Observation 3: In our preliminary evaluation results for AI/ML-based spatial domain beam prediction, the AI/ML-based scheme under 64-DFT codebook is close to the exhaustive 64 Tx beam sweeping but achieves significant sweeping overhead reduction.

Sparse beam sweeping based on 256-super-narrow codebook

The schemes for the 256-super-narrow codebook is provided in Table 2, where two non-AI/ML schemes are also considered for comparison. The exhaustive 64 Tx beam sweeping described in Section 3.3.1 is considered as the ceiling of the legacy beam sweeping under the 64-DFT codebook, and the exhaustive 256 Tx beam sweeping under the supper-narrow codebook of 256 Tx beams described in Section 3.3.2 is considered as the upper bound of 256 Tx beams. For the AI/ML-based scheme, the values of K are assumed as 2, 4, 5, and 8 for the inference of Top-K beams for Stage-2, so the overhead is the same as the AI/ML-based scheme in Section 4.1, but the inferred beams are from the supper-narrow codebook of 256 Tx beams.

Table 2 Schemes for evaluating the 256-super-narrow codebook case

| Schemes | Stage-1 SSB | Stage-2 CSI-RS |  |
| --- | --- | --- | --- |
| Exhaustive 64 | BS 64 * UE 4 Exhaustive sweep | Optimal beam |  |
| Exhaustive 256 | BS 256 * UE 4 Exhaustive sweep | Optimal Beam |  |
| AI/ML | BS 16 * UE 4 Sparse beam sweep | Predicted Top-K beams from 256-super-narrow beams |  |

The evaluation result is provided in Figure 5 where the RSRP is provided also as the relative gain over the baseline of traditional sparse beam sweeping provided by Section 4.1. It can be observed that, the AI/ML-based scheme based on the supper-narrow codebook of 256 Tx beams (denoted as ‘256CB’) can provide 0.9dB~1.9dB over the exhaustive 64 Tx beam sweeping. It shows that the AI/ML-based beam prediction can achieve better coverage than the ceiling of the legacy 64 Tx beams under only small beam sweeping overhead, which is comparable with the baseline sparse beam sweeping and much smaller than exhaustive beam sweeping. This is due to the beam prediction accuracy and the use of the super-narrow beam.

On the other hand, the RSRP margin of the AI/ML-based beam prediction is 0.2dB when compared with the upper bound performance of the exhaustive 256 Tx beam sweeping, which is quite close.

Figure 5 RSRP gain over baseline of traditional 16 sparse sweeping

Observation 4: In our preliminary evaluation results for AI/ML-based spatial domain beam prediction, the AI/ML-based scheme under the supper-narrow codebook of 256 Tx beams can provide RSRP gain over the exhaustive 64 Tx beam sweeping under 64-DFT codebook with much smaller beam sweeping overhead.

Conclusion

Based on the previous discussions, following observations and proposals are provided.

Observation 1: Legacy beam management faces the challenge to achieve a high beam selection accuracy with low beam sweeping overhead.

Proposal 1: The evaluation for beam prediction can take the NW-oriented operation mode where the AI/ML model is trained and inferred at the gNB as a starting point.

Proposal 2: For AI/ML-based spatial domain beam prediction evaluation, adopt the RSRP for sparse beams and the optimal beam ID as the training inputs.

Proposal 3: For AI/ML-based beam management evaluation, training inputs generated from simulation platform should be a baseline.

Proposal 4: The calibration of the AI/ML model and dataset for beam management evaluations can be achieved by aligning simulation assumptions and calibrating intermediate results such as beam prediction accuracy.

Proposal 5: For AI/ML-based beam management evaluation, study how to evaluate the generalization of the AI/ML model in simulation.

Proposal 6: Consider the evaluation of the AI/ML based spatial domain beam prediction with the target to obtain accurate narrow beams for CSI-RS beam sweeping to reduce the UE’s overhead and power consumption.

Proposal 7: Consider a super-narrow codebook (e.g., with 256 Tx beams) for the evaluation of the AI/ML-based spatial domain beam prediction to improve the system performance (e.g., coverage, throughput) from the network perspective without increasing the maximum number of CSI-RS resources.

Observation 2: In our preliminary evaluation results for AI/ML-based spatial domain beam prediction, the AI/ML-based scheme under 64-DFT codebook can provide RSRP gain over the traditional sparse beam sweeping under the same beam sweeping overhead.

Observation 3: In our preliminary evaluation results for AI/ML-based spatial domain beam prediction, the AI/ML-based scheme under 64-DFT codebook is close to the exhaustive 64 Tx beam sweeping but achieves significant sweeping overhead reduction.

Observation 4: In our preliminary evaluation results for AI/ML-based spatial domain beam prediction, the AI/ML-based scheme under the supper-narrow codebook of 256 Tx beams can provide RSRP gain over the exhaustive 64 Tx beam sweeping under 64-DFT codebook with much smaller beam sweeping overhead.

- References

- RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, RAN#94e, Electronic Meeting, Dec. 6 - 17, 2021.

- R1-2203143, “Discussion on AI/ML for beam management”, RAN1#109e, Huawei, HiSilicon, May 9-20, 2022.

- 3GPP TR 38.802 Study on New Radio Access Technology - Physical Layer Aspects

- 3GPP TS 38.901 Study on channel model for frequencies from 0.5 to 100 GHz (Release 16)

- Appendix

Objectives in SID

Study the 3GPP framework for AI/ML for air-interface corresponding to each target use case regarding aspects such as performance, complexity, and potential specification impact. Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels Note: the selection of use cases for this study solely targets the formulation of a framework to apply AI/ML to the air-interface for these and other use cases. The selection itself does not intend to provide any indication of the prospects of any future normative project. …… For the use cases under consideration: - Evaluate performance benefits of AI/ML based algorithms for the agreed use cases in the final representative set: - Methodology based on statistical models (from TR 38.901 and TR 38.857 [positioning]), for link and system level simulations. - Extensions of 3GPP evaluation methodology for better suitability to AI/ML based techniques should be considered as needed. - Whether field data are optionally needed to further assess the performance and robustness in real-world environments should be discussed as part of the study. - Need for common assumptions in dataset construction for training, validation and test for the selected use cases. - Consider adequate model training strategy, collaboration levels and associated implications - Consider agreed-upon base AI model(s) for calibration - AI model description and training methodology used for evaluation should be reported for information and cross-checking purposes - KPIs: Determine the common KPIs and corresponding requirements for the AI/ML operations. Determine the use-case specific KPIs and benchmarks of the selected use-cases. - Performance, inference latency and computational complexity of AI/ML based algorithms should be compared to that of a state-of-the-art baseline - Overhead, power consumption (including computational), memory storage, and hardware requirements (including for given processing delays) associated with enabling respective AI/ML scheme, as well as generalization capability should be considered. …… Note 1: specific AI/ML models are not expected to be specified and are left to implementation. User data privacy needs to be preserved. Note 2: The study on AI/ML for air interface is based on the current RAN architecture and new interfaces shall not be introduced.

Simulation assumptions

| Structures | AI at gNB |
| --- | --- |
| Carrier frequency | 30 GHz |
| Bandwidth | 20MHz |
| UE antenna | 4 [3] |
| gNB antenna | 64 [3] |
| Channel Model | CDLA, CDLB, CDLD [4] |
| Training Channels | Mixed CDLA30, CDLB30, CDLB300, CDLD30, CDLD300 |
| Test Channel | CDLB30 |
| Baseline | Rel-15 beam management, where 5 narrow beams in Stage-2 are artificially selected based on reported beams of SSB in Stage-1 |
| AI/ML model | CNN |