3GPP TSG RAN WG1 Meeting #109-e	R1-2203145

e-Meeting, May 9 – 20, 2022

Agenda Item:	9.2.4.2

Source:	Huawei, HiSilicon

Title:	Discussion on AI/ML for positioning accuracy enhancement

Document for:	Discussion and Decision

Introduction

In the RAN #94 e-meeting, a new SI to study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface had been approved [1], its objectives are given in the Appendix of this contribution. During the study item, three use cases should be investigated to evaluate achievable gains and complexity of AI/ML-based techniques. It should also be identified if specification effort is required to enable AI/ML methods for the air-interface. Furthermore, sub use cases for each of the three use cases should be finalized by RAN#98.

This contribution discusses the sub use cases, the categorization of the AI/ML operation mode and potential specification impact for AI/ML-positioning accuracy enhancement.

Sub use cases for AI/ML-based positioning

As described in the SID, AI/ML-based positioning accuracy enhancements should consider different scenarios including those with heavy NLOS components. In addition, it is our view that conditions with more LOS paths should also be considered, because AI/ML-based techniques can even help here to significantly improve the performance.

Sub use case 1 - AI/ML-based LOS/NLOS identification

In Rel-17, LOS/NLOS indicator reporting is supported to improve the positioning accuracy. The evaluation of the legacy LOS/NLOS method shows that it can enable sub-meter level positioning accuracy @ 90% when a large number of TRP antennas, e.g., 16 or 32, is employed. However, the performance degrades significantly for a smaller number of antennas. For example, as shown in our companion paper [2], when 4 antennas are used at the TRPs the positioning accuracy is several meters, if the non-AI identification method relies on the energy consistency and phase consistency of the first path across different antennas.

In typical indoor scenarios when the number of antennas per TRP is not very large, by learning the difference between features associated with LOS and NLOS paths, AI/ML-based LOS/NLOS identification followed by the traditional TDOA positioning technique can achieve sub-meter level positioning accuracy @ 90% even with 4 TRP antennas [2].

That means, AI/ML-based LOS/NLOS identification can be studied to improve the positioning accuracy with a small number of TRP antennas.

Sub use case 2 - AI/ML-based positioning in heavy NLOS scenarios

The traditional non-AI positioning techniques, e.g., TDOA, RTT, are relying on the LOS paths between the UE and multiple TRP(s). For indoor scenarios without a sufficient number of LOS paths, the traditional non-AI positioning techniques cannot work properly. By learning the relationship between channel features and UE coordinates, AI/ML-based positioning can achieve sub-meter level positioning accuracy @ 90% even in heavy NLOS scenario [2]. Therefore, AI/ML-based positioning can be considered to improve the positioning accuracy for scenarios with heavy NLOS conditions.

Based on the above discussion of potential sub use cases, we are making the following proposal:

Proposal 1: For AI/ML-based positioning accuracy enhancements, the following two sub use cases should be studied:

- AI/ML-based LOS/NLOS identification

- AI/ML-based positioning in heavy NLOS scenarios

Categorization of AI/ML operation mode

For AI/ML-based positioning, the AI/ML operation modes can include UE-oriented operation mode (training and inference at UE) and NW-oriented operation mode (training and inference at NW), where NW-oriented operation mode can include gNB-oriented mode and LMF-oriented mode. In this section, the AI/ML operation modes are analyzed from the following aspects:

Model training

- Training inputs collection

Training inputs for AI/ML-based positioning include channel measurements and ground-truth labels. The ground-truth labels are LOS/NLOS tags for LOS/NLOS identification sub use case and UE coordinates for positioning in heavy NLOS scenarios. It is hard to obtain these ground-truth labels for normal UEs. Fortunately, positioning reference units (PRUs), with known locations have already been discussed in Rel-17 to support the mitigation of timing errors. By utilizing the PRU, the ground-truth LOS/NLOS tags or UE coordinates can be collected.

It would be more convenient to collect sufficient training inputs at the gNB and LMF rather than doing it at the UE, since the network side is more efficient to collect substantial labels for training from all PRUs in the cell. Therefore, AI/ML model training at gNB and LMF may be easier to implement in term of training inputs collection.

Observation 1: For AI/ML-based positioning, ground-truth labels of LOS/NLOS tags or UE real coordinates for AI/ML model training can be obtained by positioning reference units.

- Training capability

Generally speaking, network devices, both gNB and LMF, have much more computing power and memory than the UE. This enables the network devices to train and store more powerful AI/ML models that can achieve a higher positioning accuracy.

Model updating

When the UE environment changes, e.g., when moving from one room to another, the AI/ML model should be updated correspondingly to match the new scenario. Since in principle there is no restriction for UE’s movement, it is difficult to pre-train and to store all possible AI/ML models. If the AI/ML model is deployed at the UE side, then the UE needs to retrain the AI/ML model. Alternatively, UE needs to download a new AI/ML model from NW side frequently. In contrast, the environments are generally correlated with the NW nodes (e.g., gNB). It can be known beforehand what kind of environments will be encountered when the NW nodes are deployed. Therefore, the gNB or LMF can pre-train and store multiple AI/ML models suitable for its coverage area.

Observation 2: For AI/ML-based positioning, it is more convenient for gNB and LMF to perform the updating of AI/ML models which could be scenario specific.

Model inference

- AI/ML-based LOS/NLOS identification

For LMF-oriented LOS/NLOS identification, UE or gNB needs to feed back the channel measurements to LMF since LMF doesn’t perform channel measurement. The inputs of the AI/ML model for positioning may be the power delay profile (PDP) or the channel impulse response (CIR), which have large packet size.  This may introduce latency and overhead for information exchange.

For gNB-oriented and UE-oriented LOS/NLOS identification, the gNB and the UE can use the LOS/NLOS tags derived by an AI/ML model to remove the channel without LOS path and to transmit intermediate results such as TOA, RSRP, and/or AoA of channels to the LMF. Then the eventual UE coordinates can be calculated based on these intermediate results of LOS paths by, e.g., LMF. The latency and overhead can be reduced since the size of intermediate results are much smaller than PDP or CIR.

- AI/ML-based positioning in heavy NLOS scenario

Given the current Location Services architecture specified in TS 38.305 and TS 23.273, the gNB is forbidden to obtain the UE locations. Similarly, in some cases, the information about the gNB locations should also not be disclosed to the UE. Therefore, for AI/ML-based positioning in heavy NLOS scenarios, if the AI/ML model is used to derive UE coordinates, i.e., fingerprint based, a universal solution is to deploy the AI/ML model at the LMF. On the other hand, if the AI/ML model is used to derive some intermediate results, the pros/cons of the AI/ML operation modes is similar as the LOS/NLOS identification analyzed previously.

Observation 3: For the LOS/NLOS identification sub use case, gNB-oriented/UE-oriented operation mode can achieve lower latency than LMF-oriented operation mode.

Observation 4: For the AI/ML-based fingerprint positioning in heavy NLOS scenario, LMF-oriented operation mode would be a universal solution.

Potential specification impact

The procedures of LMF-oriented, gNB-oriented and UE-oriented AI/ML-based positioning are shown in Figure 1. As we discussed in our companion contribution for the AI/ML framework [3], AI/ML model exchange faces the issues of AI/ML model delivery overhead and AI/ML model representation format (MRF) across platforms. Therefore, we only discuss the cases that model training and model inference are performed by the same nodes.

| LMF-oriented | gNB-oriented | UE-oriented |
| --- | --- | --- |

Figure 1 AI/ML-based positioning procedures for different operation modes

For LMF-oriented AI/ML operation, the LMF needs to collect training inputs and ground-truth labels, for example, from PRUs. Then, LMF can train the AI/ML models for coordinates or LOS/NLOS identification. In model inference phase, the gNBs can measure SRS and transmit the measurements, e.g., PDP, CIR or pre-processed CIR, to LMF. Alternatively, the UE can measure PRS and transmit the measurements to LMF. Then, the LMF can perform AI/ML model inference to derive UE coordinates. The potential specification impact may include: feedback of channel measurements (e.g., CIR, PDP) to LMF.

For gNB-oriented AI/ML operation, gNB needs to collect training inputs and ground-truth labels, for example, from PRUs. Then gNB can train the AI/ML models for LOS/NLOS identification. In model inference phase, gNBs measure SRS and perform AI/ML model inference to derive LOS/NLOS states. Then, intermediate results such as TOA of channels with LOS path can be transmitted to LMF to facilitate the final positioning.

For UE-oriented AI/ML operation, the UE needs to collect training inputs and ground-truth labels, for example, from PRUs. Then, UE can train the AI/ML models for LOS/NLOS identification. In model inference phase, UE can measure PRS and perform AI/ML model inference to derive LOS/NLOS states. Then intermediate results such as TOA of channels with LOS path can be transmitted to LMF to facilitate the final positioning.

Proposal 2: Study whether potential specification impact is needed for AI/ML-based positioning, including feedback of channel measurements (e.g., CIR, PDP) to LMF.

Conclusions

In this contribution, we discussed the potential specification impact and the consideration of sub use cases for positioning accuracy enhancements. Based on the discussion, we have the following observations and proposals.

Proposal 1: For AI/ML-based positioning accuracy enhancements, the following two sub use cases should be studied:

- AI/ML-based LOS/NLOS identification

- AI/ML-based positioning in heavy NLOS scenarios

Observation 1: For AI/ML-based positioning, ground-truth labels of LOS/NLOS tags or UE real coordinates for AI/ML model training can be obtained by positioning reference units.

Observation 2: For AI/ML-based positioning, it is more convenient for gNB and LMF to perform the updating of AI/ML models which could be scenario specific.

Observation 3: For the LOS/NLOS identification sub use case, gNB-oriented/UE-oriented operation mode can achieve lower latency than LMF-oriented operation mode.

Observation 4: For the AI/ML-based fingerprint positioning in heavy NLOS scenario, LMF-oriented operation mode would be a universal solution.

Proposal 2: Study whether potential specification impact is needed for AI/ML-based positioning, including feedback of channel measurements (e.g., CIR, PDP) to LMF.

- References

RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, RAN#94e, December 2021.

R1-2203144, “Evaluation on AI/ML for positioning accuracy enhancement”, RAN1#109e, Huawei, HiSilicon, May 2022.

R1-2203139, “Discussion on general aspects of AI/ML framework”, RAN1#109e, Huawei, HiSilicon, May 2022.

- Appendix - Objectives in WID

Study the 3GPP framework for AI/ML for air-interface corresponding to each target use case regarding aspects such as performance, complexity, and potential specification impact. Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels Note: the selection of use cases for this study solely targets the formulation of a framework to apply AI/ML to the air-interface for these and other use cases. The selection itself does not intend to provide any indication of the prospects of any future normative project. AI/ML model, terminology and description to identify common and specific characteristics for framework investigations: - Characterize the defining stages of AI/ML related algorithms and associated complexity: - Model generation, e.g., model training (including input/output, pre-/post-process, online/offline as applicable), model validation, model testing, as applicable - Inference operation, e.g., input/output, pre-/post-process, as applicable - Identify various levels of collaboration between UE and gNB pertinent to the selected use cases, e.g., - No collaboration: implementation-based only AI/ML algorithms without information exchange [for comparison purposes] - Various levels of UE/gNB collaboration targeting at separate or joint ML operation. - Characterize lifecycle management of AI/ML model: e.g., model training, model deployment , model inference, model monitoring, model updating - Dataset(s) for training, validation, testing, and inference - Identify common notation and terminology for AI/ML related functions, procedures and interfaces - Note: Consider the work done for FS_NR_ENDC_data_collect when appropriate For the use cases under consideration: - Evaluate performance benefits of AI/ML based algorithms for the agreed use cases in the final representative set: - Methodology based on statistical models (from TR 38.901 and TR 38.857 [positioning]), for link and system level simulations. - Extensions of 3GPP evaluation methodology for better suitability to AI/ML based techniques should be considered as needed. - Whether field data are optionally needed to further assess the performance and robustness in real-world environments should be discussed as part of the study. - Need for common assumptions in dataset construction for training, validation and test for the selected use cases. - Consider adequate model training strategy, collaboration levels and associated implications - Consider agreed-upon base AI model(s) for calibration - AI model description and training methodology used for evaluation should be reported for information and cross-checking purposes - KPIs: Determine the common KPIs and corresponding requirements for the AI/ML operations. Determine the use-case specific KPIs and benchmarks of the selected use-cases. - Performance, inference latency and computational complexity of AI/ML based algorithms should be compared to that of a state-of-the-art baseline - Overhead, power consumption (including computational), memory storage, and hardware requirements (including for given processing delays) associated with enabling respective AI/ML scheme, as well as generalization capability should be considered. - Assess potential specification impact, specifically for the agreed use cases in the final representative set and for a common framework: - PHY layer aspects, e.g., (RAN1) - Consider aspects related to, e.g., the potential specification of the AI Model lifecycle management, and dataset construction for training, validation and test for the selected use cases - Use case and collaboration level specific specification impact, such as new signalling, means for training and validation data assistance, assistance information, measurement, and feedback - Protocol aspects, e.g., (RAN2) - RAN2 only starts the work after there is sufficient progress on the use case study in RAN1 - Consider aspects related to, e.g., capability indication, configuration and control procedures (training/inference), and management of data and AI/ML model, per RAN1 input - Collaboration level specific specification impact per use case - Interoperability and testability aspects, e.g., (RAN4) - RAN4 only starts the work after there is sufficient progress on use case study in RAN1 and RAN2 - Requirements and testing frameworks to validate AI/ML based performance enhancements and ensuring that UE and gNB with AI/ML meet or exceed the existing minimum requirements if applicable - Consider the need and implications for AI/ML processing capabilities definition Note 1: specific AI/ML models are not expected to be specified and are left to implementation. User data privacy needs to be preserved. Note 2: The study on AI/ML for air interface is based on the current RAN architecture and new interfaces shall not be introduced.