3GPP TSG-RAN WG1 Meeting #109-e 	R1-2203146

e-Meeting, May 9 – May 20, 2022

Agenda Item:	9.4.1.1

Source:	Huawei, HiSilicon

Title:	Channel access mechanism and resource allocation for sidelink operation over unlicensed spectrum

Document for:	Discussion and Decision

- Introduction

RAN#94-e approved a new work item on NR sidelink evolution [1]. One objective of this WI is to study and specify sidelink on unlicensed spectrum for both mode 1 and mode 2:

Study and specify support of sidelink on unlicensed spectrum for both mode 1 and mode 2 where Uu operation for mode 1 is limited to licensed spectrum only [RAN1, RAN2, RAN4]

- Channel access mechanisms from NR-U shall be reused for sidelink unlicensed operation

  - Assess the applicability of sidelink resource reservation from Rel-16/Rel-17 to sidelink unlicensed operation within the boundaries of unlicensed channel access mechanism and operation

    - No specific enhancements for Rel-17 resource allocation mechanisms

    - If the existing NR-U channel access framework does not support the required SL-U functionality, WGs will make appropriate recommendations for RAN approval.

- Physical channel design framework: Required changes to NR sidelink physical channel structures and procedures to operate on unlicensed spectrum

  - The existing NR sidelink and NR-U channel structure shall be reused as the baseline.

- No specific enhancements for existing NR SL feature

- The study should focus on FR1 unlicensed bands (n46 and n96/n102) and is to be completed by RAN#98.

According to the WID, channel access mechanisms from NR-U shall be reused for sidelink unlicensed operation. In order to focus RAN1’s efforts on where it is most needed, it is beneficial to review NR-U design as well as NR sidelink design, and determine what is applicable as is, what needs to be modified, and what needs to be redesigned from the ground up.

In this contribution we provide our considerations on SL-U design for channel access procedures for sidelink operation over unlicensed spectrum including COT (Channel Occupation Time) sharing, LBE (Load Based Equipment) and FBE (Frame Based Equipment) mechanisms, Short Control Signaling and resource allocation Mode 1 and Mode 2. In addition, evaluation methodology for sidelink unlicensed operation is studied on UE deployment, UE dropping model and traffic model.

- Channel access procedures

Background: NR-U channel access design

Channel access procedures for NR-U DL and UL transmission have been specified in TS 37.213, which mainly follow the principle of LBT (Listen Before Talk).

A gNB or a UE selects channel access parameters subject to CAPC (Channel Access Priority Class) table for channel access procedures, including maximum COT and value range of CW (Contention Window) to determine counter value N which is used to determine the duration of channel sensing.

NR-U DL transmission supports two types of channel access procedures, i.e., Type 1 and Type 2. DL Type 1 is applicable to any DL transmission initiated by a gNB. DL Type 2 is used for certain transmissions, e.g., transmission initiated by a gNB with discovery burst, or transmissions by a gNB following transmission by a UE after a gap of 25 us/16us in a shared channel occupancy.

NR-U UL channel access mechanism also involves two types, i.e., Type 1 and Type 2. UL Type 1 is applicable to PUSCH/PUCCH/SRS transmissions scheduled or configured by gNB, or any transmission related to random access procedure. UL Type 2 is applicable in the case where a UE receives a signal indicated by a gNB to perform Type 2 LBT.

Table 1 below gives a short summary on how channel sensing is performed and how long the sensing duration is.

Table 1 Summary of Type 1 and Type 2 LBT in NR-U

| Type 1 | Time duration that are sensed to be idle before transmissions is random. A gNB or a UE determines an initial counter N which is randomly selected between 0 and CWp, where CWmin,p≤CWp≤CWmax,p, CWmin,p and CWmax,p are subject to CAPC. N can be decreased when channel is sensed to be idle for a certain period of time. Transmission can only take place when N reaches 0. |
| --- | --- |
| Type 2A | Time duration that are sensed to be idle before transmissions is deterministic. The channel needs to be idle for a sensing interval of 25 us. |
| Type 2B | Time duration that are sensed to be idle before transmissions is deterministic. The channel needs to be idle for a sensing interval of 16 us. |
| Type 2C | Does not sense the channel before transmission. Duration of the corresponding transmission is at most 584 us. |

NR-U supports multi-channel access for DL and UL transmission to improve data rate. There are two types for DL multi-channel access procedures, i.e., Type A and Type B. Type A requires gNB to perform Type 1 LBT on each channel. Type B requires gNB to perform Type 1 LBT on one of the multiple channels and sense other channels for at least a sensing interval 25us before transmission, but with constraint that selection of channels should be at random or selection of the same channel should be no more frequent than once every 1 second. For UL multi-channel access procedures in NR-U, UE is either scheduled to start transmissions on channels initiated by gNB or perform uplink transmissions on configured resources, and it will only perform single-channel sensing procedure (Type 1 or Type 2) on a certain channel as indicated by gNB.

Considerations for SL-U

For SL-U, Type 1 LBT is necessary to be performed if a UE wants to initiate a COT by itself. Type 2 LBT has benefits of deterministic sensing duration and can be performed for transmissions within a shared COT. The two types of channel access procedures defined in NR-U are applicable to SL-U and can be reused.

For Type 1 LBT, gNB and UE use different CAPC tables for channel access. It can be further studied that which table should be reused for SL-U, e.g. can both tables be considered under certain conditions, or only one of the tables can be used.

Multi-channel transmission is important to meet WID target for high data rate transmission and needs to be supported for SL-U. Unlike in NR-U, sidelink UE has to be able to perform multi-channel access by itself rather than being scheduled/indicated. Thus, both NR-U multi-channel access Type A and Type B can be reused for SL-U.

Proposal 1: Channel access procedure Type 1, Type 2A, Type 2B and Type 2C, as well as multi-channel access Type A and Type B defined in NR-U are supported for SL-U.

- Further study on using which CAPC table(s) for SL-U

- COT sharing

Background: COT sharing in NR-U

NR-U supports COT sharing between UEs and gNB, i.e., a gNB can share a COT initiated by a UE, and a UE can share a COT initiated by a gNB. When a gNB shares a COT initiated by a UE, the gNB may transmit a transmission that follows UL transmission after performing Type 2 LBT. When a UE shares a COT initiated by a gNB, once gNB has informed UE the resources for UL transmission within the COT, and UE will perform Type 2 LBT ahead of UL transmission. In general, such COT sharing is beneficial for reducing channel access delay and makes utilization of a COT more efficient, thus improving system performance.

Observation 1: NR-U supports COT sharing between gNB and UE, which reduces channel access delay and improves resource utilization.

COT sharing in SL-U

The above advantages of NR-U COT sharing also apply to sidelink scenario. In NR-U, a gNB can share a COT and indicate the COT structure to a number of UEs. Similarly, a sidelink UE, after initiating a COT, can also share the COT and indicate the COT structure to a number of sidelink UEs, thus enjoying the benefits of reduced channel access delay and improved resource utilization.

Figure 1 below gives an example where a COT is shared among UE1, UE2 and UE3. In Figure 1, UE1 initiates a COT after performing Type 1 LBT, then UE1 can share the COT to UE2 and UE3 and inform them by sending COT indication. After UE2 and UE3 receive such COT indication, they can perform Type 2 LBT before transmissions at indicated resources within this shared COT.

Figure 1. Example of COT sharing for SL-U

For SL COT sharing, at least following issues can be further studied:

- Conditions for a UE to share a COT, e.g., under what conditions a UE who initiates a COT needs to share the COT to other UEs

- Conditions for a UE to use a shared COT, e.g., under what conditions a UE who receives a COT indication can use the shared COT

- COT structure, e.g., it’s better to avoid large gap between two consecutive transmissions within a COT, otherwise other UE’s LBT may be successful and access the channel, and cause the current COT on longer available

- Signaling design of COT indication, e.g., contents, container, etc.

Proposal 2: COT sharing is supported for SL-U, further study details including the following

- Conditions for a UE to share a COT

- Conditions for a UE to use a shared COT

- COT structure

- Signaling design of COT indication

- LBE and FBE

NR-U supports two types of LBT mechanisms: LBE (Load Based Equipment) and FBE (Frame Based Equipment).

Devices using LBE can sense a channel as long as they do not transmit. Sensing duration and COT size may depend on system load. There is a complete and effective collision avoidance mechanism in LBE, so that it can compete fairly with other technologies (e.g., Wi-Fi).

FBE mechanism can be used in environments where the absence of other technologies is guaranteed e.g., by level of regulations, private premises policies, etc (see TS 37.213 Clause 4.3).

SL-U deployment scenarios can be diverse, e.g., SL-U can be deployed at home or public area where other technologies also exist, or SL-U can be deployed in a factory or industrial park where the absence of other technologies can be guaranteed, etc. Therefore, both LBE and FBE modes can be supported for SL-U.

Proposal 3: Both LBE (Load Based Equipment) and FBE (Frame Based Equipment) are supported for SL-U.

- Short Control Signaling transmissions

According to regulation ETSI EN 301 893, Short Control Signaling transmissions is defined as follows:

(below is copied from regulation ETSI EN 301 893 V2.1.1) 4.2.7.3.3 Short Control Signalling Transmissions (FBE and LBE) 4.2.7.3.3.1 General Frame Based Equipment and Load Based Equipment are allowed to have Short Control Signalling Transmissions on the Operating Channel providing these transmissions comply with the requirements in clause 4.2.7.3.3. It is not required for adaptive equipment to implement Short Control Signalling Transmissions. 4.2.7.3.3.2 Definition Short Control Signalling Transmissions are transmissions used by the equipment to send management and control frames without sensing the channel for the presence of other signals. 4.2.7.3.3.3 Limits The use of SCS transmissions is constrained as follows: - within an observation period of 50 ms, the number of SCS transmissions by the equipment shall be equal to or less than 50; and - the total duration of the equipment's SCS transmissions shall be less than 2500 µs within said observation period.

Short Control Signaling transmission has benefits of no need to perform sensing before transmission, thus reducing channel access delay. However, since a transmitter does not sense the channel before transmission, it does not know the existence of other transmissions, thus collision may happen and cause transmission failure. Short Control Signaling transmission also has limitations on transmission frequency and duration as shown above. Considering such limitations, PSFCH and S-SSB might be suitable to be transmitted as Short Control Signaling and can be further studied.

To improve transmission reliability, PSFCH is introduced in NR-V and carries SL HARQ-ACK, which is a key feature of NR-V. PSFCH itself has no acknowledgment, so that transmission of PSFCH should have high reliability. Because if PSFCH is not correctly received by the transmitter, there will be unnecessary retransmissions which consumes network resource and increase interference to other UEs. To achieve such high reliability, in R16 design, PSFCH is a dedicated system-level resource which is not shared with other SL PHY channel/signals.

If PSFCH is transmitted as Short Control Signaling, collision between PSFCH and other transmissions cannot be avoided as mentioned above, which may cause unnecessary retransmissions, increased interference, etc. Thus, reliability needs to be further studied if PSFCH is transmitted as Short Control Signaling.

Observation 2: If PSFCH is transmitted as Short Control Signaling, collision between PSFCH and other transmissions cannot be avoided, which may cause unnecessary retransmissions, increased interference, etc.

In R16 NR-V design, periodicity of S-SSB transmission is 160ms, and S-SSB can be transmitted multiple times within each period depending configuration and numerology. From transmission frequency and duration perspective, S-SSB might be possible to be transmitted as Short Control Signaling. Meanwhile, it is expected that there might be some designs on S-SSB due to unlicensed spectrum requirements such as LBT, which will be further discussed under agenda item “9.4.1.2  Physical channel design framework”. Such potential designs may impact the transmission frequency and duration of S-SSB, thus impacting the feasibility of transmitting S-SSB as Short Control Signaling.

In summary, whether PSFCH and S-SSB can be transmitted as Short Control Signaling can be further studied while considering aspects including feasibility, reliability, and benefits.

Proposal 4: Further study whether PSFCH and S-SSB can be transmitted as Short Control Signaling considering aspects including feasibility, reliability, and benefits.

- Resource allocation

  - Mode 1

In existing NR SL design framework, there are three types of resource allocation schemes in Mode 1:

- DG (Dynamic Grant): the gNB dynamically schedules SL transmission via DCI format 3_0.

- CG (Configured Grant) type 1: SL transmission resources are configured by RRC only.

- CG type 2: SL transmission resources are configured by RRC and activated by DCI format 3_0.

For Mode 1, one DG/CG can indicate at most 3 resources in different slot locations in the form of TRIV/FRIV. After gNB configures resources to different UEs via Mode 1, each UE performs LBT individually. If UEs do not share the COT with each other, there might be LBT blocking issue.

For example, as shown in Figure 2, assume gNB allocates orange and green resources to UE1 and UE2 via Mode 1, respectively. Then, UE1 and UE2 need to perform LBT before each allocated resource. Assume UE1 successfully transmitted on the 1st Tx resource, then such transmission will possibly impact UE2’s LBT procedure before UE2’s 1st TX resource and cause LBT failure.

Such LBT blocking issue may cause some UEs unable to use the allocated resources by gNB via Mode 1, thus leading to low resource utilization and decreasing system performance. It can be further studied how to solve such LBT blocking issue. One potential way is COT sharing, i.e., if UEs can share COT with each other, then more UEs can access the channel.

Observation 3: For Mode 1, if UEs do not share COT with each other, there might be LBT blocking issue, i.e., some UE’s transmissions may cause other UE’s LBT failure and unable to transmit on scheduled/configured resources, resulting in low resource utilization and decreased system performance.

Proposal 5: For mode 1, further study how to address LBT blocking issue such that the granted resources to different UEs can be efficiently used.

Figure 2 Illustration of LBT blocking

  - Mode 2

In current sidelink Mode 2, transmission resource is selected within a resource selection window based on sensing results within sensing window, which is an effective interference management mechanism for distributed systems and can ensure QoS of the system. In Mode 2, by sensing and resource exclusion procedure, resource selection can be more effective and resource conflict can be alleviated to a large extent.

Sidelink Mode 2 on unlicensed band is still a distributed system, so collision and interference control should be taken into account for SL-U design. Thus, Mode 2 reservation-based resource selection should be reused to effectively alleviate resource conflict.

Proposal 6: Reservation-based resource selection is supported for SL-U Mode 2.

In unlicensed spectrum, reserved resources can only be used once LBT is successful. The relationship between reservation-based resource selection and LBT can be further studied.

For example, a straightforward way is UE selects resources first, then performs LBT. As shown in Figure 3 below, assume UE1 firstly selects resources based on R16 Mode 2 sensing and resource exclusion procedure (assume R1, R2, R3 are selected by UE1). Then, UE1 performs Type 1 LBT before each selected resource. If LBT succeeds, UE1 can transmit on such resource.

Although such mechanism seems simple and straightforward, one major drawback is it may cause LBT blocking issue as mentioned in Observation 3 above. For example, as shown in Figure 3 below, it’s possible that other high priority UE2 and UE3 reserved R4 and R5, which are close to UE1’s selected resources R1 and R2. If UE1 transmits on R1 and R2, it may possibly impact UE2’s and UE3’s LBT procedure before R4 and R5, and cause LBT failure for UE2 and UE3, respectively.

Such LBT blocking issue may decrease performance, especially for high priority UEs. Therefore, further study is necessary on solving such issues. One potential way is COT sharing, i.e., if UE1 can share COT with UE2 and UE3, then all of them can access the channel. Another way might be deferring channel access, e.g., UE1 can defer channel access by holding on LBT counter N or choose not to transmit immediately when counter N=0 as specified in TS 37.213, which allows UE2 and UE3 to access firstly.

Proposal 7: For mode 2, further study the relationship between reservation-based resource selection and LBT, including how to address LBT blocking issue such that the reserved resources of different UEs can be efficiently used.

Figure 3 Illustration of resource selection and LBT procedures for resource allocation mode 2

- Evaluation methodology

  - Background

The operation of Rel-18 sidelink over unlicensed spectrum is motivated to support high data rate transmission, such as interactive service (e.g. AR, VR, cloud gaming), for commercial usage, which is typically deployed in indoor scenario. However, neither NR-U considers UE-to-UE communication, nor Rel-16/17 NR sidelink consider indoor scenario. The need to introduce new UE deployment and UE dropping mode is necessary. In addition, SL-U transmission may be applied to various service types, such as XR, video, file transmission, etc.

In TR 38.889, evaluation methodologies for NR-U have been extensively studied, including indoor and outdoor scenarios. To reduce effort on evaluation work of SL-U, the basic evaluation methodology of the NR-U and the simulation assumptions in TR 38.889 can be reused as much as possible.

Proposal 8: The evaluation methodology of NR-U and NR sidelink should be reused as much as possible and with adjustment for sidelink unlicensed operation.

  - UE dropping model

For a typical indoor scenario, there could be a central controller in one office, and the central controller can communicate with multiple smart devices over SL-U, such as cloud gaming device, smart HD TV, wearable devices, intelligent home devices, etc. All these devices and controller can comprise a cluster which is shown in Figure 4.

Figure 4. Example of indoor scenario for SL-U

In this cluster, the center device can communicate with all surrounding devices, and can carry different traffics. Surrounding devices can only communicate with the central device with only one traffic.

About the indoor scenario, it can reuse the office layout as defined in TR 38.889, of which the size is 80m * 120m. It can be understood that, for an indoor scenario, there can be multiple rooms/blocks, and in each of them, a smart phone can connect to a number of other smart devices. So a more comprehensive layout with multiple clusters is illustrated in Figure 5. Due to limited size of the room, edge devices can be within the communication range of the central equipment, a maximum distance Rmax is set. Considering flexibility and generality of UE dropping modelling to adapt to different scenarios, a minimum distance Rmin can be added as well, where Rmax>Rmin≥0. For UE dropping, following steps are performed:

Step 1: A layout with size of 80m * 120m.

Step 2: N clusters are dropped within the layout, and each consists of M UEs:

- The cluster is formed as a circle, with a central point and radius Rmax, the circle has to be within the layout;

- N clusters are not overlapped.

Step 3: For each cluster, a UE is dropped at the central point, and remaining M-1 UEs are further randomly dropped within the range between radius Rmax  and radius Rmin with respect to the central point.

Considering practical deployment scenarios, the size of a single cluster, represented by (Rmax,Rmin), can be considered in the table as follow, other values are not precluded.

Table 2. UE deployment options.

| Parameters | Option 1 | Option 2 |
| --- | --- | --- |
| (Rmax,Rmin) | (15m, 5m) | (10m, 1m) |
| N: number of clusters | 6 | 12 |
| M: number of UEs within a cluster | 5 | 5 |

For example, 15m, 10m, N=6, M=5 are considered as in Figure 5.

Figure 5. Indoor UE dropping model

Proposal 9: Support a new UE dropping model for SL-U based on the office layout defined in TR 38.889 as follows

Step 1: A layout with size of 80m * 120m.

Step 2: N clusters are dropped within the layout, and each consists of M UEs:

- The cluster is formed as a circle, with a central point and radius R_max, the circle has to be within the layout;

- N clusters are not overlapped.

Step 3: For each cluster, a UE is dropped at the central point, and remaining M-1 UEs are further randomly dropped within the range between radius R_max and radius R_min with respect to the central point.

Proposal 10: For UE dropping model proposed in proposal 9, at least one of the options should be supported.

| Parameters | Option 1 | Option 2 |
| --- | --- | --- |
| (Rmax,Rmin) | (15m, 5m) | (10m, 1m) |
| N: number of clusters | 6 | 12 |
| M: number of UEs within a cluster | 5 | 5 |

  - Traffic model

As already mentioned in the WID, a key requirement of SL is to increase sidelink data rate and expand the applicability of sidelink to commercial use cases, and in typical indoor scenario, a central controller can communicate with multiple smart devices over SL-U, corresponding to multiple types of applications need to be transmitted simultaneously. So we identify the following traffic models for evaluations.

    - FTP

FTP is a typical traffic model for evaluation of unlicensed technologies, such as Wi-Fi, LAA, NR-U, and can be also included for SL-U. For FTP Traffic Mode 3 (FTP3), the occurrence of such aperiodic event is modeled with a Poisson process, with transmission time of a packet is counted from the time instance it arrives in the queue. Given that FTP3 is the only traffic model adopted in NR-U for evaluation, and we can consider to reuse the FTP3 in SL-U as well. The detailed model can be found in TR 36.889.

Proposal 11: FTP3 as defined in TR 36.889 should be reused for SL-U.

- FTP Model 3: Based on FTP model 2 as in TR 36.814 with the exception that packets for the same UE arrive according to a Poisson process with arrival rate 𝜆 and the transmission time of a packet is counted from the time instance it arrives in the queue.

- Poisson process with arrival rate 𝜆 = 1 or 𝜆 = 0.5

- FTP model file size: 0.5 Mbytes.

    - XR

Cloud gaming in XR, a new type of service, is becoming an important commercial application scenario. To expand commercial using of sidelink over unlicensed, many companies also mentioned that Rel-18 SL-U should consider to deploy cloud gaming service during the WI discussion. Thus, to evaluate performance of cloud gaming service transmitted over SL-U, the traffic model specified for cloud gaming in TR 38.838 should be used and typical periodic traffic model can be applied to sidelink transmission. Specifically, for a pair of UEs communicating with each other, one UE will transmit periodic data frames, and the other UE will response pose/control data. The cloud gaming traffic model and the generic pose/control stream traffic model are given as follows (copied from TR 38.838).

Table 3: Statistical parameters for single stream cloud gaming traffic model

| Parameters | unit | Baseline values for evaluation |
| --- | --- | --- |
| data rate: R | Mbps | 8,30 |
| frame generation rate: F | fps or Hz | 60 |
| PDB | ms | 15 |

Table 4: Statistical parameters for the pose/control traffic

| Parameters | unit | Baseline values for evaluation |
| --- | --- | --- |
| Periodicity | ms | 4 |
| Jitter | ms | No jitter |
| Packet size | byte | 100 |
| PDB | ms | 10 |
| Packet Success Rate X | % | 99 |

Thus, we propose the following:

Proposal 12: Cloud gaming traffic model defined in TR 38.838 can be reused for SL-U.

For UEs in the simulation, based on typical usage scenarios, there are multiple applications need to be transmitted by one central controller. Consider the dropping model of cluster, there will be one cloud gaming device and multiple other smart home devices. For cloud gaming device, the central controller will transmit game video frame to the device and the device will feedback pose control information. And to other smart home devices, the central controller will send FTP packet to the smart home devices and they will feedback the same FTP packets. Therefore, in the simulation of SL-U, mixed XR and FTP3 traffics should be supported. To simplify the design, one possible configuration is cloud gaming : FTP = 1 : 3, and other ratios are not precluded.

Proposal 13: The traffic mode of mixed XR and FTP3 is supported for Rel-18 SL-U evaluation. The ratio of Cloud gaming over FTP could be 1:3.

Simulation bandwidth of sidelink in both Rel-16 and Rel-17 already supports 40MHz, and 80MHz bandwidth in simulation is supported in NR-U, which has been specified in TR 38.338. SL-U requires large bandwidth, so it is recommended to support simulation bandwidth of 40MHz and 80MHz.

Proposal 14: Both 40MHz and 80MHz are supported as simulation bandwidth.

  - Channel model

Given UE types and channel conditions for indoor scenarios in SL-U is similar to that in NR-U, in order to reduce RAN1 workload, channel model of NR-U, i.e., NR InH Mixed Office model, can be reuse, which is defined in TR 38.901.

Proposal 15: Reuse channel model and pathloss model as defined in TR 38.901.

  - UE Antenna model

Because user devices in SL-U simulation is the same as NR-U, UE antenna model in NR-U can be directly reused in SL-U to avoid repeated definition, which is defined in TR 37.885.

Proposal 16: Reuse antenna model as defined in TR 37.885.

- Baseline Tx/Rx: (M, N, P, Mg, Ng) = (1, 1, 2, 1, 1), dH = dV = 0.5 λ

  - Performance metric

Considering real deployment, SL-U can involve multiple types of applications simultaneously, including real-time and non-real time. Real-time applications such as XR, require highly reliable data transmissions with low latency bound so that received information is not out-dated. On the other hand, non-real time applications require high throughput but relatively loose reliability requirements.

    - PRR

PRR is a performance metric that is widely used for sidelink evaluation in both Rel-16 and Rel-17. In Rel-16/17, UEs operate in the ITS or shared licensed band, and also the resource selection procedure ensures that transmission UE is capable to choose a resource within selection window to transmit the packet without exceeding its PDB.

However, this assumption may not be valid for transmission over unlicensed spectrum, where there is no guarantee that UE can always access the channel successfully and make the transmission within its PDB. Thus packet failure due to exceeding the given PDB has to be considered, and the PRR definition should be updated for SL-U accordingly. In summary, some clarification is needed for the definition of PRR (including both type 1 and type 2) in TR 37.885, i.e., for a packet of which delay exceeding a given PDB, the packet is regarded as transmission failure.

Proposal 17: Support using Rel-16 PRR for Rel-18 SL-U with further clarification that for a packet of which delay exceeding a given PDB, the packet is regarded as transmission failure.

    - UPT and Latency

Besides PRR, performance metrics to indicate channel throughout and overall delay are also needed. User Perceived Throughput (UPT) and latency (From packet arrival in devices (eNB, AP, UE, STA) MAC buffer to successful transmission (including retransmission) of packet) can be considered, which have been used in NR-U and defined in TR 36.889.

UPT and latency are defined as follow,

(below is copied from TR 36.889) … - User perceived throughput (UPT) - UPT CDF - File throughput is calculated per file - Unfinished files should be incorporated in the UPT calculation. - The number of served bits (possibly zero) of an unfinished file by the end of the simulation is divided by the served time (simulation end time – file arrival time). - User throughput is the average of all its file throughputs - Latency (From packet arrival in devices (eNB, AP, UE, STA) MAC buffer to successful transmission (including retransmission) of packet)

Proposal 18: Reuse performance metrics UPT and latency in NR-U, as defined in TR 36.889, for SL-U evaluations.

Based on discussions in Section 7, simulation parameters are summarized in the following table:

Table 5  Summary of simulation assumptions for indoor scenario

| Simulation scenario | Indoor sub 7 GHz indoor scenario, defined in TS38.889. |
| --- | --- |
| Carrier Frequency | 5GHz |
| UE Tx Power | 18dBm |
| UE dropping model | N clusters are dropped within the layout, and each consists of M UEs, except for the central UE, other UEs are distributed between a minimum distance Rmin, and a maximum distance Rmax. The corresponding parameters are suggested as follows, Parameters; Option 1; Option 2. (Rmax,Rmin); (15m, 5m); (10m, 1m). N: number of clusters; 6; 12. M: number of UEs within a cluster; 5; 5. |
| Traffic model | Reuse cloud gaming traffic model defined in TR 38.838. Reuse FTP3 traffic model defined in TR 36.889. - Poisson process with arrival rate 𝜆 = 1 or 𝜆 = 0.5 |
| Simulation Bandwidth | Both 40MHz and 80MHz are supported |
| Channel Model | Reuse InH Mixed Office model defined in TR 38.901 |
| UE antenna Model | Baseline Tx/Rx: (M, N, P, Mg, Ng) = (1, 1, 2, 1, 1), dH = dV = 0.5 λ |
| Performance metric | Support using Rel-16 PRR for Rel-18 SL-U with further clarification that for a packet of which delay exceeding a given PDB, the packet is regarded as transmission failure. Reuse performance metrics UPT and latency defined in TR36.889. |

- Conclusions

In this contribution, channel access procedures including COT sharing, LBE and FBE for NR-U and resource allocation Mode 1 and Mode 2 for NR sidelink are supported for SL-U. Further details on feasibility and conditions need to be studied including COT sharing conditions, Short Control Signaling applications and resource reservation mechanisms. Evaluation methodology for sidelink unlicensed operation is proposed on UE deployment, UE dropping model and traffic model. We have following observations and proposals:

Observation 1: NR-U supports COT sharing between gNB and UE, which reduces channel access delay and improves resource utilization.

Observation 2: If PSFCH is transmitted as Short Control Signaling, collision between PSFCH and other transmissions cannot be avoided, which may cause unnecessary retransmissions, increased interference, etc.

Observation 3: For Mode 1, if UEs does not share COT with each other, there might be LBT blocking issue, i.e., some UE’s transmissions may cause other UE’s LBT failure and unable to transmit on scheduled/configured resources, resulting in low resource utilization and decreased system performance.

Proposal 1: Channel access procedure Type 1, Type 2A, Type 2B and Type 2C, as well as multi-channel access Type A and Type B defined in NR-U are supported for SL-U.

- Further study on using which CAPC table(s) for SL-U

Proposal 2: COT sharing is supported for SL-U, further study details including the following

- Conditions for a UE to share a COT

- Conditions for a UE to use a shared COT

- COT structure

- Signaling design of COT indication

Proposal 3: Both LBE (Load Based Equipment) and FBE (Frame Based Equipment) are supported for SL-U.

Proposal 4: Further study whether PSFCH and S-SSB can be transmitted as Short Control Signaling considering aspects including feasibility, reliability, and benefits.

Proposal 5: For mode 1, further study how to address LBT blocking issue such that the granted resources to different UEs can be efficiently used.

Proposal 6: Reservation-based resource selection is supported for SL-U Mode 2.

Proposal 7: For mode 2, further study the relationship between reservation-based resource selection and LBT, including how to address LBT blocking issue such that the reserved resources of different UEs can be efficiently used.

Proposal 8: The evaluation methodology of NR-U and NR sidelink should be reused as much as possible and with adjustment for sidelink unlicensed operation.

Proposal 9: Support a new UE dropping model for SL-U based on the office layout defined in TR 38.889 as follows

Step 1: A layout with size of 80m * 120m.

Step 2: N clusters are dropped within the layout, and each consists of M UEs:

- The cluster is formed as a circle, with a central point and radius R_max, the circle has to be within the layout;

- N clusters are not overlapped.

Step 3: For each cluster, a UE is dropped at the central point, and remaining M-1 UEs are further randomly dropped within the range between radius R_max  and radius R_min with respect to the central point.

Proposal 10: For UE dropping model proposed in proposal 9, at least one of the options should be supported.

| Parameters | Option 1 | Option 2 |
| --- | --- | --- |
| (Rmax,Rmin) | (15m, 5m) | (10m, 1m) |
| N: number of clusters | 6 | 12 |
| M: number of UEs within a cluster | 5 | 5 |

Proposal 11: FTP3 as defined in TR 36.889 should be reused for SL-U.

- FTP Model 3: Based on FTP model 2 as in TR 36.814 with the exception that packets for the same UE arrive according to a Poisson process with arrival rate 𝜆 and the transmission time of a packet is counted from the time instance it arrives in the queue.

- Poisson process with arrival rate 𝜆 = 1 or 𝜆 = 0.5

- FTP model file size: 0.5 Mbytes.

Proposal 12: Cloud gaming traffic model defined in TR 38.838 can be reused for SL-U.

Proposal 13: The traffic mode of mixed XR and FTP3 is supported for Rel-18 SL-U evaluation. The ratio of Cloud gaming over FTP could be 1:3.

Proposal 14: Both 40MHz and 80MHz are supported as simulation bandwidth.

Proposal 15: Reuse channel model and pathloss model as defined in TR 38.901.

Proposal 16: Reuse antenna model as defined in TR 37.885.

- Baseline Tx/Rx: (M, N, P, Mg, Ng) = (1, 1, 2, 1, 1), dH = dV = 0.5 λ

Proposal 17: Support using Rel-16 PRR for Rel-18 SL-U with further clarification that for a packet of which delay exceeding a given PDB, the packet is regarded as transmission failure.

Proposal 18: Reuse performance metrics UPT and latency in NR-U, as defined in TR 36.889, for SL-U evaluations.

References

- RP-213678, “New WID on NR sidelink evolution”, OPPO, LG Electronics, RAN#94e, December 6 - 17, 2021.

- ETSI EN 301 893, Harmonized European Standard, “Broadband Radio Access Networks (BRAN); 5 GHz high performance RLAN”.