3GPP TSG-RAN WG1 Meeting #109-e 	R1-2203148

e-Meeting, May 9 – May 20, 2022

Agenda Item:	9.4.2

Source:	Huawei, HiSilicon

Title:	Co-channel coexistence for LTE sidelink and NR sidelink

Document for:	Discussion and Decision

- Introduction

An objective for NR sidelink evolution has been approved in RAN#94-e [1], the scope of which includes:

- Study and specify, if necessary, mechanism(s) for co-channel coexistence for LTE sidelink and NR sidelink including performance, necessity, feasibility, and potential specification impact if any [RAN1, RAN2, RAN4]

- Reuse the in-device coexistence framework defined in Rel-16 as much as possible

LTE V2X is designed to operate in band 47 only [2], and NR V2X is designed to operate in more bands including n14, n38, n47, and n79 [3]. Band 47 and band n47 share same bandwidth range 
(i.e. 5855 MHz ~ 5925 MHz). Thus, it’s possible that LTE sidelink and NR sidelink coexist on the same channel. In this contribution, the co-channel coexistence issues between LTE sidelink and NR sidelink is discussed, including potential solutions for co-channel coexistence, and associated simulation results.

- Discussion on co-channel co-existence for LTE sidelink and NR sidelink

  - Potential solutions for co-channel coexistence

Supporting co-channel coexistence of LTE sidelink and NR sidelink is particularly requested by 5GAA. The motivation is with the number of vehicles supporting NR-V growing gradually and more LTE-V equipment retiring from market, the spectrum allocated for LTE-V would become more vacant. By utilizing co-channel coexistence of LTE sidelink and NR sidelink, the released resources could be used by NR-V naturally.

In Rel-16, the in-device coexistence of NR-V and LTE-V are supported in separate resources pools which are (pre-)configured in either TDM or FDM, as shown in Figure 1. NR-V and LTE-V traffic can share the same channel and transmit on separate resource pool.

Figure 1. Resource pools for LTE sidelink and NR sidelink are TDMed or FDMed in Rel-16

To support Rel-18 coexistence, two potential solutions as below have been identified during the discussion of Rel-18 workshop in RAN. Details are discussed in section 2.2.1 and section 2.2.2.

- Alt 1: Resource pool separation between the two RATs

- Alt 2: Dynamic resource sharing using overlapping resource pools between the two RATs.

    - Co-channel coexistence solution Alt 1 (resource pool separation)

For co-channel coexistence Alt 1, part of resources in LTE-V resource pool, are re-configured to the
NR-V resource pool by updating the resource pool configuration of both LTE sidelink and NR sidelink. Taking the Figure 2 (coexistence via TDM) and Figure 3 (coexistence via FDM) as examples, the green resources, which were originally (pre-)configured to LTE-V, are re-configured to NR-V later. Then, the resource pool for NR-V consists of resources marked in both yellow and green.

Figure 2 For coexistence via TDM, resources originally (pre-)configured to LTE-V can be used by NR-V transmission

Figure 3 For coexistence operating via FDM, resources originally (pre-)configured to LTE-V can be used by NR-V transmission

To achieve the purpose of re-allocating the resources between LTE-V sidelink and NR-V sidelink, both RRC configuration and pre-configuration can be updated.

The IE of SL-ResourcePool is used to configure the parameters of NR sidelink resource pool. For the vehicles in coverage and RRC_CONNECTED state, the IE can be updated by SL-ConfigDedicatedNR within RRCReconfiguration[4]. For the UEs in coverage but in RRC_INACTIVE or RRC_IDLE state, it can be further updated by SIB12 within SystemInformation [4]. For LTE-V, SL-CommResourcePoolV2X is the corresponding IE used to configure the parameters of LTE-V resource pool. For the vehicles in coverage and RRC_CONNECTED state, the IE can be updated by SL-CommConfig within RRCConnectionReconfiguration[5]. For the UEs in coverage but in RRC_IDLE state, it can be further updated by SystemInformationBlockType21 within SystemInformation [5].

Pre-configuration can be also updated by over-the-air signaling. In 5GS (5G System), the parameters for V2X communications over PC5 are specified in Clause 5.1 in TS 23.287 [6], where the V2X parameters can be re-configured for both ME (Mobile Equipment) and UICC (Universal Integrated Circuit Card). For the parameters within ME, they cannot be modified later. However, the pre-configurations in UICC could be updated through the signaling received from the operator and those information in UICC are prioritized than those within ME [6]. Thus, the pre-configuration of resource pools in NR sidelink and LTE-V can be updated.

For the cases that the associated parameters cannot be updated over-the-air, for example, due to absence of Uu module and cannot reach the network, or out-of-coverage, updates of the configurations via a wired manner is also feasible, e.g., the vehicle owner can visit the vehicle store to update the configurations locally.

Observation 1: Current specification allows resource pool separation between LTE sidelink and NR sidelink, which is a simple and effective way to support co-channel co-existence of the two RATs.

Proposal 1: RAN1 concludes that current specifications already support co-channel co-existence for LTE sidelink and NR sidelink via resource pool separation between the two RATs.

    - Co-channel coexistence solution Alt 2 (dynamic resource sharing)

For co-channel coexistence Alt 2, shown as Figure 4, some resources configured to LTE-V pools are dynamically used by NR-V. Since LTE-V based products are already commercially deployed in some countries/regions, such kind of resource sharing shall not impact the LTE-V performance, and the configuration and behavior of LTE-V shall remain unchanged. Note that Rel-16 NR-V2X is not designed to be backward compatible to LTE-V, and there might be large amount of specification workload if design of Alt 2 is not constrained.

Figure 4. Resources belong to LTE-V are dynamically used by NR-V

Under Alt 2, there are a series of issues as follows:

- NR-V PSFCH occasion

The impact to NR-V HARQ feedback procedure is substantial. The resources dynamically shared from LTE-V resource pool cannot be allocated with any PSFCH occasion. Otherwise, resource collision may occur between LTE-V and NR-V. For LTE-V, reliability of decoding is affected on collision resources. For NR-V, a UE could be unable to decode HARQ information because it may overlap with LTE-V transmission. Given that Alt 2 is targeted for NR-V and LTE-V sharing the same time-frequency resource, thus the simplest and feasible solution is that no PSFCH resource is pre-configured in such shared resource.

Observation 2: For Alt2 (dynamic resource sharing), collision may occur when LTE-V PSCCH/PSSCH and NR-V PSFCH are transmitted in the shared resource.

- Issues due to different SCS configured for NR-V and LTE-V

LTE-V only supports 15 kHz SCS, and NR-V supports flexible numerologies with mandatory support for 30 kHz SCS. If the two RATs sharing in the same resource pool have different numerologies, e.g. 15 kHz SCS for LTE-V and 30 kHz SCS of NR-V, AGC symbols of two adjacent NR-V slots will locate at the start and middle of an LTE-V subframe. Thus, AGC of LTE-V is impacted if NR-V transmissions in the first half and second half subframe of LTE-V change sharply. In such case, the adjustment of power implemented by LTE-V UE at the start of a subframe would be no longer effective and the data in the second half subframe cannot be decoded correctly.

In addition, Alt 2 with different SCS aggravates the half-duplex issue of NR-V UE. Because if a NR-V UE is receiving a LTE-V transmission in a subframe, then this NR-V UE cannot transmit NR-V traffic in any of these two NR-V slots.

One possible direction to avoid above issues is that NR-V is assumed to be deployed with 15k Hz SCS in situation of co-channel coexistence with LTE-V.

Observation 3: For Alt2 (dynamic resource sharing), different SCS used for NR-V and LTE-V lead to AGC issue and half-duplex issue.

- Sensing and resource reservation:

Given that NR-V and LTE-V are designed differently, it is not possible to re-design a new set of NR-V channels/signals to be transmitted in a LTE-V manner such that LTE-V can detect presence of NR-V. Thus, Rel-16 in-device coexistence framework is the only way forward, as stated in the WID. For a Rel-18 
NR-V to share with LTE-V in the same resource pool, NR-V has to be able to detect reservation of 
LTE-V via its internal LTE-V module, and combine sensing results of LTE-V module and NR-V sensing results of NR-V module. In this way, resource selection of Rel-18 NR-V can avoid potential resource collisions to LTE-V.

Observation 4: For Alt 2 (dynamic resource sharing), Rel-16 in-device coexistence framework shall be used to avoid large amount of specification workload in terms of re-designing NR-V.

In addition, based on our evaluation in Section 2.2, we observe that there is no clear performance difference between the two alternatives. If Alt 2 has to be specified, it should be designed with minimum specification impact.

Proposal 2: NR-V and LTE-V co-channel co-existence via dynamic resource sharing, if deemed necessary, should be designed as simply as possible with all following constraints:

- 15kHz SCS for NR-V

- No PSFCH resource is pre-configured in the resources shared with LTE-V

- Under Rel-16 in-device coexistence framework only.

  - Simulation results

In order to compare the performance of the two alternatives for co-channel coexistence of LTE sidelink and NR sidelink, different RP configuration and traffic model should be considered. For simplification, we reuse the parameters of Rel-17 NR sidelink power consumption model with some necessary modifications as shown in Table 1 in Appendix A.

In this section, the simulation results of PRR for the two alternatives are provided. Based on the simulation results of the following 3 cases, there is no clear performance difference between Alt 1 and Alt 2 solutions.

- Case1: FDMed resource pools for NR-V and LTE-V (Alt 1 with FDM configuration)

  - Assumption: RP configuration matches traffic density.

- Case 2: TDMed resource pools for NR-V and LTE-V (Alt 1 with TDM configuration)

  - Assumption: RP configuration matches traffic density

- Case 3: Mixed NR-V and LTE-V in a shared resource pool for NR-V and LTE-V (Alt 2)

Low traffic density for LTE-V and medium traffic density for NR-V

To begin with, we simulate the cases when LTE-V UEs use the low traffic density and NR-V UEs use the medium traffic density. This is a typical scenario when LTE-V is retiring from the network. For case 1, LTE-V resource pool consists of 2-subCHs while NR-V resource pool consists of 3-subCHs in frequency domain. For case 2, the ratio of LTE-V resource pool and NR-V resource pool in time domain is 1: 2. The simulation results under urban and periodic/aperiodic traffic are given in Figure 5 and Figure 6. For mixed NR-V and LTE-V case (i.e. Case 3), where all the UEs share the same resource pool, where the NR-V UE performs sensing and resource exclusion based on both reservation from LTE-V via its internal LTE-V module and from NR-V via its internal NR-V module, i.e. a NR-V2X UE can avoid resource reservation potentially collide with LTE-V.

It can be observed that there is no clear performance difference among the three cases in both periodic/aperiodic traffic.

Figure 5: NR-V/LTE-V PRR, with low traffic density for LTE-V and medium traffic density for NR-V in aperiodic traffic

Figure 6: NR-V/LTE-V PRR with low traffic density for LTE-V and medium traffic density for NR-V in periodic traffic

Medium traffic density for LTE-V and low traffic density for NR-V

In addition, we also simulate another traffic pattern where LTE-V UEs use the medium traffic density and NR-V UEs use the low traffic density. Under this situation. For case 1, LTE-V resource pool consists of 3-subCHs while NR-V resource pool consists of 2-subCHs in frequency domain. For case 2, the ratio of LTE-V resource pool and NR-V resource pool in time domain is 2:1. The simulation results under urban and periodic/aperiodic traffic are given in Figure 7 and Figure 8. Similar with the simulation results above, it can be observed that all the three cases share similar performances.

Figure 7: NR-V/LTE-V PRR with medium traffic density for LTE-V and low traffic density for NR-V in aperiodic traffic

Figure 8: NR-V/LTE-V PRR with medium traffic density for LTE-V and low traffic density for NR-V in aperiodic traffic

Based on all the simulation results above, there is no clear performance difference between Alt 1 and Alt 2 solutions.

Observation 5: Based on simulation results, Alt 2 (dynamic resource sharing) has no obvious PRR performance gain compared with Alt 1 (resource pool separation).

- Conclusions

In this contribution, we discuss the co-channel coexistence issues between LTE sidelink and NR sidelink in Rel-18 sidelink evolution. Two potential solutions are analyzed in our paper. For solution Alt1 (resource pool separation), it is simple and feasible with no specification impact. For solution Alt2 (dynamic resource sharing), behavior and performance of LTE-V should not be impacted. We have the following observations and proposals:

Observation 1: Current specification allows resource pool separation between LTE sidelink and NR sidelink, which is a simple and effective way to support co-channel co-existence of the two RATs.

Observation 2: For Alt2 (dynamic resource sharing), collision may occur when LTE-V PSCCH/PSSCH and NR-V PSFCH are transmitted in the shared resource.

Observation 3: For Alt2 (dynamic resource sharing), different SCS used for NR-V and LTE-V lead to AGC issue and half-duplex issue.

Observation 4: For Alt 2 (dynamic resource sharing), Rel-16 in-device coexistence framework shall be used to avoid large amount of specification workload in terms of re-designing NR-V.

Observation 5: Based on simulation results, Alt 2 (dynamic resource sharing) has no obvious PRR performance gain compared with Alt 1 (resource pool separation).

Proposal 1: RAN1 concludes that current specifications already support co-channel co-existence for LTE sidelink and NR sidelink via resource pool separation between the two RATs.

Proposal 2: NR-V and LTE-V co-channel co-existence via dynamic resource sharing, if deemed necessary, should be designed as simply as possible with all following constraints:

- 15kHz SCS for NR-V;

- No PSFCH resource is pre-configured in the resources shared with LTE-V;

- Under Rel-16 in-device coexistence framework only.

References

- RP-213678, “New WID on NR sidelink evolution”, OPPO, LG Electronics, RAN#94e, December 6 - 17, 2021.

- TS 36.101, “User Equipment (UE) radio transmission and reception”.

- TS 38.101-1, “User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”.

- TS 38.331, “NR; Radio Resource Control (RRC) protocol specification”.

- TS 36.331, “LTE; Radio Resource Control (RRC) protocol specification”.

- TS 23.287, “Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services”.

Appendix A: Evaluation Assumptions

Table 1: Basic simulation assumptions for NR-V and LTE-V co-channel coexistence

| Parameter | Assumption |
| --- | --- |
| Frequency | 6 GHz |
| Simulation bandwidth | 40 MHz |
| RP configuration | One RP, where 5 sub-CHs are configured, each consists of 10 PRBs. One PSSCH consists of 2 sub-CHs |
| Resource allocation | Full-sensing mode 2 For dynamic sharing (i.e. Alt2), assuming Rel-18 UE select resource based on the reservation information from both LTE-V and NR-V. |
| Synchronization | Ideal time frequency synchronization |
| Link & cast type | Direct vehicle-to-vehicle link, unicast; Interference is considered among V2V links. |
| Antenna model | TR 37.885 Option 1 |
| Deployment and UE drop | Urban-A as defined in TR 37.885 for 500 V-UEs. |
| Traffic model | For both LTE-V and NR-V: For periodic traffic: Low traffic intensity - Inter-packet arrival time: 100 ms - Packet size: Pattern of {300 bytes, 190 bytes, 190 bytes, 190 bytes, 190 bytes} with random starting point for each UE - Latency requirement: 100 ms Medium traffic intensity - Inter-packet arrival time: 50 ms - Packet size: 1200 bytes with probability of 0.2 and 800 bytes with probability of 0.8 - Latency requirement: 50 ms For aperiodic traffic: Low traffic intensity - Inter-packet arrival time: 100 ms + an exponential random variable with the mean of 100 ms - Packet size: Pattern of {300 bytes, 190 bytes, 190 bytes, 190 bytes, 190 bytes} with random starting point for each UE - Latency requirement: 100 ms Medium traffic intensity - Inter-packet arrival time: 50 ms + an exponential random variable with the mean of 50 ms - Packet size: 1200 bytes with probability of 0.2 and 800 bytes with probability of 0.8 - Latency requirement: 50 ms |
| Retransmission | Blind-retransmission |
| Synchronization | Ideal time frequency synchronization |
| Link & cast type | Direct vehicle-to-vehicle link, unicast; Interference is considered among V2V links. |
| Antenna model | TR 37.885 Option 1 |