3GPP TSG-RAN WG1 Meeting #109-e	R1-2203150

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.1.1.2

Source:	Huawei, HiSilicon

Title:	Discussion on TA enhancement for UL M-TRP transmission

Document for:	Discussion and Decision

Introduction

In RAN plenary #94-e [1], it was agreed to study TA enhancement to support two TAs for UL multi-TRP operation.

7. Study, and if justified, specify the following - Two TAs for UL multi-DCI for multi-TRP operation - Power control for UL single DCI for multi-TRP operation where unified TCI framework extension in objective 2 is assumed. For the case of simultaneous UL transmission from multiple panels, the operation will only be limited to the objective 6 scenarios.

In this paper, we discuss the mechanism of single TA based UL transmission in current specification and provide the issues and potential solutions for the TA enhancement.

Scenarios for TA enhancement

In R17, both intra-cell and inter-cell M-TRP based UL transmission are supported. In intra-cell M-TRP case (Fig. 1-a), two TRPs correspond to one serving cell, sharing the same configuration and resources of the same cell. While, in inter-cell M-TRP case (Fig. 1-b), one TRP corresponds to one or more serving cells and the other TRP corresponds to a non-serving cell with a PCI different from the serving cell(s). For intra-cell M-TRP case, one TA is maintained for a cell and shared by the two TRPs corresponding to the cell, which may cause the issue of timing asynchronization on one TRP. For inter-cell M-TRP case, one TA is maintained only for the TRP corresponding to the serving cell(s) and no TA is maintained for the other TRP. It is up to the UE implementation to determine the TA for the UL transmission to the other TRP. For example, UE can use the TA of the TRP corresponding to the serving cell. However, this introduces a similar issue as in the intra-cell M-TRP case, i.e., timing asynchronization on the other TRP. Hence, TA enhancement should be considered for both intra-cell and inter-cell M-TRP cases.

Figure1. Intra-cell and inter-cell M-TRP scenarios

Proposal 1: RAN1 should consider TA enhancement for both intra-cell and inter-cell M-TRP cases.

Critical issues on TA enhancement

Multi-TA framework

In current specifications, UE can be configured with a group of serving cells. Each serving cell is associated with a TAG. Cells associated with the same TAG can share the same TA. Each TAG is associated with a timeAlignmentTimer which is used for the maintenance of the TA corresponding to the TAG.

In intra-cell M-TRP case, the two TRPs correspond to the same serving cell for which only one TA is maintained according to the current specifications. To avoid UL timing asynchronization for either of the TRPs, an independent TA needs to be maintained for each TRP. In other words, two TAs need to be maintained for the serving cell corresponding to the two TRPs. There are two alternatives for maintaining two TAs in one serving cell: one TAG based method and two TAGs based method. In one TAG based method, one TAG is associated with the serving cell and used for the maintenance of two TAs. While in two TAGs based method, two TAGs are associated with the serving cell and each TAG is used for the maintenance of one TA. Intuitively, two TAGs based method seems to be more compatible to the current specifications in which each TAG is used for the maintenance of one TA. Moreover, with two TAGs, it is easier to differentiate the two TAs due to the existence of two TAG IDs. Nevertheless, RAN1 still needs to discuss whether one or two TAGs based method should be adopted.

In inter-cell M-TRP case, as non-serving cell is not associated with any TAG, no TA is maintained for the TRP corresponding to the non-serving cell. How to maintain a TA value for the TRP corresponding to the non-serving cell is a critical issue that needs to be addressed. There are two alternatives to address this issue: TAG based method and non-TAG based method. In the TAG based method, TA maintenance of the non-serving cell is also based on a TAG. For example, a TAG ID can be configured for the non-serving cell for the TA maintenance. While in the non-TAG based method, TAG is not used for the TA maintenance and a new mechanism, such as a new signaling, needs to be considered for the TA maintenance of the non-serving cell.

Proposal 2: RAN1 should independently study Multi-TA framework for both intra-cell and inter-cell MTRP cases.

Initial TA indication

In the S-TRP case, initial TA value is obtained using the RACH procedure. In particular, UE sends a RACH preamble to gNB which calculates the initial TA value of the UE, and then receives the corresponding RAR which includes the initial TA value. The initial TA value is applied for the cell in which UE sends the PRACH preamble. In the M-TRP case, to maintain two TAs for two TRPs, UE needs to obtain the initial TA value of each TRP. Legacy initial TA indication mechanism may not be applicable in this case as the UE does not know the initial TA value in the RAR corresponds to which TRP. This issue exists for both intra-cell and inter-cell M-TRP cases. Solutions for each case should be studied independently.

Proposal 3: RAN1 should study initial TA indication mechanisms for each TRP for both intra-cell and inter-cell MTRP cases.

TA update

Due to UE mobility, TA value needs to be updated periodically to keep uplink synchronization. In the current specifications, UE updates the TA value for a specific TAG according to the TA command in the MAC CE for the TA update. ID of the specific TAG is indicated in the MAC CE.

In the M-TRP case, the TA update principle for two TRPs follows the TA framework discussed in Section 3.1. For the intra-cell M-TRP case, if two TAGs based TA framework is adopted with each TAG corresponding to one TRP, legacy MAC-CE can be reused for the TA update. However, if one TAG based TA framework is adopted, enhancement on the TA update signaling should be considered. For inter-cell M-TRP case, if a specific TAG ID is configured for the non-serving cell, legacy MAC-CE can be reused for TA update for the non-serving cell. While, if the non-serving cell is not configured with a TAG ID, a dedicated TA update signaling for the non-serving may need to be introduced.

Proposal 4: RAN1 should study TA update mechanisms for each TRP for both intra-cell and inter-cell M-TRP cases.

TA mapping for UL transmission

In UL M-TRP transmission with two TAs, UE shall determine the TA used for transmission towards each TRP. For example, for PUSCH repetition in which one or multiple PUSCH repetitions are transmitted towards each TRP, UE needs to determine which TA should be used for each PUSCH repetition. Similar to the TA update, such mapping rule is dependent on the TA framework discussed in Section 3.1. For example, in intra-cell M-TRP case, whether one TAG or two TAGs are used for maintaining two TAs directly impact the design of the TA mapping rule. In particular, if two TAGs are adopted, the mapping between the two TAs and UL transmissions can be based on the TAG. For example, the TAG ID can be indicated for each UL transmission. While, if only one TAG is used, TAG based TA mapping is not feasible and other mechanisms should be considered.

Proposal 5: RAN1 should study the TA mapping to UL M-TRP transmissions for both intra-cell and inter-cell M-TRP cases.

Summary and conclusion

The proposals of this contribution are summarized as follows:

Proposal 1: RAN1 should consider TA enhancement for both intra-cell and inter-cell M-TRP cases.

Proposal 2: RAN1 should independently study Multi-TA framework for both intra-cell and inter-cell MTRP cases.

Proposal 3: RAN1 should study initial TA indication mechanisms for each TRP for both intra-cell and inter-cell MTRP cases.

Proposal 4: RAN1 should study TA update mechanisms for each TRP for both intra-cell and inter-cell M-TRP cases.

Proposal 5: RAN1 should study the TA mapping to UL M-TRP transmissions for both intra-cell and inter-cell M-TRP cases.

References

[1] RP-213598, “New WID: MIMO Evolution for Downlink and Uplink”, RAN Plenary #94-e, Dec. 6 - 17, 2021.