3GPP TSG-RAN WG1 Meeting #109-e	R1-2203151

e-Meeting, May 9 - 20, 2022

Agenda Item:	9.1.2

Source:	Huawei, HiSilicon

Title:	CSI enhancement for coherent JT and mobility

Document for:	Discussion and Decision

Introduction

For Rel-18 WI NR MIMO evolution, there are following objectives on enhancements of CSI acquisition and reporting [1],

- Study, and if justified, specify CSI reporting enhancement for high/medium UE velocities by exploiting time-domain correlation/Doppler-domain information to assist DL precoding, targeting FR1, as follows: - Rel-16/17 Type-II codebook refinement, without modification to the spatial and frequency domain basis - UE reporting of time-domain channel properties measured via CSI-RS for tracking … - Study, and if justified, specify enhancements of CSI acquisition for Coherent-JT targeting FR1 and up to 4 TRPs, assuming ideal backhaul and synchronization as well as the same number of antenna ports across TRPs, as follows: - Rel-16/17 Type-II codebook refinement for CJT mTRP targeting FDD and its associated CSI reporting, taking into account throughput-overhead trade-off - SRS enhancement to manage inter-TRP cross-SRS interference targeting TDD CJT via SRS capacity enhancement and/or interference randomization, with the constraints that 1) without consuming additional resources for SRS; 2) reuse existing SRS comb structure; 3) without new SRS root sequences - Note: the maximum number of CSI-RS ports per resource remains the same as in Rel-17, i.e. 32

In this contribution, we discuss on CSI enhancement for CJT and mobility.

CSI enhancement for coherent JT

The motivation of CSI enhancement for CJT

The eMBB traffic has been increasing significantly, which results in more transmission data streams and inter-cell interference in the network. The inter-cell interference has been a severe problem for further improvement of system performance.

CJT or Distributed-MIMO, which combines multiple MIMO antenna arrays into a large antenna array, is a promising technique to resolve the issue by inter-cell coherent transmission and inter-cell interference suppression, which can improve the system spectral efficiency and user experience, especially for cell edge users.

To achieve precise coherent transmission and interference suppression between multiple TRPs, the precision of channel information is critical for each UE, and a high resolution CSI feedback designed for CJT is necessary in 5G-advanced for better joint transmission and interference suppression. On the other hand, channel measurement and reporting of more TRPs leads to increasing of both downlink reference signal resource and uplink feedback overhead. Therefore, the CSI enhancement needs to achieve fine CSI precision with limited downlink and uplink overhead.

Proposal 1: Support CSI enhancement for CJT in Rel-18.

Evaluation assumptions

Evaluation scenario

A typical scenario of coherent joint transmission by multiple TRPs is illustrated in Figure 1. There is a coordination TRP set (TRPs inside the black solid line as an example in the figure), a CSI measurement TRP set (TRPs within the dashed red line), and a coherent joint transmission TRP set. There’s backhaul connection for TRPs within the coordination TRP set. And the CSI measurement TRP set and joint transmission TRP set can be selected in a UE-centric way.  The CSI measurement TRP set is configured by RRC based on the RSRP difference with the serving cell, such that the TRPs with strongest RSRP are included in the CSI measurement set. And each UE needs to measure the CSI of TRPs within the CSI measurement TRP set and report the measurement to gNB. Then gNB can determine coherent joint transmission TRP set for each UE depending on scheduling and CSI. The transmission TRP set is usually the same with the measurement set.

Figure 1. Illustration of typical scenario for CJT

Proposal 2: The scenario where the measurement TRP set for each UE is selected based on RSRP in a UE-centric way should be considered as the evaluation scenario for CJT.

Evaluation assumptions

Evaluation assumptions for system level simulation are listed inTable 1.

Table 1. Evaluation assumptions for Coherent-JT

| Parameters | Evaluation assumptions |
| --- | --- |
| Layout | Hexagonal grid, 3 sectors per site, 7 macro sites |
| Scenario | Dense Urban |
| Carrier frequency | 2.1GHz |
| Subcarrier spacing | 15kHz |
| Simulation Bandwidth | 20MHz |
| BS antenna configuration | 32 ports: (M, N, P, Mg, Ng; Mp, Np) = (2,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ 8 ports: (M, N, P, Mg, Ng; Mp, Np) = (1,4,2,1,1,1,4), (dH,dV) = (0.5, 0.8)λ |
| Coordination TRP selection | Each UE selects N strongest TRPs based on RSRP for CJT, N = 2,3,4. |
| UE antenna configuration | (M, N, P, Mg, Ng) = (1,2,2,1,1) ; The polarization angles are 0 and 90 |
| UE distribution | 80% indoor, 3km/h; 20% outdoor, 30km/h; 10 users per BS |
| UE receiver | MMSE-IRC |
| MIMO mode | MU-MIMO with rank adaptation |
| Traffic model | FTP model 1/3 with packet size 0.5Mbytes |
| Traffic load | 50%/70% RU |
| Baseline of evaluation | Single-TRP transmission with Rel-17 FeType II CB |

For CSI enhancement for CJT based on R17 FeTypeII codebook, additional evaluation assumptions including duplexing gap between UL and DL, SRS modeling for UL channel estimation, and FDD DL/UL calibration error model are the same as that of the EVM assumptions in Section 4 of R1-2006973 for Rel-17 CSI enhancements.

Proposal 3: The system-level evaluation assumptions for CSI enhancement for CJT should be based on Table 1.

Potential codebook structure for CJT CSI enhancement

According to the WID, the CSI enhancement for multi-TRP CJT can be based on Rel-16 eTypeII and Rel-17 FeTypeII codebooks respectively. In this section, the potential enhancements based on both Rel-16 eTypeII and Rel-17 FeTypeII codebooks are introduced.

Enhancement for Rel-16 eTypeII codebook

Assuming a UE is served by N (up to 4) TRPs with CJT. To enable CJT, the information of channels between UE and the serving TRPs and the relative relationship between channels from different TRPs need to be fed back to gNB. In this purpose, an example CJT codebook design is given in Figure 2 (taking N = 3 as an example).

Since the angle-delay pairs are different between channels of different TRPs, the channel projection to spatial domain and the frequency domain basis is performed separately for different TRPs. The submatrix corresponding to each TRP in space-frequency form is represented and approximated by the combination of a set of spatial basis and a set of frequency basis.

Then the starting point of CJT codebook design can be represented as the following formulation

W=W1⋮WN=W1,1⋱WN,1W1,2⋱WN,2W1,fH⋮WN,fH=W1,1W1,2W1,fH⋮WN,1WN,2WN,fH,

where W is the NP×Nf joint feedback matrix in space-frequency form, Wn is the sub-matrix corresponding to the nth TRP with dimension of P×Nf. For the nth TRP, Wn,1 is a P×2Ln matrix consisting of 2Ln spatial basis, Wn,f is an Nf×Mn matrix composed of Mn frequency basis, and Wn,2 is the space-frequency combination coefficients with dimension  2Ln×Mn. P denotes the number of antenna ports, and Nf denotes the number of frequency units. Ln and Mn are respectively the number of spatial basis for each polarization and the number of frequency basis corresponding to TRP n. For each TRP, Kn strongest coefficients are selected from all 2LnMn coefficients for reporting. Note that the values of Ln, Mn and Kn can be s the same or different for each TRP.

Figure 2. A CJT codebook based on R16 eTypeII codebook

With channel feedback of multi-TRPs, one problem is the feedback overhead increases by N time for N cooperating TRPs. To resolve this issue, this section discusses the enhancement to reduce overhead while keeping the precision of channel feedback.

Long term eigenvector feedback

For R16 eTypeII codebook, the spatial domain basis is a set of oversampled and orthogonal 2D-DFT beams, and the frequency domain basis is a set of orthogonal DFT vectors. Channel feedback based on spatial and frequency domain basis is to utilize the correlation of channel in a specific domain and to seek a set of sparse basis for an approximate representation of the precoding matrix. The sparser the projection of the channel on the basis are, the fewer the number of basis required to represent the channel is.

For multi-TRP CJT, the eigenvectors of spatial or frequency domain statistical covariance matrix is much sparser than 2D-DFT/DFT basis. This is because the eigen-basis can match the UE-specific statistical subspace better than 2D-DFT/DFT basis. Therefore, if the spatial and/or frequency domain basis in the codebook structure are selected from the statistical eigenvectors for the corresponding domain, the CSI precision can be improved. The spatial and frequency basis Wn,1 and Wn,f can be replaced by the statistical eigen-basis. The statistical eigenvectors can be reported with longer periodicity due to its property of slowly varying. The coefficient Wn,2 can be feedback with shorter periodicity to combine the eigenvectors to re-construct the channel at gNB side. With a sparser eigen-basis, fewer combination coefficients are needed to report for a target precision. In this way, the feedback overhead can be significantly reduced while CSI precision is kept.

Observation 1: Statistical eigen-basis for Wn,1 and Wn,f is sparser than 2D-DFT/DFT basis, which can reduce feedback overhead and increase feedback precision.

Joint Space-Frequency domain basis per TRP

For each TRP, joint space-frequency domain basis can also be considered to take full advantage of the sparsity of channel in joint space-frequency domain.

The codebook W1,1W1,2W1,fH for each TRP can be rewritten as (for simplicity the TRP index is omitted here)

W=W1W2WfH=i,jw2,i,jw1,iwf,jH

where w2,i,j is the entry (i, j) of W2, w1,i is the ith basis of W1, and wf,j is the jth basis of Wf. The above formula means that the space-frequency matrix W can be a linear combination of space-frequency basis matrix w1,iwf,jH. Note that the basis matrix has a restriction of rank-1, as the product of a column vector and a row vector. A vectorized version of space-frequency matrix can be represented as

W=vecW=i,jw2,i,jw1,i⨂wf,j=i,jw2,i,jwsf,i,j

If the restriction of rank-1 is relaxed, the basis {wsf} can be any joint SD-FD basis, which means it would provide a sparser representation of vecW than the restricted SD-FD separate basis. Such a sparser joint SD-FD basis comes from the statistical eigen-basis of joint SD-FD subspace.

Based on this, a further refined CJT codebook design for the feedback of the joint precoding matrix for all TRPs is shown in Figure 3. The precoding submatrix of each TRP in joint space-frequency form is represented by the linear combination a set of joint space-frequency domain basis vectors, each of which corresponds to one angle-delay pair actually. This CJT codebook can be represented as follows

W=W1⋮WN=W1,sf⋱WN,sfW1,2⋮WN,2=W1,sfW1,2⋮WN,sfWN,2,

where W is the vectorized precoding matrix W with dimension NPNf×1, and Wn  corresponds to TRP n with dimension PNf×1. For TRP n, Wn,sf is a PNf×2Qn matrix consisting of 2Qn joint space-frequency domain basis, and Wn,2 is the combination coefficients matrix with dimension2Qn×1. Qn denotes the number of joint space-frequency domain basis (angle-delay pair) for each polarization corresponding to TRP n. For each TRP, Kn strongest coefficients are selected from all 2Qn coefficients for reporting. Note that both the value of Qn and Kn can be set as the same or different for each TRP.

Figure 3.  A CJT codebook design based on joint space-frequency basis

Observation 2: The joint space-frequency domain basis provides a sparser basis than independent spatial and frequency domain basis.

Proposal 4: To support mTRP CJT, the enhancement of Rel-16 eTypeII codebook is based on joint space-frequency domain basis per TRP as

W=W1⋮WN=W1,sfW1,2⋮WN,sfWN,2.

Enhancement for Rel-17 FeTypeII codebook

For Rel-17 PS codebook, UE can measure the beamformed downlink channel matrix for each frequency unit based on the CSI-RS beamformed by angle and delay information. For multi-TRP CJT, the port selection for angle-delay pairs are performed independently for each TRP like the Rel-17 port selection codebook for single-TRP, because the angle-delay pairs vary among different TRPs. To support multi-TRP CJT, the sub-matrix of each TRP can be combined to feedback the CSI of multi-TRP.

Then, the CJT codebook can be represented as the following formulation

W=W1⋮WN=W1,1W1,2W1,fH⋮WN,1WN,2WN,fH,

where W is the NP×Nf joint feedback matrix in space-frequency form, Wn is the precoding submatrix corresponding to TRP n with dimension P×Nf. For TRP n, Wn,1 is a P×2Ln diagonal matrix in which the elements is 0 or 1, Wn,f is a Nf×Mn matrix composed of Mn frequency basis, and Wn,2 is the space-frequency combination coefficients matrix with dimension 2Ln×Mn. P denotes the number of antenna ports, and Nf represents the number of frequency units. Ln and Mn are respectively the number of selection ports for each polarization and the number of frequency basis vector corresponding to TRP n.

Proposal 5: To support multi-TRP CJT, the enhancement of Rel-17 FeTypeII codebook is

W=W1⋮WN=W1,1W1,2W1,fH⋮WN,1WN,2WN,fH.

Detailed designs for CJT CSI enhancement

Receiver side information feedback

When the UE reports per-TRP CSI, the channel is decomposed as below for a TRP, then vector v is fed back to gNB as the precoder assuming a receiving beam u.

H=UAVH and V=HHUA-1

However, in multi-TRP CJT scenario, the UE may have different assumptions of u for different TRPs. As a result, when UE receives the coherently transmitted signal, the signal may be canceled by each other when receiving by the UE because only one receiving beam is used by the UE.

v1H,⋯,vnH=1a1u1HH1,⋯,1anunHHn

To resolve this issue, the user side information such as u matrix assumed by the UE can be feedback to gNB to assist the derivation of the precoder for CJT. Another option is that UE can have a full rank feedback, where the feedback of each rank represents the channel for a receiving antenna. In this way, the full channel matrix can be fed back to gNB.

Moreover, such a feedback is also beneficial to have a precise estimation of SINR, which can improve the scheduling of MCS. This is because gNB can better estimate the interference in MU-MIMO in CJT, thus the accuracy of SINR estimation and MCS is increased.

Proposal 6: Support receiver side information feedback for MTRP CJT.

Joint coefficients selection for W2

In M-TRP CJT scenario, the channel qualities and the number of useful spatial paths are different between the UE and different TRPs. So the number of non-zero coefficients in W2 for different TRPs is also different. Therefore, it is necessary to jointly select non-zero coefficients among all combination coefficients of multi-TRPs. Then all the non-zero coefficients for all cooperating TRPs can be normalized to the strongest coefficient, and the co-phasing and co-amplitude between TRP can be realized naturally.

Based on the above analysis and simulation results, the following proposal is suggest:

Proposal 7: Support joint coefficients selection for W2 among cooperating TRPs.

Configuration of CSI-RS resources

For CSI measurement of N TRP coherent joint transmission, there are two options for CSI-RS resources configuration.

- Option 1: N CSI-RS port groups are contained in one CSI-RS resource. TPRs can be differentiated by CSI-RS port groups.

- Option 2: N CSI-RS resources are used with each CSI-RS resource corresponding to one TRP.

Option 1 would change the current CSI-RS configuration framework, as one CSI-RS would correspond to multi-TRP thus multiple TCI states. Meanwhile, option 2 can re-use the current CSI-RS framework with less spec efforts, that one CSI-RS corresponds to one TRP associated with one QCL configuration.

Option 2 can also have less CSI-RS resource overhead. For the example shown in Figure 4, UE2 can reuse the cell specific CSI-RS #1 and #3. However, for option 1, a specific CSI-RS resource #2 has to be configured, which increases CSI-RS resource overhead.

Figure 4. CSI-RS resource configuration for CJT.

Proposal 8: For the CSI acquisition of CJT, support configuration multiple CSI-RS resources, with each CSI-RS resource corresponding to one serving TRP.

Frequency domain granularity

In the multi-TRP CJT scenario, one UE is jointly served by a set of TRPs. And the signals transmitted by different TRPs travel through different propagation paths to the UE. Since the distances of the propagation paths from the serving TRPs to the UE are different, additional delay will be introduced for the channel of the farther TRPs. As a result, the delay spread is increased. For CSI measurement of multiple TRPs, such delay spread will lead to heavier frequency selectivity compared with the single-TRP channel. Due to the heavier frequency selectivity, large subband size would cause performance loss. Hence, finer frequency domain granularity is preferred for the CJT CSI acquisition.

Proposal 9: Support finer frequency domain granularity for CJT CSI feedback.

Feedback overhead

It is obvious that the precision of CSI can be improved as the feedback overhead increases, for both CJT and single-TRP transmission. The benefits of CJT is achieved by coherent transmission and interference suppression, which depends heavily on the CSI feedback precision. In other words, the performance of CJT is more sensitive to the CSI feedback precision. In addition, the UE needs to measure the CSI of multiple TRPs. Therefore, it is reasonable that the total overhead of PMI reporting of the N TRPs with CJT should increase linearly, where the average overhead per TRP is the same as the single-TRP transmission with Rel-17 FeTypeII or Rel-16 eTypeII.

Observation 3: The Total feedback overhead for N-TRP Coherent-JT should increases by N times compared to the single-TRP CSI feedback with Rel-17 FeTypeII or Rel-16 eTypeII.

Evaluation results

Evaluation on spatial and frequency domain basis

In this section, the performances of the CJT codebook with different basis are compared and analysed. System-level simulation results with rank adaptation for inter-site CJT with 3 cooperated TRPs are provided in Figure 5 and Figure 6, corresponding to 32T4R and 8T4R for 50%/70% RU respectively. The baseline is the sing-TRP transmission with R17 FeTypeII codebook. All the cases utilize the full rank feedback with each rank corresponding to channel of one receiving antenna. And the total overhead is the same for different basis, which is three times of the R17 FeTypeII codebook overhead. The precoding algorithm is shown in Appendix II.

As shown in Figure 5 and Figure 6, the CJT codebook with joint space-frequency domain statistical eigenvectors achieves better performance than DFT basis for both mean UPT and 5% UPT.

Observation 4: The CJT codebook design with joint space-frequency domain statistical eigenvectors achieves 10~15% gain for mean UPT and 12~43% gain for 5%-tile UE UPT, compared with DFT basis.

Figure 5. Performance gain of CJT with different basis over single-TRP transmission with Rel-17 PS codebook (32T4R, 50%/70% RU)

Figure 6. Performance gain of CJT with different basis over single-TRP transmission with Rel-17 PS codebook (8T4R, 50%/70% RU)

Evaluation on receiver side information feedback

In this section, the performance of receiver side information feedback (using full rank feedback with each rank representing the channel for a receiving antenna) is evaluated. The joint space-frequency statistical eigenvectors as the joint space-frequency basis is assumed. System-level simulation results of rank adaptation for inter-site CJT with 3 cooperated TRPs are provided in Figure 7 and Figure 8, corresponding to 32T4R and 8T4R for 50%/70% RU respectively. In this simulation, the baseline is the sing-TRP transmission with R17 FeTypeII codebook, while both the total overhead of the above two feedback cases are three times of the baseline.

As shown in Figure 7 and Figure 8, there is a significant performance gain with receiver side information feedback for both mean UPT and 5% UPT. The receiver side information feedback can lead to 10~20% performance gain for mean UPT and 30~90% performance gain for 5% UPT.

Observation 5: The full channel feedback for CJT codebook can provide about 10~20% gain for mean UPT and 30~90% gain for mean UPT and 5% UPT respectively.

Figure 7. Performance gain of CJT with receiver side information feedback (full rank feedback) over single-TRP transmission with Rel-17 PS codebook (32T4R, 50%/70% RU)

Figure 8. Performance gain of CJT with receiver side information feedback (full rank feedback) over single-TRP transmission with Rel-17 PS codebook (8T4R, 50%/70% RU)

Evaluation on joint coefficients selection for W2

We evaluated the performance of combination coefficients joint selection among multi-TRPs. When the total number of combination coefficients reported by the UE is fixed, we evaluated the following two combination coefficients selection options:

Option 1 (TRP independent selection): The combination coefficient reported by the UE is equally allocated among multi-TRPs.

Option 2 (TRP joint selection): The combination coefficient reported by the UE is jointly selected among multi-TRPs

Figure 9. Performance between option 1 (TRP independent selection) and option 2 (TRP joint selection) of coefficient selection for W2 (32T4R, 50%/70% RU)

Figure 10. Performance between option 1 (TRP independent selection) and option 2 (TRP joint selection) of coefficient selection for W2 (8T4R, 50%/70% RU)

It can be observed that joint coefficient selection for W2 among TRPs can provide significant performance gain.

Observation 6:  Compared to TRP independent selection of coefficients for W2,

- Joint selection among TRPs can provide about 7~10% and 16~28% performance gains for mean UPT and edge UPT, respectively, when each TRP has 32 CSI-RS ports.

- Joint selection among TRPs can provide up to about 2~6% and 12~22% performance gains for mean UPT and edge UPT, respectively, when each TRP has 8 CSI-RS ports.

Evaluation on frequency domain granularity

In this section, the performances of different frequency domain granularity are compared and analysed, based on the receiver side information feedback of the CJT codebook design with joint space-frequency statistical eigenvectors. System-level simulation results of rank adaptation for inter-site CJT with 3 cooperated TRPs are provided in Figure 11 and Figure 12, corresponding to 32T4R and 8T4R for 50%/70% RU respectively. The baseline is the single-TRP transmission with R17 FeTypeII codebook, while both the total overhead of the above two feedback cases are three times of the baseline.

As shown in Figure 11 and Figure 12, there is a significant performance loss at both mean UPT and 5% UPT when the frequency domain granularity changes from 2RB to 4RB, especially at 5% UPT. Regarding the 32T4R CJT case, the larger frequency domain granularity leads to 10~14% and 45~70% performance loss at mean UPT and 5% UPT respectively. And for the 8T4R CJT case, the performance losses increase to 17~28% (mean UPT) and 70~130% (5% UPT). Besides, the performance loss caused by enlarging frequency domain granularity increases with the increase of RU.

Observation 7: There is a significant performance loss at both mean UPT and 5% UPT when the frequency domain granularity changes from 2RB to 4RB, especially at 5% UPT (a loss more than 26%).

Figure 11.  Performance gain of CJT with different frequency domain granularities over single-TRP transmission with Rel-17 PS codebook (32T4R, 50%/70% RU)

Figure 12. Performance gain of CJT with different frequency domain granularities over single-TRP transmission with Rel-17 PS codebook (8T4R, 50%/70% RU)

CSI enhancement for mobility

The motivation of CSI enhancement for mobility

In existing CSI measurement and feedback procedure, the base station generally assumes that the channel condition remains unchanged in the CSI reporting period, and the base station uses the latest reported channel CSI for downlink transmission precoding design.

However, the performance in mobile scenarios is greatly degraded due to the CSI expiration.  As in mobile scenarios, the channel varied rapidly, and the CSI expiration significantly deteriorates the system performance while UE scheduling and precoding are based on the expired CSI. In addition, this becomes worse in MU scenario, as the CSI mismatch will introduce the inter-user interference when multi-user pairing is performed.

In Fig. 13, the performance of Rel-16/17 TypeII codebooks with 10ms periodicity of CSI feedback and UE speeds of 3km/h, 30km or 60km/h is illustrated. It can be observed that the performance will decrease remarkably when UE has high mobility. The performances of R16 Type II at 30km/h and 60km/h UE speed have 25.8% and 35.3% loss compared with UE speed of 3km/h, and the performances of R17 Type II at 30km/h and 60km/h UE speed have 30.7% and 40.8% loss compared with UE speed of 3km/h, which implies that the codebook with higher precision has larger performance loss in medium- and high-speed scenarios.

Figure 13. Performance for different UE speeds with 10ms periodicity of CSI feedback

Observation 8: The performances of R16 Type II at 30km/h and 60km/h UE speed have 25.8% and 35.3% loss compared with UE speed of 3km/h. The performances of R17 Type II at 30km/h and 60km/h UE speed have 30.7% and 40.8% loss compared with UE speed of 3km/h.

The prediction of CSI for mobility

In MIMO transmission, a channel consists of multiple paths, and the time-varying characteristic of a single path is uniquely described by the exponential function of its Doppler frequency:

Hu,s,n,mNLOSt= αu,s,n,mtime-invariant ej2πvn,mtDoppler

As the Doppler frequency does not change rapidly in short time duration, the CSI can be enhanced by predicting the time-varying channel based on the Doppler frequency of each single path.

Based on the existing R16/R17 type II codebook structure, one angel-delay pair corresponds to a single path or a cluster with similar angel-delay. Therefore, the Doppler frequency information can be recovered from the CSI of each angel-delay pair at continuous slots, as shown in Fig. 14, where the red box in the figure represents a certain angel-delay pair.

Figure 14. CSI of certain angle delay pair at continuous slots

Evaluation assumptions

To enhance the performance in mobility scenario, CSI-RS-based Doppler measurement and CSI feedback should ensure the accuracy of Doppler information acquisition for each angel-delay pair in N-LOS scenarios with a larger number of channel paths. In addition, the number of antennas, the frequency granularity, and the number of channel paths all affect the number of Doppler frequencies on each angle-delay pair, and further affect the accuracy of Doppler acquisition and channel prediction. Therefore, we propose the following evaluation assumptions:

Proposal 10: N-LOS scenarios with a larger number of channel paths, large bandwidth, large-scale antennas should be considered for CSI enhancement for mobility.

CSI enhancement for mobility

CSI measurement enhancement

To extract more accurate Doppler information, continuous CSI-RS may be needed for UE to measure channel information. In addition, considering the UE implementation complexity and the accuracy of the Doppler information, uniformly separated CSI-RS burst is preferred. In current spec, the aperiodic CSI-RS configuration cannot trigger a group of uniformly separated CSI-RS burst. Therefore, we propose to use multiple CSI-RS resources for CSI measurement and the CSI-RS resource set with multiple CSI-RS resource can be configured to perform a Doppler information measurement, which is a unified design for periodic CSI-RS, semi-periodic CSI-RS and aperiodic CSI-RS as shown in the Fig. 15.

Figure 15. The uniformly separated CSI-RS burst of one CSI-RS resource set with multiple CSI-RS resource

Based on above analysis, the following proposal is suggested:

Proposal 11: Multiple CSI-RS resource in one CSI-RS resource set should be configured for CSI measurement for mobility.

CSI prediction based on Doppler domain information

In order to solve the problems of the performance degradation due to CSI expiration, an effective technique is to perform channel prediction to compensate the channel variation. The channel prediction can be classified into gNB-based prediction and UE-based prediction, which are illustrated in Fig. 16 and Fig. 17, respectively.

Figure 16. CSI prediction based on Doppler domain information at gNB side

Figure 17. CSI prediction based on Doppler domain information at UE side

gNB side channel prediction

For gNB-based CSI prediction, after gNB sending continuous CSI-RS burst, Doppler domain information can be further extracted by the UE based on angle-delay domain information for multiple slots, and CSI based on space domain, frequency domain and Doppler domain bases could be reported to gNB. Then gNB can predict the subsequent channel or precoder based on the reported CSI with Doppler domain information

If CSI prediction is performed at gNB side, UE would feedback the recommended precoder V by SVD decomposition of the channel matrix: H=UΛVH. However, for different CSI-RS instances at different slots, UE may have different assumption of U matrix. So there would error in tracking each paths or angle-delay pair. As a result, eigenvector feedback for multiple slots could introduce performance loss for gNB-based prediction.

UE side channel prediction

For UE-based CSI prediction, gNB also needs to send continuous CSI-RS for UE to extract Doppler domain information based on angle-delay domain information for multiple slots, then UE can predict the subsequent channel based on extracted Doppler domain information and finally UE performs PMI prediction and then send the predicted CSI to gNB by Doppler domain compression.

As there’s no quantization error for the channel measured by UE, it can has better accuracy in CSI prediction.

Doppler domain compression in UE prediction and reporting

As different paths or angle-delay pairs have different Doppler shifts, the CSI prediction should use the prediction of every path or angle-delay pair by the estimated Doppler shift for it. For Rel-16 eTypeII codebook or Rel-17 FeTypeII codebook, every coefficient in W2 corresponds to such a path or angle-delay pair, thus the change of each path is represented by change of coefficients of w2, which can be projected on a basis of DFT vectors and feedback to gNB to predict the coefficients of W2.

Proposal 12: To support CSI for mobility, the changing of each coefficient in W2 in time domain is projected to a set of DFT vectors, which is feedback to gNB for channel prediction.

Evaluation results

The UE side prediction is evaluated, assuming MU-MIMO transmission with rank 1 per UE. It is based on Rel.16/17 type II SD/FD bases with 32 ports at speed 60km/h. More detail simulation assumptions can be found in Appendix I. The evaluation results are illustrated in the figures 18 and 19.

Figure 18. Performance for UE-based CSI prediction based on Rel 17 type II SD/FD bases and R17 type II

Figure 19. Performance for UE-based CSI prediction based on Rel 16 type II SD/FD bases and R16 type II

Based on simulation results, it can be observed that UE-based CSI prediction based on R17 FeTypeII codebook can achieve 14% performance gain compared with R17 type II codebook. And UE based CSI prediction based on R16 type II code can achieve 13% performance gain compared with R16 type II.

Observation 9: For UE-based CSI prediction at speed 60km/h with 10ms periodicity of CSI feedback,

- 14% average gain compared with R17 type II

- 13% average gain compared with R16 type II.

Based on above analysis and simulations, the following proposal is suggested:

Proposal 13: Predicting precoder at UE side should be considered for accurate prediction.

Conclusions

This contribution provides our views on CSI enhancement for CJT and mobility. We have following observations and proposals.

For CSI enhancement for CJT:

Observation 1: Statistical eigen-basis for Wn,1 and Wn,f is sparser than 2D-DFT/DFT basis, which can reduce feedback overhead and increase feedback precision.

Observation 2: The joint space-frequency domain basis provides a sparser basis than independent spatial and frequency domain basis.

Observation 3: The Total feedback overhead for N-TRP Coherent-JT should increases by N times compared to the single-TRP CSI feedback with Rel-17 FeTypeII or Rel-16 eTypeII.

Observation 4: The CJT codebook design with joint space-frequency domain statistical eigenvectors achieves 10~15% gain for mean UPT and 12~43% gain for 5%-tile UE UPT, compared with DFT basis.

Observation 5: The full channel feedback for CJT codebook can provide about 10~20% gain for mean UPT and 30~90% gain for mean UPT and 5% UPT respectively.

Observation 6:  Compared to TRP independent selection of coefficients for W2,

- Joint selection among TRPs can provide about 7~10% and 16~28% performance gains for mean UPT and edge UPT, respectively, when each TRP has 32 CSI-RS ports.

- Joint selection among TRPs can provide up to about 2~6% and 12~22% performance gains for mean UPT and edge UPT, respectively, when each TRP has 8 CSI-RS ports.

Observation 7: There is a significant performance loss at both mean UPT and 5% UPT when the frequency domain granularity changes from 2RB to 4RB, especially at 5% UPT (a loss more than 26%).

Proposal 1: Support CSI enhancement for CJT in Rel-18.

Proposal 2: The scenario where the measurement TRP set for each UE is selected based on RSRP in a UE-centric way should be considered as the evaluation scenario for CJT.

Proposal 3: The system-level evaluation assumptions for CSI enhancement for CJT should be based on Table 1.

Proposal 4: To support mTRP CJT, the enhancement of Rel-16 eTypeII codebook is based on joint space-frequency domain basis per TRP as

W=W1⋮WN=W1,sfW1,2⋮WN,sfWN,2.

Proposal 5: To support multi-TRP CJT, the enhancement of Rel-17 FeTypeII codebook is

W=W1⋮WN=W1,1W1,2W1,fH⋮WN,1WN,2WN,fH.

Proposal 6: Support receiver side information feedback for MTRP CJT.

Proposal 7: Support joint coefficients selection for W2 among cooperating TRPs.

Proposal 8: For the CSI acquisition of CJT, support configuration multiple CSI-RS resources, with each CSI-RS resource corresponding to one serving TRP.

Proposal 9: Support finer frequency domain granularity for CJT CSI feedback.

For CSI enhancement for mobility:

Observation 8: The performances of R16 Type II at 30km/h and 60km/h UE speed have 25.8% and 35.3% loss compared with UE speed of 3km/h. The performances of R17 Type II at 30km/h and 60km/h UE speed have 30.7% and 40.8% loss compared with UE speed of 3km/h.

Observation 9: For UE-based CSI prediction at speed 60km/h with 10ms periodicity of CSI feedback,

- 14% average gain compared with R17 type II

- 13% average gain compared with R16 type II.

Proposal 10: N-LOS scenarios with a larger number of channel paths, large bandwidth, large-scale antennas should be considered for CSI enhancement for mobility.

Proposal 11: Multiple CSI-RS resource in one CSI-RS resource set should be configured for CSI measurement for mobility.

Proposal 12: To support CSI for mobility, the changing of each coefficient in W2 in time domain is projected to a set of DFT vectors, which is feedback to gNB for channel prediction.

Proposal 13: Predicting precoder at UE side should be considered for accurate prediction.

- References

[1] RP-213598, “New WID: MIMO Evolution for Downlink and Uplink”, RAN Plenary #94-e, Dec. 6 - 17, 2021.

# Appendix I

Table A-1 SLS assumptions for CSI enhancement

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD, OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Urban Macro |
| Frequency Range |  | FR1 only, 2.1GHz, with duplexing gap of 200MHz |
| Inter-BS distance |  | 200m |
| Channel model |  | According to the TR 38.901 |
| Antenna setup and port layouts at gNB |  | 32 ports: (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE |  | 2RX: (M, N, P, Mg, Ng; Mp, Np) = (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ Other configuration is not precluded. |
| BS Tx power |  | 44dBm |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz |
| Simulation bandwidth |  | 10 MHz |

# Appendix II

The precoding algorithm in the evaluation for CJT is provided in this section.

As mentioned in Section 2.2.1, a typical large scale multi-TRP coordination is considered in the evaluation scenario. For different UEs, the transmission TRP sets for CJT are different, while the transmission TRP set of two different UEs may overlap. In this coordination scenario, the precoding algorithm for CJT based on zero-forcing precoding is utilized to enable the joint transmission and conduct the interference suppression.

Taking all the TRPs as a whole, the precoders of all TRPs can be jointly designed via zero-forcing precoding algorithm to null the multi-user interference. If one TRP does not provide services for one UE, the corresponding precoding matrix are forcibly set to zero. However, this requires to compute a large-dimensional pseudo-inverse with high computational complexity. To reduce the computation complexity, iwe can use a UE-centric zero-forcing precoding for CJT, which is to nullify the interference caused by each user for other users with its overlapped transmission TRP sets by zero-forcing precoder.

Figure 20. Diagram of the UE-centric zero-forcing precoding algorithm for CJT

The UE-centric zero-forcing precoding algorithm for CJT is specifically described under the following scenario from the perspective of one certain UE, as an example. As shown in Figure 20, it is assumed that the transmission TRP set of UE0 consists of TRP1 and TRP2, while TRP1 serves UE0, UE1 and UE2, and TRP2 serves UE0 and UE1. Considering the transmission TRP sets of UE1 and UE2 overlap with the UE0’s, the interference caused by UE0 to the other two users has to be eliminated, which can be represented by the following formula

H1,0H2,0H1,10H1,2H2,2P1,0P2,0=100,

where Hi,j denotes the part corresponding to TRPi of CJT eigen-channel of UEj, and Pi,j is the precoding matrix from TRPi to UEj. Then, the joint precoder of UE0 can be calculated by

P1,0P2,0=HHH-1H100,  H≜H1,0H2,0H1,10H1,2H2,2.

For each user, the foregoing operation can be performed to obtain its precoder. In general, the UE-centric zero-forcing precoding algorithm for CJT described above has much lower computational complexity, and is recommended to be utilized in the evaluation for CJT.