3GPP TSG-RAN WG1 Meeting #109-e	R1-2203154

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.1.4.1

Source:	Huawei, HiSilicon

Title:	Discussion on UL precoding indication for multi-panel transmission

Document for:	Discussion and Decision

Introduction

In RAN Plenary #94-e [1], it was agreed to study simultaneous multi-panel UL transmission (STxMP) in Rel-18. In this contribution, we provide our analysis and views on STxMP.

- Study, and if needed, specify the following items to facilitate simultaneous multi-panel UL transmission for higher UL throughput/reliability, focusing on FR2 and multi-TRP, assuming up to 2 TRPs and up to 2 panels, targeting CPE/FWA/vehicle/industrial devices (if applicable) - UL precoding indication for PUSCH, where no new codebook is introduced for multi-panel simultaneous transmission - The total number of layers is up to four across all panels and total number of codewords is up to two across all panels, considering single DCI and multi-DCI based multi-TRP operation. - UL beam indication for PUCCH/PUSCH, where unified TCI framework extension in objective 2 is assumed, considering single DCI and multi-DCI based multi-TRP operation For the case of multi-DCI based multi-TRP operation, only PUSCH+PUSCH, or PUCCH+PUCCH is transmitted across two panels in a same CC.

Use case and performance analysis

Preliminary simulations

The underlying idea of STxMP is to realize UL transmission with more than one propagation paths. In order to obtain the benefit of STxMP, it requires the two panels to have similar path-losses. If the gap of path-loss between the two panels is relatively large, the benefit of STxMP over the single panel based UL transmission (TxSP) would be questionable.

Figure 1. STxMP in Case 1

Let us consider the following two typical cases for STxMP:

- Case 1: UE is located in the middle of the two TRPs with a relatively large distance to both TRPs ;

- Case 2: UE is located close to one TRP and far away from the other;

As shown in Figure 1 (Case 1), two UE panels are used for transmission towards two different TRPs and the distances from the UE to the two TRPs are comparable which may result in a small difference in path-loss between the UE and the two TRPs. This may seem advantageous for STxMP. However, as the UE is far away from both TRPs and the total transmission power of the two panels is restricted by the maximum total radiated power defined by the UE power class, the RSRP of each panel will be quite low which in turn degrades the performance of STxMP. For instance, the maximum total radiated power simultaneously transmitted from both panels of a Class 2 UE would be restricted to 23 dBm. In contrast, if TxSP is adopted, the maximum total radiated power from one panel is 23 dBm. Therefore, the performance benefit of STxMP over TxSP in improving coverage is questionable as it may be better to spend all the transmit power budget on the panel with the better channel condition to enhance the coverage.

Observation 1: For a UE with relatively large distances to both TRPs, since the total transmission power over the two panels is restricted by the UE power class, the benefit of STxMP over TxSP for coverage enhancement is questionable.

Figure 2. STxMP in case 2

As shown in Figure 2 (Case 2), the UE is located close to the first TRP and far away from the second one. For such UE, the only benefit of STxMP over TxSP is introducing an extra link for transmission of an extra part of bits. Nevertheless, as the link between the UE and the second TRP is very weak and the throughput provided by the first TRP is already very large, the improvement of the extra link is marginal.

Observation 2: For a UE located close to one TRP and far away from the other, the benefit of STxMP over TxSP is marginal.

To further validate our above concerns, some preliminary SLS results on STxMP are provided and compared with the TxSP baseline. We assume a Class 3 UE with the equal total transmit power for the baseline TxSP scheme and STxMP scheme. Evaluation assumptions are based on the dense urban scenario given in Appendix I. Simulation results are given in Figure 3.

Figure 3: THP comparison of single panel and multi-panel transmission

As shown in Figure 3, when power sharing among panels is assumed, TxSP (blue bar in the figure) outperforms STxMP (red bar) in all cases. Moreover, it can be observed from the cell average throughput that STxMP contributes little to the overall cell throughput performance even with perfect scheduling decision (i.e. perfectly selecting between TxSP and STxMP for each transmission occasion), where STxMP achieves only a 2.25% gain to the average cell throughput in comparison with TxSP.

Observation 3: Based on our simulation results and analysis, the benefit of STxMP over TxSP is marginal.

Proposal 1: RAN1 should decide whether or not to support STxMP based on the system level evaluation of STxMP in comparison with the baseline TxSP.

Evaluation methodology and assumptions

In section 2.1, both our analysis and numerical results show that the performance gain of STxMP over TxSP is marginal. Since different simulation assumptions may lead to totally different simulation results for STxMP, it is important to align the simulation parameter assumptions for STxMP before identifying its possible performance benefits in comparison with the baseline TxSP. As discussed, our simulation results in Section 2.1 use the simulation parameter assumptions given in Appendix I which itself is based on the agreed evaluation methodology for multi-beam enhancement in RAN1#102-e [2].

For simulation parameter assumptions, evaluation methodology in [2] can be used as a starting point. However, further discussions are required for some STxMP simulation parameters.

Proposal 2: RAN1 should agree on evaluation methodology for STxMP SLS.

- Evaluation methodology for multi-beam enhancement agreed in R1-2007151 can be used as a starting point.

In particular, UE maximum transmit power plays an important role in simulation results. Currently, there are 5 types of UE Power Classes defined in [3] as shown in Table 1.

Table 1: Assumption of UE Types

| UE Power class | UE type |
| --- | --- |
| 1 | Fixed wireless access (FWA) UE |
| 2 | Vehicular UE |
| 3 | Handheld UE |
| 4 | High power non-handheld UE |
| 5 | Fixed wireless access (FWA) UE |

In the WID, the target devices for STxMP are CPE/FWA/vehicle/industrial devices (if applicable), that is, UE power class 1, power class 2, power class 4, and power class 5. For each UE power class, the maximum total transmit power is defined in [3]. As discussed earlier, this means that, for TxSP, the single UE panel may transmit with the maximum power limit defined for the UE power class. In contrast, for STxMP, the two UE panels share the total transmit power, and, hence, each of the two panels cannot achieve the maximum transmit power limit defined for the UE power class. The evaluation methodology for STxMP should take into account that the sum of the total transmit power from both panels is restricted by the total transmit power limit defined for the UE power class.

Observation 4: For TxSP, the single UE panel may transmit with the maximum power limit defined for the UE power class. For STxMP, the two UE panels share the total transmit power, and, hence, each of the two panels cannot achieve the maximum transmit power limit defined for the UE power class.

Proposal 3: Evaluation methodology for STxMP should capture that the sum of the total transmit power from both panels is restricted by the total transmit power limit defined for the UE power class.

Note also that the evaluation methodology for multi-beam enhancement agreed in [2] is tailored for mobile UEs and CPEs inside a HST and its target UE type is different from Rel-18 STxMP target UE type which is CPE/FWA/vehicle/industrial devices. Therefore, the baseline evaluation methodology for multi-beam enhancement agreed in [2] should be updated so that the agreed simulation assumptions represent CPE/FWA/vehicle/industrial devices.

Observation 5: The evaluation methodology for multi-beam enhancement agreed in R1-2007151 is tailored for mobile UEs and CPEs inside a HST and its target UE type is different from Rel-18 STxMP target UE type which is CPE/FWA/vehicle/industrial devices.

Proposal 4: Simulation assumptions in the evaluation methodology for STxMP should represent its target UE type which is CPE/FWA/vehicle/industrial devices.

Another important factor to consider is the UE panel architecture. Depending on the UE type, many different components may be packed into a small area when implementing the UE, including display, camera, sensors, etc. Therefore, the space for antennas is quite limited and those spaces would not be of equal size in general. In addition, the locations of antennas are dependent on the device shape and its design and, further, different panel orientations should be considered. Considering hardware cost and power consumption, in practice, a UE may have different antenna array types and panel architectures. For example, multiple configurable directional antenna arrays can be located at the same or different places as shown in Figure 3. Furthermore, uniform or non-uniform antenna spacing, co-polarized and/or cross-polarized antennas can be implemented. Based on the above discussion, we believe that the employed panel architecture has a substantial impact on the STxMP performance and, to align the simulation assumptions and to provide results based on realistic assumptions, the employed panel architecture and antenna array configuration should be agreed in the evaluation methodology for STxMP. Other related issues that impact the simulation results and need to be agreed in the evaluation methodology for STxMP include MPE modeling and panel calibration.

Proposal 5: RAN1 should agree on the employed panel architecture, antenna array configuration, MPE modeling, and panel calibration in evaluation methodology for STxMP.

Figure 3: An illustration on different antenna arrays implementation on a UE

Impacts of different multi-panel transmission chain architecture

Different transmission chain architectures could have different impacts on STxMP functionality such as supported number of codewords, supported number of layers, the feasibility of independent per-panel power control, phase calibration, etc. In what follows, we analyze the supported functionalities for different multi-panel transmission chain architectures.

Figure 5-1: Potential multi-panel transmission chain architecture 1

In architecture 1 that is shown in Figure 5-1, each panel is associated with an independent baseband (BB), intermediate frequency (IF), PA, and antenna array (AA). This architecture can support multiple codewords since each panel has an independent BB as well as an independent power control per panel. Since each panel is associated with 2 IFs each of which associated with an antenna port, the total number of supported layers is 4 in architecture 1.

Figure 5-2: Potential multi-panel transmission chain architecture 2

In architecture 2 that is shown in Figure 5-2, the two panels share a common baseband, but each panel has its own IF, PA, and AA. This structure can support only one codeword since the two panels share a common BB. However, it still can support an independent power control per panel. Since each panel is associated with 2 IFs, the total number of supported layers is 4 for architecture 2.

Figure 5-3: Potential multi-panel transmission chain architecture 3

In architecture 3 that is shown in Figure 5-3, two panels share a common baseband and IF. This structure can also support only one codeword. However, it still can support an independent power control per panel. Since two panels share two IFs, the total number of supported layers is 2 for architecture 3.

Figure 5-4: Potential multi-panel transmission chain architecture 4

In architecture 4 that is shown in Figure 5-4, two panels share a common baseband, IF, and PA. This structure can also support only one codeword and independent power control per panel cannot be supported. Since two panels share 2 IFs, the total number of supported layers is 2 for architecture 4. For this architecture, only SDM repetition can be supported.

Based on the above analysis, it can be observed that different multi-panel transmission chain architectures can support different functionalities. Therefore, RAN1 should agree on the employed multi-panel transmission chain architecture(s) in the evaluation methodology for STxMP.

Observation 6: For STxMP, used multi-panel transmission chain architecture determines whether or not functionalities such as multiple codewords, multiple layers, or independent power control per panel is supported.

Proposal 6: RAN1 should agree on the employed multi-panel transmission chain architecture in the evaluation methodology for STxMP.

- Note: The possible support for multiple codewords, multiple layers, or independent power control per panel in STxMP depends on the employed multi-panel transmission chain architecture.

Uplink precoder indication for sTxMP

For codebook based UL transmission, the number of bits required to notify TPMI in DCI ranges from 0 to 6 bits. For non-codebook based UL transmission, the number of bits required to notify SRI in DCI is up to 4 bits depending on the number of SRS resources configured in the SRS resource set. For both cases, the TPMI/SRI indication could be doubled for STxMP, which is a substantial overhead.

Observation 7: TPMI/SRI indication overhead for STxMP is twice as large as TPMI/SRI indication overhead in DCI.

Proposal 7: If RAN1 agreed to support STxMP, solutions for reducing overhead associated with uplink precoding indication for STxMP should be discussed.

Summary and conclusion

In this contribution, we provide the following observations and proposals:

Observation 1: For a UE with relatively large distances to both TRPs, since the total transmission power over the two panels is restricted by the UE power class, the benefit of STxMP over TxSP for coverage enhancement is questionable.

Observation 2: For a UE located close to one TRP and far away from the other, the benefit of STxMP over TxSP is marginal.

Observation 3: Based on our simulation results and analysis, the benefit of STxMP over TxSP is marginal.

Proposal 1: RAN1 should decide whether or not to support STxMP based on the system level evaluation of STxMP in comparison with the baseline TxSP.

Proposal 2: RAN1 should agree on evaluation methodology for STxMP SLS.

- Evaluation methodology for multi-beam enhancement agreed in R1-2007151 can be used as a starting point.

Observation 4: For TxSP, the single UE panel may transmit with the maximum power limit defined for the UE power class. For STxMP, the two UE panels share the total transmit power, and, hence, each of the two panels cannot achieve the maximum transmit power limit defined for the UE power class.

Proposal 3: Evaluation methodology for STxMP should capture that the sum of the total transmit power from both panels is restricted by the total transmit power limit defined for the UE power class.

Observation 5: The evaluation methodology for multi-beam enhancement agreed in R1-2007151 is tailored for mobile UEs and CPEs inside a HST and its target UE type is different from Rel-18 STxMP target UE type which is CPE/FWA/vehicle/industrial devices.

Proposal 4: Simulation assumptions in the evaluation methodology for STxMP should represent its target UE type which is CPE/FWA/vehicle/industrial devices.

Proposal 5: RAN1 should agree on the employed panel architecture, antenna array configuration, MPE modeling, and panel calibration in evaluation methodology for STxMP.

Observation 6: For STxMP, used multi-panel transmission chain architecture determines whether or not functionalities such as multiple codewords, multiple layers, or independent power control per panel is supported.

Proposal 6: RAN1 should agree on the employed multi-panel transmission chain architecture in the evaluation methodology for STxMP.

- Note: The possible support for multiple codewords, multiple layers, or independent power control per panel in STxMP depends on the employed multi-panel transmission chain architecture.

Observation 7: TPMI/SRI indication overhead for STxMP is twice as large as TPMI/SRI indication overhead in DCI.

Proposal 7: If RAN1 agreed to support STxMP, solutions for reducing overhead associated with uplink precoding indication for STxMP should be discussed.

References

[1] RP-213598, New WID: MIMO Evolution for Downlink and Uplink, Samsung, RAN Plenary #94-3, Dec. 6 - 17, 2021

[2] R1-2007151, Moderator summary for multi-beam enhancement: EVM

[3] 3GPP TS 38.101, User Equipment (UE) radio transmission and reception

- Appendix I Simulation assumptions used to obtain results in this paper

Table 2 Simulation assumptions used to obtain results in this paper

| Parameters | Values |
| --- | --- |
| Frequency Range | FR2 @ 30 GHz, - SCS: 120 kHz - BW: 80 MHz |
| Scenarios | Dense urban (TR 38.901/38.913) |
| UE Speed | 3 km/hr (for outdoor UEs, Dense Urban) |
| Transmission Power | Maximum Power and Maximum EIRP for base station and UE as given by corresponding scenario in 38.802 (Table A.2.1-1 and Table A.2.1-2) |
| BS Antenna Configuration | (M, N, P, Mg, Ng) = (4, 8, 2, 2, 2). (dV, dH) = (0.5, 0.5) λ. (dg,V, dg,H) = (2.0, 4.0) λ |
| BS Antenna radiation pattern | TR 38.802 Table A.2.1-6, Table A.2.1-7 |
| UE Antenna Configuration | Number/location of Panels - 2,3,4 Panel UEs Panel structure - 1x4x2 (Baseline) - Other panel structures optional (company to report) |
| UE Antenna radiation pattern | TR 38.802 Table A.2.1-8, Table A.2.1-10 |
| Link adaptation | Based on CSI-RS |
| Traffic Model | Full Buffer |
| Control and RS overhead | Not involved |
| Control channel decoding | Ideal |
| UE receiver type | MMSE-IRC |
| BF scheme | DFT |
| Algorithm details (when applicable) | Panel selection criterion: panel(s) with best beam/beam pair Beam metric: L1-RSRP |