3GPP TSG-RAN WG1 Meeting #109-e	 R1-2203155

e-Meeting, May 9 - 20, 2022

Agenda Item:	9.1.4.2

Source:	Huawei, HiSilicon

Title:	Discussion on SRI/TPMI enhancement for enabling 8 TX UL transmission

Document for:	Discussion and Decision

Introduction

There is the following objective in Rel-18 WID NR MIMO evolution to enable 8TX UL operation [1],

- Study, and if justified, specify UL DMRS, SRS, SRI, and TPMI (including codebook) enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices

  - Note: Potential restrictions on the scope of this objective (including coherence assumption, full/non-full power modes) will be identified as part of the study.

In this contribution, we discuss SRI and TPMI enhancement for enabling 8TX UL transmission.

SRI enhancement for enabling 8TX UL transmission

SRI has been used in both codebook (CB) and non-codebook (NCB) based uplink transmission. This section discusses the enhancement of SRI for both CB and NCB PUSCH.

CB based UL transmission

For CB based UL transmission, SRI is used to indicate the selected SRS resource for uplink transmission, which is of  bits. Whether enhancement is needed depends on the design of SRS design for 8 TX UL operation. In companion contribution [2], two options are discussed for SRS to enable 8TX. If the number of antenna ports in one SRS resource is increased to 8, then no enhancement is needed for SRI. Meanwhile, if two 4-port SRS resources are used for channel sounding of 8TX, then SRI should be enhanced to support it.

Observation 1: Whether SRI enhancement for enabling 8TX in CB based UL transmission is needed depends on the SRS design for 8TX.

NCB based UL transmission

For NCB based UL transmission, SRI is used to indicate one or more SRS resources fromSRS resources to UE, where UE uses the same precoder of selected SRS resource to transmit PUSCH. The SRI overhead is  bits, where Lmax is given by the maximum number of layers. For 4TX UE, is up to 4, and the maximum SRI overhead for 4TX UE is 4 bits. Because the number of candidate precoders of 8TX UE is up to 8, the maximum SRI overhead needs to be increased to 8 bits, which will result in a large DCI overhead for SRI.

Observation 2: To support 8TX NCB based UL transmission, the legacy SRI field will introduce a large DCI overhead.

For SRS resource set with usage ‘nonCodebook’, the candidate precoders are derived by UE based on the channel measurement. One way is to use the eigenvectors of SVD decomposition of the channel. Then there will be huge SINR gap between different precoders (SRS resources).

Observation 3: For 8TX NCB based UL transmission, there will be large SINR gap between the precoders used in SRS resources with usage ‘nonCodebook’.

Due to significant SINR gaps, the probability to use some candidate precoders (SRS resources) with low SINR in cases of MU-MIMO will be very low. Therefore, the full flexibility by a 8-bit SRI may not be needed.

Proposal 1: SRI enhancement with overhead reduction to enable 8TX NCB based UL transmission should be supported.

Aspects to be considered for UL 8TX

Antenna layout for UL 8TX

To support UL 8TX antennas, it can be envisioned that more antenna ports can bring more kinds of UE antenna layouts. Due to different size limitations and working frequencies for different UEs, UE antenna layout could be uniform or non-uniform. In addition, UE may also be equipped with single polarized or dual polarized antenna arrays. For the case of 8 uplink transmission ports, there can be at least four cases of UE’s antenna layout:

- Antenna ports with low correlation. One scenario is a UE with 8 TX antennas in the below 6GHz NR, where UE antenna spacing is large.

- 8 ports divided into 2 groups. There is high correlation within each group, and low correlation between port groups. A scenario for this case is the antennas with the same polarization are high correlated, and the antennas with different polarization have low correlation.

- 8 ports divided into 4 groups. Two antennas in each group are single polarized and have small antenna spacing. Different antenna groups have low correlation between each other due to different polarization or large antenna spacing. A scenario for this case is that 4 antenna groups are distributed at different sides of the UE equipment.

- Antenna ports with high correlation. For example, for UEs with large size, the eight antennas are single polarized and the antenna spacing are small.

Proposal 2: UL 8TX codebook design should consider various antenna layouts so as to accommodate different UE implementations.

Frequency selectivity for 8TX

With the number of antennas for uplink transmission is increased to 8, the uplink beam will be much narrower. As a result, the frequency selectivity is also increased. The performance benefit of frequency selective precoding becomes larger with the increase of the number of antennas ports at UE.

The performance of frequency selectivity precoding is evaluated, with results illustrated in Figure 1 and Figure 2. A granularity of 4RB for frequency selectivity precoding is assumed.

Figure 1 Performance gain of frequency selective precoding for SU

Figure 2 Performance gain of frequency selective precoding for MU

It can be observed that with the increase of TX antenna ports, the performance gains of frequency selective precoding over frequency non-selective precoding increases. For 8TX SU, the performance gains reach up to 19.90% with SNR = -6 dB and 17.09% with SNR = 0 dB. For 8TX MU, the performance gains reach up to 13.14% with SNR = -6 dB and 9.69% with SNR = 0 dB. Therefore, frequency selective precoding should be supported for uplink transmission, especially for 8TX.

Proposal 3：Frequency selective precoding should be supported for 8TX UL transmission.

TPMI (including codebook) enhancement to enable UL 8TX

Codebook enhancement

In the existing NR specification, UL codebook for 4TX UE considers three coherence types, such as full coherence, partial coherence and no coherence. For partial coherence, ports (1,3) and (2,4) are identified as two partially coherent port groups. As we have discussed in section 3.1, there are at least four cases of UE’s antenna layout for 8TX, which also lead to three coherence types:

- No coherence: Antenna ports with low correlation.

- Partial coherence: 8 antenna ports divided into 2 partially coherent port groups or 4 partially coherent port groups. The grouping is based on the correlation level between antenna ports.

- Full coherence: antenna ports with high correlation.

To support the three coherence types and accommodate different UE antenna layouts, it is preferred to have a unified UL codebook of 8TX to different UE antenna layouts, while the specification efforts can be minimized.

To resolve these problems, a block-wise codebook can be considered, where each block is based on 2TX codebook or 4TX codebook, such as legacy 2TX or 4TX codebook. In this way, the 8 TX antenna are grouped into different blocks depending the correlation between each other. A co-phasing can be indicated between blocks, depending on the coherence between them. Then, all three coherence types can be supported. For example, for partial coherence where 8 antenna ports are divided into two blocks, with 4 antenna ports in each block. Then the legacy 4TX codebook can be used for each block. In addition, because this 8TX codebook is based on legacy 4TX codebook, the legacy TPMI and TRI indication for 4TX can also be reused for 8TX, so that the spec efforts can be minimized.

Proposal 4：Block-wise codebook based on legacy 2TX or 4TX UL codebook should be supported to enable 8TX in CB based UL transmission.

TPMI enhancement

In the existing NR specification, TPMI and TRI are jointly encoded to indicate the selected codeword and transmit rank to UE, which is indicated by the “Precoding information and number of layers” field in DCI. For 4TX UE in Rel-15, the UL transmission layer per UE is up to 4. When the UL transmission layer for 8TX UE is increased to 8, the indication of TPMI and TRI will require a larger field than the legacy. In the existing NR specification, the maximum overhead of “Precoding information and number of layers” field is 6 bits. Then for 8TX, the maximum overhead of “Precoding information and number of layers” field may reach up to 12 bits.

Observation 4: To support 8TX UL operation, the field size of Precoding information and number of layers is increased by at least twice than legacy.

In addition, as discussed in section 3.2, the frequency selectivity for 8TX is also increased.

Beamformed CSI-RS can resolve the issues of DCI overhead and frequency selectivity, where CSI-RS is used to indicate the selected codeword by TRP to UE, instead of traditional DCI. Specifically, TRP transmits a beamformed CSI-RS with precoder WDL. Then after the TRP-UE channel H, UE can receive the CSI-RS as a vector PUL with dimension nRxx1 , where nRx is the number of receive antenna ports at UE. Then the received vector can be used as a UL precoder for UE.

PUL=HWDL

By choosing the precoder WDL of CSI-RS, the selected codeword PUL can be indicated to UE with high precision. Because the CSI-RS overhead is not related to the number of codewords, we can design a high-resolution codebook with large codebook size. Note that the considered high-resolution codebook based on beamformed CSI-RS requires reciprocity between UL and DL channels, which can be used for TDD networks.

With the beamformed CSI-RS, the overhead can be reduced and the resolution of indication can be increased. Therefore, we have the following proposal.

Proposal 5: The beamformed CSI-RS should be considered to indicate UL precoders to UE.

Conclusions

The contribution discusses the SRI, TPMI (including codebook) enhancement for enabling 8TX UL transmission, based on which the following observations and proposals are made.

Observation 1: Whether SRI enhancement for enabling 8TX in CB based UL transmission is needed depends on the SRS design for 8TX.

Observation 2: To support 8TX NCB based UL transmission, the legacy SRI field will introduce a large DCI overhead.

Observation 3: For 8TX NCB based UL transmission, there will be large SINR gap between the precoders used in SRS resources with usage ‘nonCodebook’.

Observation 4: To support 8TX UL operation, the field size of Precoding information and number of layers is increased by at least twice than legacy.

Proposal 1: SRI enhancement with overhead reduction to enable 8TX NCB based UL transmission should be supported.

Proposal 2: UL 8TX codebook design should consider various antenna layouts so as to accommodate different UE implementations.

Proposal 3：Frequency selective precoding should be supported for 8TX UL transmission.

Proposal 4：Block-wise codebook based on legacy 2TX or 4TX UL codebook should be supported to enable 8TX in CB based UL transmission.

Proposal 5: The beamformed CSI-RS should be considered to indicate UL precoders to UE.

- References

RP-213598, “New WID: MIMO Evolution for Downlink and Uplink”, 3GPP RAN#94e, Electronic Meeting, 6-17 December, 2021, Samsung.

R1-2203153, “SRS enhancement for TDD CJT and 8 TX operation in Rel-18”, 3GPP RAN#109e, Electronic Meeting, May 9-20, 2022, Huawei, HiSilicon.

- Appendix I

| Parameter | Value |
| --- | --- |
| System bandwidth | 20 MHz |
| Channel model | CDL-B |
| Velocity | 3 km/h |
| Delay spread | 300 ns |
| Carrier frequency | 2 GHz |
| Subcarrier spacing | 15 kHz |
| TRP antenna | 64 |
| UE antenna | 8 |
| UE number | 1, 8 |
| Layer number | 1, 8 |
| Precoder | SVD |
| AMC | On |
| SRS estimation | Ideal + 0 dB noise |
| DMRS estimation | Ideal |
| Receiver | MMSE-IRC |