3GPP TSG-RAN WG1 Meeting #109-e	R1-2203156

e-Meeting, May 9 – 20, 2022

Agenda Item:	9.3.1

Source:	Huawei, HiSilicon

Title:	Overview of evaluation on NR duplex evolution

Document for:	Discussion and Decision

- Introduction

In RAN#94-e [1], a new SI to support duplex evolution for NR TDD in unpaired spectrum was approved:

The objective of this study is to identify and evaluate the potential enhancements to support duplex evolution for NR TDD in unpaired spectrum. In this study, the followings are assumed: - Duplex enhancement at the gNB side - Half duplex operation at the UE side - No restriction on frequency ranges The detailed objectives are as follows: - Identify applicable and relevant deployment scenarios (RAN1). - Develop evaluation methodology for duplex enhancement (RAN1). - Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4). - Identify possible schemes and evaluate their feasibility and performances (RAN1). - Study inter-gNB and inter-UE CLI handling and identify solutions to manage them (RAN1). - Consider intra-subband CLI and inter-subband CLI in case of the subband non-overlapping full duplex. - Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1). - Study the feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4). - Study the feasibility of and impact on RF requirements considering the self-interference, the inter-subband CLI, and the inter-operator CLI at gNB and the inter-subband CLI and inter-operator CLI at UE (RAN4). - Note: RAN4 should be involved early to provide necessary information to RAN1 as needed and to study the feasibility aspects due to high impact in antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression. - Summarize the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4). Note: For potential enhancements on dynamic/flexible TDD, utilize the outcome of discussion in Rel-15 and Rel-16 while avoiding the repetition of the same discussion.

In this contribution, we provide an overview of evaluation on NR duplex evolution, including deployment scenarios, interference analysis and modeling, evaluation methodology, detailed assumptions, performance metrics, and initial link budget results for interferences.

- Evaluation on evolution of NR duplex operation

  - Deployment scenarios

The first objective of the Rel-18 NR duplex evolution SI is to identify applicable and relevant deployment scenarios. Depending on the duplexing modes/TDD configurations of different base stations in the network, some example deployment scenarios are provided in Table 1-1 to 1-3 covering both homogenous networks and heterogeneous networks. Note that not all possible combinations are listed but only some potentially scenarios are provided and the feasibility of these deployment scenarios should be further studied.

Among the deployment scenarios, the interested ones from our side in this SI are listed in Table 2 and then discussed in the following sections for dynamic/flexible TDD enhancement and subband non-overlapping full duplex.

Table 1-1: Homogeneous Macro cell networks.

| Scenarios 1 | Cell A: Macro cell | Cell B: Macro cell |
| --- | --- | --- |
| 1-1 | Semi-static DL dominant TDD configuration | Semi-static UL dominant TDD configuration |
| 1-2 | Flexible TDD configuration | Flexible TDD configuration |
| 1-3 | Semi-static UL/DL subband non-overlapping full duplex | Semi-static UL/DL subband non-overlapping full duplex (same UL/DL allocation as cell A) |
| 1-4 | Flexible UL/DL subband non-overlapping full duplex | Flexible UL/DL subband non-overlapping full duplex |

Table 1-2: Heterogeneous networks.

| Scenarios 2 | Cell A: Macro cell | Cell B: small cell |
| --- | --- | --- |
| 2-1 | Semi-static DL dominant TDD configuration | Semi-static UL dominant TDD configuration |
| 2-2 | Semi-static DL dominant TDD configuration | Flexible TDD configuration |
| 2-3 | Flexible TDD configuration | Flexible TDD configuration |
| 2-4 | Semi-static DL dominant TDD configuration | Semi-static UL/DL subband non-overlapping full duplex |
| 2-5 | Semi-static DL dominant TDD configuration | Flexible UL/DL subband non-overlapping full duplex |

Table 1-3: Homogeneous small cell networks.

| Scenarios 3 | Cell A: small cell | Cell B: small cell |
| --- | --- | --- |
| 3-1 | Semi-static DL dominant TDD configuration | Semi-static UL dominant TDD configuration |
| 3-2 | Semi-static or flexible TDD configuration | Flexible TDD configuration |
| 3-3 | Semi-static UL/DL subband non-overlapping full duplex | Semi-static UL/DL subband non-overlapping full duplex (same UL/DL allocation as cell A) |
| 3-4 | Semi-static UL/DL subband non-overlapping full duplex | Semi-static UL/DL subband non-overlapping full duplex (different UL/DL allocation as cell A) |
| 3-5 | Flexible UL/DL subband non-overlapping full duplex | Flexible UL/DL subband non-overlapping full duplex |

Table 2: Deployment scenarios may be prioritized in Rel-18 NR duplex evolution.

| Scenarios | Cell A | Cell B | Potential dominant benefit |
| --- | --- | --- | --- |
| 1-3 | - Semi-static UL/DL subband non-overlapping full duplex - Macro cell | - Semi-static UL/DL subband non-overlapping full duplex (same UL/DL allocation as cell A) - Macro cell | Uplink coverage |
| 2-1 | - Semi-static DL dominant TDD configuration - Macro cell | - Semi-static UL dominant TDD configuration - Small cell | Boosted uplink capacity in the small cell area (indoor factory or hotspot etc.) |
| 3-3 | - Semi-static UL/DL subband non-overlapping full duplex - Small cell | - Semi-static UL/DL subband non-overlapping full duplex (same UL/DL allocation as cell A) - Small cell | Low latency for industrial applications |

    - Deployment scenarios for dynamic/flexible TDD

With the global commercial deployment of 5G, diverse services and applications are becoming prevalent targeting both eMBB (To C) and industry (To B) use cases. Meanwhile, there is an increasing demand for higher uplink data rate and capacity which has not been well addressed, e.g., high definition video traffic in factories [2, 3]. Therefore, the industrial interest is an important factor that should be taken into account to choose the deployment scenarios for Rel-18 NR duplex evolution.

Proposal 1: To choose the deployment scenarios for Rel-18 NR duplex evolution, the industrial interest is an important factor that should be taken into account.

To meet the uplink data rate and capacity requirement in factories, the most straightforward way is to have UL-dominant TDD configurations in the factory to provide more time-domain resources for uplink. In this background, dynamic/flexible TDD enhancement was proposed. It can accommodate more uplink traffic in factory, and meanwhile support eMBB traffic in Macro.

Naturally, Macro with DL dominant TDD configuration and Pico with UL dominant TDD configuration at local area is one typical deployment scenario for dynamic/flexible TDD enhancement (i.e., Scenario 2-1). It is typical because a same spectrum will be used for both Macro cells and Pico cells in industry to maximize the spectrum value assuming the services are provided by a same operator, due to limited wideband spectrum available for the operator. An example is shown in Fig. 1, a Macro base station with DL-dominant TDD configuration (e.g., DDDSU) for traditional eMBB services is deployed outside a factory with Pico base station with UL-dominant TDD configuration (e.g., DSUUU). In this scenario, we only focus on FR1.

Observation 1: Boosted uplink capability in local area (indoor factory or hotspot etc.) is the dominant benefit for dynamic/flexible TDD enhancement.

Proposal 2: Scenario 2-1 should be studied with a high priority for dynamic/flexible TDD enhancement.

- Scenario 2-1: Macro with DL dominant TDD configuration and Pico with UL dominant TDD configuration at local area (FR1 only).

Fig. 1 Scenario 2-1: Macro with DL dominant TDD configuration and Pico with UL dominant TDD configuration at local area.

    - Deployment scenarios for subband non-overlapping full duplex

For factory/industry Pico deployment scenarios, the subband non-overlapping full duplex can provide not only more uplink resources to satisfy the requirement of high uplink data rate and capacity requirement in factory, but also lower latency compared with legacy TDD. This is because each slot has UL resources for subband non-overlapping full duplex so that HARQ-ACK can be feedback immediately, but HARQ-ACK can only be feedback in special or UL slot for legacy TDD, thus leading to a large latency regardless of UL/DL dominant TDD configuration or almost balanced UL/DL TDD configuration.

So factory/industry Pico with subband non-overlapping full duplex is another typical deployment scenario (i.e., Scenario 3-3). In addition, the potential influence from Macro deployment with DL dominant TDD configuration to factory/industry Pico with subband non-overlapping full duplex should also be considered, similar to Scenario 2-1. As shown in Fig. 2, several Pico base stations with subband non-overlapping full duplex are located in a factory and a Macro base station with a DL dominant TDD configuration is located outside the factory. In this scenario, we only focus on FR1.

Observation 2: Low latency is the dominant benefit for subband non-overlapping full duplex under the factory/industry deployment scenario.

Proposal 3: Scenario 3-3 should be studied with a high priority for subband non-overlapping full duplex.

- Scenario 3-3: Factory/industry Pico with subband non-overlapping full duplex (and potential Macro with DL dominant TDD configuration) (FR1 only).

Fig. 2 Scenario 3-3: factory/industry Pico with subband non-overlapping full duplex (and potential Macro with DL dominant TDD configuration).

Different from the factory/industry deployment scenario, uplink coverage is critical for Macro deployment scenarios, because there are only a small number of uplink slots within one TDD configuration period in practical deployment. Subband non-overlapping full duplex can provide UL resources in each slot so that UE at cell edge has more UL resources to be scheduled and the uplink coverage can be improved.

In addition, we suggest to focus on same UL/DL subband resource configurations across all the Macro cells (i.e., Scenario 1-3). In this scenario, there is no co-channel interference between the Macro cells, and the only cross link interference is from the adjacent subband. It is much easier not only for interference management, but also for resources allocation, and compatibility with legacy Rel-15/16/17 UEs. As shown in Fig. 3, two Macro base stations with subband non-overlapping full duplex have the same UL/DL subband resource configuration. In this scenario, we focus on both FR1 and FR2.

Observation 3: Better uplink coverage is the dominant benefit for subband non-overlapping full duplex under the Macro deployment scenario.

Proposal 4: Scenario 1-3 should be studied with a high priority for subband non-overlapping full duplex.

- Scenario 1-3: Macro with subband non-overlapping full duplex with same resource configurations (FR1 and FR2).

Fig. 3 Scenario 1-3: Macro with subband non-overlapping full duplex with same resource configurations.

Some other deployment scenarios other than the ones listed in Table 1-1/1-2/1-3 should also be considered as listed below. For Scenario 4, it is challenging if the UL subband are still used for the downlink transmission in the macro deployment. For Scenario 5, one may need to take the co-existence study in Rel-16 into account to avoid duplicated work. These scenarios can be studied after the other scenarios.

- Scenario 4: Macro with subband non-overlapping full duplex and Macro with legacy TDD configuration on the same carrier (co-channel co-existence with legacy base stations scenarios);

- Scenario 5: Macro with subband non-overlapping full duplex and Macro with legacy TDD configuration on the adjacent carriers (adjacent-channel co-existence or inter-operator co-existence scenarios).

Proposal 5: The following deployment scenarios can be studied after the evaluation of other scenarios:

- Scenarios 4: Macro with subband non-overlapping full duplex and Macro with legacy TDD configuration on the same carrier (co-channel co-existence with legacy base stations scenarios);

- Scenarios 5: Macro with subband non-overlapping full duplex and Macro with legacy TDD configuration on the adjacent carriers (adjacent-channel co-existence or inter-operator co-existence scenarios).

  - Interference types and modeling

    - Interference types

The operation of dynamic/flexible TDD enhancement and subband non-overlapping full duplex suffer from more serious interferences than legacy TDD operation in unpaired spectrum, such as cross link interference (CLI), self-interference, etc., which may reduce or even negate the benefits provided by the two techniques. It is therefore significant to analyze the interferences under different deployment scenarios.

The interference types in different deployment scenarios are shown in Fig. 4 and summarized in Table 3, where UL-to-UL interference and DL-to-DL interference already exist in legacy TDD in unpaired spectrum, but BS-to-BS CLI, UE-to-UE CLI, and BS self CLI are new interferences compared with the legacy TDD in unpaired spectrum, which leads to performance degradation of the uplink/downlink desired signal. So we only focus on these new types of interference.

In addition, potential blocking issues should be considered as well. The blocking issue is due to that the current base station is using a wide band analogue filter before its low-noise amplifier (LNA). The signal received from the antenna firstly passes the filter. Assuming the filter is wideband, e.g. 100 MHz or 200 MHz depending on implementation, the downlink signal will also pass the filter and be amplified by the LNA. If the received downlink interference signal is too strong, the LNA will be saturated and the UL signals cannot be restored. This issue can also occur in the UE side.

(a) Scenario 2-1                       (b) Scenario 3-3

(c) Scenario 1-3

Fig. 4 Interference types in different deployment scenarios.

Table 3: Summary of interference types.

| Interference type | Details |
| --- | --- |
| - UL-to-UL interference | Same as the legacy TDD in unpaired spectrum. |
| - DL-to-DL interference |  |
| - Inter-subband BS-to-BS CLI | DL interference from one BS to another BS in Macro cell (or small cell) for a same operator, where the DL interference is in the adjacent frequency of the desired UL signal. |
| - Inter-subband UE-to-UE CLI | UL interference from one UE to another UE in Macro cell (or small cell) for a same operator, where the UL interference is in the adjacent frequency of the desired DL signal. |
| - In-band BS-to-BS CLI | DL interference from one BS in Macro cell (or small cell) to another BS in small cell (or Macro cell) for a same operator, where the DL interference and desired UL signal are in the same band. |
| - In-band UE-to-UE CLI | UL interference from one UE in Macro cell (or small cell) to another UE in small cell (or Macro cell) for a same operator, where the UL interference and desired DL signal are in the same band. |
| - Inter-subband BS self CLI | DL transmission leaks to UL reception in BS, where the DL signal and UL signal are in the adjacent frequency. |

Depending on whether or not the interferences are generated by non-linear components, the above types of interferences can be classified as linear and non-linear interference. The linear interference includes:

- In-band BS-to-BS CLI and in-band UE-to-UE CLI, also called linear CLI;

- Blocking caused by downlink signals from own cell, also called linear self-interference;

- Blocking caused by downlink signals from other cells, also called linear interference;

- Blocking caused by uplink signals from other UE, also called linter interference;

And the non-linear interferences includes:

- Co-site or Non-Co-site inter-subband BS-to-BS CLI and inter-subband UE-to-UE CLI, also called non-linear CLI;

- Inter-subband BS self CLI, also called BS non-linear self-interference.

Observation 4: The cross link interference can be classified as linear and non-linear interference, where downlink signals from own cell or other cells may lead to blocking, and BS-to-BS CLI, UE-to-UE CLI, and BS self CLI have impact on demodulation performance.

Proposal 6: The following interferences will impact the system performance and should be studied in Rel-18 NR duplex evolution:

- Linear interference

  - In-band BS-to-BS CLI

  - In-band UE-to-UE CLI

  - Blocking caused by DL/UL signal

- Non-linear interference

  - Inter-subband BS-to-BS CLI

  - Inter-subband UE-to-UE CLI

  - Inter-subband BS self CLI

    - Interference modeling

For dynamic/flexible TDD enhancement, the in-band BS-to-BS CLI and in-band UE-to-UE CLI have been studied in Rel-14. And the evaluation results are summarized in TR 38.802 [4]. For interference modeling, the existing methods can be reused in Rel-18 NR duplex evolution.

Proposal 7: The interference models of in-band BS-to-BS CLI and in-band UE-to-UE CLI used in TR 38.802 can be reused for Rel-18 NR duplex evolution.

For subband non-overlapping full duplex, the key distinction with respect to interference is the non-linear interference generated by the non-linear components in BS or UE. To study the feasibility and performance of subband non-overlapping full duplex, the non-linear interference should be modeled properly. For the modeling of the non-linear interference, the basic characteristics of the non-linear interference signals should be studied and some aspects may also need RAN4 expertize.

- The strength of the non-linear interference: the interference strength depends on several factors, e.g. the frequency gap between the measured frequency and the DL/UL transmission frequency. This can be studied by RAN4.

- The correlation of the non-linear interference among different antenna ports: It can be assumed that the non-linear interference between the transmit antenna ports have low correlation. This is because the DL/UL signals are usually precoded in the granularity of 2 RB or 4RB. Therefore, the DL/UL transmit signal on each antenna port will have independent peak in time-domain. The DL/UL signals with independent peaks will be fed into the PA at each antenna port, leading to independent non-linear interference at each antenna port. One exception is that when all the precoder values are equal, then the signals at each antenna port will be identical and the non-linear interference become the same on each antenna port, which will be with the same radiation direction as the DL/UL desired signal. However the probability of this is low due to multi-path channel model from the BS to the UE. So in typical cases, the non-linear interference can be modeled as uncorrelated signals. And the non-linear signal on each transmit antenna port can be modeled as Gaussian distribution as studied in [5]. This implies that beamforming should not be considered in the modeling of non-linear interference.

- Channel model for the non-linear interference: The non-linear interference generated by the aggressor will prorogate over the air and arrive at the victim. Therefore, the channel between the aggressor and the victim should be modeled. If only large scale channel is modeled, the non-linear interference becomes white and this results in a noise rise at the receiver. As analyzed in section 2.4.3, the strength of non-linear interference is high especially in for macro deployment, which impacts the feasibility of subband non-overlapping full duplex. On the other hand, if fast fading is also modeled, potential interference suppression schemes based on advanced IRC receivers can be considered since the space domain property (non-white) of the interference can be explored. In summary, both large scale and fast fading channel should be modeled for the non-linear interference.

Proposal 8: The following aspects for non-linear interference modeling should be studied in Rel-18 NR duplex evolution:

  - Strength of non-linear interference (RAN4)

  - Correlation of non-linear interference between antenna ports

    - Non-linear interference can be modeled as uncorrelated signals

    - Non-linear signal on each transmit antenna port can be modeled as Gaussian distribution, i.e., beamforming should not be considered in the modeling of non-linear interference

  - Channel for non-linear interference

    - Both large fading and fast fading should be modeled

Besides the non-linear interference, the linear interference at the aggressor will have blocking effect at the receiver assuming that the receiver is using a wideband filter before the LNA as mentioned above. Then in the evaluation, this effect should also be checked, i.e. how to model the blocking interference is an issue needs to be considered. From our side, the linear interference strength is highly related to the beamforming or radiation direction of the transmitter, and this radiation direction is related to the fast fading channel. From this point of view, the fast fading channel between BS-to-BS and the fast fading between UE-to-UE need to be modeled too. One method is to reuse existing BS-to-UE channel model given in TR 38.901 [6] with modifications given in TR 38.802 [4], which are extracted in Table 4.

However, some parameters should be checked whether they are applicable to the BS-to-BS channel model and UE-to-UE channel model in TR 38.901 and TR 38.802, e.g., LOS probability, delay spread, angle spread, etc. Taking LOS probability as an example, the LOS probability of BS-to-UE link under UMa scenario is given as follows [6]:

PrLOS=1,d2D-out≤18m18d2D-out+exp-d2D-out631-18d2D-out1+C'hUT54d2D-out1003exp-d2D-out150,18m<d2D-out

where

C'hUT=0,hUT≤13mhUT-13101.5,13m<hUT≤23m

d2D-out denotes the distance between BS and UE (assuming UE is located outdoor), and hUT denotes actual antenna heights. If this model is reused to determine the LOS probability of BS-to-BS link under UMa scenario, d2D-out should denote the distance between BS and BS and hUT should denote the BS height. Assume d2D-out=200m and hUT=25m, and then PrLOS=57.17%. However, the LOS probability of BS-to-BS link is much higher than 57.17% under UMa scenario in practice. Given that the pathloss of LOS path is much lower than that of NLOS path, it will lead to that the BS-to-BS CLI strength level in the evaluation is much lower than that in realistic network. So reusing the LOS probability of BS-to-UE link to determine that of BS-to-BS link given in TR 38.901 and TR 38.802 is inappropriate.

Observation 5: The LOS probability of BS-to-BS link determined by reusing BS-to-UE channel model in TR 38.901 with modifications in TR 38.802 is much lower than that in the realistic network. It will lead to that the BS-to-BS CLI strength level in the evaluation is much lower than that in the actual network.

Proposal 9: Reuse the existing BS-to-UE channel model in TR 38.901 and TR 38.802 as much as possible to determine the BS-to-BS channel and UE-to-UE channel in Rel-18 NR duplex evolution.

- FFS: the parameters, e.g., LOS probability, delay spread, angle spread, etc., in the BS-to-UE channel model should be checked whether they are applicable to the BS-to-BS channel model and UE-to-UE channel model.

Table 4: BS-to-BS channel model and UE-to-UE channel model.

| Parameters | Dense urban | Urban macro | Indoor hotspot |
| --- | --- | --- | --- |
| Large-scale channel parameters | Below 6GHz: - Macro-to-Macro: 3D UMa (h_UE = 25m) - Macro-to-Micro: 3D UMa (h_UE = 10m) - Micro-to-Micro: 3D UMi (h_UE = 10m) - UE-to-UE: A.2.1.2 in TR 36.843, penetration loss between UEs follows Table A.2.1-13 in TR 38.802 Above 6GHz: - Macro-to-Macro: 5GCM UMa (h_UE = 25m) - Macro-to-Micro: 5GCM UMa (h_UE = 10m) - Micro-to-Micro: UMi-Street canyon (h_UE = 10m) - UE-to-UE: UMi-Street canyon (h_BS = 1.5m ~ 22.5m), penetration loss between UEs follows Table A.2.1-12 in TR 38.802. |  | Below 6GHz: - TRP-to-TRP: ITU InH (h_UE = 3m) - UE-to-UE: A.1.2.2 in TR 36.843. Above 6GHz: - TRP-to-TRP: 5GCM Indoor-office (h_UE = 3m) - UE-to-UE: 5GCM Indoor office (h_BS = 1.5m) |
| Fast fading parameters | Below 6GHz: - Macro to Macro: 3D UMa O-to-O (h_UE=25m); ASA and ZSA statistics updated to be the same as ASD and ZSD; ZoD offset = 0 - Macro to Micro: 3D UMa O-to-O - Micro to Micro: 3D UMi O-to-O (h_UE=10m); ASA and ZSA statistics updated to be the same as ASD and ZSD; ZoD offset = 0 - UE to UE: InH for indoor to indoor, and 3D UMi for other cases. ASD and ZSD statistics updated to be the same as ASA and ZSA. Dual mobility support. Above 6GHz: - Macro to Macro: 5GCM UMa O-to-O (h_UE=25m); ASA and ZSA statistics updated to be the same as ASD and ZSD; ZoD offset = 0 - Macro to Micro: 5GCM UMa O-to-O - Micro to Micro: UMi-Street canyon O-to-O (h_UE=10m); ASA and ZSA statistics updated to be the same as ASD and ZSD; ZoD offset = 0 - UE to UE: UMi-Street canyon; ASD and ZSD statistics updated to be the same as ASA and ZSA. Dual mobility support. |  | Below 6GHz: - TRP-to-TRP: ITU InH (h_UE = 3m), ASA statistics updated to be the same as ASD - UE-to-UE: A.2.1.2 in TR 36.843 (ITU InH), ASD statistics updated to be the same as ASA. Above 6GHz: - TRP-to-TRP: 5GCM Indoor-office (h_UE = 3m), ASA and ZSA statistics updated to be the same as ASD and ZSD - UE-to-UE: 5GCM Indoor-office (h_BS = 1.5m), ASD and ZSD statistics updated to be the same as ASA and ZSA |

  - Evaluation methodology, assumptions, and metrics

From the analysis in section 2.1, the expected benefits are different under different deployment scenarios, e.g. improved UL coverage, boosted uplink capacity and low latency. However, this would require potential enhancements such as interference handling, i.e. the expected performance benefit can only be achievable if the interference can be suppressed or mitigated. To evaluate the feasibility and performance of these possible schemes, it is important to study these schemes based on system level simulations. Besides, the expected UL coverage enhancement for subband non-overlapping full duplex is highly related to the performance of the RX receiver, co-site self-interference cancellation, etc., and it is difficult to evaluate these aspects in system level simulations due to the difficulty in modeling of interference suppression algorithm in system level. From this point of view, link level simulation together with link budget analysis are efficient methods to evaluate the coverage benefit similar to what was done in the Rel-17 NR coverage enhancement study.

As discussed above, the link budget analysis, link level evaluations, and system-level evaluations should be considered to evaluate the Rel-18 NR duplex evolution.

Proposal 10: At least the following evaluation methodologies should be considered to evaluate Rel-18 NR duplex evolution:

- Link budget analysis

- Interference strength evaluation for Scenario 2-1, Scenario 3-3, and Scenario 1-3

- Coverage evaluation for Scenario 1-3

- Link level evaluation

- Coverage evaluation for Scenario 1-3

- Interference suppression evaluation for Scenario 2-1, Scenario 3-3, Scenario 1-3

- System level evaluation

- Coverage and capacity evaluation for Scenario 2-1, Scenario 3-3, and Scenario 1-3

    - Link budget analysis

Regarding to link budget for coverage performance, IMT-2020 self-evaluation has already determined a full set of evaluation parameters, and was used for Rel-17 NR coverage enhancement. So the most convenient way is to use the link budget analysis methodology, assumption, and metrics adopted in IMT-2020 self-evaluation [7] and Rel-17 NR coverage enhancement [8]. Considering this is early stage for evaluation, we reuse link budget template in TR 38.830 as a starting point. The evaluation procedure, assumptions, and metrics are listed in Table 5-1. The key change in Rel-18 NR duplex evolution is step (3a); for legacy TDD, the occupied bandwidth for uplink is 100MHz at 4GHz/2.6GHz or 200MHz at 28GHz, but for subband non-overlapping full duplex, the bandwidth of uplink subband is 20MHz at 4GHz/2.6GHz or 40MHz at 28GHz.

Regarding to link budget analysis for interference strength, especially for BS-to-BS CLI, UE-to-UE CLI, and BS self CLI, several other parameters should be considered, such as adjacent channel leakage power ratio (ACLR), antenna isolation, etc., as listed in Table 5-2, which are based on TR 38.828 [9].

Proposal 11: Reuse the evaluation methodology, assumptions, and metrics of link budget analysis in IMT-2020 self-evaluation and/or TR 38.830 of Rel-17 NR coverage enhancement for Rel-18 NR duplex evolution.

Table 5-1. Link budget template for coverage evaluation under Scenario 1-3.

| System configuration |  |
| --- | --- |
| Channel for evaluation | PUSCH |
| Scenarios and Carrier frequency (GHz) | For FR1: - Urban 4 GHz SBFD / 2.6 GHz SBFD For FR2: - Urban 28 GHz SBFD |
| BS antenna heights (m) | 25m for urban can be used as a starting point. |
| UT antenna heights (m) | 1.5m can be used as a starting point. |
| Cell area reliability (%) | 90% for data channel can be used as a starting point. |
| Lognormal shadow fading std deviation (dB) | Reported by companies |
| Tx Diversity | Reported by companies |
| Number of SSB | Reported by companies |
| Transmitter |  |
| (1) Number of transmit antenna elements | For FR1 BS: - Urban: - 192 antenna elements for 4GHz and 2.6GHz - (optional) 128 antenna elements for 4GHz For FR2 BS: - Urban: - 256, Optional: 512 For FR1 UE: - 1 - 2 (optional) For FR2 UE: - 8 |
| (2) Number of transmit TxRUs Note: this row is void (left empty) for uplink | FR1 BS: - 64TxRUs for 2.6 and 4 GHz FR2 BS: - 2 |
| (2a) Number of transmit chains modelled in LLS | For FR1 BS: - Option 1: 2 or 4 gNB transmit chains in LLS. - Option 2 (optional): Number of gNB transmit chains = number of TXRUs FR2 BS: - 2 For FR1 UE: - PUSCH: 1, 2 (optional) For FR2 UE: - Option 1: PUSCH: 1, 2 - Option 2: 8 |
| (3) Total transmit power (dBm) Note: total transmit power for system bandwidth | For FR1 UE: - 23 dBm for UE For FR2 UE: - 23 dBm and/or 12 dBm for UE (other values can be reported by companies) |
| (3a) System bandwidth for downlink, or occupied bandwidth for uplink (Hz) | For uplink: Occupied bandwidth for FR1: - 100MHz for 4GHz and 2.6GHz, where 20MHz uplink subband and 80MHz downlink subband Occupied bandwidth for FR2: - 200MHz for 28GHz, where 40MHz uplink subband and 160MHz downlink subband |
| (3b) Power Spectrum Density = (3) - 10 log( (3a) / 1000000 ) (dBm/MHz) Note: no PSD constraint for uplink |  |
| (3c) Bandwidth used for the evaluated channel (Hz) Note: (3c) is identical to the number of PRBs assigned to the channel evaluated. For uplink, (3a) = (3c) |  |
| (3bis) Total transmit power for occupied bandwidth = (3b) + 10 log ( (3c) / 1000000 ) (dBm) |  |
| (4) Total antenna gain at antenna gain component 3 & antenna gain component 4 of transmitter = (4a) – (4b) (dB) |  |
| (4a) Antenna gain at antenna gain component 3 & antenna gain component 4 of transmitter = (4c) + 10 log ( (1) / (2) ) (dB) for downlink, and = (4c) + 10 log ( (1) / (2a) ) (dB) for uplink |  |
| (4b) Antenna gain correction factor at antenna gain component 3 & antenna gain component 4 of transmitter (dB) | Reported by companies |
| (4c) Gain of antenna element (dBi) | For BS: - 8 dBi or reported by companies For UE: - 0 dBi for FR1 - 5 dBi for FR2 |
| (5) Total antenna gain at antenna gain component 2 of transmitter = (5a) - (5b) (dB) Note: zero for uplink |  |
| (5a) Antenna gain at antenna gain component 2 of transmitter = 10 log( (2)/(2a)) (dB) Note: zero for uplink |  |
| (5b) Antenna gain correction factor at antenna gain component 2 of transmitter (dB) Note: zero for uplink | Reported by companies |
| (8) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (feeder loss must be included for and only for downlink) | Reported by companies |
| (9) EIRP = (3bis) + (4) + (5) – (8) dBm |  |
| Receiver |  |
| (10) Number of receive antenna elements | For FR1 BS: Urban: - 192 antenna elements for 4GHz and 2.6GHz - (optional) 128 antenna elements for 4GHz For FR2 BS: - Urban: 256, Optional: 512 For FR1 UE: - 1 - (optional) 2 For FR2 UE: - 8 |
| (10a) Number of receive TxRUs Note: this row is void (empty) for downlink | FR1 BS: - 64TxRUs for 2.6 and 4 GHz FR2 BS: - 2 |
| (10b) Number of receive chains modelled in LLS | For FR1 BS: - Option 1: 2 or 4 gNB receive chains in LLS. - Option 2 (optional): Number of gNB receive chains = number of TXRUs FR2 BS: - 2 For FR1 UE: - 4 for 4GHz/2.6GHz For FR2 UE: - Option 1: 2 - Option 2: 8 |
| (11) Total antenna gain at antenna gain component 3 & antenna gain component 4 of receiver = (11a) - (11b) (dB) |  |
| (11a) Antenna gain at antenna gain component 3 & antenna gain component 4 of receiver = (11c) + 10 log ( (10)/(10a) ) (dB) for uplink = (11c) + 10 log ( (10)/(10b) ) (dB) for downlink |  |
| (11b) Antenna gain correction factor at antenna gain component 3 & antenna gain component 4 of receiver (dB) | Reported by companies |
| (11c) Gain of antenna element (dBi) | For BS: - 8 dBi or reported by companies For UE: - 0 dBi for FR1, - 5 dBi for FR2 |
| (11bis) Total antenna gain at antenna gain component 2 of receiver = (11bis-a) - (11bis-b) (dB) Note: zero for downlink |  |
| (11bis-a) Antenna gain at antenna gain component 2 of receiver = 10 log( (10a)/(10b)) (dB) Note: zero for downlink |  |
| (11bis-b) Antenna gain correction factor at antenna gain component 2 of receiver (dB) Note: zero for downlink | Reported by companies |
| (12) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (feeder loss must be included for and only for uplink) | Reported by companies |
| (13) Receiver noise figure (dB) | Reported by companies |
| (14) Thermal noise density (dBm/Hz) | Reported by companies |
| (15) Receiver interference density (dBm/Hz) | Reported by companies |
| (16) Total noise plus interference density = 10 log (10^(( (13) + (14))/10) + 10^((15)/10)) (dBm/Hz) |  |
| (18) Effective noise power = (16) + 10 log ((3c)) (dBm) |  |
| (19) Required SNR (dB) |  |
| (20) Receiver implementation margin (dB) | Reported by companies |
| (21) H-ARQ gain (dB) Note: Only applicable if HARQ is not considered in LLS | Reported by companies |
| (22) Receiver sensitivity = (18) + (19) + (20) – (21) (dBm) |  |
| (22bis) MCL = (3bis) – (22) + (5) + (11bis) (dB) |  |
| (23) Hardware link budget, a.k.a. MIL = (9) + (11) + (11bis) − (12) − (22) (dB) Note: MIL can also be derived by (22bis) + (4) – (8) + (11) − (12) |  |
| Calculation of available pathloss |  |
| (25) Shadow fading margin (function of the cell area reliability and lognormal shadow fading std deviation) (dB) | Reported by companies |
| (26) BS selection/macro-diversity gain (dB) | Reported by companies |
| (27) Penetration margin (dB) | Reported by companies |
| (28) Other gains (dB) (if any please specify) | Reported by companies |
| (29) Available path loss = (23) – (25) + (26) – (27) + (28) (dB) |  |
| Range/coverage efficiency calculation |  |
| (30) Maximum range (based on (29) and according to the system configuration section of the link budget) (m) |  |

Table 5-1. Extra link budget assumptions for CLI evaluation.

| ### Parameters | ### Values |
| --- | --- |
| ### ACLR (dB) | ### FR1: - BS 45dB - UE: 30dB ### FR2: - BS 28dB - UE: 17dB |
| ### Antenna isolation (dB) | ### 45dB |

    - Link level evaluation

The link level evaluation is necessary to evaluate coverage and link level performance of advanced receiver for interference suppression (e.g., IRC and SIC receiver, etc.), realistic demodulation performance due to the interferences, etc. A complete link level evaluation methodology for coverage has been used in Rel-17 NR coverage enhancement. Considering this is early stage for evaluation, so we suggest to reuse the link level evaluation methodology in TR 38.830 as a start for Rel-18 NR duplex evolution. And the evaluation assumptions and metrics are listed in Table 6.

The key change in Rel-18 NR duplex evolution is the frame structure for subband non-overlapping full duplex. We denote the slot with both UL and DL subband as “X”, called SBFD slot. And two different frame structures are considered in the link level evaluation, i.e., XXXXX and XXXXU, as shown in Fig. 5(a) and (b), respectively, where there is a guard band (GB) between the UL subband and DL subband, and there is a guard period (GP) in the DL subband of the X slot right before the U slot. The detailed subband configuration is also given in Table 6.

In addition, these evaluation assumptions can also be used for evaluation of link level signal processing algorithm.

Proposal 12: Reuse the evaluation methodology of link level evaluation in TR 38.830 of Rel-17 coverage enhancement for Rel-18 NR duplex evolution and adopt the evaluation assumptions and metrics in Table 6.

(a) XXXXX                             (b) XXXXU

Fig. 5 Subband configuration for subband non-overlapping full duplex.

Table 6 Link level valuation assumptions under Scenario 1-3.

| Parameter | Value |
| --- | --- |
| Frequency | FR1: 4GHz, 2.6GHz FR2: 28GHz |
| Frame structure | SBFD: XXXXX and XXXXU, where X denotes SBFD slot. |
| Target data rates for eMBB | FR1: DL 10Mbps, UL 1Mbps FR2: DL 25Mbps, UL 5Mbps |
| Packet size for VoIP | A packet size of 320 bits with 20ms data arriving interval is adopted. Size (bits). Payload; 256. CRC; 16 (TBS size lower than 3824 bits). MAC; 16 (with 12 bits SN size). RLC; 8 (with 6 bits SN size). PDCP; 16. RTP/UDP/IP; 24 (w RoHC). If applicable, companies report TB size assumed in evaluation. For SIP invite message - Payload of 1500 bytes can be a starting point. - The assumptions (TB size, time period etc.) are reported by companies. - Contributions R1-2003464 and R1-2005259 are taken into account for the evaluation. - In addition, 1 second time period can also be considered. |
| Latency requirements for VoIP | Latency requirements assumed in VoIP evaluation for SBFD are reported by companies. |
| Pathloss model (select from LoS or NLoS) | LoS, NLoS; Only for FR1. |
| BWP | FR1: 100MHz FR2: 100MHz, [400MHz] |
| Subband configuration for X slot | FR1: UL: 53 RB; DL: 214 RB; GB: 6RB. FR2: UL: 53 RB; DL: 214 RB; GB: 6RB. Note: DL subband of the fourth X in XXXXU: 10D: 2G: 2U. |
| Channel model for link-level simulation | FR1: TDL-C for NLOS, TDL-D for LOS FR2: CDL-A, TDL-A Note: company can provide simulation results based on either TDL channel or CDL model. |
| Delay spread | FR1: 300ns FR2: 100ns |
| UE velocity | 3km/h for indoor |
| Number of antenna elements for BS | FR1: Urban: 192 antenna elements for 4GHz and 2.6GHz, (M,N,P,Mg,Ng) = (12,8,2,1,1) (optional) 128 antenna elements for 4GHz, (M,N,P,Mg,Ng) = (8,8,2,1,1) FR2: 256, (M,N,P,Mg,Ng) = (4, 8, 2, 2, 2) Optional: 512, (M,N,P,Mg,Ng) = (8,8,2,2,2) |
| Number of TxRUs for BS | FR1: gNB architectures to study: - 64TxRUs for 2.6 and 4 GHz. - Optional: 32 TXRUs at 2 GHz gNB modeling in LLS for TDL: - Option 1: 2 or 4 gNB RF chains in LLS. - Option 2 (Optional): Number of gNB RF chains = number of TXRUs in LLS. - Companies can report if and how correlation is modelled. FR2: - 2 Note: Analog beamforming is assumed. |
| Number of UE antenna elements | Only for FR2: 8, one panel: (M, N, P) = (2, 2, 2) |
| Frequency hopping | w/ or w/o frequency hopping |
| BLER | For eMBB, w/ HARQ, 10% iBLER; w/o HARQ, 10% iBLER. For VoIP, 2% rBLER. |
| Number of UE transmit chains | FR1: 1, 2 (optional) FR2: 1T2R, 2T2R |
| DMRS configuration | For 3km/h: Type I, 1 or 2 DMRS symbol, no multiplexing with data. For frequency hopping: Type I, 1 or 2 DMRS symbol for each hop, no multiplexing with data. PUSCH mapping Type, the number of DMRS symbols and DMRS position(s) are reported by companies. |
| Waveform | FR1: DFT-s-OFDM, CP-OFDM (optional) FR2: DFT-s-OFDM |
| SCS | FR1: 30kHz FR2: 120kHz |
| PUSCH duration | 14 OS |
| Repetitions | FR1: For eMBB, w/o repetition as baseline, w/ repetition (optional). For VoIP, w/ type A repetition, optional for type B repetition. The actual number of repetitions is reported by companies. FR2: For eMBB, w/o repetition as baseline, w/ repetition (optional). For VoIP, w/ repetition. The actual number of repetitions is reported by companies. Only PUSCH repetition type A is considered for baseline performance evaluation. - Note: companies are not precluded to report results for repetition type B. |
| HARQ configuration | For eMBB, whether HARQ is adopted is reported by companies. For VoIP, w/ HARQ. The maximum number of HARQ transmission (limited by frame structure and latency requirements) can be reported by companies. |
| PRBs/TBS/MCS for eMBB | FR1: Any value of PRBs, and corresponding MCS index, reported by companies will be considered in the discussion. Companies are encouraged to use 30 PRBs for 1Mbps, 4 PRBs for 100kbps, 1 PRB for 30kbps as a starting point. TBS can be calculated based on e.g. the number of PRBs, target data rate, frame structure and overhead. FR2: Any value of PRBs, and corresponding MCS index, reported by companies will be considered in the discussion. Companies are encouraged to use [30] PRBs for 5Mbps for PUSCH as a starting point. TBS can be calculated based on e.g. the number of PRBs, target data rate, frame structure and overhead. |
| PRBs/MCS for VoIP | FR1: 4 PRBs for VoIP as starting point. Other values of PRBs can be reported by companies. QPSK, pi/2 BPSK (optional). FR2: [4 PRBs] for VoIP as starting point. Other values of PRBs can be reported by companies. QPSK for PUSCH. Optional: pi/2 BPSK for PUSCH. |

    - System level evaluation

The system level evaluation is essential to evaluate the performance of dynamic/flexible TDD enhancement and subband non-overlapping full duplex in different deployment scenarios, such as uplink capacity (uplink average cell throughput) and the coverage (cell edge throughput), downlink performance (due to UE-to-UE CLI and resource reserved for uplink), transmit beamforming for interference suppression, etc.

The system level evaluation methodology is given in ITU-R M.2412 in detail [10], and the channel models under various scenarios are given in TR 38.901 [11] and TR 38.802 [4]. They can be reused to evaluate Rel-18 NR duplex evolution, where the issues of BS-to-BS channel model and UE-to-UE channel model are discussed in section 2.2.2. And then the system level evaluation assumptions are given in Table 7.

Proposal 13: Reuse the system level evaluation methodology in ITU-R M.2412 for Rel-18 NR duplex evolution and adopt the evaluation assumptions and metrics in Table 7 for system level evaluation.

Table 7: System level evaluation assumptions under Scenario 2-1, Scenario 3-3, and Scenario 1-3.

|  | Scenario 2-1 | Scenario 3-3 | Scenario 1-3 |
| --- | --- | --- | --- |
| Layout | Two layers (Macro + Indoor Pico) - Macro: Hexagonal grid, 7 macro sites - Indoor Pico: 18 BSs per 120m × 60m |  | Single layer - Macro: Hexagonal grid, 7 macro sites |
| TRxP number per site | Macro: 3 Indoor Pico: 1 |  | Macro: 3 |
| Inter-site distance (ISD) | Macro: 200m/500m Indoor Pico: 20m |  | Macro: 200m/500m |
| UE distribution | Macro: 80% indoor and 20% outdoor Indoor Pico: 100% indoor |  | Macro: 80% indoor and 20% outdoor |
| UE density | 10 UEs per cell/TRP Randomly and uniformly distributed over the area. |  |  |
| Minimum distance of TRxP and UE (2D) | Macro: 35m Indoor Pico: 0m |  |  |
| Minimum distance of UE and UE (2D) | 3m |  |  |
| UE mobility model | Fixed and identical speed ¦v¦ of all UEs of the same mobility class, randomly and uniformly distributed direction. |  |  |
| UE speed | 3 km/h for indoor and 30 km/h for outdoor |  |  |
| Carrier Frequency | FR1: 4GHz |  | FR1: 4GHz FR2: 30GHz |
| Simulation bandwidth | FR1: 100MHz (273 PRB) |  | FR1: 100MHz (273 PRB); FR2: 200MHz (264 PRB) |
| Subcarrier spacing | FR1: 30kHz |  | FR1: 30kHz FR2: 60kHz |
| Duplex scheme | Flexible/dynamic TDD | Macro: Legacy TDD Indoor Pico: SBFD | Macro: SBFD |
| Frame structure | Macro: DDDSU Indoor Pico: DSUUU | Macro: DDDSU Indoor Pico: XXXXX/XXXXU | Macro: XXXXX/XXXXU |
| Subband configuration | -- |  | FR1: X slot: UL: 53RB; DL: 214RB; GB: 6RB FR2: X slot: UL: 52RB; DL: 206RB; GB: 6RB |
| BS antenna height | Macro: 25m Indoor Pico: 1.5m |  | Macro: 25m |
| Number of antenna elements per TRxP | FR1: Macro: - 192Tx/Rx - (M, N, P, Mg, Ng) = (12, 8, 2, 1, 1) - (dH, dV) = (0.5, 0.8)λ - ±45° polarization Indoor Pico: - 4Tx/Rx - (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) - (dH, dV) = (0.5, 0.5)λ - ±45°polarization |  | FR1: Macro: - 192Tx/Rx - (M, N, P, Mg, Ng) = (12, 8, 2, 1, 1) - (dH, dV) = (0.5, 0.8)λ - ±45° polarization FR2: Macro: - 256Tx/Rx - (M, N, P, Mg, Ng) = (4, 8, 2, 2, 2) - (dH, dV) = (0.5, 0.5)λ - (dg,H, dg,V) = (4.0, 2.0)λ - ±45° polarization |
| Number of TXRU per TRxP | FR1: Macro: - 64TXRU - (Mp, Np, P, Mg, Ng) = (4, 8, 2, 1, 1) Indoor Pico: - 4TXRU - (Mp, Np, P, Mg, Ng) = (1, 2, 2, 1, 1) |  | FR1: Macro: - 64TXRU - (Mp, Np, P, Mg, Ng) = (4, 8, 2, 1, 1) FR2: Macro: - 8TXRU - (Mp, Np, P, Mg, Ng) = (1, 1, 2, 2, 2) |
| BS TX power | FR1: Macro: - 41dBm for 10MHz bandwidth under 200m ISD - 46dBm for 10MHz bandwidth under 500m ISD Indoor Pico: - 21dBm for 10MHz bandwidth under 20m ISD Note: BS TX power is scaled up with simulation bandwidth. |  | FR1: Macro: - 41dBm for 10MHz bandwidth under 200m ISD - 46dBm for 10MHz bandwidth under 500m ISD FR2: Macro: 31dBm for 100MHz bandwidth under 200m ISD Note: BS TX power is scaled up with simulation bandwidth. |
| BS antenna element gain | Macro: 8dBi Indoor Pico: 5dBi |  | Macro: 8dBi |
| BS antenna element gain pattern | According to TR 38.901 |  |  |
| BS noise figure | FR1: 5dB |  | FR1: 5dB FR2: 7dB |
| BS receiver | MMSE-IRC |  |  |
| BS EVM | 32dB |  |  |
| UE antenna height | 1.5m |  |  |
| UE TX power | Maximum 23dBm |  |  |
| Number of UE antenna elements | FR1: - 4Tx/Rx - (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) - (dH, dV) = (0.5, 0.5)λ - 0°, 90° polarization |  | FR1: - 4Tx/Rx - (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) - (dH, dV) = (0.5, 0.5)λ - 0°, 90° polarization FR2: - 32Tx/Rx - (M, N, P, Mg, Ng) = (2, 4, 2, 1, 2) - (dH, dB) = (0.5, 0.5)λ - (dg,V, dg,H) = (0, 0)λ; θmg,ng=90; Ω0,1=Ω0,0+180; - 0°, 90° polarization |
| Number of TXRU per UE | FR1: - 4TXRU - (Mp, Np, P, Mg, Ng) = (1, 2, 2, 1, 1) |  | FR1: - 4TXRU - (Mp, Np, P, Mg, Ng) = (1, 1, 2, 1, 2) |
| UE antenna element gain | FR1: 0dBi |  | FR1: 0dBi FR2: 5dBi |
| UE antenna element gain pattern | FR1: Omnidirectional |  | FR1: Omnidirectional FR2: According to TR 38.901 |
| UE noise figure | FR1: 7dB |  | FR1: 7dB FR2: 10dB |
| UE receiver | MMSE-IRC |  |  |
| UE EVM | 32dB |  |  |
| Thermal noise level | -174 dBm/Hz |  |  |
| Channel model (BS-to-UE link) | Macro: UMa in TR 38.901 Indoor Pico: InF in TR 38.901 |  | Macro: UMa in TR 38.901 |
| Channel model (BS-to-BS link & UE-to-UE link) | FFS: reuse BS-to-UE channel model in TR 38.901 with modifications in TR 38.802 as much as possible, as discussed in section 2.2.2. |  |  |
| Channel estimation | Ideal/Real |  |  |
| Traffic | Burst buffer with FTP traffic model 3 (packet size = 0.1, 0.5, and 2.0 MB) |  |  |
| Traffic load (Resource utilization) | 25, 50, and 80% |  |  |
| MIMO scheme | MU-MIMO Rank adaption; SU: 2, MU: 12 |  |  |
| CSI scheme | Reciprocity based; 4T SRS; 10ms SRS interval |  |  |
| Scheduler scheme | Proportional fair (PF) |  |  |
| Matrices | UL/DL average cell throughput UL/DL cell edge throughput |  |  |

    - Baseline and other simulation assumptions

The evaluation methodologies, assumptions and metrics are discussed in the above sections. However, another important aspect for the evaluation is the baseline setting for each evaluation case. For the baseline choice, two aspects need to be considered; one is the real deployment limitation, and the other is the fairness of the comparison.

For dynamic/flexible TDD enhancement, there is a bunch of the interference handling solutions studied in Rel-14/16, but there are deployment limitations. For example, cancelling the interference from none co-site cells may not be feasible due to large delays and none ideal backhaul. And other solutions require exchange of dynamic information (e.g. user data, dynamic scheduling information and so on) through backhauls may have the same limitation as the interference cancellation.

Another aspect to be considered is the base station antenna configuration. There is antenna configuration antenna model in TR38.901 for evaluation. However, in real deployment, the antenna configurations may be different from the standardized models in TR38.901. For example, different antenna port radiation pattern could be used by different companies due to different implementation. And the antenna port radiation pattern is one of the key factors affecting the cross link interference in the network. Then it should allow companies to use real deployment parameters for the evaluation. Otherwise, the evaluation results do not reflect the reality.

Proposal 14: Realistic deployment limitations should be considered for the baseline of the evaluation, such as the backhaul delay, antenna port radiation pattern, etc.

From system configuration point of view, for subband non-overlapping full duplex, some of the downlink RBs are used for uplink transmission. This will have impact on the performance of the downlink. And on the other hand, the total resource for uplink transmission may be increased considering the unidirectional uplink slot and the uplink subband resources. One baseline for subband full duplex evaluation is the real deployment where DDDSU configurations are used. And another baseline may be DDSUU, where the same total uplink resource in one TDD configuration periodicity is used as illustrated in Fig.6, where baseline 1 is usually deployed in reality, while baseline 2 may be a fair baseline with the same total uplink resource and total downlink resource as the subband full duplex configuration.

Proposal 15: Realistic deployment configurations and fair configurations should be taken into account for the baseline selection, such as legacy TDD with frame structure as DDDSU and DDSUU.

(a) Baseline1                 (b) Baseline 2              (c) Subband full duplex

Fig. 6 Possible baselines and the subband full duplex resource configuration.

  - Link budget results for interference

    - Link budget for Scenario 2-1

For Scenario 2-1, two types of interference are suffered, i.e., in-band BS-to-BS CLI and in-band UE-to-UE CLI. Considering low TX power of UE and high penetration, the in-band UE-to-UE CLI can be negligible compared with DL-to-DL interference. So we only focus on in-band BS-to-BS CLI.

The link budget results for in-band BS-to-BS CLI from Macro cells to small cells is given in Table 8, and meanwhile, the link budget for UE-to-BS uplink signal of small cells are given in Table 9, where 4GHz carrier frequency and 100MHz bandwidth are assumed. It can be observed that the in-band BS-to-BS CLI is comparable with the UE-to-BS uplink signal. The in-band BS-to-BS CLI can be eliminated by applying IRC or SIC receiver at the small cell as discussed in our companion paper.

Observation 6: In Scenario 2-1, in-band BS-to-BS CLI from Macro to small cells is comparable with UE-to-BS uplink signal in small cells.

Proposal 16: Capture the link budget results in Table 8-9 and the following observations into TR 38.858:

- In-band UE-to-UE CLI can be negligible in Scenario 2-1

- Further enhancements are required to suppress the in-band BS-to-BS CLI in Scenario 2-1

Table 8: Initial link budget for in-band BS-to-BS CLI from Macro cells to small cells.

| Distance (m) | 50 | 100 | 200 |
| --- | --- | --- | --- |
| TX power (dBm) | 51 | 51 | 51 |
| TX/RX antenna array gain (dB) | 8/5 | 8/5 | 8/5 |
| Penetration loss (dB) | -28.0 | -28.0 | -28.0 |
| Shadow fading(dB) | -6 | -6 | -6 |
| Path loss outdoor (dB) | -78.4 | -84.3 | -90.7 |
| Path loss indoor (dB) | -2.5 | -2.5 | -2.5 |
| Received interference strength (dBm) | -50.9 | -56.8 | -63.2 |
| Noise power (dBm) with 5dB noise figure | -89 | -89 | -89 |
| Interference to noise ratio (dB) | 38.1 | 32.2 | 25.8 |

Table 9: Initial link budget for UE-to-BS UL signal in small cells.

| Distance (m) | 12 | 8 | 3 |
| --- | --- | --- | --- |
| TX power (dBm) | 23 | 23 | 23 |
| TX/RX antenna array gain (dB) | 0/5 | 0/5 | 0/5 |
| Shadow fading (dB) | -6 | -6 | -6 |
| Path loss (dB) | -66.5 | -62.7 | -53.5 |
| Received UL signal strength (dBm) | -44.5 | -40.7 | -31.5 |
| Noise power (dBm) with 5dB noise figure | -89 | -89 | -89 |
| UL signal to noise ratio (SNR) (dB) | 44.5 | 48.3 | 57.5 |

    - Link budget for Scenario 3-3

For Scenario 3-3, the in-band BS-to-BS CLI and in-band UE-to-UE CLI have been discussed in Scenario 2-1, so we only focus on four types of interference here: inter-subband BS-to-BS CLI, inter-subband UE-to-UE CLI, inter-subband BS self CLI, and blocking issue.

- Inter-subband BS-to-BS CLI

The initial link budget for inter-subband BS-to-BS CLI from small cells to small cells is given in Table 10, where 4GHz carrier frequency and 100MHz bandwidth are assumed. Note that 8 nearest interfering cells around the center cell are considered, as shown in Fig. 7, where D=20m. Comparing Table 9 and 10, the inter-subband BS-to-BS CLI is far less than the uplink signal in small cells. Therefore, a high UL SINR of small cells can be obtained in such a scenario even with inter-subband BS-to-BS CLI.

Observation 7: In Scenario 3-3, inter-subband BS-to-BS CLI from small cells to small cells is far less than UE-to-BS uplink signal in small cells. And therefore, a high UL SINR of small cells can be obtained even with inter-subband BS-to-BS CLI.

Fig. 7 BS deployment for indoor factory.

Table 10: Initial link budget of inter-subband BS-to-BS CLI from small cells to small cells.

| Distance (m) | 20 | 28.3 |
| --- | --- | --- |
| TX power (dBm) | 31 | 31 |
| TX/RX antenna array gain (dB) | 5/5 | 5/5 |
| ACLR | -45 | -45 |
| Shadow fading (dB) | -6 | -6 |
| Path loss (dB) | -71.3 | -74.5 |
| Received interference strength (dBm) | -81.3 | -84.5 |
| Noise power (dBm) with 5dB noise figure | -89 | -89 |
| Interference to noise ratio (dB) | 7.7 | 4.5 |
| Interference to noise ratio of 8 nearest interfering cells (dB) | 15.4 |  |

- Inter-subband UE-to-UE CLI

The initial link budget for inter-subband UE-to-UE CLI and BS-to-UE downlink signal in small cells is given in Table 11. It can be observed that the inter-subband UE-to-UE CLI is much larger than inter-subband BS-to-BS CLI (in Table 10). So the DL SINR will be reduced. But the inter-subband UE-to-UE CLI is still far less than the BS-to-UE downlink signal in small cells, i.e., 30dB SINR. So the DL SINR is still acceptable even with inter-subband UE-to-UE CLI.

Observation 8: In Scenario 3-3, inter-subband UE-to-UE CLI in small cells is much less than BS-to-UE downlink signal in small cells. The DL SINR is acceptable even with inter-subband UE-to-UE CLI.

Table 11: Initial link budget for inter-subband UE-to-UE CLI and BS-to-UE DL signal in small cells.

|  | UE-to-UE CLI | BS-to-UE DL signal |
| --- | --- | --- |
| Distance (m) | 3m | 12m |
| TX power (dBm) | 23 | 31 |
| TX/RX antenna array gain (dB) | 0/0 | 5/0 |
| ACLR (dB) | -30 | -- |
| Shadow fading (dB) | -6 | -6 |
| Path loss (dB) | -53.5 | -66.5 |
| Received interference/ DL signal strength (dBm) | -66.5 | -36.5 |
| Noise power (dBm) with 7dB noise figure | -87 | -87 |
| Interference/ DL signal to noise ratio (dB) | 20.5 | 50.5 |

- Inter-subband BS self CLI

The initial link budget for inter-subband BS self CLI in small cells is given in Table 12. It can be observed that a high inter-subband BS self CLI to noise ratio (30dB) is obtained even with -45dB antenna separation. So further study on inter-subband BS self CLI suppression is needed, such as antenna separation, subband filter, etc.

Observation 9: In Scenario 3-3, it suffers from large inter-subband BS self CLI in small cells.

Table 12: Initial link budget for inter-subband BS self CLI for small cells.

| TX power (dBm) | 31 |
| --- | --- |
| Antenna isolation (dB) | -45 |
| RX power (dBm) | -14 |
| ACLR (dB) | -45 |
| BS self CLI strength (dBm) | -59 |
| Noise power (dBm) with 5dB noise figure | -89 |
| BS self CLI to noise ratio (dB) | 30 |

- Blocking issue

The initial link budget for BS-to-BS blocking and UE-to-UE blocking in small cells is given in Table 13. The in-band blocking requirement for small cell and UE is -35dBm and -55/-44dBm (Case 1/Case 2) as defined in RAN4. Table 13 shows that the linear interference could be as high as -28.6 dBm and -36.5 dBm which is well above the typical blocking signal strength level. So the blocking issue needs to be further studied.

Observation 10: In Scenario 3-3, it suffers from serious BS-to-BS blocking and UE-to-UE blocking.

Table 13: Initial link budget for BS-to-BS blocking and UE-to-UE blocking in small cells.

|  | BS-to-BS blocking |  | UE-to-UE blocking |
| --- | --- | --- | --- |
| Distance (m) | 20m | 28.3m | 3m |
| TX power (dBm) | 31 | 31 | 23 |
| TX/RX antenna array gain (dB) | 5/5 | 5/5 | 0/0 |
| Shadow fading (dB) | -6 | -6 | -6 |
| Path loss (dB) | -71.3 | -74.5 | -53.5 |
| RX power (dBm) | -36.3 | -39.5 | -36.5 |
| RX power (dBm) from 8 nearest interfering cells | -28.6 |  | -- |

As discussed above, the following proposal can be obtained in this scenario.

Proposal 17: Capture the link budget results in Table 10-13 and the following observations into TR 38.858:

- Inter-subband BS-to-BS CLI from small cells to small cells and inter-subband UE-to-UE CLI in small cells can be negligible in Scenario 3-3

- Further enhancements are required to suppress inter-subband BS self CLI in Scenario 3-3

- Further enhancements are required to handle the BS-to-BS blocking and UE-to-UE blocking issue in Scenario 3-3

    - Link budget for Scenario 1-3

For Scenario 1-3, the results for inter-subband UE-to-UE CLI are same as Scenario 3-3. So we only focus on three types of interference: inter-subband BS-to-BS CLI, inter-subband BS self CLI, and blocking issues.

- Inter-subband BS-to-BS CLI

The initial link budget for inter-subband BS-to-BS CLI from Macro cell to Macro cell and UE-to-BS uplink signal in Macro cells is given in Table 14, where 4GHz carrier frequency and 100MHz bandwidth are assumed. Note that 4 nearest interfering cells around the center cell are considered, as shown in Fig. 8. It can be observed that the inter-subband BS-to-BS CLI is comparable with the UE-to-BS uplink signal in Macro cells. So we should study IRC and/or SIC receiver to suppress the interference in this scenario.

Observation 11: In Scenario 1-3, inter-subband BS-to-BS CLI from Macro cells to Macro cells is comparable with UE-to-BS uplink signal in Macro cells.

Table 14: Initial link budget for inter-subband BS-to-BS CLI from Macro cell to Macro cell and UE-to-BS uplink signal in Macro cells.

|  | BS-to-BS CLI | UE-to-BS uplink signal |  |  |
| --- | --- | --- | --- | --- |
| Distance (m) | 200 | 50 | 100 | 150 |
| TX power (dBm) | 51 | 23 | 23 | 23 |
| TX/RX antenna array gain (dB) | 8/8 | 0/8 | 0/8 | 0/8 |
| ACLR (dB) | -45 | -- | -- | -- |
| Shadow fading (dB) | -6 | -6 | -6 | -6 |
| Path loss (dB) | -90.7 | -78.4 | -84.3 | -88.0 |
| Received interference/ UL signal strength (dBm) | -74.7 | -53.4 | -59.3 | -63.0 |
| Noise power (dBm) with 5dB noise figure | -89 | -89 | -89 | -89 |
| Interference / UL signal to noise ratio (dB) | 14.3 | 35.6 | 29.7 | 26 |
| Interference to noise ratio of 4 nearest interfering cells (dB) | 20.3 | -- | -- | -- |

Fig. 8 BS deployment for Macro cells.

- Inter-subband BS self CLI

The initial link budget for inter-subband BS self CLI in Macro cells is given in Table 15, where 4GHz carrier frequency and 100MHz bandwidth are assumed. Similar as Scenario 3-3, the inter-subband BS self CLI is very large to deteriorate the UE-to-BS uplink signal, even with -45dB antenna separation. So we should further study the method to handle the inter-subband BS self CLI in Macro cells in such a scenario, such as antenna separation, subband filter, etc.

Observation 12: In Scenario 1-3, it suffers from large inter-subband BS self CLI in small cells.

Table 15: Initial link budget for inter-subband BS self CLI for Macro cells.

| TX power (dBm) | 51 |
| --- | --- |
| Antenna isolation (dB) | -45 |
| RX power (dBm) | 6 |
| ACLR (dB) | -45 |
| BS self CLI (dBm) | -39 |
| Noise power (dBm) with 5dB noise figure | -89 |
| BS self CLI to noise ratio (dB) | 50 |

- Blocking issue

The initial link budget for BS-to-BS blocking and UE-to-UE blocking in Macro cells is given in Table 16, where 4GHz carrier frequency and 100MHz bandwidth are assumed. Table 16 shows that the linear interference could be as high as -23.7 dBm and -33.5 dBm which is well above the typical blocking signal strength level. So it is a serious issue need us to further study in Scenario 1-3.

Observation 13: In Scenario 1-3, it suffers from serious BS-to-BS blocking and UE-to-UE blocking.

Table 16: Initial link budget for BS-to-BS blocking and UE-to-UE blocking in Macro cells.

|  | BS-to-BS blocking | UE-to-UE blocking |
| --- | --- | --- |
| Distance (m) | 200 | 3 |
| TX power (dBm) | 51 | 23 |
| TX/RX antenna gain (dB) | 8/8 | 0/0 |
| Shadow fading (dB) | -6 | -6 |
| Path loss (dB) | -90.7 | -50.5 |
| RX power (dBm) | -29.7 | -33.5 |
| RX power (dBm) from 4 nearest interfering cells | -23.7 | -- |

As discussed above, the following proposal can be obtained in this scenario.

Proposal 18: Capture the link budget results in Table 14-16 and the following observations into TR 38.858:

- Inter-subband UE-to-UE CLI in Macro cells can be negligible in Scenario 1-3

- Further enhancements are required to handle inter-subband BS-to-BS CLI from Macro cells to Macro cells in Scenario 1-3

- Further enhancements are required to handle inter-subband BS self CLI in Scenario 1-3

- Further enhancements are required to handle the BS-to-BS blocking and UE-to-UE blocking issue in Scenario 1-3

- Conclusions

In this contribution, we provide our views on evaluation on NR duplex evolution with following proposals:

Observation 1: Boosted uplink capability in local area (indoor factory or hotspot etc.) is the dominant benefit for dynamic/flexible TDD enhancement.

Observation 2: Low latency is the dominant benefit for subband non-overlapping full duplex under the factory/industry deployment scenario.

Observation 3: Better uplink coverage is the dominant benefit for subband non-overlapping full duplex under the Macro deployment scenario.

Observation 4: The cross link interference can be classified as linear and non-linear interference, where downlink signals from own cell or other cells may lead to blocking, and BS-to-BS CLI, UE-to-UE CLI, and BS self CLI have impact on demodulation performance.

Observation 5: The LOS probability of BS-to-BS link determined by reusing BS-to-UE channel model in TR 38.901 with modifications in TR 38.802 is much lower than that in the realistic network. It will lead to that the BS-to-BS CLI strength level in the evaluation is much lower than that in the actual network.

Observation 6: In Scenario 2-1, in-band BS-to-BS CLI from Macro to small cells is comparable with UE-to-BS uplink signal in small cells.

Observation 7: In Scenario 3-3, inter-subband BS-to-BS CLI from small cells to small cells is far less than UE-to-BS uplink signal in small cells. And therefore, a high UL SINR of small cells can be obtained even with inter-subband BS-to-BS CLI.

Observation 8: In Scenario 3-3, inter-subband UE-to-UE CLI in small cells is much less than BS-to-UE downlink signal in small cells. The DL SINR is acceptable even with inter-subband UE-to-UE CLI.

Observation 9: In Scenario 3-3, it suffers from large inter-subband BS self CLI in small cells.

Observation 10: In Scenario 3-3, it suffers from serious BS-to-BS blocking and UE-to-UE blocking.

Observation 11: In Scenario 1-3, inter-subband BS-to-BS CLI from Macro cells to Macro cells is comparable with UE-to-BS uplink signal in Macro cells.

Observation 12: In Scenario 1-3, it suffers from large inter-subband BS self CLI in small cells.

Observation 13: In Scenario 1-3, it suffers from serious BS-to-BS blocking and UE-to-UE blocking.

Proposal 1: To choose the deployment scenarios for Rel-18 NR duplex evolution, the industrial interest is an important factor that should be taken into account.

Proposal 2: Scenario 2-1 should be studied with a high priority for dynamic/flexible TDD enhancement.

- Scenario 2-1: Macro with DL dominant TDD configuration and Pico with UL dominant TDD configuration at local area (FR1 only).

Proposal 3: Scenario 3-3 should be studied with a high priority for subband non-overlapping full duplex.

- Scenario 3-3: Factory/industry Pico with subband non-overlapping full duplex (and potential Macro with DL dominant TDD configuration) (FR1 only).

Proposal 4: Scenario 1-3 should be studied with a high priority for subband non-overlapping full duplex.

- Scenario 1-3: Macro with subband non-overlapping full duplex with same resource configurations (FR1 and FR2).

Proposal 5: The following deployment scenarios can be studied after the evaluation of other scenarios:

- Scenarios 4: Macro with subband non-overlapping full duplex and Macro with legacy TDD configuration on the same carrier (co-channel co-existence with legacy base stations scenarios);

- Scenarios 5: Macro with subband non-overlapping full duplex and Macro with legacy TDD configuration on the adjacent carriers (adjacent-channel co-existence or inter-operator co-existence scenarios).

Proposal 6: The following interferences will impact the system performance and should be studied in Rel-18 NR duplex evolution:

- Linear interference

  - In-band BS-to-BS CLI

  - In-band UE-to-UE CLI

  - Blocking caused by DL/UL signal

- Non-linear interference

  - Inter-subband BS-to-BS CLI

  - Inter-subband UE-to-UE CLI

  - Inter-subband BS self CLI

Proposal 7: The interference models of in-band BS-to-BS CLI and in-band UE-to-UE CLI used in TR 38.802 can be reused for Rel-18 NR duplex evolution.

Proposal 8: The following aspects for non-linear interference modeling should be studied in Rel-18 NR duplex evolution:

  - Strength of non-linear interference (RAN4)

  - Correlation of non-linear interference between antenna ports

    - Non-linear interference may be modeled as uncorrelated signals

    - Non-linear signal on each transmit antenna port can be modeled as Gaussian distribution, i.e., beamforming should not be considered in the modeling of non-linear interference

  - Channel for non-linear interference

    - Both large fading and fast fading should be modeled

Proposal 9: Reuse the existing BS-to-UE channel model in TR 38.901 and TR 38.802 as much as possible to determine the BS-to-BS channel and UE-to-UE channel in Rel-18 NR duplex evolution.

- FFS: the parameters, e.g., LOS probability, delay spread, angle spread, etc., in the BS-to-UE channel model should be checked whether they are applicable to the BS-to-BS channel model and UE-to-UE channel model.

Proposal 10: At least the following evaluation methodologies should be considered to evaluate Rel-18 NR duplex evolution:

- Link budget analysis

- Interference strength evaluation for Scenario 2-1, Scenario 3-3, and Scenario 1-3

- Coverage evaluation for Scenario 1-3

- Link level evaluation

- Coverage evaluation for Scenario 1-3

- Interference suppression evaluation for Scenario 2-1, Scenario 3-3, Scenario 1-3

- System level evaluation

- Coverage and capacity evaluation for Scenario 2-1, Scenario 3-3, and Scenario 1-3

Proposal 11: Reuse the evaluation methodology, assumptions, and metrics of link budget analysis in IMT-2020 self-evaluation and/or TR 38.830 of Rel-17 NR coverage enhancement for Rel-18 NR duplex evolution.

Proposal 12: Reuse the evaluation methodology of link level evaluation in TR 38.830 of Rel-17 coverage enhancement for Rel-18 NR duplex evolution and adopt the evaluation assumptions and metrics in Table 6.

Proposal 13: Reuse the system level evaluation methodology in ITU-R M.2412 for Rel-18 NR duplex evolution and adopt the evaluation assumptions and metrics in Table 7 for system level evaluation.

Proposal 14: Realistic deployment limitations should be considered for the baseline of the evaluation, such as the backhaul delay, antenna port radiation pattern, etc.

Proposal 15: Realistic deployment configurations and fair configurations should be taken into account for the baseline selection, such as legacy TDD with frame structure as DDDSU and DDSUU.

Proposal 16: Capture the link budget results in Table 8-9 and the following observations into TR 38.858:

- In-band UE-to-UE CLI can be negligible in Scenario 2-1

- Further enhancements are required to suppress the in-band BS-to-BS CLI in Scenario 2-1

Proposal 17: Capture the link budget results in Table 10-13 and the following observations into TR 38.858:

- Inter-subband BS-to-BS CLI from small cells to small cells and inter-subband UE-to-UE CLI in small cells can be negligible in Scenario 3-3

- Further enhancements are required to suppress inter-subband BS self CLI in Scenario 3-3

- Further enhancements are required to handle the BS-to-BS blocking and UE-to-UE blocking issue in Scenario 3-3

Proposal 18: Capture the link budget results in Table 14-16 and the following observations into TR 38.858:

- Inter-subband UE-to-UE CLI in Macro cells can be negligible in Scenario 1-3

- Further enhancements are required to handle inter-subband BS-to-BS CLI from Macro cells to Macro cells in Scenario 1-3

- Further enhancements are required to handle inter-subband BS self CLI in Scenario 1-3

- Further enhancements are required to handle the BS-to-BS blocking and UE-to-UE blocking issue in Scenario 1-3

# References

- RP-213591, “New SI: Study on evolution of NR duplex operation.” Dec 6th – 17th, 2021.

- “Values of 5G Uplink Capabilities in Industry Digitalization” in Chinese, CMCC, Huawei, October, 2020.

- 3GPP TS 22.261: “Service requirements for the 5G system.”

- 3GPP TR 38.802: “Study on new radio access technology,” Jun, 2017.

- Q. Luo et al., "Linearization Angle Widened Digital Predistortion for 5G MIMO Beamforming Transmitters," in IEEE Transactions on Microwave Theory and Techniques, vol. 69, no. 11, pp. 5008-5020, Nov. 2021.

- 3GPP TR 38.901: “Study on channel model for frequencies from 0.5 to 100 GHz,” Oct, 2019.

- RP-181814, “Consideration on IMT-2020 link budget,” RAN#81, Gold Coast, Australia, 10 - 13 September 2018.

- 3GPP TR 38.830, “Study on NR coverage enhancements,” Dec, 2020.

- 3GPP TR 38.828, “Cross link interference (CLI) handling and remote interference management (RIM) for NR,” Jun, 2019.

- ITU-R M.2412-0, “Guidelines for evaluation of radio interface technologies for IMT-2020,” Oct, 2017.

- 3GPP TR 38.901: “Study on channel model for frequencies from 0.5 to 100 GHz,” Oct, 2019.