3GPP TSG-RAN WG1 Meeting #109-e	R1-2203157

e-Meeting, May 9 – 20, 2022

Agenda Item:	9.3.2

Source:	Huawei, HiSilicon

Title:	Discussion on subband non-overlapping full duplex

Document for:	Discussion and Decision

- Introduction

In RAN#94-e [1], a new SID to support duplex evolution for NR TDD in unpaired spectrum was approved:

The objective of this study is to identify and evaluate the potential enhancements to support duplex evolution for NR TDD in unpaired spectrum. In this study, the followings are assumed: - Duplex enhancement at the gNB side - Half duplex operation at the UE side - No restriction on frequency ranges The detailed objectives are as follows: - Identify applicable and relevant deployment scenarios (RAN1). - Develop evaluation methodology for duplex enhancement (RAN1). - Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4). - Identify possible schemes and evaluate their feasibility and performances (RAN1). - Study inter-gNB and inter-UE CLI handling and identify solutions to manage them (RAN1). - Consider intra-subband CLI and inter-subband CLI in case of the subband non-overlapping full duplex. - Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1). - Study the feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4). - Study the feasibility of and impact on RF requirements considering the self-interference, the inter-subband CLI, and the inter-operator CLI at gNB and the inter-subband CLI and inter-operator CLI at UE (RAN4). - Note: RAN4 should be involved early to provide necessary information to RAN1 as needed and to study the feasibility aspects due to high impact in antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression. - Summarize the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4). Note: For potential enhancements on dynamic/flexible TDD, utilize the outcome of discussion in Rel-15 and Rel-16 while avoiding the repetition of the same discussion.

In this contribution, we discuss the key issues of subband non-overlapping full duplex and provides some potential solutions to enable the operation of subband non-overlapping full duplex. The impact on legacy operation are also analyzed.

- Discussion on subband non-overlapping full duplex

  - General analysis on the potential interference handling

As analyzed in our contribution [2], several types of the interference should be considered in subband non-overlapping full duplex, as shown in Fig. 1. And the corresponding potential techniques are summarized in Table 1.

Fig. 1 Interference analysis for subband non-overlapping full duplex.

Table 1: Interference type and potential solutions.

| Interference type |  | Potential solutions |
| --- | --- | --- |
|  | - Intra-operator Co-sector BS nonlinear self-interference | - Antenna isolation - Interference cancellation in analogue and digital domain - IRC receiver - Guard band |
|  | - Intra-operator Co-sector BS blocking self-interference | - Antenna isolation - Interference cancellation in analogue and digital domain - TX beamforming - RX analogue filter - Beam pairing (for mmWave) |
|  | - Intra-operator Co-site BS-BS cross link interference nonlinear interference | - Antenna isolation - Interference cancellation in analogue and digital domain - IRC receiver - Guard band |
|  | - Intra-operator Co-site BS-BS cross link interference blocking interference | - Antenna isolation - Interference cancellation in analogue and digital domain - TX beamforming - RX analogue filter - Beam pairing (for mmWave) |
|  | - Intra-operator Non-co-site BS-BS cross link interference nonlinear interference | - IRC receiver - Guard band |
|  | - Intra-operator Non-co-site BS-BS cross link interference blocking interference | - TX beamforming - RX analogue filter - Beam pairing (for mmWave) |
|  | - Inter-operator Co-site BS-BS cross link interference nonlinear interference | - Antenna isolation - IRC receiver - Guard band |
|  | - Inter-operator Co-site BS-BS cross link interference blocking interference | - RX analogue filter - Inter-operator cooperation |
|  | - Inter-operator Non-co-site BS-BS cross link interference nonlinear interference | - Antenna isolation - IRC receiver - Guard band |
|  | - Inter-operator Non-co-site BS-BS cross link interference blocking interference | - RX analogue filter - Inter-operator cooperation |

From Table 1, it can be observed that different potential candidate solutions can be considered for each type of interference. Among the candidate solutions, beamforming, IRC receiver, interference cancellation and RX analogue filter are almost common solutions for all types of interference.

From RAN1 point of view, the feasibility and performance of advanced IRC receiver and TX beamforming can be prioritized to be studied further. In addition, beam coordination can be considered for FR2, e.g. avoid using beam pairs that are mutually interfering heavily to each other.

The rest of interference handling schemes such as antenna isolation, interference cancellation (analogue and digital domain), RX analogue filter and guard band can be studied in RAN4 and the performance of these interference suppression and/or mitigation techniques should be provided to RAN1 to check the feasibility as well as the performance of subband non-overlapping full duplex.

Proposal 1: The feasibility and performance of advanced IRC receiver, TX beamforming, and beam pairing (for FR2) should be prioritized to be studied in RAN1 for Rel-18 NR duplex evolution.

Proposal 2: Antenna isolation, interference cancellation (analogue and digital domain), RX analogue filter and guard band should be studied in RAN4 and the performance of these interference suppression and/or mitigation techniques should be provided to RAN1 to check the feasibility as well as the performance of subband non-overlapping full duplex.

  - BS-to-BS CLI management

    - Interference suppression based on advanced receiver

As discussed in [2], advanced receiver (e.g., IRC and SIC receiver) is a promising solution to suppress the (inter-subband) BS-to-BS CLI. The performance of the advanced receivers is highly dependent on the accuracy of channel estimation and interference covariance matrix estimation. However, it is difficult, if not impossible, to achieve accurate channel and interference measurement if there is strong interference on the measurement resources.

To measure the UL and DL channel without being contaminated by (inter-subband) BS-to-BS CLI and (inter-subband) UE-to-UE CLI, the BS-to-BS CLI and UE-to-UE CLI should be managed not to affect the channel estimation on UL DMRS and DL DMRS, respectively. So one potential way is to use orthogonal UL DMRS and DL DMRS in time domain. For example, the DL DMRS and UL DMRS are allocated in the third and fourth symbol, as shown in Fig. 2, and all the REs in the third symbol in uplink subband and all the REs in the fourth symbol in downlink subband are muted. So the uplink and downlink channel estimation will not be affected by BS-to-BS CLI and UE-to-UE CLI. Meanwhile, the uplink and downlink inter-cell interference can also be measured in the UL DMRS and DL DMRS with a high precision.

To measure the BS-to-BS CLI without being contaminated by uplink interferences from UEs in other cells, the uplink interferences should be managed not to affect the CLI estimation. One potential way is to define specific muting resources for BS-to-BS CLI measurement. Since the uplink signal will not be transmitted on these muting resources from UE, and only BS-to-BS CLI is present on these muting resources, so the BS-to-BS CLI can be accurately measured. An example is shown in Fig. 2. Some REs in the first/fifth symbol in uplink subband are muted, and gNB can thus measure the PDCCH CLI and PDSCH CLI in these muting REs with a high precision.

Proposal 3: Study the following aspects related to advanced IRC receivers in subband non-overlapping full duplex.

- Feasibility and performance of muting symbols for UL DMRS and DL DMRS to improve channel estimation and inter-cell interference estimation and suppression.

- Feasibility and performance of specific CLI measurement resources to improve inter-subband BS-to-BS CLI estimation and suppression.

Fig. 2 Interference management for interference suppression based on advanced receiver.

    - Interference suppression based on transmit beamforming

As analyzed in [2], there is a potential blocking issue when the receiver is using wideband filter. And the blocking may cause severe demodulation performance degradation at the receiver (there is a corresponding in-band block requirement in RAN4). Assuming that a wideband filter is used to receive the uplink signal on the subband non-overlapping full duplex slots, where the downlink signal will fall into the receiving bandwidth of the uplink receiver, as shown in Fig. 3. Although the downlink signal and the uplink desired signal are in different RBs, the strength of the downlink signal is much higher than the uplink desired signal, and the strong downlink signal will have blocking effect to degrade the receiver performance.

To suppress the downlink blocking signal, downlink beamforming considering the interference to other BS could be used, e.g. if the channel between transmitter and receiver can be measured, the DL beamforming weights can be manipulated to avoid transmitting in the direction of the receiver.

Proposal 4: Study the feasibility and performance of downlink beamforming in subband non-overlapping full duplex to avoid blocking issues caused by downlink signal transmitting from the aggressor in the direction of the victim.

Fig. 3 Illustration of the downlink signal and uplink signal assuming wideband uplink front end filter.

    - Interference suppression based on analogue filter

Analogue filters are efficient to suppress both the linear and non-linear interference at the cost of additional hardware complexity. Two kinds of analogue filters may be applicable, one is the filter at the transmitter and the other is the filter at the receiver. For the filter at the transmitter, it can help to reduce the leakage interference to the uplink subband. For the filter at the receiver, it can help to reduce the blocking interference, so that the downlink signal on the downlink RBs will not fall into the receiving bandwidth, and the potential blocking issues can be alleviated. However, additional guard band will be needed for the filter, and the amount of the guard band may degrade the overall system performance. This study should be started by RAN4, and the performances should be provided to RAN1 to check the feasibility as well as the performance of subband non-overlapping full duplex.

Proposal 5: Study the feasibility and performance of applying filters at both transmitter and receiver sides in subband non-overlapping full duplex involving RAN4 on the following aspects.

- Filter at transmitter to suppress the leakage interference.

- Filter at receiver to suppress the blocking interference.

- Guard band for filters.

    - Interference management for inter-slot interference

In legacy TDD system, UE is provided a value NTA, offset, a timing advance offset for a serving cell. It is used to reserve enough time for UL/DL switching, as shown in Fig. 4(a), where DL/UL switching is done during the GP in S slot.

In case of subband non-overlapping full duplex, from the base station point of view, a non-zero NTA, offset will result in inter-slot interference. As shown in Fig. 4(b), the UL signals in UL slot and DL signals in DL slot are interfered by each other. To avoid the inter-slot interference, one potential solution is to configure NTA, offset=0.

Proposal 6: For subband non-overlapping full duplex, the timing advance offset NTA, offset can be configured as 0 to avoid the inter-slot interference.

(a) Legacy TDD.

(b) Subband non-overlapping full duplex.

Fig. 4 Inter-slot interference for subband ono-overlapping full duplex.

    - Other interference suppression solutions

- Antenna isolation

To reduce the self-interference between the transmitter and the receiver at the base station, antenna isolation is simple and effective. The transmit antenna and receive antenna may be separated for better isolation. This will require two antenna or two antenna arrays to transmit and receive respectively. The drawback is that the cost of the BS may be increased.

- Interference cancellation

Interference cancellation can be used for the self-interference suppression because it is easier to obtain the original interference signal when the transmitter and receiver are co-located. Interference cancellation may be difficult for non-co-located aggressor and victims. The interference cancellation may be performed at RF part and/or digital part for different base station type. For example, for small cells with a small number of antennas, the RF analogue interference cancellation could be applied, while for a Macro base station equipped with 64 antenna ports, analogue interference cancellation may not be feasible due to extremely high complexity. The interference cancellation may not have any specification impact, since the transmitter and receiver belong to the same cell or co-site cells, no other cell or UEs are involved. However, some requirements may need to be defined to guarantee the demodulation performance due to the self-interference from the co-site sectors.

- Advanced DPD

To reduce the nonlinear interference, DPD may be enhanced so that the nonlinear interference from the DL RBs to the UL RBs can be reduced.

Proposal 7: For self-interference handling, the requirement of the implementation based solutions should be defined.

  - UE-to-UE CLI management

In Rel-16, UE-to-UE CLI measurement was standardized based on L3 measurement and reporting. The standardized UE-to-UE CLI measurement assume that the aggressor UE and the victim UE are in the adjacent cell, and then there will be not too many UEs generating severe interference to other UEs.

In subband non-overlapping full duplex scenario, the victim UE and the aggressor UE are in the same cell, i.e. the severe CLI will happen more frequently than dynamic/flexible TDD case. And this will require much more UE-to-UE CLI measurement resources and frequent measurement report. L3 UE-to-UE CLI measurement will be too slow and consumes a lot of time-frequency resources. One alternative is to introduce a L1/L2 based UE-to-UE CLI measurement, e.g. the measurement resources can be scheduled triggered dynamically, so that less measurement resources will be required than L3 based CLI measurement and reporting.

Proposal 8: Study L1/L2 based UE-to-UE CLI measurement solutions in subband non-overlapping full duplex.

  - Inter-operator co-existence and UL/DL subband allocation

According to the SID, the feasibility of subband non-overlapping full duplex operation considering inter-operator co-existence should be studied. Although adjacent frequency CLI could be suppressed by filtering (ACLR and ACS), extensive studies have shown that inter-operator adjacent frequency CLI can be a limiting factor in NR duplex operation [6].

To solve this issue, a potential solution is to utilize a flexible UL/DL subband allocation to avoid adjacent frequency CLI among operators. As shown in Fig. 5 (a), there are three adjacent frequency bands belonging to three different operators, i.e., operator C, A, and B from left to right, respectively. Operator A operates with subband non-overlapping full duplex, and operator B and C perform downlink transmission. To avoid strong adjacent frequency CLI from operator B and C to operator A , it is better to configure the uplink subband in the middle of the frequency band of operator A. Similarly, if only one frequency band is located neighboring to operator A, e.g., operator C, as shown in Fig. 5(b), it is better not to configure the uplink subband in the left side of the frequency band of operator A. And if there is no other frequency band located neighboring to operator A, as shown in Fig. 5(c), the uplink subband can be configured in the both sides or in the middle of the frequency band.

Regarding to the UL/DL subband allocation for subband non-overlapping full duplex, several issues should be studied.

- The number of UL/DL subband

In our view, three subband (two UL subband and one DL subband, or one UL subband and two DL subband) are sufficient in one frequency band. If the number of subband is too large, the design of BS and UE will be complex, e.g., more filters should be used at transmitter to suppress the leakage interference, and more filters should be used at receiver to suppress the blocking interferences.

- The start position and the bandwidth of each UL/DL subband

The start position and the bandwidth of each UL/DL subband should be configurable to support inter-operator co-existence as discussed above. And it is sufficient to provide several candidate starting positions and subband bandwidths for different bandwidth of frequency band.

Proposal 9: Study flexible UL/DL subband allocation in subband non-overlapping full duplex to support inter-operator co-existence, considering at least the number of UL/DL subband, the start position and the bandwidth of each UL/DL subband.

(a)

(b)

(c)

Fig. 5 Flexible UL/DL subband allocation for inter-operator co-existence.

  - Co-existence with legacy UE

For legacy UEs, the most conservative way is to schedule legacy UEs in the pure UL symbols, so that its behavior is just the same as that in a traditional TDD cell. Alternatively, the legacy UEs can also be scheduled in the UL subband in a “full duplex slot”, e.g. the “full duplex slot” is configured as flexible and the legacy UEs are scheduled dynamic in the flexible slot. For example, when the downlink traffic load is low, the CLI may not a big issue. Furthermore, solutions like the TX beamforming are still valid for legacy UE operation in the UL subband of a full duplex slot.

Proposal 10: Legacy UEs can be scheduled either in pure UL symbols or in the UL subband in the subband non-overlapping full duplex without additional specification impact.

  - UE half-duplex operation

According to the SID, the UE is supposed to be operating in half duplex mode. In Rel-15, how to handle the different UL/DL indicating signaling for the same OFDM symbol are defined for single carrier, e.g. the same symbol cannot be indicated as uplink symbol by scheduling grant if this symbol is configured as downlink symbol by tdd-UL-DL-ConfigurationCommon in SIB1.

In a Rel-16 TEI, the UE behavior is specified for UEs not capable of simultaneous UL transmission and DL reception on the same symbol on different TDD carriers. In the TEI, a UE capability to handle the conflict signaling of UL/DL indication of the same symbol is defined, as well as the UE behavior to decide the OFDM as downlink or uplink when conflict signaling is indicated in different cells for the same OFDM symbol. In subband non-overlapping full duplex, conflict signaling will happen, how to handle the conflict signaling should also be studied.

Proposal 11: UE half-duplex on handling conflict UL/DL indicating signaling for the same OFDM symbol should be studied in the subband non-overlapping full duplex.

- Conclusions

In this contribution, we provide our views on subband non-overlapping full duplex with following proposals:

Proposal 1: The feasibility and performance of advanced IRC receiver, TX beamforming, and beam pairing (for FR2) should be prioritized to be studied in RAN1 for Rel-18 NR duplex evolution.

Proposal 2: Antenna isolation, interference cancellation (analogue and digital domain), RX analogue filter and guard band should be studied in RAN4 and the performance of these interference suppression and/or mitigation techniques should be provided to RAN1 to check the feasibility as well as the performance of subband non-overlapping full duplex.

Proposal 3: Study the following aspects related to advanced IRC receivers in subband non-overlapping full duplex.

- Feasibility and performance of muting symbols for UL DMRS and DL DMRS to improve channel estimation and inter-cell interference estimation and suppression.

- Feasibility and performance of specific CLI measurement resources to improve inter-subband BS-to-BS CLI estimation and suppression.

Proposal 4: Study the feasibility and performance of downlink beamforming in subband non-overlapping full duplex to avoid blocking issues caused by downlink signal transmitting from the aggressor in the direction of the victim.

Proposal 5: Study the feasibility and performance of applying filters at both transmitter and receiver sides in subband non-overlapping full duplex involving RAN4 on the following aspects.

- Filter at transmitter to suppress the leakage interference.

- Filter at receiver to suppress the blocking interference.

- Guard band for filters.

Proposal 6: For subband non-overlapping full duplex, the timing advance offset NTA, offset can be configured as 0 to avoid the inter-slot interference.

Proposal 7: For self-interference handling, the requirement of the implementation based solutions should be defined.

Proposal 8: Study L1/L2 based UE-to-UE CLI measurement solutions in subband non-overlapping full duplex.

Proposal 9: Study flexible UL/DL subband allocation in subband non-overlapping full duplex to support inter-operator co-existence, considering at least the number of UL/DL subband, the start position and the bandwidth of each UL/DL subband.

Proposal 10: Legacy UEs can be scheduled either in pure UL symbols or in the UL subband in the subband non-overlapping full duplex without additional specification impact.

Proposal 11: UE half-duplex on handling conflict UL/DL indicating signaling for the same OFDM symbol should be studied in the subband non-overlapping full duplex.

# References

- RP-213591, “New SI: Study on evolution of NR duplex operation.” Dec 6th – 17th, 2021.

- R1-2203156, “Overview of evaluation on NR duplex evolution.” May 9th – May 20th, 2022.

- 3GPP TR 38.901: “Study on channel model for frequencies from 0.5 to 100 GHz.” Oct, 2019.

- 3GPP TR 38.802: “Study on new radio access technology.” Jun, 2017.

- R1-2203157, “Discussion on subband non-overlapping full duplex.” May 9th – May 20th, 2022.

- RP-212428, “On evolution of duplex in Rel-18.” Sep 13th – Sep 17th, 2021.