3GPP TSG-RAN WG1 Meeting #109-e	R1-2203158

e-Meeting, May 9 – 20, 2022

Agenda item:	9.3.3

Source:	Huawei, HiSilicon

Title:	On potential enhancements on dynamic/flexible TDD

Document for:	Discussion and decision

Introduction

In RAN#94-e, a new SID to support duplex evolution for NR TDD in unpaired spectrum was approved [1]:

The objective of this study is to identify and evaluate the potential enhancements to support duplex evolution for NR TDD in unpaired spectrum. In this study, the followings are assumed: - Duplex enhancement at the gNB side - Half duplex operation at the UE side - No restriction on frequency ranges The detailed objectives are as follows: - Identify applicable and relevant deployment scenarios and use cases (RAN1). - Develop evaluation methodology for duplex enhancement (RAN1). - Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD. - Identify possible schemes and evaluate their feasibility and performances (RAN1). - Study inter-gNB and inter-UE CLI handling and identify solutions to manage them (RAN1). - Study their impacts on inter-gNB interfaces if needed (RAN3). - Consider intra-subband CLI and inter-subband CLI in case of the subband non-overlapping full duplex. - Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1). - Study the impact on RF requirements considering the self-interference, the inter-subband CLI, and the inter-operator CLI at gNB and the inter-subband CLI and inter-operator CLI at UE (RAN4). - Study the impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4). - RAN4 should be involved early to provide necessary information to RAN1 as needed and to study the feasibility aspects due to high impact in antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression. - Summarize the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4). Note: For potential enhancements on dynamic/flexible TDD, utilize the outcome of discussion in Rel-15 and Rel-16 while avoiding the repetition of the same discussion.

In this contribution, we discuss the potential enhancements on dynamic/flexible TDD.

Study on potential enhancements on dynamic/flexible TDD

As discussed in [2, 3, 8], Macro with DL dominant TDD configuration and small cell with UL dominant TDD configuration at local area is one typical deployment scenario which has drawn a lot of interest from the operators. In this scenario, the main challenge is the cross link interference from the Macro base station to the small cells. Therefore, in this contribution we discuss the potential enhancements to suppress gNB-to-gNB cross link interference. In addition, the potential enhancements to handle UE-to-UE interference are also discussed.

Studied solutions for flexible duplex up to Rel-16

During the NR study item phase [6], the feasibility and performance of flexible duplex was studied and a number of cross link interference handling technologies were proposed and evaluated, including advanced receiver, hybrid dynamic/static UL/DL resource assignment, scheduling coordination, beam coordination, power control, link adaptation, and sensing etc. The enablers of these technologies are listed as bellows.

- Advanced receiver, the enabler includes

- Symmetric/Common (DMRS) RS design for DL and UL.

  - Information exchange among TRPs (e.g., DMRS port/pattern/sequence/location), i.e., use DMRS as RS for CLI measurement.

  - Signaling/mechanism to facilitate interference cancellation and cross-link timing alignment.

- Hybrid dynamic/static UL/DL resource assignment, the enabler include

  - Implementation based method.

- Coordination including scheduling coordination, beam coordination, and the enabler include

  - Specific RS, for example, CSI-RS.

  - Backhaul/ X2/ OTA signaling to exchange information (e.g., DL/UL transmission direction, resource allocation).

- Power control, the enabler include

  - Information exchange among TRPs.

  - Interference-aware power control.

- Link adaptation, the enabler include

  - DL and UL CLI measurement.

- Sensing scheme, the enabler include.

  - Sensing based CLI measurements/Busy tone for interference measurement.

  - Semi-Static/dynamic OTA measurement.

  - LBT-based CLI avoidance.

Overall, some solutions are implementation based schemes while others may require information exchange among the base stations. Due to the limited time in Rel-15/16, UE-to-UE CLI was prioritized and gNB-to-gNB CLI was de-prioritized. As a result, no sophisticated gNB-to-gNB CLI handling technologies were standardized except that an intended TDD configuration can be exchanged between gNBs in Rel-15 and Rel-16. This is not adequate to enable efficient dynamic/flexible TDD operation for indoor factory deployments, especially when coordination between the aggressor gNB and the victim gNB is not possible due to the non-ideal backhaul. There is a similar observation from NGMN and an LS is sent to 3GPP requesting additional solutions [7]. Thus, there is a need to study BS-to-BS CLI handling technologies further for dynamic/flexible TDD in Rel-18.

Possible interference handling solutions

In this section, we discuss the potential schemes and specification impact for the gNB-to-gNB CLI handling at the receiver and transmitter.

gNB-to-gNB interference handling at the receiver

To study the gNB-to-gNB CLI, we consider a Macro base station with DL-dominant TDD configuration (e.g., DDDSU) deployed outside the factory with indoor small cells with UL-dominant TDD configuration (e.g., DSUUU) is shown in Figure 1.

Figure 1：The received signals at the indoor small cell

The receiving signal of the small cells can be formulated as follows for the considered scenario.

| Y=HkSk | The target user’s uplink signal |
| --- | --- |
| +n≠kHnSn | Other target users’ uplink signal |
| +HgSg | Adjacent cells users’ uplink interference |
| +HCLIWmSm | The gNB-to-gNB interference |
| +N | Noise |

where,

- Y is the received signal at the receiver.

- Hk is the channel between the target UL user and small cell.

- Hn is the channel between other target UL users and small cell.

- Hg is the channel between adjacent small cells UL users and small cell.

- HCLI is the channel between Macro cell and small cell.

- Sk, Sn, Sg, and Sm are the target user’s uplink signal, other target users’ uplink signal, adjacent small cells users’ uplink interference, and macro cell downlink interference.

- Wm is the precoding of the downlink interference.

- N is noise, which consists of thermal noise and noise figure.

With MMSE-IRC receiver, the equalization at the UL receiver can be expressed as follows

s k=HkHHkHkH+n≠kHnHnH+RuuUL,  inter-cell+RuuDL,   CLI+σ2I-1Y

where,

- s k is the estimated signal.

- RuuUL,  inter-cell is the covariance of the interference introduced adjacent cells users’ uplink interference.

- RuuDL,   CLI is the covariance of the interference introduced by DL gNB-to-gNB CLI.

- σ2I is the covariance of noise, σ2 is the variance of noise, and I is unit matrix.

- R = HkHkH+n≠kHnHnH+RuuUL,  inter-cell+RuuDL,   CLI+σ2I is the covariance matrix including the interference.

From the analysis of the IRC receiver, it can be observed that, the demodulation performance is highly related to the accuracy of the desired uplink signal channel estimation and the interference estimation, wherein the interference includes both the UL interference from UEs in other cells and the gNB-to-gNB CLI.

As the interference covariance matrix (RuuUL,  inter-cell+RuuDL,   CLI+σ2I) is essential for the performance of IRC receiver, it is preferred that the interference covariance can be estimated as accurate as possible, i.e. the CLI RuuDL,   CLI should be accurately measured. Meanwhile, the channel estimation of the desired signal for the target users Hk  and the UL inter-cell interference n≠kHnHnH, should not be effected by gNB-to-gNB CLI.

As shown in Figure 2, the DMRS of the uplink desired signal and the downlink cross link interference can be in the same symbol, the UL channel estimation accuracy will impacted. To improve the channel estimation accuracy of the desired signal for the target user, the Macro cell can blank the DL resources overlapping with the UL DMRS. Then, the estimation of Hk and n≠kHnHnH would not be effected by gNB-to-gNB CLI. Note that DL blanking resource is already supported in current NR specification. With DL blanking resource configured at the Macro, RuuDL,   CLI cannot be estimated at the UL DMRS since there is no DL interference from Macro cell. Therefore, enhancements on gNB-to-gNB CLI covariance matrix estimation RuuDL,   CLI should be studied in order to achieve a good performance for advanced IRC receivers.

Observation 1: IRC receiver is essential to suppress the cross link interference while minimizing the impact on the downlink performance of the Macro cells.

Proposal 1: Potential enhancements on the interference covariance matrix estimation for the IRC receiver should be studied in Rel-18 dynamic/flexible TDD enhancement.

Firstly, the downlink interference can be different from symbol to symbol. For example, some of the downlink interference come from PDCCH, and some of the interference come from DL DMRS and SSB, and other symbols may only be interfered by PDSCH. The key difference of the above downlink interference is the interference signal are transmitted by the Macro cell with different precoding. PDCCH could be a broadcast PDCCH or a UE specific PDCCH, and PDSCH is similar with UE specific pre-coder for dedicated data transmission, or cell specific pre-coder for broadcast signaling transmission such as SIB1. The CSI-RS may be transmitted with different downlink precoding from the downlink data, SSB, PDCCH or other downlink signals.

Secondly, the downlink interference can be different from RB to RB. The minimum precoding resource block group (PRG) size is 2 according to the current specification and on different RBs different users may be scheduled which also result in different precoding. The different precoding of the downlink signal will result in different downlink interference to the small cells who is receiving the uplink.

Observation 2: For flexible TDD scenario, the downlink cross link interference from the interfering cell consists of various downlink signals and are precoded differently according to the signal type, user etc.

Observation 3: Various types of downlink interference signal with various precoding will result in various downlink cross link interference and should be considered separately at the victim uplink receiver.

Figure 2: The illustration of various types of signals.

To obtain an accurate estimate of the interference covariance matrix, one possible way is to define dedicated resources for measurement. In current specification, downlink reserved resources are specified for forward compatibility, to accommodate future systems or signals in the downlink. The same idea can be reused in the uplink, where, uplink reserved resources or blank resources can be defined for accurate cross link interference measurement or to skip heavily interfered REs.

As one example, uplink muting resource (i.e. uplink blank resource) in the small cell could be considered for gNB-to-gNB CLI measurement as shown in Figure 3. In one UL RB, some of the uplink REs can be muted, and correspondingly the desired uplink UE can skip these REs for uplink transmission (i.e., rate matching around these REs). As a result, only cross link interference (and thermal noise) can be observed on these REs, and an accurate interference measurement can be achieved. For the IRC receiver above, the cross link interference can be measured as RuuDL,   CLI, i.e., RuuDL,   CLI=meanykykH, where yk is the received DL signals on one muted RE. Furthermore, if DL signal is DMRS at the muted RE, the channel between the Macro cell and indoor small cell HCLI  can be estimated based on the DMRS.

Figure 3: Muting REs to obtain the RuuDL,   CLI.

Considering that the gNB-to-gNB CLI from Macro cell to indoor small cells are different with various downlink signals, the CLI should be measured separately. Besides, some of the REs may be skipped by the receiver account for the strong interference. For example, for the cross link interference caused by CSI-RS, the interference may not be measured by receiver before applied to the IRC receiver, the RE interfered by the CSI-RS could skipped, and accordingly the desired uplink UE could skip these RE for transmission. Some typical DL interference types and muting REs for the specific DL interference are provided in Table 1 below.

Table 1: Patterns of muting resources for different downlink signals

| Interference types | Interference measurement and application |
| --- | --- |
| Downlink broadcast interference: One time measurement, multiple application | - Beamforming of the broadcast channel are stable - UE muting at the RE with broadcast signal - Downlink interference is measured - The measured interference Ruu can be used by subsequent UL equalization RX - SSB, SIB1, broadcast PDCCH, cell specific CSI-RS, TRS etc. |
| Unicast PDSCH /PDCCH: One time measurement, one time application | - UE muting on the DMRS RE of downlink PDSCH - UE muting on the DMRS RE of downlink PDCCH - The measurement applied in the equalization |
| UE dedicated CSI-RS: Rate matched to avoid interference | - UE muting on the distributed downlink UE dedicated CSI-RS - Interference is avoided |

Proposal 2: Study the feasibility and performance of advanced IRC receivers based on muting resources for interference covariance matrix estimation considering different interference characteristics.

gNB-to-gNB interference handling at the transmitter

The gNB-to-gNB CLI can also be mitigated at the transmitter of Macro cell. The channel HCLI  from the Macro cell to the indoor small can be measured by the indoor small cell based on reference signals, e.g. CSI-RS of the Macro cell. With the channel HCLI is reported to Macro cell, Macro cell can use Tx beamforming taking HCLI into account to suppress the CLI to the indoor small cell. To enable the resources for estimation of channel HCLI, the parameters of resources (for example, CSI-RS of Macro cell) should be exchanged between BSs. However, the downlink performance of Macro cell would be affected Tx beamforming. Thus, Tx beamforming considering HCLI can be applied at the Macro cell if the performance of Macro cell is slightly affected or the degradation of performance is acceptable.

Proposal 3: Study the feasibility and performance of Tx beamforming based on gNB-to-gNB interference channel estimation.

Enhancement of UE-to-UE interference handling schemes

The periodic resources configured for UE-to-UE CLI measurements, i.e., SRS-RSRP, CLI-RSSI, have been agreed in Rel-16. To reduce the resources used by UE-to-UE CLI measurement, aperiodic UE-to-UE CLI measurements can be considered. With aperiodic UE-to-UE CLI measurements, the resources for CLI measurements are applicable if the triggering DCI is received.

The UE-to-UE CLI measurements are reported after layer 3 filtering in Rel-16. The UE might not transmit SRS if the SRS conflicts with other UL signals (e.g. DCI scheduled downlink PDSCH transmission), and the UE-UE CLI cannot reflect the UE-UE CLI. As a result, layer 3 filtered results might be not accurate if the interference from downlink signal is used in the filtering. With the information exchange between the Macro cell and small cell, the Macro cell knows the position of resources with uplink signals in the small cell. Then, with layer 1 based UE-to-UE CLI, the Macro cell can use the DCI to trigger the UE-to-UE CLI measurement in the resources only if the UE-to-UE CLI exists. Besides, compared with the layer 3 based report, layer 1 based report is more time-efficient.

The enhancement on the UE-to-UE interference can be also applied to subband non-overlapping full duplex especially for intra-cell UE-to-UE CLI as explained in [9].

Observation 4: The overhead of UE-to-UE CLI measurement can be reduced by aperiodic measurement, and layer 1 based UE-UE CLI report could improve the accuracy of measurement.

Proposal 4: Study the feasibility and performance of UE-to-UE CLI measurement based on muting resources, aperiodic UE-to-UE CLI measurement and layer 1 based UE-UE CLI report.

Conclusions

In this paper we discussed the interference analysis and possible interference handling solutions of dynamic/flexible TDD for factory applications. We have the below observations and proposals:

Observation 1: IRC receiver is essential to suppress the cross link interference while minimizing the impact on the downlink performance of the Macro cells.

Observation 2: For flexible TDD scenario, the downlink cross link interference from the interfering cell consists of various downlink signals and are precoded differently according to the signal type, user etc.

Observation 3: Various types of downlink interference signal with various precoding will result in various downlink cross link interference and should be considered separately at the victim uplink receiver.

Observation 4: The overhead of UE-to-UE CLI measurement can be reduced by aperiodic measurement, and layer 1 based UE-UE CLI report could improve the accuracy of measurement.

Proposal 1: Potential enhancements on the interference covariance matrix estimation for the IRC receiver should be studied in Rel-18 dynamic/flexible TDD enhancement.

Proposal 2: Study the feasibility and performance of advanced IRC receivers based on muting resources for interference covariance matrix estimation considering different interference characteristics.

Proposal 3: Study the feasibility and performance of Tx beamforming based on gNB-to-gNB interference channel estimation.

Proposal 4: Study the feasibility and performance of UE-to-UE CLI measurement based on muting resources, aperiodic UE-to-UE CLI measurement and layer 1 based UE-UE CLI report.

Reference

- RP-212707, “Draft SID on Evolution of NR Duplex Operation”, 3GPP TSG RAN#94-e, Electronic Meeting, December 6 – 17, 2021

- “Values of 5G Uplink Capabilities in Industry Digitalization” in Chinese, CMCC, Huawei, October, 2020

- RP-212153, “Updated views on Rel-18 evolution of duplex operation”, 3GPP TSG RAN Meeting #93-e, Electronic Meeting, Sept 13 – 17, 2021

- 3GPP TS 22.261: “Service requirements for the 5G system”

- RWS-210436, “NR uplink boosting”, 3GPP TSG RAN Rel-18 workshop, Electronic Meeting, June 28 – July 2, 2021

- 3GPP TR38.802, “Study on New Radio Access Technology”

- RP-211627, “NGMN Liaison Statement to 3GPP Plenary on 5G NR TDD Uplink Throughput”, NGMN，RAN#93e, September 2021

- R1-2203156, “Overview of evaluation on NR duplex evolution”, Huawei, HiSilicon

- R1-2203157, “Discussion on subband non-overlapping full duplex” , Huawei, HiSilicon