3GPP TSG-RAN WG1 Meeting #109-e	R1-2203160

e-Meeting, May 9 – May 21, 2022

Agenda Item:	9.12.2

Source:	Huawei, HiSilicon

Title:	Discussion on disabling of HARQ feedback for IoT NTN

Document for:	Discussion and Decision

- Introduction

In RAN# 94e, the work item description on IoT NTN enhancement address the following issue [1]:

This work considers existing IoT-NTN as baseline as well as Rel-17 WI outcome and the further IoT-NTN performance enhancements objectives are listed below:

- Disabling of HARQ feedback to mitigate impact of HARQ stalling on UE data rates [RAN1, RAN2]

- Study and specify, if needed, improved GNSS operations for a new position fix for UE pre-compensation during long connection times and reduced power consumption [RAN1]

This work considers existing IoT-NTN as baseline as well as Rel-17 WI outcome and the further IoT-NTN performance enhancements objectives are listed below:

- Disabling of HARQ feedback to mitigate impact of HARQ stalling on UE data rates [RAN1, RAN2]

- Study and specify, if needed, improved GNSS operations for a new position fix for UE pre-compensation during long connection times and reduced power consumption [RAN1]

In this contribution, we provide some further considerations on HARQ enhancement for IoT NTN.

- Throughput with HARQ feedback enabled or disabled

  - Peak rate of IoT NTN with HARQ enabled or disabled

For NTN, the round trip time (RTT) delay varies from tens to hundreds of milliseconds, which is lengthy compared to terrestrial networks. To accommodate long RTT and minimize the throughput loss, it is agreed to extend maximal supported HARQ processes number to 32 for both UL and DL and disabling feedback of some HARQ processes in NR NTN. As for a NB-IoT UE that supports twoHARQ-Processes-r14 or the UE is configured with higher layer parameter npdsch-MultiTB-Config, there shall be a maximum of 2 downlink HARQ processes. Furthermore, when the NPDCCH with DCI format N1 schedules two transport blocks as determined by the Number of scheduled TB for Unicast field if present, the number of HARQ process is one. And the data transmission can be scheduled with granularity of:

- NPUSCH : NumberofslotNPUSCH=NTBNRepNRUNslotsRU, where NTB is the number of scheduled TB, NRep (maximum 128) is repetition number, NRU is RU number scheduled for transport block, NslotsRU (2,4,8,16) is the number of slots in each RU;

- NPDSCH : NumberofsubframeNPDSCH=NTBNRepNSF, where NTB is the number of scheduled TB, NRep (maximum 2048) is repetition number, NSF is the number of slots scheduled for transport block;

Figure 1(a) shows the DL timing relationship with 1 HARQ process, where NPDCCH with DCI format N1 schedules two transport blocks as determined by the Number of scheduled TB for Unicast field if present, and Figure 1(b) shows the DL timing relationship with 2 HARQ processes. Value of k0 (0, 4, 8, 12, 16, 32, 64, 128) is determined by the scheduling delay field ( IDelay) in the corresponding DCI. When considering the largest TBS of 4968 bits, the transmission time of each TB is 10ms, thus the duration time of NPDSCH transmission is 20ms and 10ms for Figure 1(a) and Figure 1(b), respectively. The HARQ-ACK is carried by NPUSCH format 2, which usually occupies 1 RU in time domain. For every HARQ process in NTN, due to the long RTT, it was agreed that K_offset needs to be introduced to enhance the timing relationships involving DL-UL timing interaction. Consequently, if NPDSCH was transmitted, eNB would receive the corresponding HARQ-ACK after k0'+K_offset because of the TA applied by UE, and the value of k0' (13, 15, 17, 18) is determined by the ACK/NACK resource field in the corresponding DCI. Note that in current specification, the value of k0' indicates that the time between the end of NPDSCH and the start of the corresponding HARQ-ACK should be at least 12ms, which can be considered as the processing time for UE to process the NPDSCH and prepare the ACK/NACK feedback. Especially, K_offset equals to zero for NB-IoT in TN, so the value of K should follow the values of certain k0', whereas the value of K can be any value vary from 13 to 1042 for IoT NTN.

- NPDCCH with DCI format N1 schedules two TBs

- Two HARQ processes with each DCI schedules one TB

Figure 1 DL Timing relationship of IoT NTN with HARQ feedback enabled (without repetition)

Similarly, when HARQ feedback disabled, the DL timing relationship is illustrated in Figure 2. It can be observed that the HARQ stalling issue caused by large RTT can be solved without feedback, as eNB can schedule another NPDSCH with the same HARQ ID immediately after the ending of the NPDSCH1. Thus, the total duration time for a complete PDSCH scheduling can be greatly reduced due to the disabling of NPUSCH format 2. Based on the timing relationship in Figure 1 and Figure 2, T_total is the duration of time for complete PDSCH or PDSCHs scheduling with 1 or 2 HARQ processes. Table 1 presents the maximum data rate (without repetition) of different scenarios with HARQ feedback enabled and disabled. When disabling of HARQ feedback mechanism is applied, the maximum data rate can greatly increase compared with the data rate with HARQ feedback enabled.

- NPDCCH with DCI format N1 schedules two TBs

- Two HARQ processes with each DCI schedules one TB

Figure 2 DL Timing relationship of IoT NTN with HARQ feedback disabled (without repetition)

Table 1 Maximum data rate of different scenarios with enabling and disabling HARQ (without repetition)

| Scenario | RTT (ms) | T_total (ms) |  |  |  | Maximum data rate (kbps) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | HARQ feedback enabled |  | HARQ feedback disabled |  | HARQ feedback enabled |  | HARQ feedback disabled |  |
|  |  | 1 HARQ Process | 2 HARQ Processes | 1 HARQ Process | 2 HARQ Processes | 1 HARQ Process(with two TBs scheduling) | 2 HARQ Processes | 1 HARQ Process (with two TBs scheduling) | 2 HARQ Processes |
| GEO | 541.46 | 583.46 | 579.46 | 37 | 41 | 17.03 | 17.15 | 268.54 | 242.34 |
| LEO600 | 25.77 | 67.77 | 63.77 | 37 | 41 | 146.61 | 155.81 | 268.54 | 242.34 |
| LEO1200 | 41.77 | 83.77 | 79.77 | 37 | 41 | 118.61 | 124.56 | 268.54 | 242.34 |
| TN | 1 | 42 | 44 | -- |  | 236.57 | 225.82 | -- |  |

Observation 1: For IoT NTN with disabling HARQ mechanism, the peak rate for different scenarios can be greatly increased, and the data rate of LEOs is comparable with that of TN.

  - Data rate of repetition scenarios with HARQ feedback enabled or disabled for IoT NTN

Taking the link budget of different constellations for IoT NTN into consideration [2], the required repetition number and duration time of NPDSCH, NPDCCH and NPUSCH format2 are shown in Table2. Note that the decoding BLER target of NPDCCH is usually lower than NPDSCH, so the repetition number of NPDCCH is larger than that of NPDSCH. Besides, as NB IoT, a gap of 64ms is inserted every 256ms DL transmission for synchronization. When repetition is taken into consideration, as shown in Figure 3, the stalling issues may not exist when UE is configured with 2 HARQ processes and each HARQ process schedules one TB. Because, the duration time T of NPDCCH and NPDSCH is larger than RTT but smaller than the maximum value of K, the NPDSCH scheduling by the second HARQ process can fill the stalling issue of the NPDSCH scheduling by the first HARQ process.

Figure 3 Timing relationship when HARQ process scheduled one by one

Observation 2: When repetition is taken into consideration, the stalling issues may not exist when UE is configured with 2 HARQ processes and each HARQ process schedules one TB as the NPDSCH scheduling by the second HARQ process can fill the stalling of the NPDSCH scheduling by the first HARQ process.

Table 2 Required repetition numbers of different satellite constellations

| Constellation | NPDSCH |  |  | NPDCCH |  | NPUSCH format2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Repetition number | Duration time (ms) |  | Repetition number | Duration time (ms) | Repetition number | Duration time (ms) |
|  |  | 1 HARQ | 2 HARQ |  |  |  |  |
| GEO | 64 | 1600 | 768 | 128 | 128 | 64 | 128 |
| LEO600 | 8 | 160 | 80 | 16 | 16 | 8 | 16 |
| LEO1200 | 16 | 384 | 160 | 32 | 32 | 16 | 32 |

Based on the timing relationship of DL, the data rate of different satellite constellations under the parameters in Table 2 are illustrated in Table 3. Compared to the maximum data rate evaluated above, the data rate is reduced largely due to the long duration of NPDSCH transmission with repetition. Moreover, HARQ disabling has smaller impact on the performance when compared to the performance of PDSCH with large repetition.

Table 3 Data rate of repetition scenario with enabling and disabling HARQ

| Scenario | RTT (ms) | T_total (ms) |  |  |  | Data rate (kbps) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | HARQ feedback enabled |  | HARQ feedback disabled |  | HARQ feedback enabled |  | HARQ feedback disabled |  |
|  |  | 1 HARQ Process | 2 HARQ Processes | 1 HARQ Process | 2 HARQ Processes | 1 HARQ Process | 2 HARQ Processes | 1 HARQ Process | 2 HARQ Processes |
| GEO | 541.46 | 2416.46 | 2073 | 1744 | 1824 | 0.2119 | 0.2470 | 0.2936 | 0.2807 |
| LEO600 | 25.77 | 236.77 | 249 | 192 | 224 | 2.1624 | 2.0562 | 2.6667 | 2.2857 |
| LEO1200 | 41.77 | 508.77 | 473 | 432 | 416 | 1.0063 | 1.0825 | 1.1852 | 1.2308 |

Observation 3: In IoT NTN, the maximum data rate can be greatly impacted in the case when large number of repetition is used for link budget improvement.

Observation 4: For repetition scenario, due to the long duration of NPDCCH and NPDSCH transmission, the performance improvement by HARQ disabling is small.

- Timing relationships with HARQ feedback disabled

Based on the data rate analysis of repetition scenarios with HARQ feedback enabled or disabled, the performance improvement of HARQ disabling is due to that eNB can schedule a new NPDSCH immediately after the transmission of the NPDSCH. That is to say the scheduling period can be reduced without HARQ feedback. According to performance requirements for satellite access in [4], the experienced DL data rate for IoT is 2 kbps, which can be met by LEO600 with HARQ enabling or disabling when repetition is considered. For the cases that can meet service requirement, power consumption can be saved with the introduction of feedback disabling. For other two scenarios, i.e. LEO1200 and GEO, with disabling HARQ mechanism, eNB can schedule new TB after 12ms from the ending of PDSCH transmission to improve the throughput.

Observation 5: For the cases that can meet service requirement, power consumption can be saved due to disabling of feedback.

Observation 6: For the cases that cannot meet service requirement, with disabling HARQ mechanism, eNB can schedule new TB after 12ms from the ending of PDSCH transmission to improve the throughput.

- Conclusion

The observations and proposals made in this contribution are summarized below:

Observation 1: For IoT NTN with disabling HARQ mechanism, the peak rate for different scenarios can be greatly increased, and the data rate of LEOs is comparable with that of TN.

Observation 2: When repetition is taken into consideration, the stalling issues may not exist when UE is configured with 2 HARQ processes and each HARQ process schedules one TB as the NPDSCH scheduling by the second HARQ process can fill the stalling of the NPDSCH scheduling by the first HARQ process.

Observation 3: In IoT NTN, the maximum data rate can be greatly impacted in the case when large number of repetition is used for link budget improvement.

Observation 4: For repetition scenario, due to the long duration of NPDCCH and NPDSCH transmission, the performance improvement by HARQ disabling is small.

Observation 5: For the cases that can meet service requirement, power consumption can be saved due to disabling of feedback.

Observation 6: For the cases that cannot meet service requirement, with disabling HARQ mechanism, eNB can schedule new TB after 12ms from the ending of PDSCH transmission to improve the throughput.

# References

- RP-212729, “New WID on IoT NTN enhancements”.

- TR 36.763, “Study on Narrow-Band Internet of Things (NB-IoT) / enhanced Machine Type Communication (eMTC) support for Non-Terrestrial Networks (NTN)”.

- 3GPP TS 36.213, “LTE; Physical layer procedures”.

- TS 22.261, “Service requirements for the 5G system”.