3GPP TSG-RAN WG1 Meeting #109-e 	R1-2203162

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.5.1.1

Source:	Huawei, HiSilicon

Title:	Discussion on scenarios and requirements for sidelink positioning

Document for:	Discussion and Decision

Introduction

RAN approved the SID [1] on expanded and improved NR positioning in last December. The following objectives are identified to enable SL positioning.

- Study solutions for sidelink positioning considering the following: [RAN1, RAN2] - Scenario/requirements - Coverage scenarios to cover: in-coverage, partial-coverage and out-of-coverage - Requirements: Based on requirements identified in TR38.845 and TS22.261 and TS22.104 - Use cases: V2X (TR38.845), public safety (TR38.845), commercial (TS22.261), IIOT (TS22.104) - Spectrum: ITS, licensed - Identify specific target performance requirements to be considered for the evaluation based on existing 3GPP work and inputs from industry forums [RAN1] - Define evaluation methodology with which to evaluate SL positioning for the uses cases and coverage scenarios, reusing existing methodologies from sidelink communication and from positioning as much as possible [RAN1]. - Study and evaluate performance and feasibility of potential solutions for SL positioning, considering relative positioning, ranging and absolute positioning: [RAN1, RAN2] - Evaluate bandwidth requirement needed to meet the identified accuracy requirements [RAN1] - Study of positioning methods (e.g. TDOA, RTT, AOA/D, etc) including combination of SL positioning measurements with other RAT dependent positioning measurements (e.g. Uu based measurements) [RAN1] - Study of sidelink reference signals for positioning purposes from physical layer perspective, including signal design, resource allocation, measurements, associated procedures, etc, reusing existing reference signals, procedures, etc from sidelink communication and from positioning as much as possible [RAN1] - Study of positioning architecture and signalling procedures (e.g. configuration, measurement reporting, etc) to enable sidelink positioning covering both UE based and network based positioning [RAN2, including coordination and alignment with RAN3 and SA2 as required] Note: When the bandwidth requirements have been determined and the study of sidelink communication in unlicensed spectrum has progressed, it can be reviewed whether unlicensed spectrum can be considered in further work. Checkpoint at RAN#97 to see if sufficient information is available for this review.

In this paper, we discuss the scenarios and requirements for SL positioning.

Scenarios and requirements

The scenarios/use cases and the corresponding requirements of SL positioning are summarized below according to [2][3][5].

Table 1 Summarized use case and requirement

| Use case |  | Relative or absolute | Accuracy (H) | Accuracy (V) | Positioning service availability | Positioning service latency | Coverage, environment of use and UE velocity |
| --- | --- | --- | --- | --- | --- | --- | --- |
| V2X | Set 1 | R/A | 10-50m @68-95% | 3m@95% | 95% | 1s | Up 250km/h outdoor. In coverage and out of coverage |
|  | Set 2 | R/A | 1-3m@95-99% | 2m-3m@95% | 99-99.9% | 15ms, 1s | Up 250km/h outdoor. In coverage and out of coverage |
|  | Set 3 | R/A | 0.1-0.5m@95-99% | 2m@95% (A) 0.2m@95% (R) | 99-99.9% | 10ms, 1s | Up 250km/h outdoor. In coverage and out of coverage |
| Public safety |  | R/A | 1m | 2m (A) 0.3m (R) | 95-98% | Event triggered | Indoor and outdoors. In coverage and out of coverage |
| IIoT |  | R/A | 0.2m-5m (A) 1m (R) | 0.2-3m (A) 1m ® | 90-99.9% | 10ms-5s | <30km/h In coverage and out of coverage |
|  |  |  | Ranging distance | Ranging direction |  |  |  |
| Commercial |  | R | 10cm-50m@95% | 2-12.5°@90% | 95-99% | 50ms-1s | <10m/s Distance within 1m-1km In coverage, partial coverage, and out of coverage |

In general, both absolute positioning and relative positioning requirements should be considered.

- Proposal 1: Both absolute positioning and relative positioning requirements should be evaluated.

Ranging is considered as part of the relative positioning.

Performance metric

Since there is no SL positioning at all, we believe that the top priority of the evaluation should focus on accuracy aspects, and horizontal accuracy in particular. This includes also the relative positioning accuracy.

For evaluating the absolute positioning accuracy, horizontal accuracy and vertical accuracy could borrow what is defined as [4].

Horizontal accuracy is the difference between the calculated horizontal position and the actual horizontal position of a UE. At least CDFs of horizontal positioning errors are used as a performance metrics in NR positioning evaluations. Vertical accuracy is the difference between the calculated vertical position and the actual vertical position of a UE. Vertical error is not necessarily applicable to all solutions and/or scenarios. In terms of applicability, the CDFs of vertical positioning errors are used as a performance metrics in NR positioning evaluations.

For evaluating the relative positioning accuracy, two methods can be considered.

Method 1: the relative positioning accuracy includes the distance accuracy and direction accuracy with respect to a reference UE. The distance accuracy is the difference between the calculated distance and the actual distance between the target UE and the reference UE, while the direction accuracy is the difference between the calculated direction and the actual direction of the target UE from the reference UE.

Method 2: the relative positioning accuracy includes the relative horizontal accuracy and relative vertical accuracy. The relative horizontal accuracy is the difference between the calculated horizontal position and the actual horizontal position of the target UE in a reference coordinate system, while the relative vertical accuracy is the difference between the calculated vertical position and the actual vertical position of the target UE in a reference coordinate system.

From evaluation perspective, both methods can be considered. Method 1 can be directly mapped to the ranging requirement, while Method 2, which is aligned with the absolute positioning, by simply transferring the global coordinate system into a reference coordinate system, where e.g. the reference UE is placed at the origin of the reference coordinate system.

There are also accuracy KPIs for V2X use cases where lateral and longitudinal accuracy have been defined in TS 22.186, but in our understanding, this can be further refined if we are to split the horizontal accuracy in the reference coordinate system into x-axis accuracy and y-axis accuracy. This lateral and longitudinal accuracy also applies to relative positioning.

- Proposal 2: Focus only on accuracy KPI for Rel-18 SL positioning.

- Proposal 3: Define the horizontal accuracy and vertical accuracy for absolute positioning as per TR 38.855.

- Proposal 4: Define the following accuracy KPI for relative positioning:

The distance accuracy is the difference between the calculated distance and the actual distance between the target UE and the reference UE.

The direction accuracy is the difference between the calculated direction and the actual direction of the target UE from the reference UE.

The relative horizontal accuracy is the difference between the calculated horizontal position and the actual horizontal position of the target UE in a reference coordinate system.

The relative vertical accuracy is the difference between the calculated vertical position and the actual vertical position of the target UE in a reference coordinate system.

- Due to the diverse use cases and requirements, we suggest to focus on the most pressing needs and define the requirements accordingly in this release.

- In particular, for the corresponding KPIs, we propose to define the requirement according to requirement ‘set 2’ from TR 38.845 [2] as below.

| - Absolute positioning | - Horizontal | - 3m @90% |
| --- | --- | --- |
|  | - Vertical | - 3m @90% |
| - Relative positioning | - Horizontal | - 3m@90% |
|  | - Vertical | - 3m@90% |

- Proposal 5: Define the SL positioning accuracy requirement as below.

| - Absolute positioning | - Horizontal | - 3m @90% |
| --- | --- | --- |
|  | - Vertical | - 3m @90% |
| - Relative positioning | - Horizontal | - 3m@90% |
|  | - Vertical | - 3m@90% |

Conclusion

In this contribution, we have discussed the scenarios and requirements for SL positioning. Based on the discussion, we have the following proposals.

- Proposal 1: Both absolute positioning and relative positioning requirements should be evaluated.

Ranging is considered as part of the relative positioning.

- Proposal 2: Focus only on accuracy KPI for Rel-18 SL positioning.

- Proposal 3: Define the horizontal accuracy and vertical accuracy for absolute positioning as per TR 38.855.

- Proposal 4: Define the following accuracy KPI for relative positioning:

The distance accuracy is the difference between the calculated distance and the actual distance between the target UE and the reference UE.

The direction accuracy is the difference between the calculated direction and the actual direction of the target UE from the reference UE.

The relative horizontal accuracy is the difference between the calculated horizontal position and the actual horizontal position of the target UE in a reference coordinate system.

The relative vertical accuracy is the difference between the calculated vertical position and the actual vertical position of the target UE in a reference coordinate system.

- Proposal 5: Define the SL positioning accuracy requirement as below.

| - Absolute positioning | - Horizontal | - 3m @90% |
| --- | --- | --- |
|  | - Vertical | - 3m @90% |
| - Relative positioning | - Horizontal | - 3m@90% |
|  | - Vertical | - 3m@90% |

- References

- RP-213588, Revised SID on Study on expanded and improved NR positioning, Intel, RAN#94-e, 6th – 17th Dec, 2021.

- TR 38.845, Study on scenarios and requirements of in-coverage, partial coverage, and out-of-coverage NR positioning use cases.

- TS 22.261, Service requirements for the 5G system.

- TR 38.855, Study on NR positioning support.

- TS 22.104, Service requirements for cyber-physical control applications in vertical domains.