3GPP TSG-RAN WG1 Meeting #109-e 	R1-2203163

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.5.1.2

Source:	Huawei, HiSilicon

Title:	Evaluation of SL positioning

Document for:	Discussion and Decision

Introduction

In RAN#94e, Rel-18 NR positioning SI was approved in [1]. The objective of the SI for RAN1 sidelink positioning includes the following:

- Study solutions for sidelink positioning considering the following: [RAN1, RAN2] - Scenario/requirements - Coverage scenarios to cover: in-coverage, partial-coverage and out-of-coverage - Requirements: Based on requirements identified in TR38.845 and TS22.261 and TS22.104 - Use cases: V2X (TR38.845), public safety (TR38.845), commercial (TS22.261), IIOT (TS22.104) - Spectrum: ITS, licensed - Identify specific target performance requirements to be considered for the evaluation based on existing 3GPP work and inputs from industry forums [RAN1] - Define evaluation methodology with which to evaluate SL positioning for the uses cases and coverage scenarios, reusing existing methodologies from sidelink communication and from positioning as much as possible [RAN1]. - Study and evaluate performance and feasibility of potential solutions for SL positioning, considering relative positioning, ranging and absolute positioning: [RAN1, RAN2] - Evaluate bandwidth requirement needed to meet the identified accuracy requirements [RAN1] - Study of positioning methods (e.g. TDOA, RTT, AOA/D, etc) including combination of SL positioning measurements with other RAT dependent positioning measurements (e.g. Uu based measurements) [RAN1] - Study of sidelink reference signals for positioning purposes from physical layer perspective, including signal design, resource allocation, measurements, associated procedures, etc, reusing existing reference signals, procedures, etc from sidelink communication and from positioning as much as possible [RAN1] - Study of positioning architecture and signalling procedures (e.g. configuration, measurement reporting, etc) to enable sidelink positioning covering both UE based and network based positioning [RAN2, including coordination and alignment with RAN3 and SA2 as required]

The requirements for V2X use cases are defined in [2]. Considering the difference of service levels, the requirements for the absolute or relative position are categorized into three sets as follows:

-	Set 1: 10 – 50 m with 68 – 95 % confidence level.

-	Set 2: 1 – 3 m with 95 – 99 % confidence level.

-	Set 3: 0.1 – 0.5 m with 95 – 99 % confidence level.

The requirements for commercial use cases are defined in Table 7.9-1[4] and the evaluation parameters we used are defined in [5][6].

This contribution presents the simulation results with existing positioning techniques for the V2X scenarios specified in [3] and commercial scenarios specified in [5][6].

Scenarios for evaluation

Two scenarios regarding V2X have been defined in [3]: urban grid and highway. The road configuration is shown in Table 1. The system level parameters regarding vehicle types, dropping options and antenna settings are also defined in [3].

Table 1 Road configuration of urban grid and highway [3]

| Parameter | Urban case | Highway case |
| --- | --- | --- |
| Number of lanes | 2 in each direction (4 lanes in total in each street) | 3 in each direction (6 lanes in total in the highway) |
| Lane width | 3.5 m | 4 m |
| Road grid size by the distance between intersections | 433 m * 250 m. NOTE1 | N/A |
| Simulation area size | Minimum 1299 m * 750 m | Highway length >= 2000 m. Wrap around should be applied to the simulation area. |
| NOTE1: 3 m is reserved for sidewalk per direction (i.e., no vehicle or building in this reserved space). |  |  |

- Proposal 1:  Support to use the channel model defined in TR 37.885 for V2X use case.

For commercial use cases, based on the D2D evaluation in LTE [6], indoor to indoor channel model defined in A2.1.2 in [6] can be used. The layout parameters may reference Table 7.2-2 in [5], which is shown in Table 2. The antenna settings for pedestrian UE defined in [3] are used.

Table 2 Evaluation parameters for indoor-office scenarios [6]

| Parameters |  | Indoor - office |
| --- | --- | --- |
| Layout | Room size (WxLxH) | 120mx50mx3m |
| UT location | LOS/NLOS | LOS |
|  | Height | 1 m |
| UT mobility (horizontal plane only) |  | 3 km/h |
| UT distribution (horizontal) |  | Uniform |

Proposal 2: The channel model between UEs for commercial use cases may reference indoor to indoor channel model defined in A2.1.2 in TR 36.843.

To reduce the evaluation workload, other use cases are considered as lower priority for the evaluation purpose. The evaluation results from V2X and commercial use cases seem sufficient to provide the judgement on the performance of other use cases.

- Proposal 3: Consider only V2X use case and commercial use case for the purpose of evaluation.

Evaluation methodology and simulation assumptions

V2X use case

Considering the characteristics of scenarios, we will conduct the simulation with different assumptions and positioning methods. The V2V sidelink channel model [3] is utilized in our simulation for V2X use case.

Lane width:

3.5m

Street width：        20m

Sidewalk width: 3m

433m

250m

Lane width:

3.5m

Street width：        20m

Sidewalk width: 3m

433m

250m

For urban grid, roads are usually congested. Considering traffic safety, normally we need to pay attention to the distance and direction between two vehicles. The RTT positioning method is suitable for distance estimation and the AOA method is sufficient for direction estimation. RTT+AOA method is used for the relative positioning between two UEs. For each UE, any other vehicle in the same street is selected for positioning pairing as in Figure 1.

Figure 1 Positioning pairing for urban grid scenario

For highway, vehicles are sparse with high velocity. TDOA method is used for the absolute positioning with multiple fixed UEs (e.g. RSU). The RSUs are interlaced on both sides of the road with distance of 200m with the deployment shown in Figure 2.

100m

100m

200m

200m

200m

200m

200m

200m

Lane width:

4m

≥2km

Lane width:

4m

≥2km

Figure 2 RSU deployment in Highway scenario

The two cases in Table 3 are evaluated for FR1. Other parameters can be found in the Appendix A.

Table 3 Parameters of simulation assumption for V2X use case

| Parameters | Urban grid | Highway |
| --- | --- | --- |
| Bandwidth | 40MHz | 40MHz |
| Positioning method | RTT+AOA | TDOA |
| Antenna array configuration (M, N, P, Mg, Ng) | Vehicle UE: Rooftop antenna: (1, 2, 2, 1, 1) | Vehicle UE: Rooftop antenna: (1, 2, 2, 1, 1) RSU: (1, 2, 2, 1, 1) |
| Antenna height | Vehicle UE: 1.6m | Vehicle UE: 1.6m RSU: 5m |
| Positioning type | Relative positioning | Absolute positioning |
| Note | The absolute value of longitudinal distance (m) between two vehicles follows truncated normal distribution Nμ, σ2, a, b: N(10, 32, 7, ∞), for same lane; N(0, 32, 0, ∞), for different lane. | 6 RSUs are selected for positioning |

In TR 37.885, three different types of vehicles are defined with different location of antenna and vehicle size. In order to simplify the simulation, we take option A in UE dropping model [3] with 100% vehicle type 2.

Based on the above simulation assumptions, we have the following proposals.

- Proposal 4: Take the following two scenarios for V2X use cases for evaluations:

Urban grid:

  - The relative positioning based on sidelink between two UEs is evaluated.

  - The relative horizontal accuracy is evaluated.

Highway:

  - The absolute positioning based on sidelink between the target UE and multiple fixed UEs is evaluated.

  - The absolute horizontal accuracy is evaluated.

For both urban grid and highway scenarios, 100% Vehicle type 2 is a starting point for the UE drop model.

- Proposal 5: Adopt the table in Appendix B as the overall evaluation assumption for V2X use cases.

Commercial use case

For commercial use case simulation, the relative positioning between two UEs should be considered, where the distance and direction performance can be evaluated for relative positioning.

We adopt the InH channel model [6]. Considering the performance requirement in Table 7.9-1 [4] where the requirements are defined only for LOS only, 100% LOS probability (ideal LOS) is used. The parameters of simulation assumption is given in Table 4. Other parameters can be found in the Appendix A.

Table 4 Parameters of simulation assumption for commercial use case

| Parameters | Indoor office |
| --- | --- |
| Bandwidth | 40MHz |
| Positioning method | RTT or AOA |
| Antenna array configuration (M, N, P, Mg, Ng) | (1, 2, 2, 1, 1) |
| Antenna height | 1m |
| Positioning type | Relative positioning |
| Note | The largest distance between the two UEs is 10m. |

- Proposal 6: For commercial use cases, the relative positioning based on sidelink between two UEs is evaluated.

The distance accuracy and/or direction accuracy are considered.

- Proposal 7: Adopt the table in Appendix C as the overall evaluation assumption for commercial use cases.

Preliminary simulation results

V2X use case

The positioning error of 50%, 67%, 80%, 90%, and 95% for all cases are summarized in Table 5. The detailed simulation results can be found in Figure 3 and Figure 4. In urban scenario, only UEs in the same street could be selected for pairing, so the two vehicles is either in LOS state or NLOS state. LOS+NLOSv indicates statistics for all cases based on the above assumptions, while ideal LOS means the results of two vehicles in LOS state are selected for the statistics. In highway, the situation is basically the same except for the ideal LOS, which indicates only RSUs in LOS state from the 6 selected RSUs are used for positioning.

Table 5 CDF of positioning accuracy (in unit of meter)

|  | Case |  | 50% | 67% | 80% | 90% | 95% |
| --- | --- | --- | --- | --- | --- | --- | --- |
| FR1 | Urban grid {RTT+AOA, Relative} | LOS+NLOSv | 0.94 | 1.35 | 2.06 | 2.99 | 10.68 |
|  |  | Ideal LOS | 0.90 | 1.26 | 1.88 | 2.72 | 4.71 |
|  | Highway {TDOA, Absolute} | LOS+NLOSv | 0.372 | 0.704 | 1.191 | 2.405 | 4.791 |
|  |  | Ideal LOS | 0.334 | 0.596 | 0.961 | 1.524 | 2.341 |

Figure 3 Positioning accuracy in urban grid

Figure 4 Positioning accuracy in highway

Based on the simulation results, we have the following observation:

Observation 1: With 40 MHz bandwidth, the horizontal accuracy performance with both relative positioning and absolute positioning for V2X use cases can reach <3 m @ 90%.

Commercial use case

The distance accuracy and direction accuracy of 50%, 67%, 80%, 90% and 95% for commercial use case are summarized in Table 6. The detailed simulation results can be found in Figure 5 and Figure 6.

Table 6 CDF of positioning accuracy

|  | Case |  | 50% | 67% | 80% | 90% | 95% |
| --- | --- | --- | --- | --- | --- | --- | --- |
| FR1 | Distance (m) | RTT, Relative | 0.417 | 0.674 | 0.979 | 1.341 | 1.956 |
|  | Direction (deg) | AOA, Relative | 0.316 | 0.551 | 0.998 | 1.882 | 3.451 |

Figure 5 Distance accuracy in Indoor office

Figure 6 Direction accuracy in Indoor office

Based on the simulation results, we have the following observation:

Observation 2: With 40 MHz bandwidth, the distance accuracy performance and direction accuracy performance for commercial use cases can reach <1.4 m @ 90% and <2 degrees @ 90%, respectively.

Conclusion

In this contribution, we presented our views on the evaluation methodology for SL positioning, and the initial evaluation results. We have the following observations and proposals.

Observation 1: With 40 MHz bandwidth, the horizontal accuracy performance with both relative positioning and absolute positioning for V2X use cases can reach <3 m @ 90%.

Observation 2: With 40 MHz bandwidth, the distance accuracy performance and direction accuracy performance for commercial use cases can reach <1.4 m @ 90% and <2 degrees @ 90%, respectively.

- Proposal 1:  Support to use the channel model defined in TR 37.885 for V2X use case.

Proposal 2: The channel model between UEs for commercial use cases may reference indoor to indoor channel model defined in A2.1.2 in TR 36.843.

- Proposal 3: Consider only V2X use case and commercial use case for the purpose of evaluation.

- Proposal 4: Take the following two scenarios for V2X use cases for evaluations:

Urban grid:

  - The relative positioning based on sidelink between two UEs is evaluated.

  - The relative horizontal accuracy is evaluated.

Highway:

  - The absolute positioning based on sidelink between the target UE and multiple fixed UEs is evaluated.

  - The absolute horizontal accuracy is evaluated.

For both urban grid and highway scenarios, 100% Vehicle type 2 is a the starting point for the UE drop model.

- Proposal 5: Adopt the table in Appendix B as the overall evaluation assumption for V2X use cases.

- Proposal 6: For commercial use cases, the relative positioning based on sidelink between two UEs is evaluated.

The distance accuracy and/or direction accuracy are considered.

- Proposal 7: Adopt the table in Appendix C as the overall evaluation assumption for commercial use cases.

- Reference

- RP-213561, New SID on Study on expanded and improved NR positioning, RAN#94e, e-Meeting, Dec.6-17, 2021.

- TR 38.845, Study on scenarios and requirements of in-coverage, partial coverage, and out-of-coverage NR positioning use cases

- TR 37.885, Study on NR Vehicle-to-Everything (V2X).

- TS 22.261, Service requirements for the 5G system.

- TR 38.901, Study on channel model for frequencies from 0.5 to 100 GHz.

- TR 36.843, Study on LTE Device to Device Proximity Services.

- Appendix A

The common parameters can be found in Table 7.

Table 7 Common parameters

| Parameter | Value |
| --- | --- |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | Comb-4 |
| Reference signal | PRS: Gold, 1-port |
| Number of symbols | 4 |
| Description of positioning technique / applied positioning algorithm | MUSIC |

- Appendix B

Overall evaluation assumption for V2X use cases

|  | FR1 Specific Values |
| --- | --- |
| Carrier frequency, GHz | 6GHz |
| Bandwidth, MHz | 40MHz |
| Subcarrier spacing, kHz | 30kHz |
| RSU model parameters |  |
| RSU noise figure, dB | 5dB |
| RSU max. TX power, dBm | 23dBm |
| RSU antenna configuration | Mg = 1, Ng = 1, P = 2, dH = 0.5λ, (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) |
| RSU antenna radiation pattern | Omni, 0dBi |
| UE model parameters |  |
| UE noise figure, dB | 9dB |
| UE max. TX power, dBm | 23dBm |
| UE antenna configuration | Mg = 1, Ng = 1, P = 2, dH = 0.5λ, (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) |
| UE antenna radiation pattern | Omni, 0dBi |
| PHY/link level abstraction | Explicit simulation of all links, individual parameters estimation is applied. Companies to provide description of applied algorithms for estimation of signal location parameters. |

Other parameters according to V2X use case as defined in TR 37.885.

|  |  | FR1 Specific Values |  |
| --- | --- | --- | --- |
|  |  | Urban grid | Highway |
| Channel model |  | V2V sidelink |  |
| Road configuration | Number of lanes | 2 in each direction (4 lanes in total in each street) | 3 in each direction (6 lanes in total in the highway) |
|  | Lane width | 3.5 m | 4 m |
|  | Road grid size by the distance between intersections | 433 m * 250 m. NOTE1 | N/A |
|  | Simulation area size | Minimum 1299 m * 750 m | Highway length >= 2000 m. Wrap around should be applied to the simulation area. |
| UE horizontal drop procedure |  | Vehicles are dropped independently in each lane.All vehicles in a lane have the same speed |  |
| UE size |  | Length 5m, Width 2m, Height 1.6m |  |
| UE antenna height |  | 1.6m |  |
| UE mobility |  | 60km/h | 70km/h |
| Min RSU-UE distance (2D), m |  | 0m |  |
| Min UE-UE distance (2D), m |  | 2m (bumper-bumper distance) |  |
| RSU antenna height |  | 5m |  |
| NOTE1: 3 m is reserved for sidewalk per direction (i.e., no vehicle or building in this reserved space). |  |  |  |

- Appendix C

Overall evaluation assumption for commercial use cases

|  | FR1 Specific Values |
| --- | --- |
| Carrier frequency, GHz | 3.5GHz |
| Bandwidth, MHz | 40MHz |
| Subcarrier spacing, kHz | 30kHz |
| UE model parameters |  |
| UE noise figure, dB | 9dB |
| UE max. TX power, dBm | 23dBm |
| UE antenna configuration | Mg = 1, Ng = 1, P = 2, dH = 0.5λ, (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) |
| UE antenna radiation pattern | Omni, 0dBi |
| PHY/link level abstraction | Explicit simulation of all links, individual parameters estimation is applied. Companies to provide description of applied algorithms for estimation of signal location parameters. |

Other parameters according to InH as defined in TR 38.901.

|  |  | FR1 Specific Values |
| --- | --- | --- |
| Channel model |  | InH |
| Layout | Room size (WxLxH) | 120mx50mx3m |
| UE horizontal drop procedure |  | Uniformly distributed over the whole hall area |
| UE antenna height |  | 1m |
| UE mobility |  | 3km/h |
| Min UE-UE distance (2D), m |  | 0m |
| Max UE-UE distance (2D), m |  | 10m |