3GPP TSG-RAN WG1 Meeting #109-e 	R1-2203164

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.5.1.3

Source:	Huawei, HiSilicon

Title:	Discussion on solutions to support SL positioning

Document for:	Discussion and Decision

Introduction

In RAN#94e, Rel-18 NR positioning SI was approved [1]. With the study on “Scenarios and requirements of in-coverage, partial coverage, and out-of-coverage NR positioning use cases” in Rel-17 [4], it was agreed to further study the solutions for sidelink (SL) positioning in Rel-18 with the following objectives:

- Study solutions for sidelink positioning considering the following: [RAN1, RAN2] - Scenario/requirements - Coverage scenarios to cover: in-coverage, partial-coverage and out-of-coverage - Requirements: Based on requirements identified in TR38.845 and TS22.261 and TS22.104 - Use cases: V2X (TR38.845), public safety (TR38.845), commercial (TS22.261), IIOT (TS22.104) - Spectrum: ITS, licensed - Identify specific target performance requirements to be considered for the evaluation based on existing 3GPP work and inputs from industry forums [RAN1] - Define evaluation methodology with which to evaluate SL positioning for the uses cases and coverage scenarios, reusing existing methodologies from sidelink communication and from positioning as much as possible [RAN1]. - Study and evaluate performance and feasibility of potential solutions for SL positioning, considering relative positioning, ranging and absolute positioning: [RAN1, RAN2] - Evaluate bandwidth requirement needed to meet the identified accuracy requirements [RAN1] - Study of positioning methods (e.g. TDOA, RTT, AOA/D, etc) including combination of SL positioning measurements with other RAT dependent positioning measurements (e.g. Uu based measurements) [RAN1] - Study of sidelink reference signals for positioning purposes from physical layer perspective, including signal design, resource allocation, measurements, associated procedures, etc, reusing existing reference signals, procedures, etc from sidelink communication and from positioning as much as possible [RAN1] - Study of positioning architecture and signalling procedures (e.g. configuration, measurement reporting, etc) to enable sidelink positioning covering both UE based and network based positioning [RAN2, including coordination and alignment with RAN3 and SA2 as required] Note: When the bandwidth requirements have been determined and the study of sidelink communication in unlicensed spectrum has progressed, it can be reviewed whether unlicensed spectrum can be considered in further work. Checkpoint at RAN#97 to see if sufficient information is available for this review.

Considering the scenarios, use cases and requirements of SL positioning, in this contribution, we present our views on the SL positioning methods, RS design, resource allocation, measurements, and procedures.

Sidelink positioning methods

The SL positioning use cases and requirements are defined in [3], [4] and [5], which cover both the absolute positioning and relative positioning. Hence, the methods and measurements supporting absolute and relative positioning should be supported in Rel-18.

Sidelink positioning methods

Considering the following positioning scenarios, we introduce the positioning methods that should be supported in Rel-18 for SL positioning.

Case 1

Case 2

Case 3

RSU

RSU

RSU

RSU

RSU

RSU

RSU

RSU

Case 1

Case 2

Case 3

RSU

RSU

RSU

RSU

RSU

RSU

RSU

RSU

Figure 1: Absolute positioning scenarios

In case 1, the vehicular type UE is receiving the SL positioning reference signals (SL-PRS) from several fixed RSUs. This case is similar to the Uu downlink positioning scenario. By measuring the RSTD and/or the AoA of SL-PRSs from different RSUs, the location of the UE can be fixed.

In case 2, the vehicular type UE is sending the SL-PRS to the RSUs, similar to the Uu uplink positioning scenario. By measuring the RTOA and AoA, the location of the UE can be fixed as well. Considering case 1 and case 2, the TDOA and/or AoA positioning methods should be supported.

In case 3, the vehicular type UE is sending the SL-PRS to the RSUs as well as receiving SL-PRSs from the RSUs. In this case, by measuring the Rx-Tx time difference based on the transmitted SL-PRS and received SL-PRS, the location of the UE can be fixed. Hence, the RTT positioning method should be supported for SL positioning.

UE1

UE2

θ

UE1

UE2

θ

Figure 2: Relative positioning scenarios

In the scenario of relative positioning or ranging between two UEs, which need to acquire the distance and/or the direction between each other, the RTT-based method could be used to determine the distance and the AoA-based method could be used to determine the direction. Hence, for relative positioning or ranging, the RTT positioning and/or AoA methods, which could be used independently or jointly, should be supported.

- Proposal 1: SL positioning should support:

RTT-based method for absolute and relative positioning

AoA-based method for absolute and relative positioning

TDOA-based method for absolute positioning

- Note: For absolute positioning, the position of the transmitters, such as RSU, is known.

Sidelink positioning measurements

In Uu based positioning, RSTD measurement is supported for DL-TDOA-based positioning, RTOA measurement is supported for UL-TDOA-based positioning, gNB Rx-Tx time difference and UE Rx-Tx time difference measurements are supported for RTT-based positioning, and AOA measurement is supported for AOA-based positioning. In order to support the above positioning methods in section 2.1 for SL positioning, those measurements should also be supported.

- Proposal 2: To support the above SL positioning methods, the following measurements should be reported:

RSTD and/or RTOA measurements for TDOA-based method

Rx-Tx timing difference for RTT-based method

AoA for AoA-based method

T1

T2

RSU2

RSU4

RSU1

RSU3

T1

T2

RSU2

RSU4

RSU1

RSU3

Moreover, different from the Uu positioning, SL positioning in a V2X scenario needs to support higher mobility where the UE velocity can be up to 250 km/h. With this velocity, the distance between different UEs or between UEs and RSUs will change rapidly, for example, every 100 ms a vehicular type UE can move about 7 meters, which is a non-negligible distance compared to the distance between the UE and an RSU.

Traditional Uu positioning assumes that UE is fixed within a single location fix. Under V2X scenarios, the UE’s movement should be taken into consideration. One possible way is to consider the variation of the distance between UE and RSUs and fix the UE’s locations based on measurements at different times.

- Proposal 3: Support to fix UE locations based on measurements at different time instances considering UE’s mobility characteristics.

In RTT-based positioning or ranging, TOF can be estimated using TOF=12ta-tb, where ta is the round-trip time measured in UE a, tb is the reply time measured in UE b. However, both UEs have independent clock drift due to the imperfect oscillators. The actual values of ta and tb can be modeled as

ta=1+eatatb=1+ebtb

where  ea and eb model the deviation from the nominal frequency of UE a and UE b, which give rise to the clock drift and are typically expressed in parts per million (ppm).

tb

ta

UE a

UE b

tb

ta

UE a

UE b

Figure 3: RTT based positioning

Hence, the estimated TOF now becomes TOF=12(ta-tb), and the error caused by the clock drift is given as

Errorclock drift=TOF-TOF=ea∙TOF+12tbea-eb.

Typically, TOF is in the order of nanoseconds, and the reply time tb is in the order of milliseconds. Hence, the reply time tb is the dominant factor of the error. Moreover, gNBs usually have much better oscillators than that in UE. Hence, the Errorclock drift is larger in SL positioning, in the order of nanoseconds, which further deteriorates the accuracy performance.

- Observation 1: The clock drift would deteriorate positioning accuracy in RTT-based SL positioning or ranging.

- Proposal 4: UE timing drift should be considered in SL positioning or ranging.

Sidelink positioning reference signals design

Sequence design

In the NR Uu positioning between gNB and UE, positioning reference signals include DL-PRS transmitted from gNB to UE and SRS transmitted from UE to gNB. The DL-PRS is generated based on the pseudo-random sequence, and the SRS is generated based on Zadoff-Chu (ZC) sequence. For SL-PRS, either pseudo-random sequence or ZC sequence is a potential candidate.

However, for SL positioning, especially in V2X scenarios, UE may move fast so the Doppler resultin from the relative movement between two UEs or between the UE and RSU is large. Since the positioning performance using ZC sequence is easily affected by the Doppler, a pseudo-random sequence is more robust than the ZC sequence for SL positioning. In the SL communication, the reference signals such as DMRS, PTRS, and CSI-RS are all generated based on the Gold sequence. Consequently, in our opinion, SL-PRS sequences should be generated based on pseudo-random sequences. Apart from the existing Gold sequence, other pseudo-random sequences could be considered as well for performance evaluation.

- Proposal 5: The candidate SL-PRS sequences should support the pseudo-random sequences.

Companies should report the used sequences for SL-PRS transmission in the evaluations.

SL-PRS pattern

In Uu positioning, the comb structure is introduced to multiplex multiple transmissions on the same symbol to overcome the time drift to achieve the high accuracy. At least for the case when DL-TDOA like positioning method is supported for SL positioning, the comb structure may benefit of multiplexing multiple Tx SL-PRS resources from different UEs, e.g. RSUs. We think that the comb structure should be supported for SL-PRS pattern.

Proposal 6: The comb structure of PRS should be supported for SL-PRS design.

For SL positioning, similar to SL transmission of PSSCH/PSCCH in NR V2X, an AGC symbol should precede a SL-PRS transmission for the receiver to adjust its RF dynamic range. The same applies with the GAP symbol, which allows a UE to switch from transmission to reception and vice versa.

Figure 4 shows an example of the SL-PRS slot structure, which includes the AGC, SL-PRS, and GAP symbols.

| AGC | SL-PRS | SL-PRS | SL-PRS | SL-PRS | SL-PRS | SL-PRS | SL-PRS | SL-PRS | SL-PRS | SL-PRS | GAP |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Figure 4: An example of the SL-PRS slot structure

Proposal 7: There should be an AGC symbol before the SL-PRS symbols and a GAP symbol after the SL-PRS symbols.

Sidelink positioning resource allocation

Sidelink positioning resource pool design

Current SL resources are defined within the SL BWP. The SL BWP is divided into resource pools, and each resource pool can be further divided into a number of sub-channels. The sub-channel is the minimum unit of resource allocation. To support SL positioning, some resources need to be allocated to SL-PRS transmission. There are two choices for designing the resource for positioning. One is reusing the current communication resource pool. The other is designing a dedicated positioning resource pool.

RP1

RP2

RP3

Communication resource pool

Positioning resource pool

RP1

RP2

RP3

RP1

RP2

RP3

Communication resource pool

Positioning resource pool

RP1

RP2

RP3

Figure 5: SL dedicated positioning resource pool

The main problem of reusing current SL communication resource pool is capacity/resource limitation, especially for mode 2 based resource allocation scenarios. Since positioning operations mainly focus on parameters estimation, such as TOA estimation, whereas the estimation accuracy is highly reliant on the bandwidth of reference signals. For example, a UE may need to send a SL-PRS spanning the entire resource pool to guarantee the positioning accuracy. In this case, it is very unlikely to find a slot where all the subchannels are available. On the other hand, even if a given slot is sensed available, it will only be allocated to one UE according to the current communication resource allocation scheme. When a group of UEs exists, the system capacity/resource limitation will become a big problem for providing the positioning services.

Defining a dedicated resource pool for SL positioning alleviates the above problem and allows more flexible resource allocation. The resource structure within the resource pool could be re-designed based on the positioning purpose. In addition, the resource allocation scheme could be re-designed as well to allow more UE occupy the full band of the SL-P resource pool within the same slot to improve the system capacity.

- Proposal 8: Support a dedicated resource pool used for SL positioning only.

According to the SL resource pool configuration procedure [2], when a resource pool is configured for the UE, some slots are excluded (such as Figure 6). These excluded slots are referred to as reserved slots. These reserved slots are excluded to make the length of a bitmap field indicating time domain resource allocation in resource pool configuration aligned with the quantity of remaining slots after excluding S-SSB slots and non-SL slots (some OFDM symbols are not semi-statically configured as UL in this slot). The reserved slots are not used to perform SL communication and do not belong to the resource pool.

These reserved slots can be used to transmit the SL-PRS. Time division multiplexing is implemented naturally between the data and the positioning reference signal, so that the communication of the legacy UE (such as R16/R17 UE) is not affected by R18 UE for positioning and also ensures backward compatibility. Therefore, the reserved slots excluded from the communication resource pool as a part of the dedicated resource pool for positioning to transmit SL-PRS can be considered.

Figure 6: Procedure for determining the set of slots assigned to a SL resource pool

Proposal 9: Consider the reserved slots as a part of the dedicated resource pool for positioning to transmit SL-PRS.

Resource allocation for in-coverage scenarios

For in-coverage scenario, the SL users could utilize the radio resources from licensed bands dedicated to SL or from the licensed carriers shared between SL and UL communication. In these two scenarios, gNB should be responsible for managing and scheduling the resources for SL positioning, i.e., mode 1 based SL-PRS resource allocation. There are several benefits from different perspectives. First, the interference between SL UEs and UL UEs could be reduced and the collision between SL UEs could be avoided by scheduling different resources to different SL UEs. Second, the resource utilization efficiency could be improved. With the concern of interference, some resources would be omitted even though they are actually available due to the high probability of collision. However, under gNB’s control, the gNB has a clear awareness that which resources have been allocated and which resource are available. Hence, the resource utilization efficiency could be improved. Lastly, by scheduling the SL resource for positioning at gNB, the energy efficiency of the SL users could be greatly improved since the reduced sensing operations.

In addition to mode-1 based SL-PRS resource allocation, the network may choose to grant UE to do mode-2 based SL-PRS resource allocation, similar to SL communication.

Therefore, for in-coverage scenarios, SL positioning should support both mode 1 based resource allocation and mode 2 based resource allocation.

Proposal 10: For in-coverage scenarios, both mode 1 based and mode 2 based SL positioning resource allocation should be supported.

Resource allocation for out-of-coverage scenarios

In the out-of-coverage scenario, due to the lack of control from the network, the communication between UEs is based on a model-2 resource allocation procedure; that is, the UE selects a resource to send data by itself. Before sending data, each UE needs to perform resource sensing, and then sends data on an unoccupied resource. As shown in the following figure:

Figure 7: Model-2 resource allocation procedure

In Figure 7, UE0 keeps sensing the resource reservation information. When UE0 receives a data transmission request, it will select an appropriate time slot within the selection window based on the sensing results within the prior sensing window. For SL positioning in the out-of-coverage scenario, the sensing-based resource allocation can be adopted as well. Likewise, for SL positioning, the transmitting UE can sense the channel and select an appropriate resource for SL-PRS transmission. After the initial resource selection, the reservation scheme can be adopted to reserve a future slot for the following SL-PRS transmission.

- Proposal 11: For out-of-coverage, mode-2 based SL positioning resource allocation should be supported.

Procedure

For positioning between two UEs, the transmission of SL-PRS is only required when the location service is initiated, and thus the SL positioning may not require always-on transmission of SL-PRS, which is different from the broadcast nature of DL-PRS defined in the Uu interface.

For SL positioning, the procedures of resource reservation could be flexibly optimized per positioning QoS required by the UE. Introducing non-aperiodic SL-PRS will cause a lot of efforts of specifying the resource reservation and sensing mechanism in mode 2 operation.

With the above considerations, we think that aperiodic PRS should be supported at least.

Proposal 12: Support at least aperiodic PRS triggered by SCI for sidelink positioning.

- For the configuration of SL-PRS, basically two options can be considered. One option is to reuse the resource configuration procedure of Uu interface, i.e., SL-PRS is configured by higher layer parameters (for example, RRC signaling by gNB or PC5-RRC by another UE) and then an SCI is used to reserve the time/frequency resource for SL-PRS transmission. For example, the specific time slot of SL-PRS can be indicated in SCI.

- The other option is to indicate the SL-PRS using the SCI directly, similar to scheduling PSSCH, where the PRS parameters, such as comb size, number of symbols, start symbol within the slots, offsets, etc., are carried by an SCI, so that a more flexible resource scheduling can be implemented.

Proposal 13: Consider the following SL-PRS configuration schemes:

Alt1: SL-PRS is (pre-)configured by higher layers, and SCI is used for time/frequency reservation.

Alt2: SL-PRS is configured by SCI.

Conclusion

In this contribution, we have the following observations and proposals.

- Observation 1: The clock drift would deteriorate positioning accuracy in RTT-based SL positioning or ranging.

- Proposal 1: SL positioning should support:

RTT-based method for absolute and relative positioning

AoA-based method for absolute and relative positioning

TDOA-based method for absolute positioning

- Note: For absolute positioning, the position of the transmitters, such as RSU, is known.

- Proposal 2: To support the above SL positioning methods, the following measurements should be reported:

RSTD and/or RTOA measurements for TDOA-based method

Rx-Tx timing difference for RTT-based method

AoA for AoA-based method

- Proposal 3: Support to fix UE locations based on measurements at different time instances considering UE’s mobility characteristics.

- Proposal 4: UE timing drift should be considered in SL positioning or ranging.

- Proposal 5: The candidate SL-PRS sequences should support the pseudo-random sequences.

Companies should report the used sequences for SL-PRS transmission in the evaluations.

Proposal 6: The comb structure of PRS should be supported for SL-PRS design.

Proposal 7: There should be an AGC symbol before the SL-PRS symbols and a GAP symbol after the SL-PRS symbols.

- Proposal 8: Support a dedicated resource pool used for SL positioning only.

Proposal 9: Consider the reserved slots as a part of the dedicated resource pool for positioning to transmit SL-PRS.

Proposal 10: For in-coverage scenarios, both mode 1 based and mode 2 based SL positioning resource allocation should be supported.

- Proposal 11: For out-of-coverage, mode-2 based SL positioning resource allocation should be supported.

Proposal 12: Support at least aperiodic PRS triggered by SCI for sidelink positioning.

Proposal 13: Consider the following SL-PRS configuration schemes:

Alt1: SL-PRS is (pre-)configured by higher layers, and SCI is used for time/frequency reservation.

Alt2: SL-PRS is configured by SCI.

- Reference

- RAN1, Chair’s Notes RAN1#108-e v27, RAN1#108-e, e-Meeting, 21st Feb.  – 3rd Mar., 2022.

- 3GPP TS 38.214, Physical layer procedures for data, 2022

- 3GPP TS 22.261, Service requirements for 5G systems, 2022

- 3GPP TR38.845, Study on scenarios and requirements of in-coverage, partial coverage, and out-of-coverage NR positioning use cases, 2021.

- 3GPP TS 22.104, Service requirements for cyber-physical control applications in vertical domains, 2021.

- IEEE Standard for Low-Rate Wireless Networks--Amendment 1: Enhanced Ultra-Wideband (UWB) Physical Layers (PHYs) and Associated Ranging Techniques, 2020.