3GPP TSG-RAN WG1 Meeting #109-e 	R1-2203165

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.5.2.1

Source:	Huawei, HiSilicon

Title:	Error source for NR RAT-dependent positioning

Document for:	Discussion and Decision

Introduction

RAN approved the SID [1] on expanded and improved NR positioning in last December. The following objective is identified to support RAT-dependent integrity.

- Study solutions for Integrity for RAT dependent positioning techniques [RAN2, RAN1]: - Identify the error sources, [RAN1, RAN2]. - Study methodologies, procedures, signalling, etc for determination of positioning integrity for both UE-based and UE-assisted positioning [RAN2] - Focus on reuse of concepts and principles being developed for RAT-Independent GNSS positioning integrity, where possible.

In this paper, we discuss the error source of RAT-dependent positioning techniques.

Discussion

Positioning integrity is the capability of the positioning system that can provide the timely warning when the statistical positioning error bound, a.k.a. protection level, exceed a certain limit set for a target use case, a.k.a. alert limit.

To support the calculation of the positioning error bound used to be compared against the alert limit to issue the warning, the location engine should be able to collect all necessary information, e.g. the measurement error bound.

On the measurement error bound for a RAT-dependent positioning method, we suggest to focus on the ToA error and AoA error in RAN1.

ToA error

For the ToA estimation method using super resolution, e.g. MUSIC, the estimated ToA appears to be a normal distribution due to the central limit theorem. Figure 1 shows the evaluation results of TOA error in terms of meters, with 50MHz bandwidth, using super resolution method. The TOA error distribution shows a curve similar to a normal distribution.

Figure 1 TOA error histogram for a LOS channel using high-resolution algorithm

- Observation 1: The TOA error is close to a normal distribution if super resolution ToA estimation is used.

- Due to the distribution profile of the TOA error, the same paired over-bounding Gaussian formula as discussed in GNSS integrity [2] can be reused to determine the error bound corresponding to the allocated integrity risk, where the mean error can be set to 0 for ToA.

Bound = mean + K * stdDev (Equation 8.1.1a-2) K = normInv(IRallocation / 2) irMinimum <= IRallocation <= irMaximum where: mean: mean value for this specific error, as per Table 8.1.2.1b-1 stdDev: standard deviation for this specific error, as per Table 8.1.2.1b-1

With that, we have the following proposal.

- Proposal 1: Model the ToA error as the normal distribution, and report the error bound associated with the allocated integrity risk for the ToA measurement via the paired over-bounding Gaussian formula.

This should apply to all DL RSTD, UE Rx – Tx time difference measurement, UL RTOA, and gNB Rx – Tx time difference.

The reference timing for DL RSTD should also have its reported bound.

AoA error

Given that AoA measurement usually corresponds to the azimuth AoA and zenith AoA, we consider a better quantization of the angle quality if the angle is reported in the LCS.

The agreed definition of AOA and ZOA is not symmetric as ZOA is the angle between UE and z’-axis, while AOA is the angle between the projected UE direction in the x’Oy’ plane and x’-axis if UE is not in the same horizontal plane as the gNB. The asymmetry will result in the covariance matrix of AOA and ZOA error to be non-diagonal, i.e.,

Cθ',ϕ'≜Eθ'-θ0'ϕ'-ϕ0'θ'-θ0'ϕ'-ϕ0'T=c11c12c21c22

With c12=c21≠0.

y'

x'

z'

ϕ'

θ'

φ'

y'

x'

z'

ϕ'

θ'

φ'

Figure 2 Introducing φ' as the angle between UE and y’-axis

To diagonalize the covariance matrix, i.e. de-correlate the errors, we can consider selecting a symmetric angle measurement to model the error. Since θ' is the angle between UE and z’-axis, we can use another angle φ' defined as the angle between UE and y’-axis, shown in Figure 2. It can be shown that
 cosφ'=sinθ'sinϕ', and one can prove that Cθ',φ' is diagonal. This is because θ' is estimated based on phase difference between the antenna elements along z’-axis, while φ' is estimated based on the phase difference between antenna elements along y’-axis, and the underlying error is independent of each other.

Cθ',φ'≜Eθ'-θ0'φ'-φ0'θ'-θ0'φ'-φ0'T

Furthermore, we notice that angle error is a function of the angle itself. For example, when the UE direction is close to the boresight of the antenna array, the angle error is small; however, when the UE direction is close to the endfire of the antenna array, the angle error is large as the beam width is also large. To remove the angle error dependence on angle itself, the angle error can be further modelled by the error of the following two quantities

cosθ'

cosφ'=sinθ'sinϕ'

Then with Ccosθ',sinθ'sinϕ' not only it is a diagonal matrix, but also the diagonal elements is not a function of reported angle θ' and ϕ'.

Option 1: Cθ',ϕ'

Option 2: Cθ',φ'

Option 3: Ccosθ',cosφ'=sinθ'sinϕ'

Option 1: Cθ',ϕ'

Option 2: Cθ',φ'

Option 3: Ccosθ',cosφ'=sinθ'sinϕ'

Figure 3 Covariance matrix distribution under three UE directions

Figure 3 shows the coefficients in the covariance matrix for three UE directions. Detailed simulation assumptions can be found in Appendix B.

θ'=90°, ϕ'=45°

θ'=120°, ϕ'=0°

θ'=120°, ϕ'=45°

Under each UE direction, we compare the three covariance matrices

Option 1: Cθ',ϕ'

Option 2: Cθ',φ'

Option 3: Ccosθ',cosφ'=sinθ'sinϕ'

It can be seen that

Option 1 suffers from strong cross-correlation (large green components) between zenith error and azimuth error when the UE direction is off the boresight of both zenith and azimuth, e.g., θ'≠90° and  ϕ'≠0°.

Option 2 suffers from angle-dependent error power. For example, when θ'=90°, ϕ'=45°, one can find that φ'=45°. The error of φ' (yellow) is larger than that of θ' (blue)

Option 3 can have both small cross-correlation and angle-independent error power.

Proposal 2: The AoA error is represented by the error of the following two quantities

cosθ'

sinθ'sinϕ'

Where θ' and ϕ' are the reported ZOA and AOA in the local coordinate system

With the decorrelation of the angle measurement error, the error bound of each individual error component can be estimated via the similar scheme as ToA error.

- Proposal 3: Model the AoA error quantities cosθ'and sinθ'sinϕ'  as the normal distribution, and report the error bound associated with the allocated integrity risk for the AoA measurement via the paired over-bounding Gaussian formula.

Other aspects

Other aspects affecting the error source may include NLoS condition and multi-path.

For the case of NLoS condition, Rel-17 already supports NLoS indictor including soft value and hard value. The soft value can serve as the priority indictor for the location server to perform the RAIM algorithm for monitoring whether a specific TRP should be considered as the outlier, while the hard value can serve as the DNU suggestion from the node that has performed the measurement.

For the case of multi-path, the understanding from our side is that multi-path will affect the ToA estimate for the first path, because there may be aliasing in the time domain. However, since distribution of ToA is already up to UE/TRP, it should also be up to UE/TRP to adjust the variance of ToA measurement for the case of the dense multi-path condition.

- Observation 2: No additional work is needed for handling the measurement reporting to support integrity with respect to the NLoS condition and the multi-path.

Conclusion

In this contribution, we discuss the error source for integrity and generic handling of the measurement error distribution in the assistance of calculating the protection level. Based on the discussion, we have the following observations and proposals.

- Observation 1: The TOA error is close to a normal distribution if super resolution ToA estimation is used.

- Observation 2: No additional work is needed for handling the measurement reporting to support integrity with respect to the NLoS condition and the multi-path.

- Proposal 1: Model the ToA error as the normal distribution, and report the error bound associated with the allocated integrity risk for the ToA measurement via the paired over-bounding Gaussian formula.

This should apply to all DL RSTD, UE Rx – Tx time difference measurement, UL RTOA, and gNB Rx – Tx time difference.

The reference timing for DL RSTD should also have its reported bound.

Proposal 2: The AoA error is represented by the error of the following two quantities

cosθ'

sinθ'sinϕ'

Where θ' and ϕ' are the reported ZOA and AOA in the local coordinate system

- Proposal 3: Model the AoA error quantities cosθ'and sinθ'sinϕ'  as the normal distribution, and report the error bound associated with the allocated integrity risk for the AoA measurement via the paired over-bounding Gaussian formula.

- Reference

- RP-213588, Revised SID on Study on expanded and improved NR positioning, Intel, RAN#94-e, Electronic Meeting, 6th – 17th Dec, 2021.

- TS 38.305, NG Radio Access Network (NG-RAN); Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN.