3GPP TSG-RAN WG1 Meeting #109-e 	R1-2203166

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.5.2.2

Source:	Huawei, HiSilicon

Title:	Discussion on NR carrier phase positioning

Document for:	Discussion and Decision

Introduction

In RAN#94e, Rel-18 NR positioning Objective of SI or Core part WI or Testing part WI was approved in [1], which includes NR carrier phase for higher positioning accuracy.

The objective for RAN1 includes the following:

Study solutions for accuracy improvement based on NR carrier phase measurements [RAN1, RAN4] - Reference signals, physical layer measurements, physical layer procedures to enable positioning based on NR carrier phase measurements for both UE-based and UE-assisted positioning [RAN1] - Focus on reuse of existing PRS and SRS, with new reference signals only considered if found necessary

This paper discusses NR carrier phase and evaluate its positioning accuracy.

Theoretical analysis

CRLB analysis

The accuracy of TDOA positioning methods depends on the reference signal bandwidth. When the reference signal bandwidth is 100 MHz, decimeter-level positioning accuracy can be achieved, which was evaluated in [2]. However, carrier phase positioning does not depend on the bandwidth of the reference signal, and its positioning accuracy is related to the radio frequency. According to [3], the Cramer-Rao Lower Bound (CRLB) of the carrier phase positioning accuracy is as follows:

|  | var(d)≥2*c2M*(2πf)2*SNR | (1) |
| --- | --- | --- |

where var(d) is the ranging error variance, c is the speed of light, f is the radio frequency, SNR is the signal-to-noise ratio, and M is the number of measurements.

Also, the Cramer-Rao Lower Bound (CRLB) of the TDOA positioning accuracy is as follows:

|  | var(d)≥c2B2*SNR | (2) |
| --- | --- | --- |

where B2=-∞∞(2πf)2|S(f)|2df-∞∞|S(f)|2df is the mean square bandwidth of the signal and S(f) is the Fourier transform of the signal s(t).

The signal wavelength of FR1 is decimeter-level. Therefore, theoretically, carrier phase positioning can achieve centimeter-level to millimeter-level positioning accuracy based on carrier phase measurements.

Comparing with TDOA positioning methods, we have the following observation:

- Observation 1: Theoretically, the carrier phase positioning can improve the accuracy by one or two orders of magnitude compared with timing based positioning methods, reaching centimeter-level or millimeter-level positioning accuracy.

Carrier phase measurement

Although the carrier phase quantizes the phase of carrier frequency of the received signals, at the receiver side (UE for downlink/TRP for uplink), by RF down-converting, the carrier phase can be obtained in the baseband.

The mathematical derivation is expressed in the following:

The transmitted signal is given by:

|  | xt=stexpj2πfct+jϕ0 | (3) |
| --- | --- | --- |

where s(t) denotes the baseband signals, fc denotes the carrier frequency, ϕ0 denotes the RF phase. After passing the channel, the transmitted signal xt could be received at the receiver side. The received signal is given by:

|  | yt=st-τ0expj2πfct-τ0+jϕ0 | (4) |
| --- | --- | --- |

where τ0 denotes the transmission delay. By down-converting yt to the baseband with receive LO of exp-j2πfct-jϕ1, we have

|  | ybbt=ytexp-j2πfct-jϕ1=st-τ0exp-j2πfcτ0+j(ϕ0-ϕ1) | (5) |
| --- | --- | --- |

where ϕ1 denotes the receiver RF phase. The carrier phase can be measured as Φ=-2πfcτ0+ϕ0-ϕ1, where the term -2πfcτ0 is of interest.

- Observation 2: Carrier phase can be measured at the base band.

- By channel estimation in the frequency domain, the channel can be obtained as

|  | Hk=expjΦexp-j2πkfscsτ0,k∈{-K,…,K} | (6) |
| --- | --- | --- |

- where k denotes the index of the subcarrier, fscs is the subcarrier spacing. Then, performing the IFFT operation with the FFT size N, the carrier phase Φ of the center frequency is equivalent to the phase of the first path, where

|  | hn=expjΦsin2πKn-Nfscsτ0Nsinπn-Nfscsτ0N | (7) |
| --- | --- | --- |
|  | Φ= phase(h(n)) | (8) |

- Looking at (7) and (8), the time domain phase observation is constant for the time domain taps located at each side of the real LOS delay, while the LOS peak correspond to the tap with delay n=Nfscsτ0. In the presence of noise and multi-path, the estimated LOS tap can be set to the rounding value of the actual LOS path TOA.

- Observation 3: Carrier phase can be measured both in the frequency domain or time domain.

hn

Actual LOS path

Estimated CIR

hn

Actual LOS path

Estimated CIR

Figure 1 error between actual LOS path and estimated CIR

- Observation 4: Carrier phase of the center frequency of the LOS path is equivalent to the phase of the LOS path in the time domain.

- Under the multi-path environment, in the frequency domain, the phase of the center frequency in the frequency domain incorporates the phase of all the paths.

|  | Hk=iexpjΦiexp-j2πkfscsτi | (9) |
| --- | --- | --- |

- where τi and Φi denote the time delay and phase of the i-th path. Hence, the desired carrier phase of the LOS path will be deteriorated by the NLOS paths. By converting to the time domain, the paths of different delays can be separated. Hence, the interference of the NLOS path could be greatly suppressed.

- Observation 5: Measuring the carrier phase in the time domain can effectively mitigate the interference of the NLOS path.

Phase difference of arrival

- Similar to TDOA methods, the common unknown UE RF phase within the phases between UE and different TRPs can be eliminated by differential method, so that the DL phase difference of arrival (DL-PDOA) between the Rx carrier phase of different TRPs measured by the UE.

- Observation 6: Phase difference of arrival (PDOA) can eliminate the unknown phase offset of UE’s RF chains.

PRU deployment and double difference phase

To overcome the gNB carrier phase synchronization error, PRU (positioning reference unit) as discussed in Rel-17 can be considered.

It is well understood that PRU can mitigate the gNB timing synchronization error and group delay error, which now can also be applied to the carrier phase synchronization error. PDOA using PRU is also referred to as double difference methods.

The double difference carrier phase (inter-UE inter-gNB difference) can be calculated by:

|  | Φtpij=Φtij-Φpij=-2πfτtpij-2πNtpij | (10) |
| --- | --- | --- |

where τtpij=τtij-τpij represents the double difference propagation delay between PRU, target UE, i-th gNB and j-th gNB.

The detailed analysis can be found in Appendix.

- Observation 7: For both DL and UL positioning, the double difference carrier phase measurement based on PRU can compensate the gNB carrier phase synchronization error.

Location estimate via carrier phase measurement

Single frequency case

The carrier phase measurement can be used for positioning because it is related with to the distance between the gNB and the UE. The specific relationship is show as follows:

|  | d=cfN+Φ2π | (11) |
| --- | --- | --- |

where d is the distance between the gNB and the UE; c is the speed of light; N is the corresponding carrier phase integer ambiguity and is an integer; f is the carrier frequency; Φ is the corresponding carrier phase measurement. The following figure shows the relationship between carrier phase and distance.

Figure 2 relationship between carrier phase and distance

In order to estimate the distance d, it is necessary to estimate the integer ambiguity N correctly. Only when the integer ambiguity is estimated correctly, can the positioning accuracy of carrier phase reach the CRLB. We would like to note that the CRLB depicts the local gradient of the error at the ground truth, without taking into the account the potential miscalculation due to the phase ambiguity of 2π. The integer ambiguity and the UE position can be solved by using multiple gNB Antenna Reference Point (ARP) positions and the integer property of the integer ambiguity. Detailed location fix is in Section 4.4.

Multi-frequency case

Multiple carrier phases can be measured at different subcarriers. Carrier phases corresponding to virtual frequencies may be synthesized by using carrier phases on different frequencies.

Assume that the carrier phases measured on multiple frequencies (f1, f2, ⋯, fL) are Φ1, Φ2, ⋯, ΦL, the carrier phase measurement value on the i-th frequency satisfies the following formula:

|  | Φi=2πfi*dc-Ni, i=1,2,⋯,L | (12) |
| --- | --- | --- |

where d is the distance between the gNB and the UE; c is the speed of light; Ni is the corresponding carrier phase integer ambiguity.

The multi-frequency carrier phase measurements can be linearly combined to generate the virtual carrier phase measurement corresponding to the virtual frequency:

|  | Φa=i=1LkiΦi=2πi=1Lkifi*dc-Na | (13) |
| --- | --- | --- |

where Φa is the virtual carrier phase measurement and fa=i=1Lkifi is the corresponding virtual frequency; ki is the linear combination coefficient and is an integer; Na=i=1LkiNi is the corresponding carrier phase integer ambiguity. Low frequency corresponds to long wavelength, for which carrier phase integer ambiguity is easy to resolve, but the ranging accuracy using the corresponding carrier phase measurement is poor. On the other hand, high frequency corresponds to short wavelength, for which carrier phase integer ambiguity is difficult to be solved, but the ranging accuracy using the corresponding carrier phase measurement is high. Thus, K virtual frequencies with gradients (fa1, fa2, ⋯, faK) can be synthesized using different linear combination coefficients and the corresponding carrier phases are (Φa1, Φa2, ⋯, ΦaK) .

The integer ambiguity corresponding to the low frequency fa1 is solved by using the coarse range measurements firstly.

|  | Na1=ρ×fa1c-Φa12π | (14) |
| --- | --- | --- |

where * indicates rounding.

Combining with the carrier phase φa1 corresponding to fa1, the ranging result ρ1 with improved accuracy can be obtained.

|  | ρ1=cfa1(Na1+Φa12π) | (15) |
| --- | --- | --- |

Further, the integer ambiguity Na2 corresponding to fa2 can be obtained by bringing ρ1、fa2、Φa2 to Formula (14). In this way, the integer ambiguity is determined step by step, and a high-accuracy ranging result corresponding to the high frequency faK can be obtained.

Figure 3 stepwise integer ambiguity solution

- Observation 8: The virtual frequencies can be synthesized by multi-frequency carrier phase measurements, and the integer ambiguity can be solved step by step.

Target requirement for positioning using carrier phase measurement

- As explained, positioning using carrier phase measurement can achieve extremely high accuracy, to the level of centimeters and millimeters, yet the availability of such a high accuracy depends on whether the integer ambiguity can be properly resolved. In addition, there may also be implementation impairments as discussed in section 4.2 of the paper that hinder the wide coverage of such a service.

- We think that 50% should be considered as the starting point in Rel-18, so that at least for half the cases/time, the integer ambiguity resolution is successful.

- Proposal 1:  Rel-18 shall target the accuracy requirement of 1cm@50% for positioning using carrier phase measurement.

Simulation assumption

General simulation assumption

For the purpose of evaluating the accuracy achieved by positioning using carrier phase measurement, we suggest to consider the following simulation assumptions for InF-SH and InF-DH scenarios.

|  | FR1 Specific Values |
| --- | --- |
| Carrier frequency, GHz | 3.5GHz |
| Bandwidth, MHz | 100MHz |
| Subcarrier spacing, kHz | 30kHz for 100MHz |
| gNB model parameters |  |
| gNB noise figure, dB | 5dB |
| UE model parameters |  |
| UE noise figure, dB | 9dB – Note 1 |
| UE max. TX power, dBm | 23dBm – Note 1 |
| UE antenna configuration | Panel model 1 – Note 1 Mg = 1, Ng = 1, P = 2, dH = 0.5λ, (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) |
| UE antenna radiation pattern | Omni, 0dBi |
| PHY/link level abstraction | Explicit simulation of all links, individual parameters estimation is applied. Companies to provide description of applied algorithms for estimation of signal location parameters. |
| Network synchronization | The network synchronization error, per UE dropping, is defined as a truncated Gaussian distribution of (T1 ns) rms values between an eNB and a timing reference source which is assumed to have perfect timing, subject to a largest timing difference of T2 ns, where T2 = 2*T1 – That is, the range of timing errors is [-T2, T2] – T1: 0ns (perfectly synchronized), 50ns (Optional) |
| Network carrier phase synchronization | Random within [0:2pi] |
| UE/Network carrier frequency synchronization | See 4.2 |
| gNB ARP error | See 4.2 |
| Note 1: According to TR 38.802 Note 2: According to TR 38.901 |  |

Other parameters according to InF-SH/DH as defined in TR 38.857.

|  |  | FR1 Specific Values |
| --- | --- | --- |
| Channel model |  | InF-SH, InF-DH |
| Layout | Hall size | InF-SH: (baseline) 300x150 m (optional) 120x60 m InF-DH: (baseline) 120x60 m (optional) 300x150 m |
|  | BS locations | 18 BSs on a square lattice with spacing D, located D/2 from the walls. - for the small hall (L=120m x W=60m): D=20m - for the big hall (L=300m x W=150m): D=50m |
|  | Room height | 10m |
| Total gNB TX power, dBm |  | 24dBm |
| gNB antenna configuration |  | (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1), dH=dV=0.5λ – Note 1 |
| gNB antenna radiation pattern |  | Single sector – Note 1 |
| Penetration loss |  | 0dB |
| Number of floors |  | 1 |
| UE horizontal drop procedure |  | Uniformly distributed over the horizontal evaluation area for obtaining the CDF values for positioning accuracy, The evaluation area should be - (baseline) at least the convex hull of the horizontal BS deployment. - (optional) It can also be the whole hall area if the CDF values for positioning accuracy is obtained from whole hall area. |
| UE antenna height |  | Baseline: 1.5m (Optional): uniformly distributed within [0.5, X2]m, where X2 = 2m for scenario 1(InF-SH) and X2= for scenario 2 (InF-DH) |
| UE mobility |  | 3km/h |
| Min gNB-UE distance (2D), m |  | 0m |
| gNB antenna height |  | Baseline: 8m (Optional): two fixed heights, either {4, 8} m, or {max (4,), 8}. |
| Clutter parameters: {density , height ,size } |  | Low clutter density: {20%, 2m, 10m} High clutter density: - Baseline): {40%, 2m, 2m} for fixed UE antenna height and gNB antenna height - (Optional): {40%, 3m, 5m} - (Optional): {60%, 6m, 2m} |
| Note 1: According to Table A.2.1-7 in TR 38.802 |  |  |

Imperfect factors

Network carrier phase synchronization error

For the ideal case of location via PDOA, it is assumed that the RF carrier phase between TRPs are perfectly synchronized. However, it may not be the case for some commercial deployments.

This is analogous to gNB synchronization error for TDOA methods.

To model the RF carrier phase synchronization error between TRPs, the RF phase from each TRP could be independently set to be uniformly distributed within 0, 2π.

UE/Network carrier frequency synchronization

In the above analysis, the impact from the carrier frequency offset between gNB and UE is not considered. If such a frequency offset needs to be modelled, (5) can be rewritten as

|  | ybbt=st-τ0expj2πf0t-τ0+jϕ0exp-j2πf1t-jϕ1=st-τ0exp-j2πf1-f0t-j2πf0τ0+j(ϕ0-ϕ1) | (16) |
| --- | --- | --- |

And the corresponding carrier phase measurement is dependent on the measurement time t.

|  | Φ'=-2πf0τ0-2πf1-f0t+ϕ0-ϕ1-2πN' | (17) |
| --- | --- | --- |

where N' is the corresponding integer part.

gNB ARP error

For the carrier phase positioning, the gNB Antenna Reference Point (ARP) error will cause the mismatch between phase measurement and UE-gNB distance, which affects the positioning accuracy.

If we assume that the previous analysis is based on the ground truth of the gNB ARP, i.e. BS1, BS2, BS3 shown in Figure 4, the carrier phase measurement is also sufficiently accurate in accordance with the gNB ARP ground truth. However, the location fix engine may use the gNB location with some error, i.e. BS1’, BS2’, BS3’ as shown in Figure 4.

Modeling of the gNB ARP error can be done via:

Use the ground truth of gNB ARP location for channel generation.

Use the deviated gNB ARP location for location fix.

Figure 4 the schematic drawing for the gNB ARP error

Summary of impairments

With the above, we have the following proposal.

- Proposal 2:  To evaluate positioning using carrier phase measurement, the following impairment should be considered

Carrier phase synchronization between TRPs

Carrier frequency error between TRP and UE

gNB ARP error

Carrier phase estimation

The previous discussion assumes single antenna case. In multi-antenna scenarios, multi-antenna phase combination can be used to improve phase observation quality.

Figure 5 Schematic illustration for multi-antenna carrier phase combination based on AoA/ZoA

First, the AoA/ZoA is obtained based on the phases on different antennas. Different antenna array elements have different carrier phases to be observed. Then, the carrier phases of multiple antennas are combined based on the AoA/ZoA, the carrier phase after multi-antenna combination is:

|  | Φ=anglekronexpj2πndhsinθsinφ,expj2πmdvcosθ'×H | (18) |
| --- | --- | --- |

where dv and dh represent the vertical and horizontal antenna spacing. θ and φ represent the estimated ZoA and AoA, respectively. m=0,1,2,…,M-1 with M being the number of horizontal antennas and n=0,1,2,…N-1 with N being the number of vertical antennas. H is the channel response (including amplitude and phase) for the first path in the time domain.

Location fix utilizing carrier phase measurements

Based on the TDOA measurements, the rough UE position and the integer ambiguity floating point solution to each gNB can be obtained. Although the TDOA positioning result is not accurate, the search range of carrier phase positioning solution can be determined. There are two search forms:

Search mode 1: Taking TDOA position as the center, the position solutions are searched in the location space. The UE location candidate that minimizes the cost function is the search outcome.

Search mode 2: Taking integer ambiguity floating point solution as the center, the position solutions are searched in the integer ambiguity space. In other words, the search engine tries the candidate combination of integers, and UE location can be fixed for each combination with the resultant cost function. The combination of integers that minimizes the cost function is the search outcome and the corresponding UE location is determined.

Then the cost functions for modes can be either:

|  | x=argminx∈xTDOA±∆xTDOAiΦti(x)-Φti2 | (19) |
| --- | --- | --- |

Or if we also take the contribution of TDOA measurement into account

|  | x=argminx∈xTDOA±∆xTDOAidti(x)-dti2σi2+iΦti(x)-Φti2σΦi2 | (20) |
| --- | --- | --- |

Where

Φti is the i-th carrier phase measurement;

Φti(x)=2πf*||x-gposi||c-Nti, is the i-th carrier phase calculation;

x  is the UE position; Nti is the i-th integer ambiguity;

dti is the product of the TOA measurement and the speed of the light c; σi2 is the corresponding measurement variance;

Φti is the carrier phase measurement and σΦi2 is the variance of the carrier phase measurement .

dti(x)=||x-gposi|| is the theoretical value calculated from the search point position x and gNB ARP positions.

- x and Nti can be obtained based on each search point. gposi is the i-th gNB ARP position. For phase difference of arrival, Φti can be replaced by Φtij=Φtj-Φti where Φtj is the j-th carrier phase measurement. And Φti(x) can be replaced by Φtij(x)=Φtj(x)-Φti(x). For TDOA, dt(i)can be replaced by dt(ij)=dt(j)-dt(i) where dt(ij) is the product of the TDOA measurement and the speed of the light c. And dti(x) can be replaced by dt(ij)=dt(j)-dt(i). For the double difference carrier phase, the same can be obtained.

For search mode 1,  xTDOA is the TDOA positioning result and ∆xTDOA is the UE position search range. For search mode 2, the search space is Nti∈NTDOAi±∆NTDOAi, 1≤i≤3. At each search point, the UE position x can be obtained. Then the cost functions can be calculated.

To reduce the search process, some fast solving algorithms, such as PSO (Particle Swarm Optimization) and LAMBDA (Least square AMBiguity Decorrelation Adjustment) algorithms, can also be applied.

Evaluation results

Baseline evaluation

To compare the performance, we present the positioning accuracy in the InF-SH and InF-DH scenario. The UL-TDOA is evaluated as a comparison. Assuming that gNBs are time-synchronized, the simulation results of single difference carrier phase positioning are as follows:

Figure 6 The CDF curve of single difference carrier phase positioning under synchronous conditions

| Simulation Settings | Scenario | Positioning method | Positioning Error |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 50% | 67% | 80% | 90% |
| FR1, 100MHz, 30kHz SCS, 7 TRPs | InF-SH | UL-TDOA | 0.029m | 0.049m | 0.082m | 0.159m |
|  |  | Carrier phase | 0.00045m | 0.00056m | 0.00068m | 0.00091m |
|  | InF-DH | UL-TDOA | 0.047m | 0.084m | 0.155m | 0.403m |
|  |  | Carrier phase | 0.00048m | 0.00064m | 0.00097m | 0.389m |

Further, assuming that gNBs are asynchronous and the time synchronization error is 0.3 ns or the signals have random initial phase, the simulation results of single difference carrier phase positioning are as follows:

Figure 7 The CDF curve of single difference carrier phase positioning under asynchronous conditions

| Simulation Settings | Scenario | Positioning method | Positioning Error |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 50% | 67% | 80% | 90% |
| FR1, 100MHz, 30kHz SCS, 7 TRPs | InF-SH | UL-TDOA | 0.052m | 0.079m | 0.117m | 0.209m |
|  |  | Carrier phase | 0.339m | 0.397m | 0.446m | 0.509m |
|  | InF-DH | UL-TDOA | 0.065m | 0.104m | 0.176m | 0.480m |
|  |  | Carrier phase | 0.358m | 0.421m | 0.486m | 0.651m |

Theoretically, the double difference carrier phase positioning can eliminate the above errors, the simulation results of double difference carrier phase positioning under asynchronous conditions are as follows:

Figure 8 The CDF curve of double difference carrier phase positioning under asynchronous conditions

| Simulation Settings | Scenario | Positioning method | Positioning Error |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 50% | 67% | 80% | 90% |
| FR1, 100MHz, 30kHz SCS, 7 TRPs | InF-SH | UL-TDOA | 0.051m | 0.077m | 0.120m | 0.220m |
|  |  | Carrier phase | 0.00065m | 0.00084m | 0.00107m | 0.00171m |
|  | InF-DH | UL-TDOA | 0.071m | 0.113m | 0.202m | 0.460m |
|  |  | Carrier phase | 0.00086m | 0.00142m | 0.268m | 0.555m |

Evaluation of gNB ARP error

In this section, we provided our evaluation results for the cases with gNB ARP error. Both single difference positioning assuming gNB are perfectly carrier phase synchronized and double difference positioning without any synchronization assumption between gNB are shown for InF-SH only.

Figure 9 shows the evaluation results for gNB ARP error of 1cm under InF-SH with 100MHz positioning for single difference method and double difference method. The stair-case CDF plot shows the impact on the integer ambiguity resolution with a step size of a wavelength.

Figure 9 The CDF curve of single (left) and double (right) difference carrier phase positioning with gNB ARP error of 1cm

| Simulation Settings | Scenario | Positioning method | Positioning Error |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 50% | 67% | 80% | 90% |
| FR1, 100MHz, 30kHz SCS, 7 TRPs | InF-SH Single diff | UL-TDOA | 0.036m | 0.058m | 0.088m | 0.144m |
|  |  | Carrier phase | 0.007m | 0.010m | 0.090m | 0.183m |
|  | InF-SH Double diff | UL-TDOA | 0.058m | 0.090m | 0.141m | 0.237m |
|  |  | Carrier phase | 0.011m | 0.097m | 0.176m | 0.209m |

Figure 10 shows the evaluation results for gNB ARP error of 5cm under InF-SH with 100MHz positioning for single difference method and double difference method. When the gNB error is comparable with the wavelength, it is not possible for the carrier phase positioning to resolve the integer ambiguity.

Figure 10 The CDF curve of single (left) and double (right) difference carrier phase positioning with gNB ARP error of 5cm

| Simulation Settings | Scenario | Positioning method | Positioning Error |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 50% | 67% | 80% | 90% |
| FR1, 100MHz, 30kHz SCS, 7 TRPs | InF-SH Single diff | UL-TDOA | 0.050m | 0.072m | 0.107m | 0.169m |
|  |  | Carrier phase | 0.168m | 0.200m | 0.235m | 0.280m |
|  | InF-SH Double diff | UL-TDOA | 0.073m | 0.106m | 0.155m | 0.267m |
|  |  | Carrier phase | 0.167m | 0.195m | 0.233m | 0.276m |

Based on the evaluation results, we have the following observations.

- Observation 9: The gNB ARP error deteriorates the carrier phase positioning accuracy for both single difference method (assuming perfect synchronization) and double difference method (with PRU).

Identified specification impact

UE carrier phase measurement reporting

- Although the time domain measuring could mitigate the impact of the NLOS path. The measured carrier phase Φ is affected by the receiver imperfections. For the downlink carrier phase positioning, the Rx phase error can be mitigated by differentiating the measured phase of the LOS path.

|  | Φi,j=Φj-Φi | (21) |
| --- | --- | --- |

- where Φi denotes the carrier phase of the i-th TRP. In our opinion, reporting the differentiated phase value is helpful for mitigating the phase offset incurred by UE’s imperfect factors. Similar to the DL-TDOA method, the carrier phase can be reported along with the RSTD measurements, where the phase of the reference TRP can be normalized to 0.

- Proposal 3: For the downlink positioning methods, a reference TRP can be selected and the phase difference of other TRPs compared to the reference TRP can be reported.

Conclusion

In this contribution, we have the following observations and proposals with regards to the NR carrier phase positioning.

- Observation 1: Theoretically, the carrier phase positioning can improve the accuracy by one or two orders of magnitude compared with timing based positioning methods, reaching centimeter-level or millimeter-level positioning accuracy.

- Observation 2: Carrier phase can be measured at the base band.

- Observation 3: Carrier phase can be measured both in the frequency domain or time domain.

- Observation 4: Carrier phase of the center frequency of the LOS path is equivalent to the phase of the LOS path in the time domain.

- Observation 5: Measuring the carrier phase in the time domain can effectively mitigate the interference of the NLOS path.

- Observation 6: Phase difference of arrival (PDOA) can eliminate the unknown phase offset of UE’s RF chains.

- Observation 7: For both DL and UL positioning, the double difference carrier phase measurement based on PRU can compensate the gNB carrier phase synchronization error.

- Observation 8: The virtual frequencies can be synthesized by multi-frequency carrier phase measurements, and the integer ambiguity can be solved step by step.

- Observation 9: The gNB ARP error deteriorates the carrier phase positioning accuracy for both single difference method (assuming perfect synchronization) and double difference method (with PRU).

- Proposal 1:  Rel-18 shall target the accuracy requirement of 1cm@50% for positioning using carrier phase measurement.

- Proposal 2:  To evaluate positioning using carrier phase measurement, the following impairment should be considered

Carrier phase synchronization between TRPs

Carrier frequency error between TRP and UE

gNB ARP error

- Proposal 3: For the downlink positioning methods, a reference TRP can be selected and the phase difference of other TRPs compared to the reference TRP can be reported.

- Reference

- 3GPP RP-213561, New SID on expanded and improved NR positioning, Intel Incorporated, Electronic Meeting, Dec. 6 - 17, 2021

- 3GPP TR 38.857, Performance evaluation for Rel-17 positioning.

- Steven M K, Fundamentals of Statistical Signal Processing, Englewood Cliffs, NJ, USA: Prentice Hall PTR, 1993.

- Appendix

To analyze the phase correction via a PRU, we only consider random initial phase in this section.

For DL positioning, the carrier phase measurement obtained by PRU between the i-th gNB and the PRU is:

|  | Φpi=-2πfτpi-ϕp-ϕi-2πNpi | (22) |
| --- | --- | --- |

Npi is the i-th integer ambiguity. Similarly, the carrier phase measurement obtained by PRU between the j-th gNB and the PRU is:

|  | Φpj=-2πfτpj-ϕp-ϕj-2πNpj | (23) |
| --- | --- | --- |

So, the single difference carrier phase (inter-gNB difference) can be obtained at the PRU:

|  | Φpij=Φpj-Φpi=-2πfτpij-ϕij-2πNpij | (24) |
| --- | --- | --- |

where τpij=τpj-τpi represents the single difference propagation delay from the i-th gNB and the j-th gNB to the PRU, and ϕij=ϕi-ϕj represents the random initial phase error between the i-th gNB and the j-th gNB. Npij=Npj-Npi represents the single difference integer ambiguity.

Similarly, for target UE, the single difference carrier phase (inter-gNB difference) also can be obtained by:

|  | Φtij=Φtj-Φti=-2πfτtij-ϕij-2πNtij | (25) |
| --- | --- | --- |

So, the double difference carrier phase (inter-UE inter-gNB difference) can be calculated by (24) and (25):

|  | Φtpij=Φtij-Φpij=-2πfτtpij-2πNtpij | (26) |
| --- | --- | --- |

where τtpij=τtij-τpij represents the double difference propagation delay between PRU, target UE, i-th gNB and j-th gNB. Ntpij=Ntij-Npij represents the double difference integer ambiguity. With the help of PRU, it can be observed that the double difference carrier phase can compensate these non-ideal factors caused by random initial phase.

For UL positioning, the similar principles about the double difference carrier phase method based on PRU are still applicable, we won’t repeat it.

Figure 11  the double difference method based on PRU