3GPP TSG-RAN WG1 Meeting #109-e 	R1-2203167

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.5.2.3

Source:	Huawei, HiSilicon

Title:	Requirements and evaluation methodology for LPHAP

Document for:	Discussion and Decision

Introduction

RAN#94-e approve the SID in [1] with the LPHAP objective as below.

- Study the requirements on LPHAP as developed by SA1 and evaluate whether existing RAN functionality can support these power consumption and positioning requirements. Based on the evaluation, and, if found beneficial, study potential enhancements to help address any limitations [RAN2, RAN1] - Study is limited to a single representative use case (use case 6 as defined TS 22.104). The choice of selected use case can be reviewed at the start of the study. - Study is limited to enhancements to RRC_INACTIVE and/or RRC_IDLE state

In this paper, we discuss the requirements and evaluation methodology for LPHAP.

Target use cases and requirement

The SID used use case 6 in [2] as the representative use case for evaluation, which can be reviewed at the study.

Table 1 LPHAP use cases defined in [2]

| Use Case # | Use Case description | Horizontal accuracy | Corresponding service level (22.261) | Positioning interval/ duty cycle | battery life time/ minimum operation time |
| --- | --- | --- | --- | --- | --- |
| 1 | Process automation: Dolly tracking (outdoor). | 10 m | Service Level 1 | on request | 24 months |
| 2 | Process automation: Asset tracking. | 2 m to 3 m | Service Level 2 | < 4 seconds | > 6 months |
| 3 | Flexible modular assembly area: Tool tracking in flexible, modular assembly areas in smart factories. | < 1 m | Service Level 3 | no indication | 1 work shift - 8 hours (up to 3 days, 1 month for inventory purposes) |
| 4 | Process automation: Sequence container (Intralogistics). | < 1 m | Service Level 3 | 1 second | 6 - 8 years |
| 5 | Process automation: Palette tracking (e.g. in turbine construction). | < 1 m | Service Level 3 | 5 seconds - 15 minutes | 18 months |
| 6 | Flexible modular assembly area: Tracking of workpiece (in- and outdoor) in assembly area and warehouse. | < 1 m | Service Level 3 | 15 s to 30 s | 6 - 12 months |
| 7 | Flexible modular assembly area: Tool assignment (assign tool to vehicles in a production line, left/right) in flexible, modular assembly area in smart factories. | 30 cm | Service Level 5 | 250 ms | 18 months |
| 8 | Flexible modular assembly area: Positioning of autonomous vehicles for monitoring purposes (vehicles in line, distance 1.5 meter). | 30 cm | Service Level 5 | 1 second | 6 - 8 years (no strong limitation in battery size) |
| 9 | (Intra-)logistics: Asset tracking | 10m | Service level 1 | 20 minutes | 12 years (@20mJ/positioning fix) |

We believe that use case 6 can serve as the representative one that draws interest in the industry.

The target accuracy can meet most industry IoT requirements.

The positioning interval of tens of seconds offers the real-time update of the device location, e.g. three times per minute.

The battery life of half a year to one year appeals to the general impression of a long battery life.

We propose to consider use case 6 for the purpose of RAN1 evaluation.

- Proposal 1: Adopt use case 6 of TS 22.104 for power evaluation in RAN1.

Overall evaluation methodology

Accuracy aspects

On accuracy aspects, given that we already spent two releases to evaluate the positioning accuracy, it should be clear that for IIoT use case, with the positioning bandwidth of 100MHz for FR1 and FR2, the target accuracy under InF-DH/SH channel is able to meet the 1m accuracy.

- Observation 1: The existing evaluation already showed that 100MHz bandwidth can already meet the 1m accuracy.

To reduce the workload, we think there is no need to further optimize the accuracy aspects for LPHAP, but rather conclude that LPHAP target accuracy can be met for IIoT scenarios.

- Proposal 2: Conclude in RAN1 that LPHAP target accuracy can already be met for IIoT scenarios with the Rel-17 positioning techniques, with at least 100MHz positioning bandwidth.

Power aspects

The NR power evaluation used relative evaluation, assuming the number of power units for each reception/transmission model (e.g. PDCCH, PDSCH, UL, etc.). The power saving gain of any enhancement is shown in comparison with the baseline power consumption in terms of power units.

This evaluation methodology avoids disclosing the vendor-specific current/power profiles. However, using relative power evaluation does not directly convert to the absolute battery life, whereas LPHAP use cases are defined based on absolute battery life.

To overcome this, we suggest to also consider the relative evaluation of battery life, which can be done in the following steps.

First, consider a reference device, which can be e.g. a commercial cell-phone.

Second, assume a reference traffic for the reference device, which can be e.g. the traffic model used for calibration in TR 38.840, and calculate the power consumption rate, e.g. X1 power unit per second

Third, assume the battery volume/energy allocated for the reference traffic for the reference device, e.g. E1 mAh and the expected battery life, T1 hours.

Fourth, consider a target device to evaluate the battery life.

Fifth, assume a traffic for the target device, and calculate the power consumption rate, e.g. X2 power unit per second.

Sixth, assume the battery volume/energy allocated for the traffic for the target device, e.g. E2 mAh.

Last, calculate the expected battery life for the target device as

- T2=T1×E2E1×X1X2

If the total energy used for the reference traffic of the reference device is the same as that used for the traffic of the target device, i.e. E2=E1, the equation can be reduced to

- T2=T1×X1X2

This E2 = E1 is under the assumption that even if the reference device (commercial cell-phone) has a large battery volume, but most of the energy are consumed on the screen and applications, while the target device considered for positioning may concentrate most of its energy to positioning related procedures, and thus a smaller battery volume is expected, justifying using the same energy for both devices on their concerned traffic.

If the LPHAP target requirement is expressed in absolute battery life (e.g. 1 year), then the target is set as T2 ≥ 1 year and RAN1 evaluations can evaluate X2 and determine whether X2 can be made small enough to meet the target. This only requires RAN1 to agree on an assumption for T1 for the reference device (e.g. a commercial cellphone, for which T1 could be set to e.g. 8 hours). With such evaluation methodology, RAN1 could then inform RAN2 that RAN1 has the capability to evaluate the feasibility of a requirement expressed in absolute battery life for LPHAP.

It is also worth noting that this approach may also be used to estimate the increase of the battery volume to reach an even longer battery life.

Proposal 3: For the purpose of battery life evaluation for LPHAP, define a reference device (e.g. a commercial cellphone) with a reference traffic and a reference battery life.

Proposal 4: Adopt the following steps in the TR for evaluating the battery life, and inform RAN2 that RAN1 is able to evaluate the feasibility of a requirement expressed in absolute battery life for LPHAP.

First, consider a reference device, which can be e.g. a commercial cell-phone. Second, assume a reference traffic for the reference device, which can be e.g. the traffic model used for calibration in TR 38.840, and calculate the power consumption rate, e.g. X1 power unit per second Third, assume the battery volume/energy allocated for the reference traffic for the reference device, e.g. E1 mAh and the expected battery life, T1 hours. Fourth, consider a target device to evaluate the battery life. Fifth, assume a traffic for the target device, and calculate the power consumption rate, e.g. X2 power unit per second. Sixth, assume the battery volume/energy allocated for the traffic for the target device, e.g. E2 mAh. Last, calculate the expected battery life for the target device as - T2=T1×E2E1×X1X2

Power model

UL positioning

For Rel-17 positioning procedures to support UL positioning, the existing deferred MT-LR with event report is considered, as shown in Figure 1 [3].

Figure 1 Low Power Periodic and Triggered 5GC-MT-LR Procedure with SDT (UL-only positioning)

According to the TS 24.572/TS 24.080 [5][4], the event report may be configured to “area event”, which may be set to the UE serving cell only. Under this configuration, UE may send the SRS according to the periodicity, until UE leaves the area, in which case UE may report the event to the LMF, triggering the above procedure and get a new SRS configuration.

LCS-PeriodicTriggeredInvokeArg ::= SEQUENCE { referenceNumber [0] LCS-ReferenceNumber, h-gmlc-address [1] GSN-Address, qoS [2] LCS-QoS OPTIONAL, reportingPLMNList [3] ReportingPLMNList OPTIONAL, periodicLocation [4] PeriodicLocation OPTIONAL, areaEventReporting [5] AreaEventReporting OPTIONAL, motionEventReporting [6] MotionEventReporting OPTIONAL, ..., referenceNumberExt [7] LCS-ReferenceNumberExt OPTIONAL, h-gmlc-callBackUri [8] UTF8String OPTIONAL, supportedGADShapes [9] SupportedGADShapes OPTIONAL, deferredRoutingIdentifier [10] OCTET STRING OPTIONAL, reportingAccessTypes [11] ReportingAccessTypes OPTIONAL, multiplePositioningProtocolPDUs [12] MultiplePositioningProtocolPDUs OPTIONAL, controlPlane-CIoT-5GS-Optimisation [13] ControlPlane-CIoT-5GS-Optimisation OPTIONAL, scheduledLocTime [14] DateTime } -- Only one of periodicLocation, areaEventReporting and motionEventReporting shall be included. -- responseTime and velocityRequest are not applicable in LCS-QoS. -- reportingPLMNList provides a list of PLMNs in which event reporting is allowed. -- If referenceNumberExt is included, an MS shall ignore referenceNumber. -- h-gmlc-address shall be ignored by a UE for 5GS access. -- referenceNumberExt, h-gmlc-callBackUri, supportedGADShapes, deferredRoutingIdentifier, -- reportingAccessTypes, multiplePositioningProtocolPDUs and controlPlane-CIoT-5GS-Optimisation -- shall not be included for E-UTRA access to EPC. -- reportingPLMNList shall not be included for 5GS access. -- scheduledLocTime is only applicable for periodic location reporting, which indicates a time in future for the first periodic location to be reported. AreaEventReporting ::= SEQUENCE { deferredLocationEventType [0] DeferredLocationEventType, areaList [1] AreaList, occurrenceInfo [2] OccurrenceInfo OPTIONAL, intervalTime [3] IntervalTime OPTIONAL, maximumInterval [4] MaximumInterval OPTIONAL, samplingInterval [5] SamplingInterval OPTIONAL, duration [6] Duration OPTIONAL, locationInfo [7] LocationInfo OPTIONAL, ... } -- msAvailable and periodicLDR in DeferredLocationEventType are not applicable. -- intervalTime and maximumInterval are not applicable when OccurenceInfo is present with -- the value of oneTimeEvent. AreaList ::= SEQUENCE SIZE (1..maxAreas) OF Area maxAreas INTEGER ::= 250 Area ::= SEQUENCE { areaType [0] AreaType, areaIdentification [1] AreaIdentification, ..., areaIdentificationExt [2] AreaIdentificationExt } AreaType ::= ENUMERATED { trackingArea (0), ecgi (1), ..., trackingArea5GS (2), ncgi (3) } -- trackingArea5GS and ncgi shall not be included for an MS without 5GS access

- Observation 2: According to Rel-17 discussion and CT1 specification, the UE may be configured to transmit the periodic SRS without reporting anything to LMF if SRS is valid when UE is still within the last serving cell for the configured area event reporting.

Therefore, UE should be able to wake up to only transmit SRS for most of the time, and the power model for SRS transmission can be shown in Figure 2. UE needs to sync to SSB first, and the duration between SSB and SRS transmission is because of the typical TDD configuration of DDDDD-DDSUU with 30kHz SCS. For the SRS transmission, in order to increase the coverage so that the SRS can be received by a neighbouring cell, 23dBm transmission power may be assumed.

Deep sleep

Ramp down

SRS [23dBm]

SSB

Micro sleep

Ramp up

Deep sleep

4ms

Deep sleep

Ramp down

SRS [23dBm]

SSB

Micro sleep

Ramp up

Deep sleep

4ms

Figure 2 Power model for SRS transmission in RRC_INACTIVE

- Proposal 5: Adopt the following model for SRS transmission in RRC_INACTIVE for evaluating power consumption of UL positioning.

Deep sleep

Ramp down

SRS [23dBm]

SSB

Micro sleep

Ramp up

Deep sleep

4ms

Deep sleep

Ramp down

SRS [23dBm]

SSB

Micro sleep

Ramp up

Deep sleep

4ms

DL positioning

For Rel-17 positioning procedures to support DL positioning, the existing deferred MT-LR with event report is considered, as shown in Figure 3 [3].

Figure 3 Low Power Periodic and Triggered 5GC-MT-LR Procedure with SDT – DL-only and RAT-Independent positioning

It can be observed that for DL positioning, UE needs to report the measurement to the LMF along with event report configured by the LMF, and UE needs to receive the event report ACK and the RRCRelease message to terminate the SDT procedure.

Given that it may be difficult to define the timeline between SDT initiation and the subsequent RRCRelease message, we suggest to simply adopt the power model for PRS reception and measurement reporting shown in Figure 4, where PRS reception is within a slot, and UE needs to process the PRS in 20ms with light sleep given that UE is not required to maintain synchronization. UE receives the SSB, and selects the CG-SDT resource to report the measurement and potentially the event to the LMF, without further considering the PDCCH/PDSCH after the SDT that is used to convey the subsequent DL. Since the CG-SDT resource is only intended for the serving gNB, 0dBm transmission power may be assumed.

Deep sleep

Ramp down

CG-PUSCH [0dBm]

PRS

Ramp up

Deep sleep

[20] ms

SSB

Light sleep

4 ms

Micro sleep

Deep sleep

Ramp down

CG-PUSCH [0dBm]

PRS

Ramp up

Deep sleep

[20] ms

SSB

Light sleep

4 ms

Micro sleep

Figure 4 Power model for PRS reception and measurement reporting in RRC_INACTIVE

- Proposal 6: Adopt the following model for PRS reception and measurement reporting in RRC_INACTIVE for evaluating power consumption of UL positioning.

Deep sleep

Ramp down

CG-PUSCH [0dBm]

PRS

Ramp up

Deep sleep

[20] ms

SSB

Light sleep

4 ms

Micro sleep

Deep sleep

Ramp down

CG-PUSCH [0dBm]

PRS

Ramp up

Deep sleep

[20] ms

SSB

Light sleep

4 ms

Micro sleep

For PRS reception power, we suggest to follow the RRM model of intra-frequency measurement from TR 38.840, which is reflected in Table 2. It is worth noting that the actual power may be distributed after the buffering time, i.e. in the micro-sleep after the PRS slot, but here for simplicity, we only count them in the PRS slot.

Table 2 Power model for PRS reception

| N: Number of TRPs for positioning | Synchronous case |  |
| --- | --- | --- |
|  | FR1 | FR2 |
| N=4 | 120 | 195 |

- Proposal 7: Adopt the following power model for PRS reception.

| N: Number of TRPs for positioning | Synchronous case |  |
| --- | --- | --- |
|  | FR1 | FR2 |
| N=4 | 120 | 195 |

Deep sleep power

The current TR 38.840 assumed a deep sleep mode with 1 power unit per slot with the corresponding transition period

| Sleep type | Additional transition energy: (Relative power x ms) | Total transition time |
| --- | --- | --- |
| Deep sleep | 450 | 20 ms |
| Light sleep | 100 | 6 ms |
| Micro sleep | 0 | 0 ms* |
| * Immediate transition is assumed for power saving study purpose from or to a non-sleep state |  |  |

It can be perceived that for the UE doing positioning during RRC_INACTIVE state, most of the time will be in the deep sleep mode, and the power contribution in deep sleep constitutes most part of the UE power consumption. An ultra-deep sleep can be considered in the evaluation.

Ultra-deep sleep is not new to either 3GPP or positioning industry.

In 3GPP, during the CIoT study TR 45.820, the evaluation assumes “power saving state” (PSS) in the power evaluation, and it has been stated in the TR that:

PSS denotes a Power Saving State such as that achieved with the Rel-12 Power Save Mode feature. In Idle, the device may be consuming more power than in the PSS state because, for example, it is maintaining a more accurate time/frequency synchronization with the network.

Figure 5 Example of events affecting energy consumption for IP packet exchange (Figure 5.4-1 [7])

The power of PSS state is assumed as 15uW according to table 5.4-1 [7], which translates to 5uA if the voltage is 3.3V.

Table 3 Key input parameters for energy consumption analysis (Table 5.4-1 [7])

| (1) Battery capacity (Wh) | (2) Battery power during Tx (mW) | (3) Battery power for Rx (mW) | (4) Battery power when Idle but not in PSS (mW) | (5) Battery power in Power Save State (PSS) (mW) | (6) Time between end of IP packet carrying "report" and start of IP packet carrying "ack" on radio (ms) | (7) Number of reports per day |
| --- | --- | --- | --- | --- | --- | --- |
| 5 |  |  |  | [0,015] | 1000 |  |

In positioning industry, as shown in Figure 6 from the Decawave 1000 data sheet for the UWB product, the sleep current and deep sleep current can be as low as 1uA and 50nA, respectively, which helps UWB to claim the battery life of more than a year. Note that the current of the sleep mode for the UWB device is less than a thousandth of the IDLE mode.

Figure 6 Operating States and their effect on power consumption (Table 24 [8])

- Observation 3: Both CIoT and UWB considered a very deep sleep mode with current in the order of uA, which is much lower than the assumption of deep sleep in TR 38.840.

- Proposal 8: Define a new ultra-deep sleep type for the evaluation of LPHAP.

The power unit of ultra-deep sleep can take 0.01 power unit as the starting point.

UE enters ultra-deep sleep when UE is not performing positioning.

FFS any change to the transition energy and/or period for the ultra-deep sleep compared with that of the current deep sleep mode.

Initial evaluation results

Power consumption for the reference device

The simulation for the reference device can be implemented via the same calibration effort as conducted during Rel-16.

The assumption of the reference traffic for the reference device is listed in Table 4.

Table 4 Reference traffic for the reference device

| Parameters | Reference traffic |
| --- | --- |
| Traffic model | FTP (model 3) |
| Packet Size | 0.5 Mbytes (TR 38.840) |
| Mean interval | 200 ms |
| DRX setting | Period: 160ms On duration: 8ms Inactivity timer: 100ms |
| Positioning period | N/A |
| RRM | Not accounting for RRM |
| Deep sleep power unit | 1 |

The power consumption model considered in the evaluation is listed in Table 5.

Table 5 Power model assumption for the reference device

| UE power state | Power units per slot |
| --- | --- |
| PDCCH only | 100 |
| PDCCH+PDSCH | 300 |
| Micro sleep | 45 |
| Light sleep | 20 Note: additional 100 PU of 6ms transition time is required. |
| Deep sleep | 1 Note: additional 450 PU of 20ms transition time is required |

Based on the assumption, the power consumption can be evaluated as below, and X1, the power consumption rate, for the reference device is given by Table 6.

State Time TimeRatio Power PwrRatio LightSleep 2903 0.14515% 69760 0.09395% MicroSleep 214 0.01070% 9630 0.01297% PDCCH_Only 689137 34.45685% 68913700 92.80733% PDCCH_PDSCH 4896 0.24480% 1468800 1.97806% DeepSleep 1302850 65.14250% 3792700 5.10770% Total Number of State: 5 Total Number of Slots: 2000000 Total Power: 74254590

Table 6 Power consumption rate for the reference device

| Power consumption rate | X1 = 74254590 per 1000s = 74000 [PU/s] |
| --- | --- |
| Battery volume for the traffic | E1 |
| Battery life | T1 |

Power consumption evaluation for Rel-17 baseline

UL positioning

The assumption of the traffic for the Rel-17 UL positioning in RRC_INACTIVE state is listed in Table 7.

Table 7 Traffic for UL positioning in RRC_INACTIVE state for Rel-17

| Parameters | Reference traffic |
| --- | --- |
| Traffic model | N/A |
| Packet Size | N/A |
| Mean interval | N/A |
| DRX setting | I-DRX with 10.24s periodicity |
| Positioning period | 20.48s |
| RRM | Not accounting for RRM |
| Deep sleep power unit | 1 |

The power consumption model considered in the evaluation is listed in Table 8

Table 8 Power model assumption for the Rel-17 device with UL positioning in RRC_INACTIVE state

| UE power state | Power units per slot |
| --- | --- |
| SSB synchronization | 100 |
| SRS Tx | 210 |
| Micro sleep | 45 |
| Light sleep | 20 Note: additional 100 PU of 6ms transition time is required. |
| Deep sleep | 1 Note: additional 450 PU of 20ms transition time is required |

Based on the assumption, the power consumption can be evaluated as below, and X2, the power consumption rate, for the Rel-17 device is given by Table 9.

Table 9 Power consumption rate of the Rel-17 device with UL positioning

| SRS Tx | Duration (ms) | Power within the duration |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Micro sleep | 3 | 270 |  |  |  |  |  |
| SSB Sync | 0.5 | 100 |  |  |  |  |  |
| SRS Tx (23dBm) | 0.5 | 210 |  |  |  |  |  |
| Total | 4 | 580 |  |  |  |  |  |
| INACTIVE state UL positioning every 20.48s |  |  |  |  |  |  |  |
| Power state | Power unit | Duration (ms) |  | Power unit | Sum duration (ms) | Instances | Power ratio |
| Ramp up + down | 450 | 20 |  | 450 | 20 | 1 | 1.0729% |
| SRS Tx | 580 | 4 |  | 580 | 4 | 1 | 1.3829% |
| Deep sleep | 2 | 1 |  | 40912 | 20456 | 20456 | 97.5442% |
| Total |  |  |  | 41942 | 20480 |  | 100.0000% |
| Power consumption rate |  |  | X2UL, Rel-17 = 41942 per 20.48s = 2000 [PU/s] |  |  |  |  |
| Battery volume for the traffic |  |  | E2 |  |  |  |  |
| Battery life |  |  | T2UL, Rel-17 |  |  |  |  |

DL positioning

The assumption of the traffic for the Rel-17 DL positioning in RRC_INACTIVE state is listed in Table 10.

Table 10 Traffic for DL positioning in RRC_INACTIVE state for Rel-17

| Parameters | Reference traffic |
| --- | --- |
| Traffic model | Periodic |
| Packet Size | Small packet via a single slot PUSCH |
| Mean interval | 20.48s |
| DRX setting | I-DRX with 10.24s periodicity |
| Positioning period | 20.48s |
| RRM | Not accounting for RRM |
| Deep sleep power unit | 1 |

The power consumption model considered in the evaluation is listed in Table 11.

Table 11 Power model assumption for the Rel-17 device with DL positioning in RRC_INACTIVE state

| UE power state | Power units per slot |
| --- | --- |
| PRS Rx | 120 |
| SSB synchronization | 100 |
| PUSCH Tx (0dBm) | 250 |
| Micro sleep | 45 |
| Light sleep | 20 Note: additional 100 PU of 6ms transition time is required. |
| Deep sleep | 1 Note: additional 450 PU of 20ms transition time is required |

Based on the assumption, the power consumption can be evaluated as below, and X1, the power consumption rate, for the Rel-17 device is given by Table 12.

Table 12 Power consumption rate for the Rel-17 device with DL positioning

| PRS meas./report | Duration (ms) | Power within the duration |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PRS Rx | 0.5 | 120 |  |  |  |  |  |
| Light sleep | 13.5 | 540 |  |  |  |  |  |
| Ramp up/down for light sleep | 6 | 100 |  |  |  |  |  |
| SSB Sync | 0.5 | 100 |  |  |  |  |  |
| Micro sleep | 3 | 270 |  |  |  |  |  |
| PUSCH (0dBm) | 0.5 | 250 |  |  |  |  |  |
| Total | 24 | 1380 |  |  |  |  |  |
| INACTIVE state DL positioning every 20.48s |  |  |  |  |  |  |  |
| Power state | Power unit | Duration (ms) |  | Power unit | Sum duration (ms) | Instances | Power ratio |
| Ramp up + down | 450 | 20 |  | 450 | 20 | 1 | 1.0538% |
| PRS meas./report | 1380 | 24 |  | 1380 | 24 | 1 | 3.2317% |
| Deep sleep | 2 | 1 |  | 40872 | 20436 | 20436 | 95.7145% |
| Total |  |  |  | 42702 | 20480 |  | 100.0000% |
| Power consumption rate |  |  | X2DL, Rel-17 = 42702 per 20.48s = 2000 [PU/s] |  |  |  |  |
| Battery volume for the traffic |  |  | E2 |  |  |  |  |
| Battery life |  |  | T2DL, Rel-17 |  |  |  |  |

Summary for power consumption evaluations of Rel-17 device

Based on the evaluation in section 5.2.1 and 5.2.2, the following observations are summarized:

- Observation 4: The ratio of power consumption rate between Rel-17 UL positioning/DL positioning and the reference device is approximately 1:37.

Observation 5: 97.5% and 95% of the total power consumption is on the deep sleep for UL and DL positioning, respectively.

Observation 6: Assuming the same energy used for the reference traffic on reference device (for communication) as that used for positioning on the Rel-17 device, and assuming T1, i.e. the battery life for the reference device, is 8 hours, the battery life of Rel-17 device is approximately 300 hours (8x37), which cannot meet LPHAP requirements for use case 6.

Power consumption evaluation for ultra-deep sleep

The evaluation can be adjusted if the ultra-deep sleep replaces deep sleep between each positioning interval.

| Deep sleep | 1 Note: additional 450 PU of 20ms transition time is required |
| --- | --- |
| Ultra-deep sleep | 0.01 Note: additional 450 PU of 25ms transition time is required |

The evaluation results for UL and DL are provided for ultra-deep sleep in Table 13 and Table 14, respectively.

Table 13 Power consumption rate for ultra-deep sleep with UL positioning

| INACTIVE state UL positioning every 20.48s |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Power state | Power unit | Duration (ms) |  | Power unit | Sum duration (ms) | Instances | Power ratio |
| Ramp up + down | 450 | 25 |  | 450 | 25 | 1 | 31.2713% |
| SRS Tx | 580 | 4 |  | 580 | 4 | 1 | 40.3052% |
| Ultra-deep sleep | 0.02 | 1 |  | 409.02 | 20451 | 20451 | 28.4235% |
| Total |  |  |  | 1439.02 | 20480 |  | 100.0000% |
| Power consumption rate |  |  | X2UL, Rel-18 = 1439.02 per 20.48s = 70 [PU/s] |  |  |  |  |
| Battery volume for the traffic |  |  | E2 |  |  |  |  |
| Battery life |  |  | T2DL, Rel-17 |  |  |  |  |

Table 14 Power consumption rate for ultra-deep sleep with DL positioning

| INACTIVE state DL positioning every 20.48s |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Power state | Power unit | Duration (ms) |  | Power unit | Sum duration (ms) | Instances | Power ratio |
| Ramp up + down | 450 | 25 |  | 450 | 25 | 1 | 20.1017% |
| PRS meas./report | 1380 | 24 |  | 1380 | 24 | 1 | 61.6451% |
| Ultra-deep sleep | 0.02 | 1 |  | 408.62 | 20431 | 20431 | 18.2532% |
| Total |  |  |  | 2238.62 | 20480 |  | 100.0000% |
| Power consumption rate |  |  | X2DL, Rel-17 = 2238.62 per 20.48s = 109 [PU/s] |  |  |  |  |
| Battery volume for the traffic |  |  | E2 |  |  |  |  |
| Battery life |  |  | T2DL, Rel-17 |  |  |  |  |

With the evaluations shown in Table 13 and Table 14, the following observation are summarized:

- Observation 7: The ratio of power consumption rate between the LPHAP device with ultra-deep sleep and

- The Rel-17 baseline is approximately 1:28 for UL and 1:18 for DL;

The reference device is approximately 1:1000 for UL and 1:700 for DL.

Observation 8: Assuming the same energy used for the reference traffic on reference device as that used for positioning on the Rel-18 LPHAP device, and assuming T1, i.e. the battery life for the reference device, is 8 hours, the battery life of the Rel-18 LPHAP device with ultra-deep sleep is approximately 5600~8000 hours, which can meet LPHAP requirements for use case 6.

Conclusion

In this contribution, we have discussed the requirements and evaluation methodology for LPHAP. In addition, initial battery life estimation is provided. With the discussion, we have the following observations and proposals.

- Observation 1: The existing evaluation already showed that 100MHz bandwidth can already meet the 1m accuracy.

- Observation 2: According to Rel-17 discussion and CT1 specification, the UE may be configured to transmit the periodic SRS without reporting anything to LMF if SRS is valid when UE is still within the last serving cell for the configured area event reporting.

- Observation 3: Both CIoT and UWB considered a very deep sleep mode with current in the order of uA, which is much lower than the assumption of deep sleep in TR 38.840.

- Observation 4: The ratio of power consumption rate between Rel-17 UL positioning/DL positioning and the reference device is approximately 1:37.

Observation 5: 97.5% and 95% of the total power consumption is on the deep sleep for UL and DL positioning, respectively.

Observation 6: Assuming the same energy used for the reference traffic on reference device (for communication) as that used for positioning on the Rel-17 device, and assuming T1, i.e. the battery life for the reference device, is 8 hours, the battery life of Rel-17 device is approximately 300 hours (8x37), which cannot meet LPHAP requirements for use case 6.

- Observation 7: The ratio of power consumption rate between the LPHAP device with ultra-deep sleep and

The Rel-17 baseline is approximately 1:28 for UL and 1:18 for DL;

The reference device is approximately 1:1000 for UL and 1:700 for DL.

Observation 8: Assuming the same energy used for the reference traffic on reference device as that used for positioning on the Rel-18 LPHAP device, and assuming T1, i.e. the battery life for the reference device, is 8 hours, the battery life of the Rel-18 LPHAP device with ultra-deep sleep is approximately 5600~8000 hours, which can meet LPHAP requirements for use case 6.

- Proposal 1: Adopt use case 6 of TS 22.104 for power evaluation in RAN1.

- Proposal 2: Conclude in RAN1 that LPHAP target accuracy can already be met for IIoT scenarios with the Rel-17 positioning techniques, with at least 100MHz positioning bandwidth.

Proposal 3: For the purpose of battery life evaluation for LPHAP, define a reference device (e.g. a commercial cellphone) with a reference traffic and a reference battery life.

Proposal 4: Adopt the following steps in the TR for evaluating the battery life, and inform RAN2 that RAN1 is able to evaluate the feasibility of a requirement expressed in absolute battery life for LPHAP.

First, consider a reference device, which can be e.g. a commercial cell-phone. Second, assume a reference traffic for the reference device, which can be e.g. the traffic model used for calibration in TR 38.840, and calculate the power consumption rate, e.g. X1 power unit per second Third, assume the battery volume/energy allocated for the reference traffic for the reference device, e.g. E1 mAh and the expected battery life, T1 hours. Fourth, consider a target device to evaluate the battery life. Fifth, assume a traffic for the target device, and calculate the power consumption rate, e.g. X2 power unit per second. Sixth, assume the battery volume/energy allocated for the traffic for the target device, e.g. E2 mAh. Last, calculate the expected battery life for the target device as - T2=T1×E2E1×X1X2

- Proposal 5: Adopt the following model for SRS transmission in RRC_INACTIVE for evaluating power consumption of UL positioning.

Deep sleep

Ramp down

SRS [23dBm]

SSB

Micro sleep

Ramp up

Deep sleep

4ms

Deep sleep

Ramp down

SRS [23dBm]

SSB

Micro sleep

Ramp up

Deep sleep

4ms

- Proposal 6: Adopt the following model for PRS reception and measurement reporting in RRC_INACTIVE for evaluating power consumption of UL positioning.

Deep sleep

Ramp down

CG-PUSCH [0dBm]

PRS

Ramp up

Deep sleep

[20] ms

SSB

Light sleep

4 ms

Micro sleep

Deep sleep

Ramp down

CG-PUSCH [0dBm]

PRS

Ramp up

Deep sleep

[20] ms

SSB

Light sleep

4 ms

Micro sleep

- Proposal 7: Adopt the following power model for PRS reception.

| N: Number of TRPs for positioning | Synchronous case |  |
| --- | --- | --- |
|  | FR1 | FR2 |
| N=4 | 120 | 195 |

- Proposal 8: Define a new ultra-deep sleep type for the evaluation of LPHAP.

The power unit of ultra-deep sleep can take 0.01 power unit as the starting point.

UE enters ultra-deep sleep when UE is not performing positioning.

FFS any change to the transition energy and/or period for the ultra-deep sleep compared with that of the current deep sleep mode.

- Reference

- RP-213588, Revised SID on Study on expanded and improved NR positioning, Intel, RAN#94-e, Electronic Meeting, 6th – 17th Dec, 2021.

- TS 22.104, Service requirements for cyber-physical control applications in vertical domains.

- R2-2203949, LS on Positioning in RRC_INACTIVE State, RAN2 (Qualcomm, Huawei), RAN2#117-e, Electronic, 21st Feb. – 3rd March, 2022

- TS 24.080, Supplementary services specification; Formats and coding.

- TS 24.571, 5G System (5GS); Control plane Location Services (LCS) procedures; Stage 3.

- TR 38.840, NR; Study on User Equipment (UE) power saving in NR.

- TR 45.820, Cellular system support for ultra-low complexity and low throughput Internet of Things (CIoT).

- https://www.decawave.com/wp-content/uploads/2020/04/DW1000_Datasheet.pdf