3GPP TSG-RAN WG1 Meeting #109-e 	R1-2203168

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.5.3

Source:	Huawei, HiSilicon

Title:	Discussion on RedCap positioning

Document for:	Discussion and Decision

Introduction

In RAN#94e, the SID of Rel-18 expanded and improved NR positioning was approved as in [1], where the objective of RedCap positioning is excerpted as follows:

- Positioning support for RedCap UEs, considering the following: - Evaluate positioning performance of existing positioning procedures and measurements with RedCap UEs[RAN1] - Based on the evaluation, assess the necessity of enhancements and, if needed, identify enhancements to help address limitations associated with for RedCap UEs [RAN1, RAN2]

In Rel-17 and previous releases, the support of NR positioning is in the form of introducing additional UE capability, where the positioning performance of normal eMBB devices are discussed and evaluated in most cases. The objective of studying RedCap positioning includes evaluating the positioning performance of RedCap UEs and further identifying enhancements. We would like to first introduce some potential use cases and identify the target requirements for RedCap positioning. Then the positioning performance of existing positioning methods is evaluated for RedCap UE. Finally, we provide our views on the necessity of enhancements for RedCap positioning to fulfill the identified performance requirement.

Discussion

Use cases and requirements for RedCap positioning

Rel-17 introduces RedCap UE with reduced capabilities compared to the existing eMBB device [2]:

- The maximum bandwidth is 20 MHz for FR1, and is 100 MHz for FR2. UE features and corresponding capabilities related to UE bandwidths wider than 20 MHz in FR1 or wider than 100 MHz in FR2 are not supported by RedCap UEs;

- The maximum mandatory supported DRB number is 8;

- The mandatory supported PDCP SN length is 12 bits while 18 bits being optional;

- The mandatory supported RLC AM SN length is 12 bits while 18 bits being optional;

- 1 DL MIMO layer if 1 Rx branch is supported, and 2 DL MIMO layers if 2 Rx branches are supported. UE features and corresponding capabilities related to more than 2 UE Rx branches and more than 2 DL MIMO layers, as well as UE features and capabilities related to more than 2 UE Tx branches and more than 2 UL MIMO layers are not supported by RedCap UEs;

- CA, MR-DC, DAPS, CPAC and IAB ( i.e., the RedCap UE is not expected to act as IAB node) related UE features and corresponding capabilities are not supported by RedCap UEs. All other feature groups or components of the feature groups as captured in TR 38.822 [24] as well as capabilities specified in this specification remain applicable for RedCap UEs same as non-RedCap UEs, unless indicated otherwise.

For RedCap UE, some typical use cases and specific requirements were discussed and identified in WI phase [3], which are summarized as follows:

Figure 2.1-1 Use case and specific requirements for RedCap UE

|  | Industrial wireless sensors | Video surveillance | Wearables |
| --- | --- | --- | --- |
| Communication service availability | 99.99% | 99-99.9% | 99% |
| Latency | < 100ms (5-10ms for safety) | < 500ms | < 50ms |
| Bit rate | < 2Mbps | 2-25Mbps | 150Mbps/50Mbps (as LTE cat 4) |
| Mobility | Low | Low | High |
| Battery life | A few years | - | 1-2 weeks |

Observing Figure 2.1-1, we can find the typical use cases such as sensors and wearables mostly appear in vertical domains, e.g. industrial environments. According to TS 22.104 [4], the positioning performance requirements for the RedCap-like UE (e.g. wearables in augmented reality, sensors in inbound logistics) are highlighted as follows.

Figure 2.1-2 Positioning performance requirements

| Scenario | Horizontal accuracy | Vertical accuracy | Availability | Heading | Latency for position estimation of UE | UE speed | Corresponding Positioning Service Level in TS 22.261 [5] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Mobile control panels with safety functions (non-danger zones) | < 5 m | < 3 m | 90 % | n/a | < 5 s | n/a | Service Level 2 |
| Process automation – plant asset management | < 1 m | < 3 m | 90 % | n/a | < 2 s | < 30 km/h | Service Level 3 |
| Flexible, modular assembly area in smart factories (for tracking of tools at the work-place location) | < 1 m (relative positioning) | n/a | 99 % | n/a | 1 s | < 30 km/h | Service Level 3 |
| Augmented reality in smart factories | < 1 m | < 3 m | 99 % | < 0.17 rad | < 15 ms | < 10 km/h | Service Level 4 |
| Mobile control panels with safety functions in smart factories (within factory danger zones) | < 1 m | < 3 m | 99.9 % | < 0.54 rad | < 1 s | n/a | Service Level 4 |
| Flexible, modular assembly area in smart factories (for autonomous vehicles, only for monitoring purposes) | < 50 cm | < 3 m | 99 % | n/a | 1 s | < 30 km/h | Service Level 5 |
| Inbound logistics for manufacturing (for driving trajectories (if supported by further sensors like camera, GNSS, IMU) of indoor autonomous driving systems)) | < 30 cm (if supported by further sensors like camera, GNSS, IMU) | < 3 m | 99.9 % | n/a | 10 ms | < 30 km/h | Service Level 6 |
| Inbound logistics for manufacturing (for storage of goods) | < 20 cm | < 20 cm | 99 % | n/a | < 1 s | < 30 km/h | Service Level 7 |

Considering the reduced capabilities and positioning requirements, we have the following observation and proposal for RedCap positioning:

- Observation 1: The use cases of RedCap UEs (e.g., sensors, wearables) would require a sub-meter (< 1m in horizontal) positioning accuracy considering the reduced capabilities:

Bandwidth: 20MHz at FR1

Antenna setting: 1Tx&1Rx, 1Tx&2Rx

- Proposal 1: Set the following target requirements for RedCap positioning:

Horizontal positioning accuracy: <1m

Vertical positioning accuracy: <3m

Performance evaluation

In this section, the simulation results of achievable positioning accuracy for RedCap positioning using existing positioning methods, e.g. UL-TDOA, are provided. As is shown in Figure 2.2-1, we compare the positioning performance of eMBB UE with 100MHz bandwidth (Case 1) and that of RedCap UE with 20MHz bandwidth (Case 2).

Case 1. Single 100MHz

(eMBB UE)

Case 2. Single 20MHz

(RedCap UE)

Case 1. Single 100MHz

(eMBB UE)

Case 2. Single 20MHz

(RedCap UE)

Figure 2.2-1 Simulation scenarios for the RedCap positioning

- Case 1: SRS transmission with a single 100MHz (eMBB UE).

- Case 2: SRS transmission with a single 20MHz (RedCap UE).

Table 2.2-1 shows the positioning accuracy of the considered simulation scenarios with simulation assumptions in Appendix A. Note that according to the study on NR positioning in Rel-17, the typical positioning method, e.g., DL/UL TDOA, a larger bandwidth (e.g., 100MHz) is needed to achieve high accuracy positioning (sub-meter positioning accuracy) [6].  Therefore, RedCap positioning is facing the risk of degraded positioning accuracy due to the reduced bandwidth (i.e., 20MHz), which is also shown by the deteriorated performance of Case 2 in Table 2.2-1.

Table 2.2- 1 Positioning accuracy comparison of different cases (UL-TDOA positioning, FR1)

| Bandwidth | LOS/NLOS | Positioning accuracy (in unit of meter) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50% | 67% | 80% | 90% |
| Case 1: 100MHz | Ideal LOS | 0.034 | 0.057 | 0.103 | 0.205 |
|  | NLOS detection | 0.035 | 0.061 | 0.117 | 0.258 |
| Case 2: 20MHz | Ideal LOS | 0.617 | 0.979 | 1.553 | 2.593 |
|  | NLOS detection | 0.632 | 1.035 | 1.677 | 3.119 |
| Note: 1) Ideal LOS means LOS path is perfectly identified. 2) NLOS detection means NLOS path is deleted via some NLOS identification algorithms. |  |  |  |  |  |

Based on the simulation results of Table 2.2-1, we have the following observations:

Observation 2: Compared with eMBB UE, the positioning accuracy for RedCap UE is seriously degraded due to the reduced bandwidth.

- Observation 3: The performance for RedCap UE with existing positioning methods (>3m@90% with NLOS detection) cannot meet the sub-meter positioning accuracy requirement.

Therefore, it’s necessary to further study how to achieve a relatively high positioning accuracy for RedCap positioning, in order to meet the performance requirement of RedCap UE.

- Proposal 2: Identify positioning solutions suitable for RedCap UEs to achieve high accuracy positioning, i.e., sub-meter positioning accuracy.

Conclusion

In this contribution, we mainly provide our opinions on the target requirements and the necessity of potential enhancement for RedCap positioning. Based on the discussion, we have the following observations and proposals.

Target requirements for RedCap positioning

- Observation 1: The use cases of RedCap UEs (e.g., sensors, wearables) would require a sub-meter (< 1m in horizontal) positioning accuracy considering the reduced capabilities:

Bandwidth: 20MHz at FR1

Antenna setting: 1Tx&1Rx, 1Tx&2Rx

- Proposal 1: Set the following target requirements for RedCap positioning:

Horizontal positioning accuracy: <1m

Vertical positioning accuracy: <3m

Evaluation and enhancements for RedCap positioning

Observation 2: Compared with eMBB UE, the positioning accuracy for RedCap UE is seriously degraded due to the reduced bandwidth.

- Observation 3: The performance for RedCap UE with existing positioning methods (>3m@90% with NLOS detection) cannot meet the sub-meter positioning accuracy requirement.

- Proposal 2: Identify positioning solutions suitable for RedCap UEs to achieve high accuracy positioning, i.e., sub-meter positioning accuracy.

- References

- 3GPP RP-213561, New SID on expanded and improved NR positioning, Intel Incorporated, Electronic Meeting, Dec. 6 - 17, 2021

- 3GPP R2-2204040, Running 38.306 CR for the RedCap capabilities, Intel Corporation, Electronic Meeting, 21 Feb- 3 March, 2022

- 3GPP RP-210918, Revised WID on support of reduced capability NR devices, Nokia, Ericsson, Electronic Meeting, 22nd – 26th March, 2021

- 3GPP TS 22.104, “Service requirements for cyber-physical control applications in vertical domains (Release 17)”

- 3GPP TS 22.261, “Service requirements for the 5G system (Release 17)”

- TR 38.857, Study on NR positioning enhancements, 2021-03.

- Appendix A

In this part, the simulation scenarios and parameters are presented for the evaluation of RedCap positioning.

1) Scenario

We consider an InF-SH (Indoor Factory, Sparse-clutter) scenario as follows, where the clutter density is 20%, the clutter height is set as 2m, and the clutter size is 10m.

2) System Parameters

| System Parameters | value |
| --- | --- |
| Carrier Frequency | 3.5 GHz |
| Sub-Carrier Spacing | 30 kHz |
| Channel model | InF-SH |
| Reference signal physical structure | UL-SRS(comb-2,1 symbol) |
| Number of TRPs | 18 (7 TRPs are selected for positioning) |
| TRP Height | 8m |
| UE Height | 1.5m |
| Description of Measurement Algorithm | MUSIC |
| Description of positioning technique / Applied positioning algorithm | UL-TDOA/PSO |
| Network synchronization assumptions | ideal |
| Number of UEs | 2000 |
| Beam-related assumption | Tx beam sweeping |
| Antenna config. for TRP | (4,4,2,1,1) |
| Antenna config. for UE | (1,2,2,1,1) |