3GPP TSG-RAN WG1 Meeting #109-e	R1-2203172

e-Meeting, May 9 – May 20, 2022

Agenda Item:	9.7.1

Source:	Huawei, HiSilicon

Title:	Discussion on performance evaluation for network energy saving

Document for:	Discussion and Decision

Introduction

A new study item on network energy saving was approved in RAN plenary #94e with the following objectives:

- Definition of a base station energy consumption model [RAN1] - Adapt the framework of the power consumption modelling and evaluation methodology of TR38.840 to the base station side, including relative energy consumption for DL and UL (considering factors like PA efficiency, number of TxRU, base station load, etc), sleep modes and the associated transition times, and one or more reference parameters/configurations. - Definition of an evaluation methodology and KPIs [RAN1] - The evaluation methodology should target for evaluating system-level network energy consumption and energy savings gains, as well as assessing/balancing impact to network and user performance (e.g. spectral efficiency, capacity, UPT, latency, handover performance, call drop rate, initial access performance, SLA assurance related KPIs), energy efficiency, and UE power consumption, complexity. The evaluation methodology should not focus on a single KPI, and should reuse existing KPIs whenever applicable; where existing KPIs are found to be insufficient new KPIs may be developed as needed. Note: WGs will decide KPIs to evaluate and how. - Study and identify techniques on the gNB and UE side to improve network energy savings in terms of both BS transmission and reception, which may include: - How to achieve more efficient operation dynamically and/or semi-statically and finer granularity adaptation of transmissions and/or receptions in one or more of network energy saving techniques in time, frequency, spatial, and power domains, with potential support/feedback from UE, and potential UE assistance information [RAN1, RAN2] - Information exchange/coordination over network interfaces [RAN3] Note: Other techniques are not precluded The study should prioritize idle/empty and low/medium load scenarios (the exact definition of such loads is left to the study), and different loads among carriers and neighbor cells are allowed. The following example scenarios (mapping between scenarios and network loads is left to the study) including single-carrier and multi-carrier deployments are used as the starting point for discussion on prioritized scenarios for the study. The following example scenarios are listed in no particular order. - Urban micro in FR1, including TDD massive MIMO (note: this scenario can also model small cells) - FR2 beam-based scenarios (note: this scenario can also model small cells) - Urban/Rural macro in FR1 with/without DSS (no impact to LTE expected in case of DSS) - EN-DC/NR-DC macro with FDD PCell and TDD/Massive MIMO on higher FR1/FR2 frequency Note 1: legacy UEs should be able to continue accessing a network implementing Rel-18 network energy savings techniques, with the possible exception of techniques developed specifically for greenfield deployments. Note 2: the study of energy savings specifically for IAB is not part of the scope. The study should coordinate with RAN4 as needed.

In this contribution, the definition of base station (BS) power consumption model is discussed based on the UE power consumption model in TR38.840, and the framework of relative BS power consumption model is proposed for evaluation, including the reference configurations, the related impact factors like power amplifier (PA), number of TxRU, static circuit part, and also considering the sleep modes (SM) and the associated transition times. Then, the evaluation methodology and KPIs are discussed aiming to achieve the energy saving gain with acceptable KPI guarantees. In addition, prioritized and typical scenarios and evaluation assumptions are also provided.

BS power consumption model

2.1 Framework of BS power consumption model

Building a reliable BS power consumption model is the first important objective for the study on network energy saving. Such a model should reflect the key influence of power consumption factors for power calculation.

In UE power saving study, the power consumption model consists of several items:

1. Reference configuration assumptions, including bandwidth, subcarrier spacing (SCS), antenna configurations, etc.

2. Different power states with corresponding relative power consumptions, including deep sleep, light sleep, micro sleep, PDCCH-only, PDCCH+PDSCH, SSB or CSI-RS processing and UL transmission.

3. The transition times among sleeping states and the additional relative power consumptions for the transition.

4. The scaling method of the relative power for different configurations in the non-sleeping state.

5. The relative power of RRM measurement.

The modeling of UE power consumption serves a good starting point, by providing a general applicable framework for BS energy consumption analysis. On one hand, for the common parts that can be utilized for BS energy modeling on high level,

- Reference configuration assumptions are needed and factors such as BW, SCS are also critical configuration factors that can lead to different BS power as well

- Different power states with associated transition times are also to be considered, in order to better match BS operations in real sites and enable analysis of potential techniques targeting different achievements/KPIs

- Scaling operation is important for generating relative values for typical cases based on different implementations, even if only for evaluation purpose, which could be representative and scalable with more various configurations when needed. This only applies to non-sleeping modes, similar to that at UE side.

On the other hand, the needed adaptation stands due to the implementation/functional difference between BS and UE. For example, UE absolute power is much smaller than BS, therefore finer sleep modes are sensitive and helpful, and divisions among different channels can be critical for UE power consumption. While for BS, the general base value of power is much higher, and normally the BS needs to serve multiple users for transmission and reception, therefore the case of a slot with PDCCH-only rarely happens, and RRM measurement at individual UE side does not seem to differentiate the SSB(s) being broadcasted from gNB side.

Based on the above, it could be proposed that

Proposal 1: Consider to adopt the relative BS power consumption model consisting of

- Reference configuration assumptions including at least bandwidth, subcarrier spacing, antenna configurations,

- Different power states with corresponding relative power consumptions and the corresponding transition times among the power states,

- Scaling of the relative power for different configurations in the non-sleep/active mode(s).

2.2 BS power consumption model adaptations

The major power difference for the BS from the UE is that the power consumption for BS DL transmission takes much larger portion than that for BS UL reception over the total BS energy consumption. For BS UL reception, since PA is not used, the consumed power can be considered relatively static, or does not drastically/obviously change during various conditions. Therefore, we primarily focus on the power consumption model for DL transmission in the present discussion, including DL reference configuration and relative BS DL power consumption model in this section.

### Reference configuration

According to section 2.1, similar to the UL reference configuration for the UE power model, the SCS, the number of carriers and system bandwidth, transmit antenna configuration and power levels need to be determined.

Since the study includes both FR1 and FR2. For FR1, similar to the study in UE power saving, a respective set of DL reference configurations for BS power model for FR1 TDD and FR2 TDD referring to IMT-2020 can be considered as below:

- BS DL: FR1 TDD

  - SCS: 30 kHz

  - Number of carrier: 1 Component carrier (CC)

  - System bandwidth: 100 MHz

  - Transmit antenna configuration: 64 Tx

  - Power levels: [FFS] dBm

- BS DL: FR2 TDD

  - SCS: 120 kHz

  - Number of carrier: 1 Component carrier (CC)

  - System bandwidth: 100 MHz

  - Transmit antenna configuration: 8 Tx

  - Power levels: [FFS] dBm

For FR1 FDD, the power consumption is also interested thus could be modeled. There are two approaches to obtain this. For example, a set of reference configurations referring to IMT-2020 can be provided as an example below, with antenna configurations varying per deployment scenarios (rural or dense urban). Alternatively, it may also be possible to scale the power of FR1 TDD reference configuration for FR1 FDD, to ease the evaluation efforts when the implementations can be common for TDD and FDD in the same BS.

- BS DL: FR1 FDD

  - SCS: 15 kHz

  - Number of carrier: 1 Component carrier (CC)

  - System bandwidth: 20 MHz

  - Transmit antenna configuration: [8, 16 or 32] Tx

  - Power levels: [FFS] dBm

Proposal 2: For BS DL reference configuration at least for FR1 TDD and FR2 TDD, consider the following assumptions respectively,

- FR1 TDD

  - SCS: 30 kHz

  - Number of carrier: 1 Component carrier (CC)

  - System bandwidth: 100 MHz

  - Transmit antenna configuration: 64 Tx

  - Power levels: [FFS] dBm

- FR2 TDD

  - SCS: 120 kHz

  - Number of carrier: 1 Component carrier (CC)

  - System bandwidth: 100 MHz

  - Transmit antenna configuration: 8 Tx

  - Power levels: [FFS] dBm

### Power state and transition time

Next, based on the UE power model, several power states, transition times among the power states and the relative power need to be defined for the BS.

In general, the total energy consumption Etotal consists of energy consumption of active modes Eactive, sleep modes Esleep, and mode transitions Etransition, as shown in the following formula.

Etotal=Eactive+Esleep+Etransition

On one hand, considering that the base value for the power consumption for the BS is much larger than that for the UE once the BS is working actively, it might be possible to use one single active mode without differentiation of the power consumed by various channels/signals, e.g., PDCCH-only, PDCCH+PDSCH, SSB or CSI-RS processing. This can greatly simplify the modeling of BS on work. The goal for reducing the BS energy for active mode would simply be to improve the scheduling efficiency when there is data transmission, or to reduce the need of common channels/signals when there is no data request. On the other hand, since common signals/channels may usually apply power boosting for ensuring cell edge user performance, reduction of the ratio of power consumption of common signals/channels-only such as SSB-only seems to be straightforward and important, especially when the traffic load is quite low.

For the divisions of power sates when BS is going to sleep, i.e. certain gNB functions may be off, the content of energy savings and impact of such operation specific to performance is concerned. Therefore, it would largely depend on how long/often it may happen/resume, and whether/how it may affect the behavior of UEs (which also leads to specification impact).

For example, if the BS has no DL to transmit at all for a certain time (e.g., one or several symbols or slots), one sleep mode, denoted as dynamic sleep mode, can be introduced to at least shut down the dynamic part of the power consumption, and the sleep mode can be dynamically switched to the DL active mode for transmission, in order to guarantee the communication performance.

Besides the DL active mode and the dynamic sleep mode defined above, another sleep mode, e.g., deep sleep mode, could be introduced to shut down the additional static part of the power consumption on top of dynamic part of the power consumption for the dynamic sleep mode. Such a deep sleep mode can achieve further power saving compared to the dynamic sleep mode. However, generally the deeper the BS can sleep to achieve larger power saving gain, the longer transition time or wakeup time it would take, e.g., up to at least second level.

Figure 1 Illustrations of active and sleep modes

Based on the above discussion, an illustration for the power states and the transition times is shown in Figure 1, assuming one active mode and two sleep modes (wherein SM 1 stands for dynamic deep mode and SM 2 stands for deep sleep mode mentioned above). Also, the relative power for the power states and the related transition times for BS are given in Table 1 and Table 2 respectively, which would be determined based on the determined reference configurations. For FR2 TDD, the relative power for active mode may be different from that for FR1 TDD, and the relative power for other parts including sleep modes and transition times can be the same as that for FR1 TDD based on the similar rule from UE power model.

Table 1 BS power consumption model

| Power state | Characteristic | Relative Power |
| --- | --- | --- |
| SM 2 | Deep sleep with lower power for static part | Psleep,2 (normalized to 1) |
| SM 1 | Dynamic shutdown with higher power for static part | Psleep,1 |
| Active DL | Transmission with power for dynamic part and higher power for static part | Pactive |

Table 2 BS DL power consumption during the state transition

| Power state | Additional transition energy: (product of relative power and duration in ms) | Transition time |
| --- | --- | --- |
| SM 2 | [X] | At least [x] seconds |
| SM 1 | [Y] | [y] micro seconds |
| Active DL | 0 | 0 ms |
| Note: the transition time is from a power state to non-sleep state (active state). |  |  |

Proposal 3: Consider to introduce one DL active mode and two sleep modes for modeling the BS power consumption and determine the corresponding values of relative power, transmission time for each mode.

### Scaling for adaptation

Last, based on the reference configuration and the defined relative power, the final relative energy consumption can be determined. As previously explained, scaling is important to be implemented in the modeling and obtaining the final values. However the scaling is not needed for the energy consumption for sleep modes which can be calculated as below also.

For the sleep modes, they mainly contain the static part of the energy consumption since no signal is transmitted by the PA. Assuming there are L sleep modes, the specific energy consumption for the sleep modes and the corresponding transition time can be represented by the following respectively, wherein tsleep,l and Psleep,l are the time duration and power for each sleep mode, ttrans,l and Ptrans,l are transition time and the power for each sleep mode.

Esleep=l=1Ltsleep,lPsleep,l

Etransition=l=1Lttrans,lPtrans,l

Proposal 4: For modeling and evaluation, the energy consumption of the sleep modes is sum of the products of the time duration and static power for each sleep mode.

For the active mode with realistic scheduling and configuration, energy consumption can be scaled according to the reference configuration and the relative power based on the proper scaling methods.

In UE power model, the linear scaling rule is used to determine the final relative power consumption by linearly scaling the relative power for the reference configuration. The similar scaling rule can also be considered in the BS power model.

However, there can be differences for the BS from the UE power model.

- First, we may be able to use a general scaling rule for the power consumption regardless of signal types for transmission, e.g., no differentiation among the scaling rules for SSB/CSI-RS, PDCCH-only, etc. as that simplifies the modeling of BS energy consumption.

- Second, some of parameters may not be linearly scaled, e.g., the back-off of the PA efficiency due to output power back-off may not be linear.

- Third, the output power for CA case can be different. The total output power cannot be increased for the UE, e.g., maximum 23 dBm for 1 CC and CA. The BS can use common RF chain and PA for contiguous CA or non-contiguous CA with neighboring CC frequencies, while the BS should use separate RF chains and PAs for inter-band CA with distanced CC frequencies.

Based on the above analysis on the differences between the UE and BS for the power scaling, the specific BS power scaling rule is further formulated in the following. Generally, the BS power consumption for the active mode contains mainly three parts, including PA part, RF chain part and the static power part, which is shown by the following formula. The PA part should capture the impact factors including the PA efficiency (η), the number of activated RF chains (β1), scheduled RB occupancy ratio (β2), other factors (β3) like the power spectrum density (PSD), etc. The RF chain part contains dynamic power part for the activated RF chain except the PA part. The static power part is the same as the power consumption for the dynamic sleep mode (Psleep,1), which means dynamic mode switch can be performed between the active mode and the dynamic sleep mode as stated above.

Eactive=tactive(Psleep,1+fβ1,β2,β3,η)

In summary, the framework for the BS power scaling rule can be summarized as following:

- Antenna scaling for PA and RF chain: linearly scaling by β1

- BWP with 1 CC: linearly scaling by β2

- CA:

  - Common PA and RF chain: linearly scaling assuming the common output power and total bandwidth among carriers

  - Separate PA and RF chain: sum of separate single-carrier power consumption

Proposal 5: For modeling and evaluation, the energy consumption for DL active mode consists of dynamic and static power parts

- The dynamic power part is related to PA efficiency, spatial domain activated RF chain ratio, frequency domain scheduled RB ratio and other factors such as PSD, etc.

- The static power part is same as the power consumption for the dynamic sleep mode.

Proposal 6: For evaluation, the scaling for adaptation in a BS power model can include the following.

- Antenna scaling for PA and RF chain: linearly scaling by  β1

- BWP with 1 CC: linearly scaling by β2

- CA: scaling by β3 and η

  - Common PA and RF chain: linearly scaling assuming the common output power and total bandwidth among carriers

  - Separate PA and RF chain: sum of separate single-carrier power consumption

Evaluation methodology

In this section, we discuss the evaluation methodology including the performance metrics, simulation assumptions and the prioritized scenarios.

3.1 Evaluation metrics and KPIs

For the BS power consumption perspective, one essential evaluation metric would be the power saving gain based on the relative power consumption model. Specifically, by setting the system-level simulation, the power consumption can be calculated in a dynamic basis using the above BS power consumption model.

Proposal 7: Conduct system-level simulation that allows dynamic calculation of BS power consumption and comparison of power saving of each proposed technique.

The energy efficiency (EE) which is commonly denoted as bit per Joule can also be considered. However, the EE metric is not complete and may not be appropriate to the idle/empty and low/medium load scenarios which are prioritized by the SID, since in such load scenarios power consumption is dominated by the common signal/channel transmission.

Proposal 8: Study the definition of energy efficiency and necessary revisions/additions for evaluation of the power consumption for the prioritized idle/empty and low/medium load scenarios.

In addition of the metric of the relative BS power consumption, the performance metrics should also be considered to the performance guarantee, since reducing the power consumption may bring other resource cost, such as more RB and time usage. So, the power saving gain should be obtained with acceptable/minimized loss of the performance metrics.

For evaluation of impact on performance, one of the widely used network metric is UPT, which is also adopted in the study of UE power saving. Other metrics can also be considered, which could be associated with proposed techniques for further discussion, e.g., latency, UE complexity and UE power consumption.

Proposal 9: Include UPT in the system-level evaluation. Further study and identify other appropriate metrics for each proposed technique.

3.2 Evaluation scenarios and assumptions

Several evaluation scenarios are mentioned in the SID as shown as following:

- Urban micro in FR1, including TDD massive MIMO (note: this scenario can also model small cells)

- FR2 beam-based scenarios (note: this scenario can also model small cells)

- Urban/Rural macro in FR1 with/without DSS (no impact to LTE expected in case of DSS)

- EN-DC/NR-DC macro with FDD PCell and TDD/Massive MIMO on higher FR1/FR2 frequency

According to the industry research, it is well understood that the large number of antennas equipped with massive MIMO occupies a relative considerable portion of power consumption within a BS site. Therefore, those scenarios are preferably to be included, especially considering the trend of more antennas to be implemented in higher frequency.

Also, it is understood that there is much more room for energy saving for idle/empty and low/medium load scenarios as prioritized in the SID, since there is relatively good transmission efficiency already when the load is heavy and the BS is considered to be working without resource/power waste. For such reason, the energy saving potential for the scenarios with DSS carrier, or with FDD PCell in DC case may not be very promising due to already high traffic load or spectrum utilization. Therefore, it can be proposed that

Proposal 10: Include TDD urban or FDD urban/rural equipped with massive MIMO for evaluations. Study other scenarios including whether DSS is specifically represented and whether DC scenarios can be represented using other assumptions.

A set of system-level simulation assumptions is also listed as in Annex A. Basically, the IMT-2020 simulation assumptions are used as reference. And some parameters, such as carrier frequency, bandwidth, TxRU and traffic model, etc, are adjusted with reference of actual deployment, so that the evaluation results could be closer to reality. With that, we consider this set of assumptions could be adopted for BS power evaluations.

Proposal 11: Consider to adopt the parameters in Annex A for system-level simulation assumptions.

Preliminary simulation results

In this section, some preliminary system-level simulation results on the relative power saving ratio for several technical directions on different domains are provided for interest, even though the specific BS power model and evaluation methodologies are to be further determined. The detailed descriptions on the technical directions can be found in the companion contributions [2].

Light common signalling

As explained in section 2.2 and analysed in [2], there can be up to 30% symbols for FR1 and 15% symbols for FR2 being active in time for the network to only transmit SSB and SIB1. Therefore, methods can be studied to reduce the time transmission ratio of common signals, e.g. SSB and SIB1.

As shown in Figure 2, the power saving of SSB/SIB1-less operation on ES CC is simulated at different resource utilization (RU) cases. From the simulation results, we can see that compared to the baseline power consumption where SSB and SIB1 are both transmitted with four beams in NR FDD, more than 30% and 20% power saving gain can be achieved for SSB/SIB1-less and SIB1-less only operations respectively for the low load case (e.g., <10% RU), as shown in the left figure. In addition, we also simulate the power increase for the anchor CC to carry the additional SIB1 of the ES CC as shown in the right figure, wherein we can see less than 10% power increase in anchor CC for the low load case.

Figure 2 Results for SSB/SIB1-less operation

TRX chain shutdown

As analysed in [2], in the spatial domain, we can reduce the number of active TRX chains to achieve power saving, especially when the traffic is low and the transmission capacity is redundant. The resource allocation of frequency and space domain can be optimized to achieve the minimum power consumption without causing performance loss.

As shown in Figure 3, the power saving of TRX chain shutdown for both TDD and FDD is simulated at 30%RU case for different performance loss situations (e.g., 100% means no performance loss and 95% means 5% performance loss). Multiple CSIs with various TRX number are also compared. From those, considerable power saving gains may be obtained depending on the performance impact.

Figure 3 Results for TRX chain shutdown

PSD back-off and frequency extension

As analysed in [2], according to the Shannon capacity formula, i.e., B=Wlog21+P/N, for a given data rate B, we can get different combinations of bandwidth W and transmit power P. Specifically, the channel capacity increases linearly with the bandwidth but logarithmically with the transmit power spectrum density (PSD), and hence it is preferable to transmit a given amount of data with a larger bandwidth and hence a much smaller PSD, to achieve a smaller total power consumption.

As shown in Figure 4, the power saving of PSD back-off for both TDD and FDD is simulated at 30%RU case. Multiple CSI with different PSD back-off ratios are considered. Similarly, considerable power saving gains can be expected depending on the performance impact.

Figure 4 Results for PSD back-off and frequency extension

Conclusions

According to the discussion, following proposals and observations are provided:

Proposal 1: Consider to adopt the relative BS power consumption model consisting of

- Reference configuration assumptions including at least bandwidth, subcarrier spacing, antenna configurations,

- Different power states with corresponding relative power consumptions and the corresponding transition times among the power states,

- Scaling of the relative power for different configurations in the non-sleep/active mode(s).

Proposal 2: For BS DL reference configuration at least for FR1 TDD and FR2 TDD, consider the following assumptions respectively,

- FR1 TDD

  - SCS: 30 kHz

  - Number of carrier: 1 Component carrier (CC)

  - System bandwidth: 100 MHz

  - Transmit antenna configuration: 64 Tx

  - Power levels: [FFS] dBm

- FR2 TDD

  - SCS: 120 kHz

  - Number of carrier: 1 Component carrier (CC)

  - System bandwidth: 100 MHz

  - Transmit antenna configuration: 8 Tx

  - Power levels: [FFS] dBm

Proposal 3: Consider to introduce one DL active mode and two sleep modes for modeling the BS power consumption and determine the corresponding values of relative power, transmission time for each mode.

Proposal 4: For modeling and evaluation, the energy consumption of the sleep modes is sum of the products of the time duration and static power for each sleep mode.

Proposal 5: For modeling and evaluation, the energy consumption for DL active mode consists of dynamic and static power parts

- The dynamic power part is related to PA efficiency, spatial domain activated RF chain ratio, frequency domain scheduled RB ratio and other factors such as PSD, etc.

- The static power part is same as the power consumption for the dynamic sleep mode.

Proposal 6: For evaluation, the scaling for adaptation in a BS power model can include the following.

- Antenna scaling for PA and RF chain: linearly scaling by  β1

- BWP with 1 CC: linearly scaling by β2

- CA: scaling by β3 and η

  - Common PA and RF chain: linearly scaling assuming the common output power and total bandwidth among carriers

  - Separate PA and RF chain: sum of separate single-carrier power consumption

Proposal 7: Conduct system-level simulation that allows dynamic calculation of BS power consumption and comparison of power saving of each proposed technique.

Proposal 8: Study the definition of energy efficiency and necessary revisions/additions for evaluation of the power consumption for the prioritized idle/empty and low/medium load scenarios.

Proposal 9: Include UPT in the system-level evaluation. Further study and identify other appropriate metrics for each proposed technique.

Proposal 10: Include TDD urban or FDD urban/rural equipped with massive MIMO for evaluations. Study other scenarios including whether DSS is specifically represented and whether DC scenarios can be represented using other assumptions.

Proposal 11: Consider to adopt the parameters in Annex A for system-level simulation assumptions.

- References

- RP-220297, Revised SI: Study on network energy savings for NR, March 17 – March 23, 2022.

- R1-2203173, Huawei, HiSilicon, Discussion on network energy saving techniques, May 9 – May 20, 2022.

- Annex A. The evaluation assumption

Table A The evaluation assumption for BS power consumption model

|  | Parameters |  |  |
| --- | --- | --- | --- |
| Basic parameters | Channel model | 3D/HF-Uma based on TR 38.901 | 3D/HF-Uma based on TR 38.901 |
|  | Device deployment | 80% indoor, 20% outdoor | 80% indoor, 20% outdoor |
|  | Inter-site distance | 350m | 350m |
|  | Network Topology | 7*3 Sector | 7*3 Sector |
|  | Carrier Frequency | 2.1GHz | 4GHz |
|  | Multiple access | OFDMA | OFDMA |
|  | Duplexing | FDD | TDD |
|  | Numerology | 15KHz, 14 OFDM symbol slot | 30kHz, 14 OFDM symbol slot |
|  | Guard band ratio on simulation bandwidth | FDD: 6.4% (104RB for 15kHz SCS and 20 MHz BW) | TDD: 2.08% (272 RB for 30kHz SCS and 100 MHz bandwidth) |
|  | Simulation bandwidth | FDD: 20 MHz | TDD: 100 MHz |
|  | Frame structure | Full downlink | DDDSU |
|  | UE density | 20 UEs per TRxP | 20 UEs per TRxP |
|  | UT attachment | Based on RSRP | Based on RSRP |
|  | Wrapping around method | Geographical distance based wrapping | Geographical distance based wrapping |
|  | Traffic model | Burst buffer with load <10%, 30%, 50% Packet size: 0.5M, 0.1M, 20k, 5k, 2k | Burst buffer with load <10%, 30%, 50% Packet size: 0.5M, 0.1M, 20k, 5k, 2k |
| BS parameters | BS antenna height | 25 m | 25 m |
|  | BS noise figure | 5 dB | 5 dB |
|  | BS antenna element gain | 8 dBi | 8 dBi |
|  | Antenna configuration at TRxP | For 32T: (M,N,P,Mg,Ng; Mp,Np) = (8,8,2,1,1;2,8) (dH, dV)=(0.5, 0.8)λ | For 64T: (M,N,P,Mg,Ng; Mp,Np) = (12,8,2,1,1;4,8) (dH, dV)=(0.5, 0.8)λ; |
| UE parameters | UE power class | 23dBm | 23dBm |
|  | UE noise figure | 9 dB | 7 dB |
|  | UE antenna element gain | 0 dBi | 0 dBi |
|  | UE antenna height | Outdoor UEs: 1.5 m; Indoor Uts: 1.5m or consider floor height | Outdoor UEs: 1.5 m; Indoor Uts: 1.5m or consider floor height |
|  | Antenna configuration at UE | For 4R: (M,N,P,Mg,Ng; Mp,Np)= (1,2,2,1,1; 1,2) (dH, dV)=(0.5, N/A)λ | For 4R: (M,N,P,Mg,Ng; Mp,Np)= (1,2,2,1,1; 1,2) (dH, dV)=(0.5, N/A)λ |
| Transmission parameters | Modulation | Up to 256 QAM | Up to 256 QAM |
|  | Transmission scheme | SU-MIMO | SU-MIMO |
|  | SU dimension | For 4Rx: Up to 4 layers | For 4Rx: Up to 4 layers |
|  | DL CSI measurement | Non-precoded CSI-RS based | Precoded CSI-RS based |
|  | DL codebook | Type I/II codebook | non-PMI tranmission |
|  | SRS transmission | N/A | For UE 4 Tx ports: Non-precoded SRS, |
|  | CSI feedback | PMI, CQI, RI: every 5 slot; Subband based | CQI, RI: every 5 slot; Subband based |
|  | Interference measurement | SU-CQI; CSI-IM for inter-cell interference measurement | SU-CQI; CSI-IM for inter-cell interference measurement |
|  | Scheduling | PF | PF |
|  | Receiver | MMSE-IRC | MMSE-IRC |
|  | Channel estimation | Non-ideal | Non-ideal |

(M, N, P, Mg, Ng; Mp, Np)

- M: Number of vertical antenna elements within a panel, on one polarization

- N: Number of horizontal antenna elements within a panel, on one polarization

- P: Number of polarizations

- Mg: Number of panels in a column;

- Ng: Number of panels in a row;

- Mp: Number of vertical TXRUs within a panel, on one polarization

- Np: Number of horizontal TXRUs within a panel, on one polarization