3GPP TSG RAN WG1 Meeting #109-e	R1-2203174

e-meeting, May 9th – May 20th, 2022

Source:             CEWiT, IITM, IITK, RIL, Saankhya Labs

Title:	             Discussion on Unified TCI framework extension for multi-TRP

Agenda Item:	 9.1.1.1

Document for: Discussion

# Introduction

In RAN#94e [1], a work item for NR MIMO evolution for downlink and uplink was agreed with the following objectives.

- Specify extension of Rel-17 Unified TCI framework for indication of multiple DL and UL TCI states focusing on multi-TRP use case, using Rel-17 unified TCI framework.

MIMO is one of the key technologies in NR systems and is successful in commercial deployment. MIMO technique is a feature to increase channel capacity, and its enhancement has conventionally shown in each release. In Rel 16/17, PDXCH/ PUXCH enhancements for multi TRP was introduced to improve the spectral efficiency and reliability via aggregating resources from different TRPs. The unified TCI framework was introduced in Rel-17 which facilitates streamlined multi-beam operation targeting FR2.

For Rel-18, one of the potential enhancements for UL/DL MIMO is the introduction of unified TCI framework for multi-TRP. As Rel-17 focused on single-TRP use cases, extension of unified TCI framework that focuses on multi-TRP use cases is beneficial. In this contribution, we provide our views on required enhancements to support unified TCI framework for multi- TRP.

- Discussion

  - Unified TCI framework for multi-TRP

In Rel-15, beam management was introduced to support independent analog beam selection especially in high frequency range. In Rel-16/17, TCI framework used for beam indication has been further enhanced to achieve low overhead and low latency for single TRP use cases. In Rel-17 NR, unified TCI framework was introduced for further reduction in overhead and latency for high intra-/inter-cell mobility and large number of configured TCI states with high efficiency. In unified TCI framework, BS supports L1-based beam indication (TCI update) to enable common beam for DL data and control and common beam for UL data and control beam operation. There are two types of unified TCI framework which are defined based on the condition of existence of beam correspondence between between DL and UL, such as separate TCI state indication used for DL reception and UL transmission if beam correspondence between DL and UL is not assumed, and a joint (common)TCI state indication used for DL reception and UL transmission when beam correspondence between DL and UL is assumed.

But in multi TRP scenario, latency and overhead is still an issue in case of beam indication for UL and DL when large number of TCI states are configured. Hence, in Rel.18 it has been agreed to extend the unified TCI framework for multi-TRP scenario as well. To support unified TCI framework for multi-TRP, some of the issues need to be addressed and are explained below.

    - Potential enhancement to L2- based signaling

In Rel-16 NR, if a UE is configured for M-TRP operation, BS configures the UE with up to 128 TCI states using RRC signaling and signals the UE with a MAC-CE that consists of up to eight code points where each code point maps to one or two TCI states out of the configured TCI states. BS signals one out of the eight code points (from MAC CE) using a DCI to the UE for activating the corresponding TCI states. In Rel-17 NR, UE can be configured with up to 128 unified TCI states for joint/DL TCI state indication and 64 TCI states for UL TCI state indication. Further, the BS signals a MAC CE to the UE that consists of up to eight code points where each code point maps to either one joint TCI state in case of a UE configured with joint unified TCI operation or one DL TCI state and one UL TCI state in case of a UE is configured with separate unified TCI operation.

When a Multi-TRP architecture is combined with unified TCI framework, BS needs to signal a TCI state common for data/channel/RS for each TRP in case of both separate and joint TCI indication to the UE. In order to signal the Common TCI state to a UE using DCI for each TRP for both separate TCI and joint TCI operation, codepoints should map to multiple DL/UL TCI states in MAC CE structure. Therefore, the MAC CE structure needs to be further studied and modified for indicating multiple DL/UL TCI states to support unified TCI framework for multi-TRP transmission.

Proposal 1: Support modification to L2 based signaling for indicating multiple DL/UL TCI states to support unified TCI framework for multi TRP transmission.

    - TCI configuration enhancement of PDCCH/PDSCH

In Rel-15/16 NR, each CORESET is configured with up to 64 possible TCI states and BS signals a UE to activate one out of these 64 TCI states using a MAC-CE. When a UE monitors a search space set associated with a CORESET, the UE uses the activated TCI state for that corresponding CORESET. In Rel-17 NR, if a UE is configured with a set of 128 unified TCI states, one of these TCI states is activated using a DCI for a UE. UE uses the activated unified TCI state to receive following CORESETs.

If a UE is configured to be operating in unified TCI and is connected to Multi-TRP, BS activates a pair of unified TCI states to, support M-TRP operation. Then UE needs to determine which one of the two TCI states to be used to receive PDCCH. So, there is a need to investigate this impact on PDCCH monitoring when COEREST is associated with multiple TCI states.

Proposal 2: Investigate impact on PDCCH monitoring.

In Rel-16 NR, there are various M-TRP schemes that are introduced by 3GPP. In the case of a multi TRP scenario, the transmission and reception to and from different TRPs can be multiplexed in any one of time, frequency, and space, leading to FDM Scheme A, FDM scheme B, TDM scheme A, SDM scheme etc. To support the various M-TRP schemes as mentioned above, CORESETPoolIndex variable was introduced in multi TRP transmission. A CORESET pool index is a variable that will be present inside the ControlResourceSet IE. This variable can take a value of either 0 or 1. If this variable is not configured for a CORESET, UE assumes default value that is 0 for that CORESET. When a UE is configured by higher layer parameter PDCCH-Config that contains two different values of CORESETPoolIndex in ControlResourceSet, the UE may expect to receive multiple PDCCHs scheduling fully/partially/non-overlapped PDSCHs in time and frequency domain. Similarly, when a UE is configured with 0 CORESETPoolIndex by higher layer parameter then the UE may expect to be scheduled with full/partially/non-overlapped PDSCHs in time and frequency domain, the full scheduling information for receiving a PDSCH is indicated and carried only by the scheduling PDCCH.

In Rel-17, multi- TRP framework was extended to support PDCCH transmission for more reliability. In this PDCCH repetition was introduced where PDCCH is repeated multiple times in TDM manner and each repetition from different TRP maps to one out of two TCI states. This framework can be a baseline for further extension in supporting unified TCI framework for multi-TRP.

Proposal 3: For beam indication to receive PDCCH and PDSCH, consider existing Rel. 16/17 multi TRP framework as the baseline.

    - Collision between channel and signals

A UE that is connected with multiple TRPs, can be scheduled to receive signals/channels simultaneously from both the TRPs using full or partially overlapping resources sometimes. If the UE is not capable of receiving with different spatial Rx filters simultaneously, then collision occurs. Therefore, mechanism to handle collisions between channels and signals should be investigated to support unified TCI framework for multi-TRP.

Proposal 4: Need to investigate the methods to avoid collisions between different channels and signals.

# 3   Conclusion

This contribution we discussed potential enhancements to support unified TCI framework for multi- TRP.

Further, we summarize the proposals as follows:

Proposal 1: Support modification to L2 based signaling for indicating multiple DL/UL TCI states to support unified TCI framework for multi TRP transmission.

Proposal 2: Investigate impact on PDCCH monitoring.

Proposal 3: For beam indication to receive PDCCH and PDSCH, consider existing Rel. 16/17 multi TRP framework as the baseline.

Proposal 4: Need to investigate the methods to avoid collisions between different channels and signals.

Reference

[1] RP-213598, “New WID: MIMO Evolution for Downlink and Uplink”, Samsung.