3GPP TSG RAN WG1 #109	R1-2203178

e-Meeting, 09 – 20 May, 2022

Agenda item:		9.5.2.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Initial Views on improved accuracy based on NR carrier phase measurement

Document for:		Discussion and Decision

# Introduction

In Rel-16 native NR positioning support was standardized and in Rel-17 enhancements were made. At RAN#94 a new SI was approved on enhancements for Rel-18 NR positioning [1]. This contribution discussed our views related to RedCap positioning. Our companion contributions discuss our other views [2-8]. The objective in the SID is:

  - Study solutions for accuracy improvement based on NR carrier phase measurements [RAN1, RAN4]

    - Reference signals, physical layer measurements, physical layer procedures to enable positioning based on NR carrier phase measurements for both UE-based and UE-assisted positioning [RAN1]

    - Focus on reuse of existing PRS and SRS, with new reference signals only considered if found necessary

# Discussion

NR positioning would serve various use cases such as V2X positioning, IIoT, and consumer use cases (e.g., XR, gaming) beyond Rel-17 positioning accuracy requirement. Carrier Phase (CP) positioning is a promising candidate technique that could provide less than 10 cm positioning accuracy performance as CP measurement can potentially capture sub-sample level of measurement accracy. The CP measurement error could be within one wavelength, for example, one wavelength is about 8.6 cm for 3.5 GHz carrier frequency.

Rel-16 NR positioning supported DL PRS and UL SRS as dedicated RS for positioning and it was designed for UE/TRP to well estimate timing measurement. As a simple way to obtain phase measurement, the UE can extract phase value from the first arrival path of the received signal, and the UE already needs to detect the first path to measure timing measurement for the current NR positioning techniques, so it might not be difficult to introduce CP positioning to NR positioning on top of the current positioning techniques. If the current PRS could be utilized for CP positioning, it would not lead to additional RS overhead to introduce CP positioning. Thus, the reuse of the current reference signals should be the first priority of this study as mentioned in SID.

In this paper, we provide introduction on CP positioning technique, initial evaluation result showing feasibility of CP technique in NR positioning system, and also discuss key issues to be studied to support CP positioning.

## Carrier phase positioning method

Fundamentally, carrier phase positioning technique utilizes phase measurements for multiple TRPs to estimate location of target UEs. The general concept of the carrier phase positioning is shown in Figure 1, where a target UE and a reference device measures carrier phase measurement for PRS resources transmitted from multiple gNBs/TRPs. In this section, we describe the key principle of CP positioning in more detail.

Figure 1. An illustrative example of overall concept of carrier phase positioning

For mathematical notation, we denote the target UE by the k-th UE. The estimated phase measurement from this k-th UE for a PRS resource transmitted from the i-th TRP is denoted by

λφik=dik +c δk-δi-λ Nik											(1)

Where φik=12πφik to denote the phase measurement in distance/cycles and to leave out repeated use of 2π, and dik, c, δk, δi, and Nik represent respectively actual geographical distance between the k-th UE and the i-th TRP, speed of light, internal clock bias at the k-th UE, internal clock bias at the i-th TRP, and integer ambiguity of the propagated wavelength. Similar to (1), the similar equation is derived for the different PRS transmitted from the j-th TRP such that λφjk=djk+c δk-δj-λNjk.

After the UE measures phase measurement for PRS resources transmitted from both TRPs, the UE calculates the phase difference measurement for two different phase measurements, and gets the following phase difference measurement.

λΔφijk=Δdk+cΔδij-λΔNijk												(2)

Where Δφijk=φik-φjk, Δdk=dik-djk,Δδij=δj-δi,  and ΔNijk=Nik-Njk. From single difference operation, the clock bias from UE side was cancelled out similar to the RSTD measurement of DL-TDOA. The k-th UE needs to report this measurement to the LMF.

After single difference operation, the clock error between two TRPs are remaining in the phase difference measurement, and it needs to be compensated for. We propose using reference UE to remove this error. The clock error between TRPs is remaining but it is cancelled by double differential operation using phase difference measurements of reference device such as reference UE. The location of the reference UE should be already known to the LMF accurately.

Let us assume that the K-th UE is a reference device. For the transmitted PRS from the i-th and j-th TRP, the phase difference measurement at the reference UE is denoted by λΔφijK=ΔdK+cΔδij-λΔNijK. The reference UE also reports the phase difference measurement to the LMF, and then it calculates double difference measurement as follows:

λΔΔφijkK=ΔΔdkK+λ×ΔΔNijkK											(3)

Where ΔΔφijkK= Δφijk-ΔφijK, ΔΔdkK=Δdk-ΔdK, and ΔΔNijkK=ΔNijk-ΔNijK. In the end, the clock error between TRPs are cancelled out. The only remaining ambiguity is the integer numbers and location of the target UE and the LMF should find it. We discuss how to find the integer ambiguity in more detail in the following section.

Observation 1: Double difference operation removes the phase offset between different TRPs.

The above description is based on the DL PRS but the TRP also can measure CP measurement from received SRS signals (SRS for positioning) and the LMF can calculate phase difference measurement for the target UE. It would be unnecessary to describe a similar equation for the UL case but the study on CP positioning should be for both DL and UL-based positioning.

Proposal 1: RAN1 to study CP positioning for both DL and UL positioning based on the existing DL PRS and UL positioning SRS.

## CP Evaluation Methodology & Feasibility

In this section, we provides initial evaluation results for system-level simulation to show the feasibility of the CP positioning technique in NR system. Also, we includes link-level evaluation results in appendix. As an initial investigation into the accuracy of CP positioning in the IIoT use cases (Indoor factory, InF-SH model) we provide initial evaluation result for the current DL PRS, and our assumption on simulation parameter is aligned to the agreed parameters in Rel-17 SI [9], which is given by a table below. The channel model for InF-SH is in [10].

| Parameter |  |
| --- | --- |
| Channel model (baseline, otherwise state any modifications) | Baseline |
| PRS resource configuration (RE pattern) | DL : Comb-6, 6 symbol NR PRS |
| Reference signal (type of sequence, number of ports, …) | DL : NR PRS UL : NR SRS-P |
| Number of TRPs and frequency layers | 18 and 1 |
| Number of symbols used per slot per positioning estimate | 6 symbols per slot |
| Number of slots per positioning estimate | 1 slot |
| Power-boosting level | N/A |
| interference modelling (ideal muting, or other) | Ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | Cross-correlation receiver |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, taylor series, etc) | Taylor Series, Least Squares |
| Network synchronization assumptions | Non-ideal synchronization |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) | N/A |
| Precoding assumptions (codebook, nr of antenna elements used, etc) | N/A, single antenna at TRP |
| Additional notes, if any | Ideal fixing of integer ambiguity based on virtual wavelength method plus DL-TDOA 4 reference UEs (PRUs) modeled |
| Carrier frequency, PRS BW, and SCS | 3.5 GHz/ 100 MHz/ 30KHz |

The effect of channel propagation delay corresponding to the distance between UE and a TRP is seen in the phase value of channel tap. Thus, the UE can extract the phase measurement from the first arrival path by simple correlation method. Simply put, if the PRS signal is s(t) and channel gain is ai for the i-th arrivla path, the UE can extract e-i2πfcτ1 from the received signal yt= i=1Lais(t -τi)e-i2πfcτi, where τ1=d/c and d is the distance between the UE and the TRP.

Figure 2. Phase measurement error for InF-SH moedel (the right-hand figure shows zoom-in range between 0 and 2 cm of the left figure)

Figure 2 shows phase measurement error in distance assuming InF-SH (Indoor Factory, spare clutter and high BS), which shows the feasibility of CP measurement in NR system. This figure shows phase measurement error between a UE and 18 TRPs without fitering measurement where the error means distance error between UE and TRPs. From the simulation result assuming 100% LoS channel, which means each channel between a UE and each TRP has LoS signal path component, if LoS path is guaranteed, simple correlation method also can extract the phase measurement pretty accurately. In normal InF-SH model, the phase measurement error is less than 1 cm for 80 percent of UEs and the error is less than 4 cm error for 90 percent of the UEs.

Observation 2: The phase measurement error is less than 1 cm for 80 percent of UEs and the error is less than 4 cm error for 90 percent of the UEs in InF-SH model.

Based on these measurement results, we show initial result of positioning accuracy performance for CP positioning to provide feasibility. To remove the clock bias between TRPs/gNBs, we introduced four reference UEs and they are placed as shown in Figure 3 below. For each target UE, a single reference UE measurement was used for double difference operation. We assumed that the integer ambiguity is fixed without error as an initial simulation, but it shound be noted that the existing solutions [11][12] already show the error is not substantial.

Figure 3. Reference UEs and TRPs in InF-SH model

Figure 4. Positioning accuracy performance for CP of InF-SH model.

The simulation results show that horizontal positioning accuracy is about 2 cm at 90% of UEs and vertical positioning accuracuy is 13 cm at 90% UEs in case that integer ambiguity is resolved without errors. It shows centi-meter level of horizontal positioning accuracy is achievable.

Observation 3: Horizontal positioning accuracy is about 2 cm at 90% of UEs and vertical positioning accuracuy is 13 cm at 90% UEs in case that integer ambiguity is resolved without errors.

Proposal 2: Capture the above simulation results in the TR.

Proposal 3: RAN1 to conclude that carrier phase positioning can significantly improve the accuracy of the NR positioning.

## Potential Enhancements

In this section, we discuss the necessary enhancement to introduce CP positioning on top of the currently supported positioning techniques. For support of the CP positioning, there are four key issues that should be considered in study item phase.

Additional measurement:

The first one is that the UE needs to support phase measurement for a TRP or phase difference measurement for two different TRPs. Considering the clock bias between the UE and the TRP, the phase difference might be considered for measurement reporting, which is similar to DL-TDOA, but the phase measurement (without difference) is also useful for integer ambiguity resolution. To estimate timing measurement or RSRPP measurement for the first path, the UE already measures the first arrial path of wireless channel and hence, as a way of measuring the phase measurement, the UE can additionally extract phase value of the first path in addition to timing measurement. The CP positioning might be also effective for UL-based positioning. In addition to the DL measurement, this AI should include UL CP measurement at least for SRS for positioning.

Proposal 4: RAN1 to study and identify necessary measurements to support CP positioning for both DL-based positioning and UL-based positioning.

Integer ambiguity resolution:

Carrier phase measurement error is within one wavelength and hence the maximum error is within one wavelength and approximately 8.6 cm for 3.5 GHz carrier frequency. As shown in the above, the phase error is very small, but the key factor affecting the performance error might be unknown integer numbers of the wavelength (i.e., ΔNijk). Thankfully, this problem has been studied and there are couple of well-known solutions for GNSS system [11][12]. In this contribution, we briefly discuss virtual wavelength method [13], which is also known as wide-lane method. By this method, the integer ambiguity could be effectively found. This method uses two different carrier frequency measurements and ToA measurements together.

Fundamentally, as we mentioned above, the wavelength is quite small value such as 8.6 cm for 3.5 GHz carrier frequency, so search space for integer ambiguity is too large and it may lead to inaccurate interger ambiguity fixing. The concept of virtual wavelength method provides reduced search space to find the integer ambiguity. More specifically, let us assume two PRS resources are transmitted with different carrier freqnecy such as f1 and f2. Wavelength for each f1 and f2 are represented by λ1 and λ2. Then the carrier phase measurements can be denoted by as follows:

λ1φik1=dik +c δk-δi-λ1 Nik1

λ2φik2=dik +c δk-δi-λ2 Nik2

Where φik1 and φik2 mean the carrier phase measurement for carrier frequency f1 and f2, respectively. Nik1 and Nik2 respectively represent the integer ambiguity of the carrier phase measurement from carrier f1 and f2. Simply, multiply λ2λ2-λ1 and λ1λ2-λ1 to the first and the second carrier phase measurement, and then we can attain the following equation by subtract the second from the first one.  
λLφikL=dik +c δk-δi-λL Nik

Where λL=λ2-λ1λ1λ2, and the distance and clock bias term are not affected by this. The bottom line is that network can make λL is much greater than both λ1 and λ2. From the previous section, we are assuming 3.5 GHz carrier frequency. If f1=3.7 GHz and f2=3.5 GHz, λ1=8.1 cm and λ2=8.6 cm, but λL=1.5 m. Thus, the search space is highly reduced. If the UE location estimation from timing based techniques alone (e.g., DL-TDOA) is smaller than λL then the integer ambiguity can be resolved. This can be done by using the UE position estimate from timing measurements first to then know the integer ambiguity then use the carrier phase estimation procedure to find the UE location with even higher accuracy.

Observation 4: The virtual wavelength method may provide the reduced search space of integer ambiguity variables.

Given that Rel-17 positioning supports accuracy level less than 30 cm, we may be able to expect that the integer ambiguity fixing could be addressed by introducing the virtual wavelength technique.

Proposal 5: RAN1 to study the impact of integer ambiguity on CP positioning and identify potential solutions with necessary physical layer procedures.

Antenna Phase center offset:

In whole Rel-17 WI, issue on UE antenna array phase center offset has been discussed but it has not been addressed. By CP positioning, we are targeting centi-meter level of positioning accuracy, and it is critical to have a precise measure of the effective phase reference position for the signal received or transmitted via the UE mmWave antenna array. We name this position the antenna array phase center and ideally this antenna array phase center is fixed and aligned with e.g. the physical Antenna Reference Point (ARP). However, the UE antenna array phase center may not always align with the physical Antenna Reference Point (ARP) and its position may in fact be highly sensitive to multiple parameters of which some will vary during live operation in the field. The UE antenna array phase center location is for example sensitive to UE formfactor, cover, coating, beam steering angle, polarization and angle of arrival/departure both for narrow and broad beam antenna array configurations.

Observation 5: The antenna array phase center may not always aligne with the physical antenna reference point and it affects to phase and timing measurement accuracy.

RAN1 needs to study antenna phase center offset impact on the carrier phase positioning accuracy, and identify the physical layer impact to address this issue.

Proposal 6: RAN1 needs to study antenna phase center offset impact on the carrier phase positioning accuracy and identify potential solutions.

Reference device:

The double difference operation is essential to support cm-level positioning by avoiding the clock bias between multiple TRPs. The reference UE (PRU, Positioning Reference Unit) has been discussed in Rel-17 to improve the accuracy performance of timing measurement-based techniques. Many companies shared a similar view on the necessity and RAN1 made the following agreement, but still it is unclear how/whether it has been supported in NR positioning. For further accuracy performance improvement, the reference UE is essential and we should discuss further in this AI how to support reference UE. The starting point would be the early agreement in RAN1.

Agreement:(RAN1#105-e)

Send an LS to RAN2/RAN3 (cc SA2), including the following content:

- RAN1 has evaluated the use of positioning reference units (PRUs) with known locations for positioning and observes improvements in using PRUs for enhancing the positioning performance. But, RAN1 has not identified specification enhancements needed in RAN1 specifications. RAN1 kindly requests RAN2/RAN3 (cc SA2) to determine if and what specification enhancements are adopted for PRUs for positioning.

- Notes:

  - The term “positioning reference unit (PRU)” is only used as a terminology in this discussion.  PRU does not necessarily mean an introduction of a new network node.

  - PRU may support, at least, some of the Rel-16 positioning functionalities of UE, if agreed, which is up to RAN2.  The positioning functionalities may include, but not limited to, the following:

    - Provide the positioning measurements (e.g., RSTD, RSRP, Rx-Tx time differences)

    - Transmit the UL SRS signals for positioning

  - PRU may be requested by the LMF to provide its own known location coordinate information to the LMF. If the antenna orientation information of the PRU is known, the information may also be requested by the LMF.

In Rel-17, RAN1 has not identified RAN1 impact related to reference UE, but this decision was made in the early stage of Rel-17 WI. Now we need to reconsider it. From our perspective, the reference UE may not be able to have unlimited processing capability. If the reference UE is requested to measure/report many measurements in time-domain (e.g., every slot), frequency-domain (e.g., multiple frequency layers) and/or too much measurements from many TRPs, the reference UE might not be able to follow all the requests from network.

For double differential operation, the LMF would subtract the phase difference measurements between the target UE and the reference UE, i.e., ΔΔφijkK= Δφijk-ΔφijK. The two phase difference measurements should be measured with respect to the same two TRPs. The TRP clock bias might not be highly fluctuated but it might not be a constant value in time-domain, which should be considered. Furthermore, RAN1 needs to consider the impact of TRP Tx timing error group introduced in Rel-17. In our initial understanding, the target UE and reference UE may need to measure the phase difference measurements for the same TRP Tx TEGs.

Proposal 7: RAN1 to study the CP positioning performance impact by introducing reference UE (PRU) for UE-assisted positioning, and identify the necessary physical layer procedure including aligning measurements by PRU and target UE.

For UE-based CP positioning, the target UE also perform double differential operation, so the measurements from reference UE are also useful for target UE. For the accuracy enhancement of the UE-based CP positioning, we may need to study whether and how the target UE request CP measurements of the reference UE.

Proposal 8: RAN1 to study/identify the CP positioning performance impact by introducing reference UE (PRU) for UE-based positioning, and identify the necessary physical layer procedure.

# Conclusion

In this contribution we made the following proposals and observations:

Observation 1: Double difference operation removes the phase offset between different TRPs.

Proposal 1: RAN1 to study CP positioning for both DL and UL positioning based on the existing DL PRS and UL positioning SRS.

Observation 2: The phase measurement error is less than 1 cm for 80 percent of UEs and the error is less than 4 cm error for 90 percent of the UEs in InF-SH model.

Observation 3: Horizontal positioning accuracy is about 2 cm at 90% of UEs and vertical positioning accuracuy is 13 cm at 90% UEs in case that integer ambiguity is resolved without errors.

Proposal 2: Capture the above simulation results in the TR.

Proposal 3: RAN1 to conclude that carrier phase positioning can significantly improve the accuracy of the NR positioning.

Proposal 4: RAN1 to study and identify necessary measurements to support CP positioning for both DL-based positioning and UL-based positioning.

Observation 4: The virtual wavelength method may provide the reduced search space of integer ambiguity variables.

Proposal 5: RAN1 to study the impact of integer ambiguity on CP positioning and identify potential solutions with necessary physical layer procedures.

Observation 5: The antenna array phase center may not always aligne with the physical antenna reference point and it affects to phase and timing measurement accuracy.

Proposal 6: RAN1 needs to study antenna phase center offset impact on the carrier phase positioning accuracy and identify potential solutions.

Proposal 7: RAN1 to study the CP positioning performance impact by introducing reference UE (PRU) for UE-assisted positioning, and identify the necessary physical layer procedure including aligning measurements by PRU and target UE.

Proposal 8: RAN1 to study/identify the CP positioning performance impact by introducing reference UE (PRU) for UE-based positioning, and identify the necessary physical layer procedure.

# Appendix

We provide our initial performance analysis in the selected scenarios to read the performance trends accuracy. To understand the accuracy performance of simple correlation based method for the phase study, we limit our simulations for the use case with the dominant LOS scnenario, and hence we consider TDL-D and TDL-E channels. As a baseline, we have also provided results for the AWGN channel.

We provide the evaluation result for the DL PRS, and our assumption on simulation parameters (i.e., PRS BW (PRBs), sample rate, DL PRS NumSymbols, DL PRS CombSize, etc.) are aligned to the agreed parameters in [9], and we limit to a single antenna at TRP with no mobility. For TDL-D and TDL-E channels delay spread is set to 20ns by using delay-profile provided in Table 7.7.3-2 of TR38.901 [10]. Default values of K-factor for TDL-D (K1 = 13.3dB) and TDL-E (K1 = 22 dB) are used.

Figure 5 Phase error measurements in distance (cm) of AWGN, TDL-E, and TDL-D channels for BW = 5 MHz

This figure shows the CDF of the phase error measuremenets in distance (cm) of TDL-E, and TDL-D channels for 5 MHz BW. The carrier frequency is set to 2GHz. Results show that the estimation accuracy of TDL-E is higher compared with that of TDL-D, as its K1 factor value is larger and hence small phase interference from the NLOS taps is expected. Furthermore, the first tap of TDL-D and TDL-E follows the Ricean distribution and their exist inherent channel-model phase interference on the LOS path, which cannot be resolved by increasing the sampling rate (or BW). So, in order to properly evaluate/study the phase estimation error of the LOS path with the link level TDL channels, an agreegement on the treatment (e.g., introduction of delta timing offset between NLOS and LOS path in tap1, K1 values such that channel-model phase interference is negligible, etc.) of Rayleigh path of the first tab is necessary.

Observation 6: The ranging error that introduced due to the estimation error in phase is less than 1cm for 80% of realizations for TDL channels (D and E) with dominant LOS path.

Observation 7: The first arrival path of TDL model follows the Ricean distribution, resulting in inherent phase interference introduce due to TDL channel model. An agreement on the treatment of Rayleigh path of the first arrival path is necessary for the LOS path phase study.

# References

- RP-213588, Revised SID on Study on Expanded and Improved positioning.

- R1-2203127, SL positioning scenarios and requirements, Nokia, Nokia Shanghai Bell.

- R1-2203128, Evaluation of SL positioning, Nokia, Nokia Shanghai Bell.

- R1-2203129, Potential solutions for SL positioning, Nokia, Nokia Shanghai Bell.

- R1-2203177, Initial views on solutions for integrity of RAT-dependent positioning technques, Nokia, Nokia Shanghai Bell.

- R1-2203181, Initial views on Other topics for positioning, Nokia, Nokia Shanghai Bell.

- R1-2203179, Initial views on LPHAP, Nokia, Nokia Shanghai Bell.

- R1-2203180, Initial views on Positioning for RedCap UEs, Nokia, Nokia Shanghai Bell.

- 3GPP TR 38.857, Study on NR Positioning Enhancements (Release 17)

- 3GPP TR 38.901, Study on channel model for frequencies from 0.5 to 100 GHz.

- C. Günther and P. Henkel, IEEE, “Integer Ambiguity Estimation for Satellite Navigation”, IEEE Trans. Signal Processing, vol. 60, no. 7, July 2012

- Henkel, Patrick, Günther, Christoph, "Reliable Integer Ambiguity Resolution: Multi-Frequency Code Carrier Linear Combinations and Statistical A Priori Knowledge of Attitude", NAVIGATION: Journal of The Institute of Navigation, Vol. 59, No. 1, Spring 2012, pp. 61-75.

- R1-2003642, Discussion of NR positioning enhancements, CATT.

- R4-21108313, NR Pos performance simulation results collection and requirement summary.