3GPP TSG RAN WG1 #109	R1-2203179

e-Meeting, 09 – 20 May, 2022

Agenda item:		9.5.2.3

Source:	Nokia, Nokia Shanghai Bell

Title:	Initial Views on LPHAP

Document for:		Discussion and Decision

# Introduction

In Rel-16 native NR positioning support was standardized and in Rel-17 enhancements were made. At RAN#94 a new SI was approved on enhancements for Rel-18 NR positioning [1]. This contribution discussed our views related to LPHAP. Our companion contributions discuss our other views [2-8]. The objective in the SID is:

  - Study the requirements on LPHAP as developed by SA1 and evaluate whether existing RAN functionality can support these power consumption and positioning requirements. Based on the evaluation, and, if found beneficial, study potential enhancements to help address any limitations [RAN2, RAN1]

    - Study is limited to a single representative use case (use case 6 as defined TS 22.104). The choice of selected use case can be reviewed at the start of the study.

    - Study is limited to enhancements to RRC_INACTIVE and/or RRC_IDLE state

# Discussion

In the SID [1] it is stated that the use case for LPHAP, as developed by SA1, should be a single representative use case. In this case that is use case 6 as defined in [9]. The requirements, or targets, for this use case are defined in Section A.7 of [9] and are target hoiztonal accuracy of <1 m, 15 to 30 second positioning interval/duty cycle, and battery life of 6-12 months. The description of use case 6 is flexible modular assembly area: tracking of workpiece (in and outdoor) in assembly area and warehouse. In our view this use case should be confirmed as the only use case to be studied as part of the LPHAP study.

Proposal 1: Confirm that use case 6 as defined by TS 22.104 is the only use case to be studied in RAN1 during the LPHAP work.

RAN1 should also confirm the target requirements and inform RAN2 of these decisions. The current requirements from TS 22.104 also do not specify the number of UEs (e.g., 80% or 90%) for which the accuracy requirement holds. RAN1 should determine the appropriate requirement for LPHAP.

Proposal 2: Confirm the target requirements of at least horiztonal accuracy of <1 m, 30 second positioning interval/duty cycle, and battery life of 6 months.

Proposal 3: RAN1 to discuss the number of UEs for which the horizational accuracy requirement holds (e.g., 90%).

Rel-16/17 NR positioning supports various techniques using DL and/or UL reference signals, showing different performances. This study does not necessarily need to exclude a specific method as each one has pros and cons, but it should be noted that this target requirement is not necessarily met by all NR positioning techniques.

## LPHAP Evaluation Methodology

After these target requirements have been confirm then the next step is to agree on the evaluation methodology for meeting these requirements. The target accuracy has a well established methodology from Rel-16/17 and should be reused. For battery life there needs to be some agreements made. Some power consumption evaluation methodology from the UE power savings work (e.g., TR 38.840) can be used as a starting point when applicable. Another open point is if the battery life should be directly evaluated or only relative power will be used similar to the UE power saving study. If the later is chosen then it should be discussed how this relates to the 6+ month requirement from the SID.

Proposal 4: RAN1 to discuss evaluation methology for UE battery life in positioning and reuse assumptions from TR 38.840 when possible.

Proposal 5: RAN1 to decide if battery life will be directly evaluated or if only relative power as an evaluation metric.

As the study objective in the SID is limited to enhancement of RRC Inactive and/or RRC Idle states our understanding is these are the only states that we need to evaluate the power consumption/battery life for.

Observation 1: RRC Connected state does not need to be evaluated for UE power consumption/battery life as part of the LPHAP study.

In many ways PRS measurement is similar to RRM measurement and we can use the power consumption model from Section 8.1.4 of TR 38.840. We also reuse some evaluation assumptions from [10]. At least for an initial evaluation we make some assumption as below. There may be further combinations of scenarios that RAN1 will investigate later. RAN1 should discuss what the priority is (e.g., FR1 vs FR2, indoor or outdoor deployment, Inactive or idle mode, UE based or UE assisted positioning).

Proposal 6: RAN1 to discuss the baseline scenario assumptions for power consumption evaluations (e.g., FR1 vs FR2 , indoor vs outdoor).

Some general assumptions on PRS measurements should be made:

- Carrier Frequency: 3.5 GHz

- Bandwidth: 100MHz

- SCS:30kHz

- RRC State: Inactive

- PRS Periodicity: 1280ms

- Single Sample PRS measurement

- RE mapping: Comb-6, 6 symbols

- 1 PRS resource per TRP

- Repetition factor=4, time gap=2 slot

- Single positioning frequency layer

- 12 TRPs, TDM across 2 slots for 6 of the TRPs

Some general assumptions on SRS transmission should be made:

- Carrier Frequency: 3.5 GHz

- Bandwidth: 100MHz

- SCS:30kHz

- RRC State: Inactive

- SRS Periodicity: 1280ms

- RE mapping: Comb-2, 4 symbols

- 1 SRS resource

- Repetition factor=4, time gap=1 slot

Proposal 7: Capture the above power consumption assumptions for PRS measurement and SRS transmission in idle/inactive mode in the TR.

Additional considerations include synchronization, control channel data, and any data channel traffic. To meet the challenging requirements for use case 6 we propose to assume the UE has no data traffic and the only control channel data is used for positioning purposes. The UE should still monitor the paging and synchronize by making SSB RRM measurements periodicially.

Observation 2: For idle/inactive mode UE power consumption modelling the UE should monitor paging and synchronize by making periodic SSB RRM measurements.

Proposal 8: For LPHAP evaluations do not model any data traffic power consumption.

From Section 8.1.1 in TR 38.840 [11] we can make use of the following agreed power consumption numbers from Table 18 and Table 19:

| Power State | Characteristics | Relative Power |
| --- | --- | --- |
| Deep Sleep | Time interval for the sleep should be larger than the total transition time entering and leaving this state. Accurate timing may not be maintained. | 1 (Optional: 0.5) |
| Light Sleep | Time interval for the sleep should be larger than the total transition time entering and leaving this state. | 20 |
| Micro sleep | Immediate transition is assumed for power saving study purpose from or to a non-sleep state | 45 |
| PDCCH-only | No PDSCH and same-slot scheduling; this includes time for PDCCH decoding and any micro-sleep within the slot. | 100 |
| SSB or CSI-RS proc. | SSB can be used for fine time-frequency sync. and RSRP measurement of the serving/camping cell. TRS is the considered CSI-RS for sync. FFS the power scaling for processing other configurations of CSI-RS. | 100 |
| PDCCH + PDSCH | PDCCH + PDSCH. ACK/NACK in long PUCCH is modeled by UL power state. | 300 |
| UL | Long PUCCH or PUSCH. | 250 (0 dBm) 700 (23 dBm) |

| Sleep type | Additional transition energy: (Relative power x ms) | Total transition time |
| --- | --- | --- |
| Deep sleep | 450 | 20 ms |
| Light sleep | 100 | 6 ms |
| Micro sleep | 0 | 0 ms* |
| * Immediate transition is assumed for power saving study purpose from or to a non-sleep state |  |  |

It is also defined in Table 21 how to do BW scaling and that the power of SRS is = 0.3 x uplink power.

| Scaling for FR1 | Proposal | Comment |
| --- | --- | --- |
| BWP Bandwidth (DL) | Scaling of X MHz = 0.4 + 0.6 * (X - 20) / 80. Linear interpolation for intermediate bandwidths. Valid only for X = 10, 20, 40, 80, and 100. Above scaling is applicable for FR1 only. In case scaling is needed for FR2, companies can report the assumed scaling factor. | For 10MHz BW, only AL up to 8 can be used for PDCCH The transition time is the same as DCI-based BWP switching delay for Rel-15. If the power after scaling is smaller than the BWP transition power, assume the BWP transition power as the output of scaling unless otherwise justified. |
| BWP Bandwidth (UL) | No scaling at 0dBm or 23dBm Above scaling is applicable for FR1 only. In case scaling is needed for FR2, companies can report the assumed scaling factor. |  |
| SRS | SRS power = 0.3 x uplink power | Applicable for FR1 and FR2. |

Proposal 9: Reuse Tables 18, 19, and 21 from [11] for LPHAP power consumption evaluations.

Table 1. Summary of relevant states for LPHAP.

| Power State | Reference Relative Power |
| --- | --- |
| Deep Sleep | 1 (Optional: 0.5) |
| Light Sleep | 20 |
| Micro sleep | 45 |
| PDCCH-only | 100 |
| SSB RRM intra-freq | 150 |
| SSB Inter-freq | 200 |
| PDCCH + PDSCH | 300 |
| UL | 250 (0 dBm) 700 (23 dBm) |
| PRS (6 TRPs, 6 symbols) | 375 |
| SRS for positioning (4 symbols) | 420 (23 dBm) |

Proposal 10: For the values of PRS (6 TRPs, 6 symbols) and SRS for positioning (4 symbols) power states adopt 375 and 420 respectively.

## Initial LPHAP Power Consumption Evaluation

In this section we present our initial power consumption evaluation. Figure 1 shows the proposed DL timeline and Figure 2 shows the propose UL timeline. Table 2 then shows the energy consumption for the DL case and Table 3 shows the energy consumption for the UL case. For the DL case we assume that the UE’s DRX cycle is not aligned with the PRS reception.

Figure 1. Initial DL Timeline for RRC Inactive positioning evaluation.

Figure 2. Initial UL Timeline for RRC Inactive positioning evaluation

Table 2. Initial DL LPHAP power consumption evaluation.

| Power State | Relative Power | Time (ms)/# of Transitions | Relative Energy |
| --- | --- | --- | --- |
| Deep Sleep | 1 | 1255 | 1255 |
| Light Sleep | 20 | 0 | 0 |
| Micro sleep | 45 | 10 | 450 |
| PDCCH + PDSCH | 300 | 2 | 600 |
| SSB RRM | 150 | 2 | 300 |
| SSB Inter-freq | 200 | 7 | 1400 |
| PRS (6 TRPs) | 375 | 4 | 1500 |
| Deep sleep transition | n/a | 2 | 900 |
| Light sleep transition | n/a | 0 | 0 |
| Micro sleep transition | n/a | 2 | 0 |
| Total Relative Energy |  |  | 6405 |

Table 3. Initial UL LPHAP power consumption evaluation.

| Power State | Relative Power | Time (ms)/# of Transitions | Relative Energy |
| --- | --- | --- | --- |
| Deep Sleep | 1 | 1252 | 1252 |
| Light Sleep | 20 | 0 | 0 |
| Micro sleep | 45 | 15 | 675 |
| PDCCH + PDSCH | 300 | 2 | 600 |
| SSB RRM | 150 | 2 | 300 |
| SSB Inter-freq | 200 | 7 | 1400 |
| SRS for positioning (4 symbols) | 420 | 2 | 840 |
| Deep sleep transition | n/a | 1 | 450 |
| Light sleep transition | n/a | 0 | 0 |
| Micro sleep transition | n/a | 3 | 0 |
| Total Relative Energy |  |  | 5517 |

## Potential LPHAP Enhancements

As pointed out in Section 2.1 the battery life of the UE can not be directly evaluated based on the above power consumption model. Our understanding based on this analysis however, is that enhancements will be needed to lower the power consumption of the UE during positioning procedures. Especially considering some impacts that were so far ignored in the analysis in Section 2.2, for example UE measurement report and assistance data update. Some potential areas for enhancement that RAN1 can consider are:

- Measurement report power consumption reduction

- Optimzed delivery of assistance data updates

The UE measurement report takes considerable power consumption as it needs to transmit/receive multiple messages to/from the network. Even when using SDT the UE may still incur considerable power consumption to make the measurement report. In fact many of these measurement reports may be redundant if the UE has not moved much or the measurement results are similar to previous reported UE measurements. One potential solution to consider allowing the UE to skip some measurement reports.

Proposal 11: RAN1 to study allowing UE to skip some measurement reports (e.g., when measurement results are similar).

We are expecting that the mobility of this type of UE would be quite low than normal UE. Although the UE has moved to another place, it does not necessarily report all positioning measurements so that the LMF calculates the location of this UE with low latency. For the power consumption reduction, it would be better to use the minimum power as less as possible. For example, the UE could report a part of measurements at a reporting instance and reports another part of measurements after a long time, so that LMF could estimate UE location using a part of accumulated reported measurements. It should be noted that the current LPP may not be able to utilize a part of reported measurements by message segmentation. RAN1 needs to study the partial measurement report functionality. Inherently, RRC_INACTIVE UE might have a limited reporting overhead.

Proposal 12: For purpose of the power consumption reduction, RAN1 investigates the impact of the partial measurement reporting functionality and identifies the necessary physical layer procedure.

In addition, the overhead of updating the AD should be considered as the UE power consumption is targeted to be mitigated. One potential way to reduce the overhead of updating the AD is to consider a partial update of the assistance data. For example, the UE may receive AD for a new TRP only and the rest of the AD is left unchanged.

Proposal 13: RAN1 to study partial updates of PRS AD for UEs in RRC_INACTIVE mode to reduce overhead and power consumption.

When RRC-inactive/idle UE moves from one serving gNB to a new serving gNB, the configured SRS-pos (from original cell) becomes invalid. UE needs to resume RRC connection to get new SRS-pos configuration from the new serving cell, in order to maintain the positioning service. Obviously, the related procedure would cause high power consumption at the UE and large positioning latency due to RRC resume process including UE AS Context fetch or a handover and RRC connection state transition from RRC INACTIVE to RRC CONNECTED.

Proposal 14: RAN1 to study methods to reduce the frequency of UL SRS for positioning configuration update, e.g., by configuring common UL SRS for positioning within RAN-based Notification Area (RNA).

# Conclusion

In this contribution we made the following proposals and observations:

Proposal 1: Confirm that use case 6 as defined by TS 22.104 is the only use case to be studied in RAN1 during the LPHAP work.

Proposal 2: Confirm the target requirements of at least horiztonal accuracy of <1 m, 30 second positioning interval/duty cycle, and battery life of 6 months.

Proposal 3: RAN1 to discuss the number of UEs for which the horizational accuracy requirement holds (e.g., 90%).

Proposal 4: RAN1 to discuss evaluation methology for UE battery life in positioning and reuse assumptions from TR 38.840 when possible.

Proposal 5: RAN1 to decide if battery life will be directly evaluated or if only relative power as an evaluation metric.

Observation 1: RRC Connected state does not need to be evaluated for UE power consumption/battery life as part of the LPHAP study.

Proposal 6: RAN1 to discuss the baseline scenario assumptions for power consumption evaluations (e.g., FR1 vs FR2 , indoor vs outdoor).

Proposal 7: Capture the above power consumption assumptions for PRS measurement and SRS transmission in idle/inactive mode in the TR.

Observation 2: For idle/inactive mode UE power consumption modelling the UE should monitor paging and synchronize by making periodic SSB RRM measurements.

Proposal 8: For LPHAP evaluations do not model any data traffic power consumption.

Proposal 9: Reuse Tables 18, 19, and 21 from [11] for LPHAP power consumption evaluations.

Proposal 10: For the values of PRS (6 TRPs, 6 symbols) and SRS for positioning (4 symbols) power states adopt 375 and 420 respectively.

Proposal 11: RAN1 to study allowing UE to skip some measurement reports (e.g., when measurement results are similar).

Proposal 12: For purpose of the power consumption reduction, RAN1 investigates the impact of the partial measurement reporting functionality and identifies the necessary physical layer procedure.

Proposal 13: RAN1 to study partial updates of PRS AD for UEs in RRC_INACTIVE mode to reduce overhead and power consumption.

Proposal 14: RAN1 to study methods to reduce the frequency of UL SRS for positioning configuration update, e.g., by configuring common UL SRS for positioning within RAN-based Notification Area (RNA).

# References

- RP-213588, Revised SID on Study on Expanded and Improved positioning.

- R1-2203127, SL positioning scenarios and requirements, Nokia, Nokia Shanghai Bell.

- R1-2203128, Evaluation of SL positioning, Nokia, Nokia Shanghai Bell.

- R1-2203129, Potential solutions for SL positioning, Nokia, Nokia Shanghai Bell.

- R1-2203177, Initial views on solutions for integrity of RAT-dependent positioning technques, Nokia, Nokia Shanghai Bell.

- R1-2203178, Initial views on improved accuracy based on NR carrier phase measurement, Nokia, Nokia Shanghai Bell.

- R1-2203181, Initial views on Other topics for Positioning, Nokia, Nokia Shanghai Bell.

- R1-2203180, Initial views on Positioning for RedCap UEs, Nokia, Nokia Shanghai Bell.

- TS 22.104, Service requirement for cyber-physical control applications in vertical domains Stage1.

- R1-2101664, Evaluation of potential paging enhancements, Nokia, Nokia Shanghai Bell.

- TR 38.840. Study on User Equipment power saving in NR.