3GPP TSG RAN WG1 #109	R1-2203180

e-Meeting, 09 – 20 May, 2022

Agenda item:		9.5.3

Source:	Nokia, Nokia Shanghai Bell

Title:	Initial Views on Positioning for RedCap UEs

Document for:		Discussion and Decision

# Introduction

In Rel-16 native NR positioning support was standardized and in Rel-17 enhancements were made. At RAN#94 a new SI was approved on enhancements for Rel-18 NR positioning [1]. This contribution discusses our views related to RedCap positioning. Our companion contributions discuss our other views [2-8]. The objective in the SID is:

- Positioning support for RedCap UEs, considering the following:

  - Evaluate positioning performance of existing positioning procedures and measurements with RedCap UEs[RAN1]

  - Based on the evaluation, assess the necessity of enhancements and, if needed, identify enhancements to help address limitations associated with for RedCap UEs [RAN1, RAN2]

# Discussion

## Evaluations

Figure 1 shows the evaluated performance of DL-TDOA for a RedCap device with a PRS BW of 5 MHz. We initially focused on the indoor factory (InF) model and reused many of the assumptions from TR 38.857 in the Rel-17 NR positioning evaluations.

Figure 1. RedCap UE performance with 5 MHz BW and 1 Rx antenna.

This figure shows the hotizontal positioning accuracy for InF-SH. The purple curve shows less than 10 m horizontal accuracy error at 80% of UEs, where the oversampling factor is 4 and measurement filtering method is used to select 8 out of 18 TRPs. Other simulation assumption parameters are below:

| Parameter | Value |
| --- | --- |
| Carrier Frequency | 3.5 GHz |
| PRS bandwidth | 5 MHz |
| SCS | 15 kHz |
| PRS assumptions | 6 symbols PRS resource with comb-6. |
| # of UE antennas | 1 |
| # of gNB antennas | 4 |
| Positioning Method | DL-TDOA |
| Parameter Estimation Method | ToA estimation using thresholding technique |
| Position Estimation Method | Taylor Series, Least Squares |

Table 1 Evaluation assumption parameters

Observation 1: The horizatonal positioning accuracy performance of RedCap UEs is significantly degraded compare with normal NR UEs.

RAN1 should determine the performance target of RedCap UEs in order to decide what enhancements need to be met. It should be clear that at least some degradation compared with normal NR UEs will be needed.

Proposal 1: RAN1 to discuss the performance targets for RedCap UEs.

The evaluation scenarios should also be discussed for RedCap devices. For example, which scenarios of are most interest to find enhancements. For example, should the focus like in Rel-17 be on the indoor factory environment. RAN1 may need a discussion to determine assumptions on simulation parameters for performance evaluation. The indoor factory model could be considered as a baseline so we could reuse the simulation assumption of Rel-17 SI, but the key different assumption might be the number of Rx antennas and PRS BW.

Proposal 2: RAN1 to consider evaluation assumption parameters of Table 1 as a baseline for accuracy performance evaluation of RedCap UE.

## Potential Solutions

### Dynamic SRS Transmissions

Rel-16/17 NR defines a fixed SRS configuration that the UE should use at each positioning period, regardless of the UE mobility change and/or power limitations. Specifically, an SRS transmission is characterized by a given time-frequency density which is maintained for all occurrences during a periodic localization process. This approach is sub-optimal for RedCap devices which would need to ensure a periodic SRS transmission even though the device position remains fixed from one location update to the other. Thus, a flexible UL positioning session w.r.t. time-frequency resource allocation and the ability to tune the periodicity of the session would be beneficial in terms of power savings.

Proposal 3: RAN1 to study more dynamic SRS transmissions for RedCap UEs.

### Group Positioning

SRS transmission for UL positioning is an expensive operation for a power-limited device like RedCap. That is because:

- SRS for positioning are typically transmitted at full power, and most often, their transmission is repeated for several consecutive OFDM symbols in a slot.

- SRS are often configured periodically, which means that the RedCap UE needs to transmit full-power SRS every X subframes, even though its position may have not changed in between consecutive transmissions.

Schemes that reduce the power consumption associated with SRS transmission according to the above are thus desirable for RedCap devices. Given that RedCap devices in some use cases are likely to be collocated with other similar devices, their collocation could be exploited by the LMF to locate the group of devices instead of each device independently, reducing thus the signalling load of each independent device.

Proposal 4: RAN1 to investigate group based positioning schemes for RedCap UEs.

### Narrow band measurement on wideband signal Positioning

With the limitation of the bandwidth from 100 MHz to 20MHz (or even 5MHz) there will be a degradation of the timing estimation used for the position estimate with a factor of four. To overcome this alternative solutions could be to continue transmitting the PRS signal with 20 MHz (or 5 MHz) chunks repeated in multiple symbols and introduce frequency shift and thereby re-gain the combined timing estimate as if it was a wideband receiver architecture.

The same method could be considered on the transmitter side- apply frequency shift symbol based and send the combined SRS signal as a virtual wideband signal in chunks of 20 Mhz (or 5MHz) divided over multiple symbols.

Proposal 5: RAN1 to study the ability to receive wideband PRS signals with a narrow band receiver (e.g., over multiple 20 MHz chunks) and transmit wideband signal on a narrow band transmitter (e.g., frequency hopping over multiple 20 MHz chunks). Identifying any specification impact should be part of the study.

### Positioning Impacts on Reduced Capability Features

A RedCap UE may be implementing power saving or other reduced capability features which could negatively impact the positioning performance of the device. Outside of the obvious constraints from limited BW and reduced antennas there could be other features such as relaxed RRM measurements or relaxed beam measurements When the UE is implementing these features it may reduce the positioning measurement accuracy of the device. This is further complicated by the fact that the gNB may not be aware the UE is performing positioning measurements or the requirements associated with those measurements (due to LPP being transparent to the gNB).

Observation 2: RedCap UEs may have positioning measurement performance degradation due to power saving/reduced capability features it is implementing.

It should be ensured that a balance between power savings/reduced capabilities and positioning performance is reached. RAN1 should consider the impact of these reduced capability features from Rel-17 and minimize any impact to the accuracy of positioning measurements.

Proposal 6: RAN1 to study methods for reducing the impact of reduced capability features (e.g., RRM measurement relaxation) on the positioning measurement accuracy of RedCap UEs.

# Conclusion

In this contribution we made the following observations and proposals:

Observation 1: The horizatonal positioning accuracy performance of RedCap UEs is significantly degraded compare with normal NR UEs.

Proposal 1: RAN1 to discuss the performance targets for RedCap UEs.

Proposal 2: RAN1 to consider evaluation assumption parameters of Table 1 as a baseline for accuracy performance evaluation of RedCap UE.

Proposal 3: RAN1 to study more dynamic SRS transmissions for RedCap UEs.

Proposal 4: RAN1 to investigate group based positioning schemes for RedCap UEs.

Proposal 5: RAN1 to study the ability to receive wideband PRS signals with a narrow band receiver (e.g., over multiple 20 MHz chunks) and transmit wideband signal on a narrow band transmitter (e.g., frequency hopping over multiple 20 MHz chunks). Identifying any specification impact should be part of the study.

Observation 2: RedCap UEs may have positioning measurement performance degradation due to power saving/reduced capability features it is implementing.

Proposal 6: RAN1 to study methods for reducing the impact of reduced capability features (e.g., RRM measurement relaxation) on the positioning measurement accuracy of RedCap UEs.

# References

- RP-213588, Revised SID on Study on Expanded and Improved positioning.

- R1-2203127, SL positioning scenarios and requirements, Nokia, Nokia Shanghai Bell.

- R1-2203128, Evaluation of SL positioning, Nokia, Nokia Shanghai Bell.

- R1-2203129, Potential solutions for SL positioning, Nokia, Nokia Shanghai Bell.

- R1-2203177, Initial views on solutions for integrity of RAT-dependent positioning technques, Nokia, Nokia Shanghai Bell.

- R1-2203178, Initial views on improved accuracy based on NR carrier phase measurement, Nokia, Nokia Shanghai Bell.

- R1-2203179, Initial views on LPHAP, Nokia, Nokia Shanghai Bell.

- R1-2203181, Initial views on Other topics for Positioning, Nokia, Nokia Shanghai Bell.