3GPP TSG RAN WG1 #109-e                                         R1-2203185

e-Meeting, May 9th – May 20th, 2022

Source:	ZTE

Title:          Discussion on non-overlapping PUCCHs with repetition

Agenda item:	7.2.5

Document for: Discussion and Decision

- Introduction

In RAN1#108-e meeting, a conclusion is achieved for the clarification on HARQ-ACK feedback of multiple SPS configurations with PUCCH repetition. But there is still a note in the conclusion to be further clarified [1].

Conclusion For collision handling among PUCCH with repetitions with the same UCI type priority, - the starting slot of PUCCH is the PUCCH slot before deferral, i.e. the starting slot of PUCCH is - the slot determined as described in clause 9.2.3 for HARQ-ACK reporting. - the slot determined as described in clause 9.2.4 for SR reporting - the slot determined as described in clause 5.2.1.4 of TS 38.214 for CSI reporting. - if a UE would transmit at least two PUCCHs with UCI of same UCI type priority that start in different slots and the earliest PUCCH is with repetitions, the UE drops transmission of a later PUCCH in every slot where the transmission of the later PUCCH would overlap with a repetition of the earliest PUCCH transmission. - Note: The related handling of at least two non-overlapping PUCCHs with HARQ-ACK within a PUCCH slot requires further clarification.

In this contribution, we further discuss the potential remaining issue of handling of at least two non-overlapping PUCCHs with HARQ-ACK within a PUCCH slot and provide our views.

- Multiplexing UCIs of different priorities in a PUCCH

The issue for further clarification is related to the handling of at least two non-overlapping PUCCHs with HARQ-ACK within a PUCCH slot which is not explicitly covered by the current specification as cited.

A UE does not multiplex different UCI types in a PUCCH transmission with repetitions over slots. If a UE would transmit a first PUCCH over more than one slot and at least a second PUCCH over one or more slots, and the transmissions of the first PUCCH and the second PUCCH would overlap in a number of slots then, for each slot of the number of slots and with UCI type priority of HARQ-ACK > SR > CSI with higher priority > CSI with lower priority - the UE does not expect the first PUCCH and any of the second PUCCHs to start at a same slot and include a UCI type with same priority - if the first PUCCH and any of the second PUCCHs include a UCI type with same priority, the UE transmits the PUCCH starting at an earlier slot and does not transmit the PUCCH starting at a later slot - if the first PUCCH and any of the second PUCCHs do not include a UCI type with same priority, the UE transmits the PUCCH that includes the UCI type with higher priority and does not transmit the PUCCH that include the UCI type with lower priority

Current specification only explicitly describe the case that the first PUCCH and the second PUCCH would overlap in a slots, but not explicitly describe the case of non-overlapping PUCCHs.

The case of non-overlapping PUCCHs can be further split to two individual cases, one is for the PUCCHs with the same physical priority, and the other is for the PUCCHs with different physical priorities. The Figure 1 illustrates the typical case of non-overlapping PUCCHs with repetition. Based on Figure 1, some analyses are provided as follows.

Figure 1 non-overlapping PUCCHs with repetition

There are two PUCCHs with repetition factor 4, PUCCH1 starts earlier than PUCCH2, PUCCH1 and PUCCH2 are not overlapped in the slot 2, 3, 4. One case is the PUCCH1 and PUCCH2 have the same physical priority, the other case is the PUCCH1 and PUCCH2 have different physical priorities.

If the non-overlapping PUCCHs with repetition have the same physical priority in the same slot, especially when the second PUCCH is a dynamic PUCCH, it should be an error case to schedule a second dynamic PUCCH, as current specification doesn’t support more than one PUCCH with same priority in the same slot. In addition, PUCCH2 can also be without repetition, i.e., the repetition factor of PUCCH2 is equal to 1. Then a unified possible conclusion can be proposed for the case with PUCCH repetition as below:

Proposal 1: In a slot determined for a HARQ-ACK/SR/CSI PUCCH repetition, other PUCCHs that are non-overlapping in the time domain with the HARQ-ACK/SR/CSI PUCCH repetition are not expected to be dynamically scheduled.

- The other PUCCHs have the same physical priority as the HARQ-ACK/SR/CSI PUCCH repetition.

- The other PUCCHs are to be transmitted over one or more slots.

If the PUCCH2 is the configured grant PUCCH or semi-static PUCCH, to avoid the non-overlapping PUCCHs with repetition and with the same physical priority to be transmitted in the same slot is very difficult and restrictive for gNB to configure the semi-static PUCCH. It is better not to restrict the gNB configuration and only cancel the second PUCCH if the non-overlapping PUCCHs with repetition and with the same physical priority to be transmitted in the same slot. Then another possible conclusion can be proposed for the case with PUCCH repetition as below:

Proposal 2: In a slot determined for a HARQ-ACK/SR/CSI PUCCH repetition, other PUCCHs are cancelled if the other PUCCHs are semi-statically configured which are non-overlapping with the HARQ-ACK/SR/CSI PUCCH repetition in the time domain.

- The other PUCCHs have the same physical priority as the HARQ-ACK/SR/CSI PUCCH repetition.

- The other PUCCHs are to be transmitted over one or more slots.

If the non-overlapping PUCCHs with repetition have the different physical priorities in a slot determined for a HARQ-ACK/SR/CSI PUCCH repetition, it is allowed in the current specification as the two PUCCHs with higher and lower priorities. It means that the two PUCCHs can both survive in the same slot, even if the PUCCH is with repetition.

Proposal 3: In a slot determined for a HARQ-ACK/SR/CSI PUCCH repetition, other PUCCHs are allowed to be semi-statically configured or dynamically scheduled which are non-overlapping with the HARQ-ACK/SR/CSI PUCCH repetition in the time domain if the other PUCCHs have different physical priorities with the HARQ-ACK/SR/CSI PUCCH repetition.

- The other PUCCHs are to be transmitted over one or more slots.

For more than two non-overlapping PUCCHs in one slot, for example, three PUCCHs are in the same slot, anyway it is not allowed by the current specifications and can be regarded as an error case.

- Conclusion

According to the analysis given above, we have the following proposals:

Proposal 1: In a slot determined for a HARQ-ACK/SR/CSI PUCCH repetition, other PUCCHs that are non-overlapping in the time domain with the HARQ-ACK/SR/CSI PUCCH repetition are not expected to be dynamically scheduled.

- The other PUCCHs have the same physical priority as the HARQ-ACK/SR/CSI PUCCH repetition.

- The other PUCCHs are to be transmitted over one or more slots.

Proposal 2: In a slot determined for a HARQ-ACK/SR/CSI PUCCH repetition, other PUCCHs are cancelled if the other PUCCHs are semi-statically configured which are non-overlapping with the HARQ-ACK/SR/CSI PUCCH repetition in the time domain.

- The other PUCCHs have the same physical priority as the HARQ-ACK/SR/CSI PUCCH repetition.

- The other PUCCHs are to be transmitted over one or more slots.

Proposal 3: In a slot determined for a HARQ-ACK/SR/CSI PUCCH repetition, other PUCCHs are allowed to be semi-statically configured or dynamically scheduled which are non-overlapping with the HARQ-ACK/SR/CSI PUCCH repetition in the time domain if the other PUCCHs have different physical priorities with the HARQ-ACK/SR/CSI PUCCH repetition.

- The other PUCCHs are to be transmitted over one or more slots.

- Reference

- R1-2202772, Summary of [108-e-R16-URLLC-03] Issue#4: Clarification on HARQ-ACK feedback of multiple SPS configurations with PUCCH, Moderator (Nokia)