3GPP TSG RAN WG1 #109-e                                         R1-2203189

e-Meeting, May 9th – May 20th, 2022

Source:	ZTE

Title:          Discussion on enhanced intra-UE multiplexing

Agenda item:	8.3.2

Document for: Discussion and Decision

- Introduction

In the RAN1#108-e meeting, lots of agreements on the potential enhancements for intra-UE multiplexing between different priorities and simultaneously PUCCH/PUSCH transmission were achieved [1], and the WI has been considered as completed except some remaining minor issues to be solved in maintenance stage, such as:

- Overlapping between a HP PUCCH carrying SR and HP HARQ with PUCCH format 0/1 and a LP PUCCH carrying HARQ-ACK and CSI/SR (if any)

- Handling of LP/HP HARQ-ACK on HP/LP PUSCH with CG-UCI

In this contribution, we further discuss the potential remaining issues and provide our views.

- Multiplexing UCIs of different priorities in a PUCCH

  - Multiplexing PUCCH carrying HP SR and HP HARQ with PF0/1 with PUCCH carrying LP HARQ-ACK with and CSI/SR (if any)

In the RAN1#108-e meeting, one remaining issue is the overlapping between a HP PUCCH carrying SR and HP HARQ with PUCCH format 0/1 and a LP PUCCH carrying HARQ-ACK and CSI/SR (if any), a possible proposal was discussed.

When a PUCCH carrying explicit/implicit HP SR and HP HARQ-ACK with PUCCH format 0/1 overlaps with a PUCCH carrying LP HARQ-ACK, where the original PUCCH carrying the HP HARQ-ACK before Step 1 overlaps with K HP SRs which are all negative or include at least one positive, - In Step 1, 1-bit HP SR is multiplexed to HP HARQ-ACK bits (based on Rel-15 rules) for determining the resultant PUCCH resource. - Note: The description of Step 1 here is only for information purpose, which has no spec impact. - In Step 2, down-select from the two options: - Option 0: LP HARQ-ACK is dropped. When a PUCCH carrying positive HP SR and HP HARQ-ACK with PUCCH format 0/1 overlaps with a PUCCH carrying LP HARQ-ACK, the PUCCH carrying LP HARQ-ACK is dropped. - Option 2: If HP HARQ-ACK is dynamic HARQ-ACK or SPS HARQ-ACK with sps-PUCCH-AN-List, 1-bit HP SR is appended to HP HARQ-ACK bits. The number of HP UCI bits is OUCI=OACK,1+1 - The resultant PUCCH resource for multiplexing HP SR + HP HARQ-ACK + LP HARQ-ACK is either of PF2, PF3, or PF4. If HP HARQ-ACK is SPS HARQ-ACK with n1PUCCH-AN, LP HARQ-ACK is dropped.

In this case, there are both HP SR and HP HARQ-ACK in the HP PUCCH overlapping with LP PUCCH with HARQ-ACK. The first step is 1-bit HP SR multiplexed to HP HARQ-ACK bits (based on Rel-15 rules) for determining the resultant PUCCH resource. Then let’s review the Rel-15 multiplexing rules below.

In Rel-15, if an SR overlaps with a HARQ-ACK, the PUCCH resource used for transmission after multiplexing are summarized in Table 1 according to the different PUCCH formats used for HARQ-ACK and SR.

Table 1. PUCCH resource after multiplexing HARQ-ACK and SR in Rel-15

| HARQ-ACK SR | PUCCH format 0 | PUCCH format 1 | PUCCH format 2/3/4 |
| --- | --- | --- | --- |
| PUCCH format 0 | Case 1: The UE transmits the PUCCH in the HARQ-ACK resource using PUCCH format 0. The mcs are differently specified to identify the positive or negative SR. | Case 2: The UE only transmits a PUCCH with the HARQ-ACK information bits in the resource using PUCCH format 1. | Case 4: The UE transmits the concatenated UCI bits of HARQ-ACK information and SR in a PUCCH using a resource with PUCCH format 2 or PUCCH format 3 or PUCCH format 4. |
| PUCCH format 1 |  | Case 3: For positive SR, the UE transmits a PUCCH for HARQ-ACK information in the SR resource using PUCCH format 1. For negative SR, the UE transmits a PUCCH for HARQ-ACK information in the HARQ-ACK resource using PUCCH format 1. |  |

For the resultant PUCCH carrying HP SR and HP HARQ-ACK with PUCCH format 0/1, the multiplexing results are diverse depending on the PUCCH format 0 or 1 for HARQ-ACK and SR, and also depending on the positive or negative SR.

For HP SR multiplexed with HP HARQ-ACK with PUCCH format 0, the resultant PUCCH resource is only the HARQ-ACK resource using PUCCH format 0 but the mcs are differently specified to identify the positive or negative SR. The information of HP SR and HP HARQ-ACK are all carried and the HP SR is implicitly carried by the different cyclic shift. But for HP SR PUCCH on PF0 overlapping with HP HARQ-ACK on PF1, HP SR seems to be dropped and no information of SR is carried. So the step 1 in proposal is not accurate as no SR information is carried. Actually for HP SR overlapping with HP HARQ-ACK with PUCCH format 0/1, the HP SR is implicitly indicated in all sub-cases.

The simplest way for this issue is dropping the PUCCH carrying LP HARQ-ACK regardless the SR is positive or negative SR. The Option 0 in above proposal is not reasonable for dropping, as for legacy Rel-15 rules, the negative SR information is also carried in resultant PUCCH in some cases. So the dropping based only on positive SR in Option 0 above is not a clean dropping.

The Option 0 can be revised to:

When a PUCCH carrying explicit/implicit HP SR and HP HARQ-ACK with PUCCH format 0/1 overlaps with a PUCCH carrying LP HARQ-ACK, where the original PUCCH carrying the HP HARQ-ACK before Step 1 overlaps with K HP SRs, - LP HARQ-ACK is dropped regardless the PUCCH carrying positive or negative HP SR.

If we want to avoid the harm to LP HARQ-ACK as possible, then the HP UCI multiplexing with LP HARQ-ACK should be considered. But the current Option 2 seems not accurate for the multiplexing as the legacy multiplexing between HP SR and HP HARQ-ACK may not make a real 1-bit HP SR information. In some cases, the HP SR is implicitly indicated. And in other case that if HP SR PUCCH on PF0 overlapping with HP HARQ-ACK on PF1, there is no SR indication.

The resultant PUCCH resource format should be determined based on the total number of LP and HP HARQ-ACK bits.

If the total number of LP and HP HARQ-ACK bits is 2, the multiplexing follows the previous agreement for multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, i.e., treat the two bits as HARQ-ACK bits with high priority and the resultant PUCCH format is 0/1.

If the total number of LP and HP HARQ-ACK bits is larger than 2, 1-bit HP SR is appended to HP HARQ-ACK bits. The number of HP UCI bits is OUCI=OACK,1+1. The resultant PUCCH resource for multiplexing HP SR + HP HARQ-ACK + LP HARQ-ACK is either of PF2, PF3, or PF4. One exception is the resultant PUCCH after multiplexing between HP SR and HP HARQ-ACK uses the PUCCH resource with n1PUCCH-AN or SR resource with PF0/1, in this special case, the PUCCH resource can’t afford the overhead of total number of LP and HP HARQ-ACK bits, then the LP HARQ-ACK is dropped.

The Option 2 can be revised to:

When a PUCCH carrying HP SR and HP HARQ-ACK with PUCCH format 0/1 overlaps with a PUCCH carrying LP HARQ-ACK, where the original PUCCH carrying the HP HARQ-ACK before Step 1 overlaps with K HP SRs, - If the total number of LP and HP HARQ-ACK bits is 2, the multiplexing follows the previous agreement for multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, i.e., treat the two bits as HARQ-ACK bits with high priority and the resultant PUCCH format is 0/1. - If the total number of LP and HP HARQ-ACK bits is larger than 2, - If the resultant PUCCH after multiplexing between HP SR and HP HARQ-ACK doesn't use the PUCCH resource with n1PUCCH-AN or SR resource with PF0/1, 1-bit HP SR is appended to HP HARQ-ACK bits. The number of HP UCI bits is OUCI=OACK,1+1. The resultant PUCCH resource for multiplexing HP SR + HP HARQ-ACK + LP HARQ-ACK is either of PF2, PF3, or PF4. - Otherwise, LP HARQ-ACK is dropped.

Either the clean dropping or the multiplexing rule above can work, but the multiplexing to protect the LP HARQ-ACK is more preferable.

Proposal 1: When a PUCCH carrying HP SR and HP HARQ-ACK with PUCCH format 0/1 overlaps with a PUCCH carrying LP HARQ-ACK, where the original PUCCH carrying the HP HARQ-ACK before Step 1 overlaps with K HP SRs,

  - If the total number of LP and HP HARQ-ACK bits is 2, the multiplexing follows the previous agreement for multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, i.e., treat the two bits as HARQ-ACK bits with high priority and the resultant PUCCH format is 0/1.

  - If the total number of LP and HP HARQ-ACK bits is larger than 2,

    - If the resultant PUCCH after multiplexing between HP SR and HP HARQ-ACK doesn't use the PUCCH resource with n1PUCCH-AN or SR resource with PF0/1, 1-bit HP SR is appended to HP HARQ-ACK bits. The number of HP UCI bits is OUCI=OACK,1+1. The resultant PUCCH resource for multiplexing HP SR + HP HARQ-ACK + LP HARQ-ACK is either of PF2, PF3, or PF4.

    - Otherwise, LP HARQ-ACK is dropped.

- Multiplexing UCIs of different priorities in a PUSCH

  - Handling of LP/HP HARQ-ACK on HP/LP PUSCH with CG-UCI

In RAN1#108-e meeting, there were two agreements about the LP/HP HARQ-ACK only on HP/LP PUSCH without CG-UCI, but for the PUSCH with CG-UCI, it is not clear and for further study.

Agreement If HP HARQ-ACK without LP HARQ-ACK would be transmitted on LP PUSCH, the HP HARQ-ACK should be multiplexed on the LP PUSCH by reusing the rate matching/puncturing and RE mapping for the legacy HARQ-ACK. Agreement If LP HARQ-ACK without HP HARQ-ACK would be transmitted on HP PUSCH, down-select from the options: - Option 2: UE follows the same behaviour as that in case of PUSCH with HP HARQ-ACK assuming two bits - FFS for CG-UCI PUSCH.

The FFS issue for CG-UCI PUSCH is not only including LP HARQ-ACK only or LP + HP HARQ-ACK on HP PUSCH with CG-UCI, but also including HP HARQ-ACK only or LP + HP HARQ-ACK on LP PUSCH with CG-UCI.

The HARQ-ACK multiplex with the PUSCH with CG UCI can be clearly separated to the below four individual cases:

- LP HARQ-ACK only multiplexes on HP PUSCH with CG-UCI;

- LP HARQ-ACK + HP HARQ-ACK multiplex on HP PUSCH with CG-UCI;

- HP HARQ-ACK only multiplex on LP PUSCH with CG-UCI;

- LP HARQ-ACK + HP HARQ-ACK multiplex on LP PUSCH with CG-UCI.

CG-UCI is the configured grant UCI, which normally includes the HARQ ID、RV ID or COT information for the PUSCH to be transmitted. If there is no other UCI except CG-UCI will be multiplexed into the PUSCH, the CG-UCI is multiplexed in the PUSCH which the information in CG-UCI is corresponding to this PUSCH. The CG-UCI is treated as HARQ to multiplex on the corresponding PUSCH. The physical priority of CG-UCI shares the same physical priority of PUSCH which carries the CG-UCI.

But if other UCI like LP HARQ-ACK and/or HP HARQ-ACK are involved in the multiplexing on the PUSCH with the CG-UCI due to the overlapping or collision between the HP/LP PUCCH with HARQ-ACK and the PUSCH with CG-UCI when the inter-priority multiplexing function is enabled, generally the possible solution is, the CG-UCI could be concatenated with the HARQ-ACK UCI with the same priority firstly, and then the resultant concatenated UCI  should be treated as HARQ-ACK information to be multiplexed on the PUSCH as legacy way.

The solutions for the four individual cases above are illustrated:

- LP HARQ-ACK only multiplexes on HP PUSCH with CG-UCI: CG-UCI shares the same high priority with PUSCH, and it can’t concatenated with LP HARQ-ACK. Then the CG-UCI is treated as HP HARQ-ACK, the multiplexing will follow the rule that HP HARQ-ACK + LP HARQ-ACK on HP PUSCH.

- LP HARQ-ACK + HP HARQ-ACK multiplex on HP PUSCH with CG-UCI: CG-UCI shares the same high priority with PUSCH, and it is concatenated with the HP HARQ-ACK firstly. The resultant after concatenation will be treated as HP HARQ-ACK. The multiplexing will follow the rule that HP HARQ-ACK + LP HARQ-ACK on HP PUSCH.

- HP HARQ-ACK only multiplex on LP PUSCH with CG-UCI: CG-UCI shares the same low priority with PUSCH, and it can’t concatenated with HP HARQ-ACK. Then the CG-UCI is treated as LP HARQ-ACK, the multiplexing will follow the rule that HP HARQ-ACK + LP HARQ-ACK on HP PUSCH.

- LP HARQ-ACK + HP HARQ-ACK multiplex on LP PUSCH with CG-UCI: CG-UCI shares the same low priority with PUSCH, and it is concatenated with LP HARQ-ACK firstly. The resultant after concatenation will be treated as LP HARQ-ACK. The multiplexing will follow the rule that HP HARQ-ACK + LP HARQ-ACK on HP PUSCH.

This way aligns the CG-UCI design principle and can reuse the multiplexing rules of HP HARQ-ACK + LP HARQ-ACK on HP PUSCH. There is no big specification impact and easy for implementation.

Then we propose: For HARQ-ACK multiplex on the PUSCH with CG UCI, the CG-UCI could be concatenated with the HARQ-ACK with the same priority firstly, and then the resultant concatenated UCI should be treated as HARQ-ACK to be multiplexed on the PUSCH by legacy rules.

Proposal 2: For HARQ-ACK multiplex on the PUSCH with CG UCI, if the CG-UCI could be concatenated with the HARQ-ACK with the same priority, the resultant concatenated UCI should be treated as HARQ-ACK to be multiplexed on the PUSCH by legacy rules.

Proposal 3: For HARQ-ACK multiplex on the PUSCH with CG UCI, if the CG-UCI could not be concatenated with the HARQ-ACK with the same priority, the CG-UCI should be treated as HARQ-ACK to be multiplexed on the PUSCH by legacy rules.

  - Low-priority HARQ-ACK resource mapping in case insufficient resource

When the LP HARQ-ACK is multiplexed in HP PUSCH, sufficient resource may not be available for accommodating LP HARQ-ACK on HP PUSCH since the HP UCI on HP PUSCH should not be affected as possible. There are several alternatives to handle the LP HARQ-ACK when the LP HARQ-ACK tries to multiplex in HP PUSCH when no sufficient resource is available for accommodating LP HARQ-ACK on HP PUSCH.

- LP HARQ-ACK is entirely dropped;

- LP HARQ-ACKs are mapped to the rest REs of the PUSCH based on the rate matching equation, if HP HARQ-ACK and/or HP CSI have been mapped in prior on the PUSCH;

- LP UCI compression;

- UE does not expect insufficient resource for multiplexing low-priority HARQ-ACK.

For each alternatives, the detail analysis are provided.

- LP HARQ-ACK is entirely dropped

This is the easy way, but it causes the unnecessary loss of LP HARQ-ACK. For the intention of R17 Intra-UE multiplexing, the LP HARQ-ACK should be protected as possible as we can.

Even if the LP HARQ-ACK is entirely dropped, whether the leftover REs for LP HARQ-ACK can be reallocated to the HP UCI or PUSCH? From the spectrum efficiency aspect, the leftover REs for the dropped LP HARQ-ACK should be reallocated to the HP UCI or HP PUSCH. For the simplicity of UE, it doesn’t need UE to come back to the RE occupation calculation again, the easier way is that the REs for LP HARQ-ACK are absorbed by data payload of UL-SCH.

- LP HARQ-ACK are mapped to the rest REs of the PUSCH

This is the most straightforward way and can be considered as a natural partially LP UCI dropping. The UE just drop some UCI bits depending on the leftover resource. gNB will try to recover the broken HARQ information. From efficiency aspect, it is better than the operation in Rel-16, where the LP HARQ-ACK is totally dropped. This is also a unified solution for both multiplexing on PUSCH and on PUCCH and no more specification effort is needed.

- LP UCI compression

Another solution is LP UCI compression. For example, HARQ-ACK bundling is widely used to reduce the HARQ-ACK codebook overhead in LTE and NR. After bundling, if the NACK is transmitted, the network still does not know the exact HARQ-ACK for each PDSCH. This situation is the same as the partial LP UCI dropping. However, if ACK is obtained by bundling, it means all the corresponding PDSCH is ACK and the network can get the exact feedback for each PDSCH. Since the possibility of ACK is much higher than NACK in practice, it can further avoid the unnecessary retransmission, which can further improve the resource efficiency compared to partial LP UCI dropping. But LP UCI compression need some specification effort.

- UE does not expect insufficient resource for multiplexing low-priority HARQ-ACK

This alternative is too restricted to gNB, gNB can’t make sure there is sufficient resource for multiplexing low-priority HARQ-ACK.

For the tradeoff between the specification effort and the performance protection of LP HARQ-ACK, we prefer that LP HARQ-ACK are mapped to the rest REs of the PUSCH.

Proposal 4: LP HARQ-ACKs are mapped to the rest REs of the PUSCH based on the rate matching equation, if HP HARQ-ACK and/or HP CSI have been mapped in prior on the PUSCH.

- PHY prioritization between DG and CG PUSCHs with different priorities

There are a few agreements on the PHY prioritization between DG and CG PUSCHs with different priorities.

Agreement For collision between HP CG PUSCH and LP DG PUSCH, if MAC delivers two MAC PDUs to PHY, PHY layer can make the prioritization so that the UE is expected to transmit the CG PUSCH and cancel the DG PUSCH at latest from the first symbol that is overlapping with the CG PUSCH. - Note: For the DG PUSCH, it is up to UE implementation to handle the OFDM symbols of the DG PUSCH before the start of HP CG PUSCH which are nonoverlapping with the HP CG PUSCH. - FFS: How to handle the collision when there is repetition for CG and/or DG PUSCH Agreement For collision of HP DG-PUSCH and LP CG-PUSCH, the cancellation is applied per actual repetition, if HP DG-PUSCH and/or LP CG-PUSCH is repeated. Agreement For collision of LP DG-PUSCH and HP CG-PUSCH of different priorities, the cancellation is applied per actual repetition, if LP DG-PUSCH and/or HP CG-PUSCH is repeated. Agreement For the overlapping between LP CG and HP DG, if MAC delivers two MAC PDUs to PHY, PHY layer can make the prioritization so that the UE is expected to cancel the overlapping low priority CG PUSCH by the first overlapping symbol at the latest. - On top of Rel-16 cancellation time (N2+d1) for PUCCH/PUCCH or PUCCH/PUSCH collision, additional time d3 is needed (which results N2+d1+d3 in total cancellation time) for LP CG-PUSCH and HP DG-PUSCH collision resolution. - (Working assumption) d3 = {0, 1,…,2μ+1}symbol(s) upon UE capability report, where μ=0,1,2,3 for SCS=15/30/60/120kHz, respectively. Agreement The following working assumption is confirmed For the overlapping between LP CG and HP DG, if MAC delivers two MAC PDUs to PHY, PHY layer can make the prioritization so that the UE is expected to cancel the overlapping low priority CG PUSCH by the first overlapping symbol at the latest. - On top of Rel-16 cancellation time (N2+d1) for PUCCH/PUCCH or PUCCH/PUSCH collision, additional time d3 is needed (which results N2+d1+d3 in total cancellation time) for LP CG-PUSCH and HP DG-PUSCH collision resolution. - d3 = {0, 1,…,2μ+1}symbol(s) upon UE capability report, where μ=0,1,2,3 for SCS=15/30/60/120kHz, respectively.

It is allowed MAC can deliver two MAC PDUs to PHY for collision between CG PUSCH and DG PUSCH with different priorities, and PHY layer can make the prioritization to cancel the overlapping low priority PUSCH. It seems the Rel-16 UE behaviour handling about overlapping between CG PUSCH and DG PUSCH should exclude the case that the CG PUSCH and DG PUSCH have the different priority index and the UE is provided prioritizationBetweenLP-DG-PUSCHandHP-CG-PUSCH or prioritizationBetweenHP-DG-PUSCHandLP-CG-PUSCH.

So a TP for 38.214-h10 is proposed to update the section 6.1.

Proposal 5: A TP for 38.214 is proposed to update the section 6.1 on collision between CG PUSCH and DG PUSCH with different priorities.

Reason for change: In Rel-17, when DG PUSCH overlaps with CG PUSCH with different priority, the UE can cancel the PUSCH with lower priority and then transmit the PUSCH with higher priority. The timeline defined in Rel-15 is not applicable to this case.

Summary of the change: The DG PUSCH overlapping with CG PUSCH with different priority in Rel-17 is excluded.

Consequences if not approved: The UE may determine the Rel-17 overlapping is error case if the timeline is not satisfied.

# 6.1 UE procedure for transmitting the physical uplink shared channel < Unchanged part is omitted > A UE is not expected to be scheduled by a PDCCH ending in symbol i to transmit a first PUSCH on a given serving cell overlapping in time with a transmission occasion, where the UE is allowed to transmit a second PUSCH with configured grant according to [10, TS38.321], starting in a symbol j on the same serving cell if the end of symbol i is not at least N2 symbols before the beginning of symbol j unless the first PUSCH and the second PUSCH have the different priority index [9, TS38.213] and the UE is provided prioritizationBetweenLP-DG-PUSCHandHP-CG-PUSCH or prioritizationBetweenHP-DG-PUSCHandLP-CG-PUSCH. The value N2 in symbols is determined according to the UE processing capability defined in Clause 6.4, and N2 and the symbol duration are based on the minimum of the subcarrier spacing corresponding to the PUSCH with configured grant and the subcarrier spacing of the PDCCH scheduling the PUSCH. < Unchanged part is omitted >

- Conclusion

According to the analysis given above, we have the following proposals:

Proposal 1: When a PUCCH carrying HP SR and HP HARQ-ACK with PUCCH format 0/1 overlaps with a PUCCH carrying LP HARQ-ACK, where the original PUCCH carrying the HP HARQ-ACK before Step 1 overlaps with K HP SRs,

  - If the total number of LP and HP HARQ-ACK bits is 2, the multiplexing follows the previous agreement for multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, i.e., treat the two bits as HARQ-ACK bits with high priority and the resultant PUCCH format is 0/1.

  - If the total number of LP and HP HARQ-ACK bits is larger than 2,

    - If the resultant PUCCH after multiplexing between HP SR and HP HARQ-ACK doesn't use the PUCCH resource with n1PUCCH-AN or SR resource with PF0/1, 1-bit HP SR is appended to HP HARQ-ACK bits. The number of HP UCI bits is OUCI=OACK,1+1. The resultant PUCCH resource for multiplexing HP SR + HP HARQ-ACK + LP HARQ-ACK is either of PF2, PF3, or PF4.

    - Otherwise, LP HARQ-ACK is dropped.

Proposal 2: For HARQ-ACK multiplex on the PUSCH with CG UCI, if the CG-UCI could be concatenated with the HARQ-ACK with the same priority, the resultant concatenated UCI should be treated as HARQ-ACK to be multiplexed on the PUSCH by legacy rules.

Proposal 3: For HARQ-ACK multiplex on the PUSCH with CG UCI, if the CG-UCI could not be concatenated with the HARQ-ACK with the same priority, the CG-UCI should be treated as HARQ-ACK to be multiplexed on the PUSCH by legacy rules.

Proposal 4: LP HARQ-ACKs are mapped to the rest REs of the PUSCH based on the rate matching equation, if HP HARQ-ACK and/or HP CSI have been mapped in prior on the PUSCH.

Proposal 5: A TP for 38.214 is proposed to update the section 6.1 on collision between CG PUSCH and DG PUSCH with different priorities.

Reason for change: In Rel-17, when DG PUSCH overlaps with CG PUSCH with different priority, the UE can cancel the PUSCH with lower priority and then transmit the PUSCH with higher priority. The timeline defined in Rel-15 is not applicable to this case.

Summary of the change: The DG PUSCH overlapping with CG PUSCH with different priority in Rel-17 is excluded.

Consequences if not approved: The UE may determine the Rel-17 overlapping is error case if the timeline is not satisfied.

# 6.1 UE procedure for transmitting the physical uplink shared channel < Unchanged part is omitted > A UE is not expected to be scheduled by a PDCCH ending in symbol i to transmit a first PUSCH on a given serving cell overlapping in time with a transmission occasion, where the UE is allowed to transmit a second PUSCH with configured grant according to [10, TS38.321], starting in a symbol j on the same serving cell if the end of symbol i is not at least N2 symbols before the beginning of symbol j unless the first PUSCH and the second PUSCH have the different priority index [9, TS38.213] and the UE is provided prioritizationBetweenLP-DG-PUSCHandHP-CG-PUSCH or prioritizationBetweenHP-DG-PUSCHandLP-CG-PUSCH. The value N2 in symbols is determined according to the UE processing capability defined in Clause 6.4, and N2 and the symbol duration are based on the minimum of the subcarrier spacing corresponding to the PUSCH with configured grant and the subcarrier spacing of the PDCCH scheduling the PUSCH. < Unchanged part is omitted >

- Reference

- 3GPP, Chairman's Notes RAN1#108-e v25.