# 3GPP TSG RAN WG1 #109                                                  R1-2203193

# e-Meeting, May 9th – May 20th, 2022

Source: 	ZTE Corporation

Title: 	Discussion on remaining issues for coverage enhancements for PUCCH

Agenda item:	8.8.3

Document for:	Discussion/Decision

Introduction

For for PUCCH enhancements, most of the critical issues have been resolved. In this contribution, we provide our views on two remaining issues of enhancements for PUCCH.

Dynamic PUCCH repetition factor indication

Till now, there is still no explicit conclusion made on whether or not to support dynamic PUCCH repetition factor indication for HARQ-ACK for the first SPS PDSCH. Regarding the PUCCH resource for the first SPS PDSCH with a corresponding activation DCI, some clarifications were made in RAN1#108 e-meeting for Rel-15/16. It was clarified that the PDCCH with the activation DCI does not correspond to the 1st SPS-PDSCH. Thus, the PUCCH resource for the HARQ-ACK of the first SPS-PDSCH is not determined by the PRI in the activation DCI. It is determined by n1PUCCH-AN or sps-PUCCH-AN-List-r16 in RRC instead.

Based on the above clarifications, the dynamic PUCCH repetition factor indication mechanism agreed in RAN1 106e can not be applied to HARQ-ACK for the first SPS PDSCH associated with the activation DCI. Therefore, we propose:

Proposal 1: In NR Rel-17, for HARQ-ACK for SPS PDSCH, it is clarified that the dynamic PUCCH repetition factor indication mechanism agreed in RAN1 106e applies to HARQ-ACK corresponding to the SPS release DCI.

- Note: dynamic PUCCH repetition factor indication mechanism is not applied to HARQ-ACK for the first SPS PDSCH associated with the activation DCI.

- No specification impact is expected.

Frequency hopping design for PUCCH/PUSCH

Regarding the frequency hopping design for PUSCH, the following agreement were achieved during RAN1 108e. But how to capture these agreement into the spec is still pending. We think we should first align the understanding regarding the term “physical slot” used in the agreements.

Agreement - Inter-slot frequency hopping pattern for PUSCH repetitions with DMRS bundling is determined based on physical slot index

From our side, regarding the term “physical slot index”, there are at least three understandings:

- Alt-1: the physical slot index is counted within a radio frame, and restarts at the frame boundary, which is shown in Figure 1(a);

- Alt-2: the physical slot index “0” starts from the radio frame where the PUSCH is scheduled and increases from then on, which is shown in Figure 1(b);

- Alt-3: the physical slot index “0” starts from the radio frame with SNF=0, and increases from then on, which is shown in Figure 1(c);

- Figure 1 (a) Alt-1

- Figure 1 (b) Alt-2

- Figure 1 (c) Alt-3

- Figure 1: illustration of the physical slot index counting.

For Alt-1, there could be some issues if the PUCCH/PUSCH repetitions crosses the frame boundary and the hopping interval is not configured appropriately, which will result in no frequency hopping or unequal hopping interval. Two examples are shown in Figure-2 to illustrate such issue. For the first example, the number of repetitions N = 24 and hopping interval H = 12 slots. For Alt-1, the hopping position has to re-start at each radio frame boundary, which ends up with no frequency hopping (i.e., always Hop#0). For the second example, due to similar reason, the hopping interval would have different lengths, which may degrade the performance.

Figure-2. Illustration of hopping pattern for physical slot index based solution

The issue of Alt-1 that no frequency hopping is applied when the hopping interval is configured larger than the number of slots within a frame does not exists for Alt-2 and Alt-3. The main advantage of Alt-3 is that if the hopping interval is configured the same among different UEs, same hopping pattern will be achieved for those UEs. It is beneficial for the gNB to schedule those UEs to multiplex on the same frequency hopping RB pairs. But the actual performance advantage of Alt-3 over Alt-2 in practical scenarios is not justified. So in this point of view, both Alt-2 and Alt-3 seem OK for us.

Regarding the specification impact, for Alt-1, the hopping for PUSCH can be specified something like:

Frequency hopping pattern based on physical slot indices is realized as following. - The starting RB during slot ns is given by - RBstartns=RBstartns/NFH mod 2=0RBstart+RBoffset mod NBWPsizens/NFH mod 2=1 - ns is the current slot number within a radio frame - RBstart is the starting RB within the UL BWP as calculated from the resource block assignment information - RBoffset is the frequency offset in RBs between the two frequency hops - NFH is the length of hopping interval

For Alt-2, the hopping for PUSCH will be specified something like:

Frequency hopping pattern based on physical slot indices is realized as following. - The starting RB during slot ns is given by - RBstartns=RBstartns/NFH mod 2=0RBstart+RBoffset mod NBWPsizens/NFH mod 2=1 - ns is the slot number within a radio frame where the first PUSCH is transmitted and each subsequent slot increasing accordingly regardless of whether or not the UE transmits the PUSCH in the slot and whether or not the slot is across a different radio frame - RBstart is the starting RB within the UL BWP as calculated from the resource block assignment information - RBoffset is the frequency offset in RBs between the two frequency hops - NFH is the length of hopping interval

For Alt-3, the hopping for PUSCH will be specified something like:

Frequency hopping pattern based on physical slot indices is realized as following. - The starting RB during slot ns,fμ of radio frame nf, is given by - RBstartnf,ns,fμ=RBstart, &(Nslotframe,μnf+ns,fμ)/H mod 2=0RBstart+RBoffset mod NBWPsize, &(Nslotframe,μnf+ns,fμ)/H mod 2=1 - nf is the radio frame index, - ns,fμ is the current slot number within a radio frame nf, - RBstart is the starting RB within the UL BWP as calculated from the resource block assignment information - RBoffset is the frequency offset in RBs between the two frequency hops - NFH is the length of hopping interval

Proposal 2: RAN1 needs to down select the interpretation of “physical slot index” from one of the following alternatives.

- Alt-1: the physical slot index is counted within a radio frame, and restarts at the frame boundary.

- Alt-2: the physical slot index “0” starts from the radio frame where the first PUSCH is scheduled and increases from then on.

- Alt-3: the physical slot index “0” starts from the radio frame with SNF=0, and increases from then on.

Conclusion

According to the analysis given above, we have the following proposals:

Proposal 1:  In NR Rel-17, for HARQ-ACK for SPS PDSCH, it is clarified that the dynamic PUCCH repetition factor indication mechanism agreed in RAN1 106e applies to HARQ-ACK corresponding to the SPS release DCI.

- Note: dynamic PUCCH repetition factor indication mechanism is not applied to HARQ-ACK for the first SPS PDSCH associated with the activation DCI.

- No specification impact is expected.

Proposal 2: RAN1 needs to down select the interpretation of “physical slot index” from one of the following alternatives.

- Alt-1: the physical slot index is counted within a radio frame, and restarts at the frame boundary.

- Alt-2: the physical slot index “0” starts from the radio frame where the first PUSCH is scheduled and increases from then on.

- Alt-3: the physical slot index “0” starts from the radio frame with SNF=0, and increases from then on.

Reference

3GPP RAN1#108-e, R1-2202833, [108-e-NR-CRs-02] Issue#3 SPS PDSCH activation and PUCCH resource selection for the 1st SPS PDSCH. Moderator (Nokia)

3GPP RAN1 #108e, R1-2101167, Discussion on remaining issues for coverage enhancements for PUCCH, ZTE