3GPP TSG RAN WG1 Meeting #109-e	R1-2203195

e-Meeting, May 9th – 20th, 2022

Source: 	ZTE

Title: 	Maintenance of other issues for broadcast and multicast

Agenda item:	8.12.2

Document for:	Discussion/Decision

# Introduction

Up to RAN1#108 e-meeting, the critical issues for group scheduling mechanism for RRC_CONNECTED UEs and functions for broadcast or multicast for RRC_IDLE or RRC_INACTIVE UEs have been resolved. In this contribution, we provide our views on the remaining issues.

# Multicast

## 2.1	FDMed SPS PDSCH reception

During RAN1#108-e, how to support FDMed SPS PDSCH reception was discussed without consensus. The following alternatives were proposed during the discussion.

Updated proposal 5-2a (v1): If a UE supports FDM reception between unicast SPS PDSCH and multicast SPS PDSCH in a slot, and if more than one PDSCH on a serving cell each without a corresponding PDCCH transmission are in a slot, - if the PDSCHs include both unicast SPS PDSCH(s) and multicast SPS PDSCH(s), - Alt1: - the UE resolves collisions among unicast SPS PDSCHs resulting in one unicast SPS PDSCH and collisions among multicast SPS PDSCHs resulting in one multicast SPS PDSCH as in Rel-16, respectively. If the resulting unicast SPS PDSCH and multicast SPS PDSCH overlap in both time and frequency, the UE receives the one with lower configured sps-ConfigIndex; else, the UE receives both PDSCHs. - Alt2 (reuse the current procedure in TS38.214h00 as much as possible): - Step 0: set j=0, where j is the number of selected PDSCH(s) for decoding. Q is the set of activated PDSCHs without corresponding PDCCH transmissions within the slot - Step 1: A UE receives one PDSCH with the lowest configured sps-ConfigIndex within Q, set j=j+1. Designate the received PDSCH as survivor PDSCH. - Step 2: The survivor PDSCH in step 1 and any other PDSCH(s) overlapping in both time and frequency with the survivor PDSCH in step 1 are excluded from Q. - Step 3: Repeat step 1 and 2 until Q is empty or j is equal to 2 - Alt3 (based on vivo’s suggestion in the 1st round): - Step 0: Q is the set of activated PDSCHs without corresponding PDCCH transmissions within the slot. - Step 1: A UE receives one PDSCH with the lowest configured sps-ConfigIndex within Q. Designate the received PDSCH as survivor PDSCH. - Step 2: If the survivor PDSCH in step 1 is unicast PDSCH, a UE receives one multicast PDSCH with the lowest configured sps-ConfigIndex within Q (if any), where the multicast PDSCH and the survivor PDSCH in step 1 are FDMed in frequency. If the survivor PDSCH in step 1 is multicast PDSCH, a UE receives one unicast PDSCH with the lowest configured sps-ConfigIndex within Q (if any), where the unicast PDSCH and the survivor PDSCH in step 1 are FDMed in frequency. - if the PDSCHs only include unicast SPS PDSCH(s) or only include multicast SPS PDSCH(s), the legacy procedure is applied. - FFS: whether a separate UE capability is needed for FDM reception between unicast SPS PDSCH and multicast SPS PDSCH in a slot. - FFS for DG unicast PDSCHs and/or DG multicast PDSCHs

One key question is that which of following cases in Figure-1 can be supported by a UE, which reports capability of FDMed reception of unicast/multicast PDSCH in a slot.

Case 1 should be supported undoubtedly. Regarding Case 2, a lower capability is required comparing with that of Case 1. So it can also be supported. The main divergence is whether Case 3 should also be supported by this type of UE. From our opinion, it is weird for excluding case 3. Since UE is capable of receiving Case 1 (FDM) and Case 2 (FDM+TDM) already. And Case 3 is just a legacy TDM case and should be also supported.

Figure-1: Cases of SPS PDSCH reception

Proposal 1: If a UE reports capability of FDMed reception of unicast/multicast PDSCH in a slot, it can at most receive one multicast SPS PDSCH and one unicast SPS PDSCH no mather they are FMDed or TDMed.

Following the above rule, Alt 1 is a simplest way for supporting FDMed reception between a multicast SPS PDSCH and a unicast SPS PDSCH.

Regarding Alt 2, it may lead an error case that the survivor PDSCHs are both unicast and FDMed with each other. While for Alt 3, both of Case 2 and Case 3 will be excluded, which brings unnecessary restriction to scheduling.

Proposal 2: If a UE supports FDM reception between unicast SPS PDSCH and multicast SPS PDSCH in a slot, and if more than one PDSCH on a serving cell each without a corresponding PDCCH transmission are in a slot, Alt1 is support, i.e.,

- if the PDSCHs include both unicast SPS PDSCH(s) and multicast SPS PDSCH(s),

  - the UE resolves collisions among unicast SPS PDSCHs resulting in one unicast SPS PDSCH and collisions among multicast SPS PDSCHs resulting in one multicast SPS PDSCH as in Rel-16, respectively. If the resulting unicast SPS PDSCH and multicast SPS PDSCH overlap in both time and frequency, the UE receives the one with lower configured sps-ConfigIndex; else, the UE receives both PDSCHs.

## 2.2	HARQ process management

Regarding HARQ process management between unicast and multicast, whether to support dynamic HARQ process sharing have been discussed in several RAN1 e-meetings, but companies’ views are still not converged. The main issues of supporting dynamic HARQ process sharing include: 1) NDI conflict issue and 2) the differentiation of PTP (Re)Tx for unicast and PTP ReTx for multicastThese two issues have been discussed for several meetings with no progress.

From our perspective, if there is no consensus on how to solve the issues under dynamic HARQ process sharing. Semi-static HARQ process sharing mechanism can be considered in Rel-17. And most issues can be avoided. For example, a value range of HARQ process for multicast is configured via RRC signalling or defined in the spec, e.g., {0-7}. Then, the HPN in DCI format with CRC scrambled by G-RNTI should be limited within the value range {0-7}. And for DCI format with CRC scrambled by C-RNTI, if the value of HPN field located within {0-7}, it indicates that the retransmission of a multicast TB is scheduled.

Proposal 3: If companies could NOT achieve consensus on how to address the 1) NDI conflict issue and 2) the differentiation of PTP (Re)Tx for unicast and PTP ReTx for multicast, NR MBS should support semi-static HARQ process sharing between unicast and multicast.

- A value range of HARQ process for multicast is configured via RRC signalling or defined in the spec, e.g., {0-7}.

- For a DCI format with CRC scrambled by G-RNTI, the value of HPN in the DCI format should be within the configured or defined value range.

- For a DCI format with CRC scrambled by C-RNTI, if the value of HPN is located within the configured or defined value range, it indicates that the retransmission of a multicast TB is scheduled by the DCI format.

# Broadcast

## 3.1	Monitoring capability of GC-PDCCH

During RAN1#108-e, the issue on PDCCH monitoring capability sharing among unicast and MBS was discussed. And the following agreement is made [2].

Agreement Adopt the following TP for Clause 10.1 in TS 38.213: - note: further clarification may be needed for the case of receiving broadcast, and MCCH-RNTI ----------------- Start of TP ---------------- 10.1 UE procedure for determining physical downlink control channel assignment <Unchanged text is omitted> For a scheduled cell and at any time, if a UE is provided a C-RNTI, athe UE expects to have received at most 16 PDCCHs for DCI formats with CRC scrambled by C-RNTI, CS-RNTI, G-RNTI, G-CS-RNTI or MCS-C-RNTI scheduling 16 PDSCH receptions for which the UE has not received any corresponding PDSCH symbol and at most 16 PDCCHs for DCI formats with CRC scrambled by C-RNTI, CS-RNTI, or MCS-C-RNTI scheduling 16 PUSCH transmissions for which the UE has not transmitted any corresponding PUSCH symbol. <Unchanged text is omitted> ----------------- End of TP ----------------

The divergence is how to deal with PDCCH monitoring for broadcast, including, MCCH PDCCH and MTCH PDCCH. From our point of view, broadcast PDCCH cannot be considered as a unicast PDCCH and sharing the monitoring capability with unicast PDCCH. The reason is that, as agreement [3] copied below, network will configure beam sweeping for MCCH/MTCH, and it is up to UE to receive one or more than one repetition. The network even doesn’t know how many PDCCHs that UE will monitor for MCCH/MTCH in each periodicity. So it should also be a UE implementation on how to guarantee the PDCCH monitoring budget. For example, MCCH/MTCH PDCCH will be dropped if the number of PDCCH has reached monitoring budget.

Agreement: For RRC_IDLE/RRC_INACTIVE UEs, for broadcast reception, the UE may assume that group-common PDCCH/PDSCH is QCL’d with SSB. - It is up to UE implementation whether UE monitors monitoring occasions corresponding to all SSB indexes or monitoring occasions corresponding to a subset of all SSB indexes. - FFS: association rules between SSB indexes and UE monitoring occasions. - FFS: group-common PDCCH/PDSCH is QCl’d with TRS if configured

According to the above analysis, we have the following proposal.

Proposal 4: For RRC_CONNECTED UEs, a MCCH/MTCH PDCCH cannot be considered as a unicast PDCCH and it should be a UE implementation on how to guarantee the PDCCH monitoring budget. For example, MCCH/MTCH PDCCH will be dropped if the number of PDCCH has reached monitoring budget.

## 3.2	HARQ process management for MCCH and MTCH

During RAN2#117-e, HARQ process management for MCCH and MTCH was discussed by RAN2 with the following agreement. And RAN2 sent a LS on this in [4].

There are no dedicated HARQ process IDs for MCCH and Broadcast MTCH (assumption: single HARQ process for MCCH and single HARQ process for MTCH, not clear whether they can share the same, details would be RAN1 scope)

Regarding reception of MCCH and Broadcast MTCH(s), it is similar as reception of SIB1 and OSI, in which a dedicated HARQ process is shared among them by assuming that there is no requirement for receiving them simultaneously. Therefore, from our perspective, a same HARQ process can also be shared for reception of MCCH and Broadcast MTCH(s).

Proposal 5: Reply RAN2 LS [4]	R1-2203044/R2-2204017 with the following response [5].

- RAN1 confirms the RAN2 assumption on single HARQ process for MCCH and single HARQ process for Broadcast MTCH(s).

- Further, a same HARQ process can be shared for reception of MCCH and Broadcast MTCH(s) from RAN1 perspective. It is similar as reception of SIB1 and OSI, in which a dedicated HARQ process is shared among them by assuming that there is no requirement for receiving them simultaneously.

# Conclusion

In this contribution, the discussion and analysis on remaining issues are presented. And the following proposals are also provided.

Multicast

Proposal 1: If a UE reports capability of FDMed reception of unicast/multicast PDSCH in a slot, it can at most receive one multicast SPS PDSCH and one unicast SPS PDSCH no mather they are FMDed or TDMed.

Proposal 2: If a UE supports FDM reception between unicast SPS PDSCH and multicast SPS PDSCH in a slot, and if more than one PDSCH on a serving cell each without a corresponding PDCCH transmission are in a slot, Alt1 is support, i.e.,

- if the PDSCHs include both unicast SPS PDSCH(s) and multicast SPS PDSCH(s),

  - the UE resolves collisions among unicast SPS PDSCHs resulting in one unicast SPS PDSCH and collisions among multicast SPS PDSCHs resulting in one multicast SPS PDSCH as in Rel-16, respectively. If the resulting unicast SPS PDSCH and multicast SPS PDSCH overlap in both time and frequency, the UE receives the one with lower configured sps-ConfigIndex; else, the UE receives both PDSCHs.

Proposal 3: If companies could NOT achieve consensus on how to address the 1) NDI conflict issue and 2) the differentiation of PTP (Re)Tx for unicast and PTP ReTx for multicast, NR MBS should support semi-static HARQ process sharing between unicast and multicast.

- A value range of HARQ process for multicast is configured via RRC signalling or defined in the spec, e.g., {0-7}.

- For a DCI format with CRC scrambled by G-RNTI, the value of HPN in the DCI format should be within the configured or defined value range.

- For a DCI format with CRC scrambled by C-RNTI, if the value of HPN is located within the configured or defined value range, it indicates that the retransmission of a multicast TB is scheduled by the DCI format.

Broadcast

Proposal 4: For RRC_CONNECTED UEs, a MCCH/MTCH PDCCH cannot be considered as a unicast PDCCH and it should be a UE implementation on how to guarantee the PDCCH monitoring budget. For example, MCCH/MTCH PDCCH will be dropped if the number of PDCCH has reached monitoring budget.

Proposal 5: Reply RAN2 LS [4]	R1-2203044/R2-2204017 with the following response [5].

- RAN1 confirms the RAN2 assumption on single HARQ process for MCCH and single HARQ process for Broadcast MTCH(s).

- Further, a same HARQ process can be shared for reception of MCCH and Broadcasdt MTCH(s) from RAN1 perspective. It is similar as reception of SIB1 and OSI, in which a dedicated HARQ process is shared among them by assuming that there is no requirement for receiving them simultaneously.

# Reference

RP-201038, WID revision: NR Multicast and Broadcast Services, RAN#88e.

3GPP RAN1#108-e, Chairman’s notes.

3GPP RAN1#104-e, Chairman’s notes.

R1-2203044/R2-2204017, LS on HARQ process for MCCH and Broadcast MTCH(s), RAN2.

R1-220xxxx, [Draft] Reply LS on HARQ process for MCCH and Broadcast MTCH(s), ZTE.