3GPP TSG RAN WG1 #109-e	R1-2203196

e-Meeting, May 9th – 20th, 2022

Source: 	ZTE

Title: 	Maintenance of DSS and MR-DC

Agenda item:	8.13

Document for:	Discussion/Decision

Introduction

During RAN#92e meeting, a revised work item(WI) on DSS (dynamic spectrum sharing) was approved[1]. The objective of this WI is to support PDCCH of SCell scheduling PDSCH or PUSCH on P(S)Cell. During RAN#88e meeting, a revised WI on enhancements of MR-DC related scenarios was approved [2]. For this WI, RAN1’s work is to specify efficient activation/de-activation mechanism for SCells based on RAN1 leading mechanism. From RAN1#103-e, the progress were made for DSS and MR-DC [3-10].

In this contribution, we provide our analysis on the remaining issues on cross-carrier scheduling from SCell to P(S)Cell and fast SCell activation.

DSS

FFS other s1 and s2

In Rel-16 CA, there is only one scheduling cell for a scheduled cell. And the BD/CCE limit on the scheduling cell for the scheduled cell is determined based on the numerology of the scheduling Cell. In Rel-17 DSS with support of SCell scheduling PCell, there are two scheduling cells (PCell and sSCell) for a scheduled cell (PCell). The remaining issue on BD/CCE handling is whether introduce other s1 and s2 besides of (s1, s2) = (1, 0). Agreement in RAN1#106b-e meeting and RAN1 #107-e meeting are listed below.

[RAN1#106b-e] Agreement: Option A is supported in Rel-17 - At least for Type B UE, when the UE is configured for CCS from sSCell to P(S)Cell and when P(S)Cell SCS (μ) is less than or equal to sSCell SCS (μ1),[and at least when UE is not provided monitoringCapabilityConfig for any cell, ] - Option A - On P(S)Cell (for self-scheduling) - UE is not required to monitor more than α*minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ PDCCH BD candidates per P(S)Cell slot - On sSCell (for cross-carrier scheduling to P(S)Cell) - UE is not required to monitor more than [minMPDCCHmax,slot,μ1,MPDCCHtotal,slot,μ1 or MPDCCHmax,slot,μ1] PDCCH BD candidates per sSCell slot - UE is additionally not required to monitor more than (1-α)*minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ PDCCH BD candidates per P(S)Cell slot - 0≤α≤1 is based on RRC configuration - α*minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ is used for P(S)Cell overbooking procedure - When determining MPDCCHtotal,slot,μ and MPDCCHtotal,slot,μ1 - P(S)Cell self-scheduling is counted by applying scaling factor s1 - sSCell to P(S)Cell scheduling is counted additionally (assuming SCS of sSCell) by applying scaling factor s2 - s1=1 and s2=0, FFS other s1 and s2 - 0≤s1≤1 and 0≤s2≤1 are based on RRC configuration - FFS: additional constraints on s1 and s2 e.g., 1 ≤ s1+s2 ≤ 2 or s1 + s2 ≤ 1 - Note: Ncellscap is as in Rel16 - UE capability/incapability indication for below to be discussed as part of UE features discussion - All search space configurations monitored on sSCell for cross-carrier scheduling to P(S)Cell are within a single span of [3] consecutive OFDM symbols within a duration spanning P(S)Cell slot - Same approach as above is used for CCE limits - FFS: Separate vs. same RRC configured scaling factors (corresponding to α) for BD and CCE limits. - When P(S)Cell SCS (μ) is larger than sSCell SCS (μ1), for CCS from sSCell to P(S)Cell and, it is not supported Rel-17 DSS.

[RAN1#107b-e] Agreement: If no additional set of (s1, s2) is introduced, - For Option A BD/CCE limit handling and (s1=1, s2=0) agreed in RAN1#106bis-e, down-select from - Option 2 - On sSCell (for cross-carrier scheduling to P(S)Cell) - UE is not required to monitor more than MPDCCHmax,slot,μ1 PDCCH BD candidates per sSCell slot

If a UE is configured with  downlink cells (Note, assume without M-TRP, that is = and =0, and also assume without span based monitoring) with DL BWPs of the scheduling cells using SCS configuration , where , a DL BWP of an activated cell is the active DL BWP of the activated cell, and a DL BWP of a deactivated cell is the DL BWP with index provided by firstActiveDownlinkBWP-Id for the deactivated cell, the UE is not required to monitor more than  PDCCH candidates or more than  non-overlapped CCEs per slot on the active DL BWP(s) of scheduling cell(s) from the  downlink cells.

In case that PDCCH of SCell scheduling PDSCH or PUSCH on P(S)Cell is supported, PDCCH blind decoding will be performed on two cells for this P(S)Cell. The issue is focus on how to calculate the number of P(S)Cell in parameter . For example, assume 5 cells are configured for a UE shown in Figure 1, wherein the SCS = 15kHz for PCell #1 and the SCS = 30kHz for sSCell #2. In Rel-15/Rel-16 case PCell cannot be scheduled by sSCell (the red arrow in Figure 1 is not presented), M_total_15khz = floor(4*44*1/5) = 35; M_total_30khz = floor(4*36*4/5) = 115. In case PCell can be scheduled by sSCell (the red arrow in Figure 1 is presented), PCell is counted as s1=1 cell and s2=0 cell for the two scheduling cells respectively, which is corresponding to the case 1 analyzed below.

Figure 1 CA with SCell-schedule-PCell

- Case 1: s1=1 and s2=0

      - The number of PCell is still counted as 1 only for PCell. This is aligned with the intention of the guidance proposed in RAN1#105-e that the M_total calculation is the same as in Rel-16. That is, M_total_15khz = floor(4*44*1/5) = 35;  M_total_30khz = floor(4*36*4/5) = 115.

      - In this case, the definition of M_total in the specification has been reflected that PCell is only counted into the cell number of M_total_μp, where μp is the numerology for PCell; and is not counted into the cell number of M_total_μs, where μs is the numerology for sSCell. To be more specific, the BD for each scheduling are listed in Table 1. This is aligned with Option A and the agreement in RAN1 #107-e which the total BD per PCell slot for p-p scheduling and s-p scheduling is same as that for PCell in Rel-16.

      - Table 1 BD for Rel-17 DSS and the BD budget is not changed compared with Rel-16

| - Scheduling | - PDCCH BDs |
| --- | --- |
| - P1->P1 | - b1<=α*35 per 1ms |
| - S2->S2 | - b2<=36 per 0.5ms such that b2+b3+b4+b5 <= 115 |
| - S2->S3 | - b3<=36 per 0.5ms such that b2+b3+b4+b5 <= 115 |
| - S2->S4 | - b4<=36 per 0.5ms such that b2+b3+b4+b5 <= 115 |
| - S2->S5 | - b5<=36 per 0.5ms such that b2+b3+b4+b5 <= 115 |
| - S2->P1 | - b6<=36 per 0.5ms; - 2*b6<= (1-α)*35 per 1ms |
|  | - total = (115 @30kHz) +35 @15kHz |

- Case 2: Other s1 and s2 values

      - The number of PCell is counted as s1 and s2 for the two scheduling cells (PCell and sSCell) respectively. For example, s1 = s2 = 0.5, M_total_15khz = floor(4*44*0.5/5) = 17; M_total_30khz = floor(4*36*4.5/5) = 129. For another example, s1 = 1 and s2 = 1, M_total_15khz = floor(4*44*1/6) = 29; M_total_30khz = floor(4*36*5/6) = 120.

      - If other s1 and s2 values are introduced, these cases will result the BD budget based on cross carrier scheduling from sSCell to PCell in Rel-16. Furthermore, the second BD/CCE handling manner may be introduced for CA scaling and BD/CCE counting. That is, PCell shall be also counted as s1 and s2 cells into the cell number of  Ncells,0DL,μP and Ncells,0DL,μS respectively when calculating M_total_μp and M_total_μs. And UE is not required to monitor more than  and PDCCH BD candidates per PCell slot on PCell for self-scheduling and per sSCell slot on sSCell for cross-carrier scheduling to PCell.

      - During the discussion in RAN1#108-e meeting, candidates of (s1, s2) = {(1,1), (0.5, 0.5), (α, 1-α)} are proposed. We are open to consider values other than s1=1 and s2=0 and admit the additional flexibility based on the capability of Rel-16 cross-carrier scheduling, especially for value (0.5, 0.5), (α, 1-α). It is preferred to keep the previous principle s1+s2=1. Since 0.5 is one of candidate value of α, support (s1, s2) = (α, 1-α) is enough. Considering some UE vendors have concern on the complexity if all 7 values of α are supported, it seems at least one additional (s1, s2) = (0.5, 0.5) can be supported as a trade-off.

- Proposal 1: If other value of (s1, s2) besides (s1, s2) = (1, 0) can be supported, (s1, s2) = (0.5, 0.5) is slightly preferred as a trade-off.

USS sets for non-fallback DCI

In RAN1#105-e meeting, the following agreement was achieved on two types of UEs for Rel-17 DSS. It was agreed Type B UE with higher capability can monitor PDCCH for scheduling PCell from PCell and sSCell simultaneously. While Type A UE with lower capability can be specified or as Type B UE with restrictions.

Agreement: Two types of UEs (Type A and Type B) can support CCS from sSCell to P(S)Cell - For Type A UE - At least following search space sets on P(S)Cell and search space sets on sSCell are configured so that the UE does not monitor them in overlapping [slot/symbol] of P(S)Cell and sSCell - search space sets on P(S)Cell - USS sets for DCI formats 0_1,1_1,0_2,1_2 (if supported for Type A UE) - USS sets for DCI formats 0_0,1_0 - Type3-CSS set(s) for DCI formats 1_0/0_0 with C-RNTI/CS-RNTI/MCS-C-RNTI - search space sets on sSCell - USS set(s) for scheduling P(S)Cell - FFS: BD/CCE handling - For Type B UE - Following search space sets on P(S)Cell and search space sets on sSCell can be configured so that the UE monitors them in overlapping [slot/symbol] of P(S)Cell and sSCell - search space sets on P(S)Cell - USS sets for DCI formats 0_0,1_0 - Type3-CSS set(s) for DCI formats 1_0/0_0 with C-RNTI/CS-RNTI/MCS-C-RNTI - search space sets on sSCell - USS set(s) for scheduling P(S)Cell - For handling ‘USS sets for scheduling P(S)Cell’ on P(S)Cell and/or on sSCell for DCI formats 0_1,1_1,0_2,1_2 - Alt 2-1 is adopted - There is no restriction on Type-0/0A/1/2-CSS sets configurations - FFS: BD/CCE handling - For Type A and/or Type B UE - FFS: switching to ‘normal’ PDCCH monitoring on P(S)Cell when sSCell is deactivated - FFS: Whether Type A is specified or is Type-B with restrictions (as part of UE features discussion) - FFS: Whether the UE can be configured with unaligned CA - FFS: Whether the above applies for multicast PDSCH

For Type B UE, it was agreed that Alt 2-1 is adopted to handle USS sets for non-fallback DCI. That means type B UE can monitor DCI formats 0_1,1_1,0_2,1_2 on both PCell USS set(s) and sSCell USS sets simultaneously. Type A UE agreed in RAN1#105-e is almost based on Alt 2-4 that UE does not monitor DCI formats 0_1,1_1,0_2,1_2 on both PCell USS set(s) and sSCell USS sets simultaneously by gNB configuration. In case USS sets for DCI formats 0_1,1_1,0_2,1_2 on PCell is supported by Type A, it is similar for Type A UE and Type B UE to switch to normal PDCCH monitoring on PCell when sSCell is deactivated by not applying scaling factor α. In case USS sets with DCI formats 0_1,1_1,0_2,1_2 on PCell is not supported by Type A UE, some additional mechanism may be considered to recover non-fallback DCI formats when sSCell is deactivated. Since TDM monitoring on the two scheduling cells is supported by Type A UE only except PDCCH monitoring for SIB/RAR/Paging, that is not monitoring DCI formats for unicast simultaneously, no matter fallback DCI or non-fallback DCI. Type A UE with support of USS sets for DCI formats 0_1,1_1,0_2,1_2 on PCell is preferred.

For the Scell dormancy Indication in non-fallback DCI, it is only present when this format is carried by PDCCH on the primary cell. In case Rel-17 DSS is configured, the off-loading PDCCH from PCell to sSCell is achieved. But there is no agreement that the non-fallback DCI formats used for scheduling PCell on sSCell includes same number of the SCell dormancy indication bits as the corresponding non-fallback DCI formats on PCell that are used for PCell self-scheduling. If Type-A is not allowed to receive non-fallback DCI, it means type-A UE is not allowed to indicate Scell dormancy via DCI format 0_1/1_1.

Above all, Type A UE is preferred to support USS sets for DCI formats 0_1,1_1,0_2,1_2 on PCell.

Proposal 2: Type A UE supports USS sets for DCI formats 0_1,1_1,0_2,1_2 on PCell.

MR-DC

In clause 5.2.1.5.3 in TS38.214-h10, the following red sentence is ambiguous because the the cell of the reference slot n+k is a PUCCH cell which may be different with the cell of MAC-CE received in slot n. The clarification is needed for this issue.

- 5.2.1.5.3 Aperiodic CSI-RS for tracking for fast SCell activation When the UE receives an Enhanced Scell Activation/Deactivation MAC-CE that triggers one or two CSI-RS bursts for fast SCell activation for a (set of) deactivated SCell(s), - if the MAC-CE indicates that the first CSI-RS burst for SCell activation is present in an SCell, then the UE may assume that the first CSI-RS burst for SCell activation is present in that SCell. The first slot of the first CSI-RS burst starts at the m1th SCell slot after the last SCell slot coinciding with the reference slot n+k, as defined in clause 4.3 of [6, TS38.213], of the cell in which the MAC-CE was received. - if the MAC-CE indicates that the second CSI-RS burst for SCell activation is present in an SCell, then the UE may assume that the second CSI-RS burst for SCell activation is present in that SCell. The first slot of the second CSI-RS burst starts at the m2th SCell slot after the end of the first CSI-RS burst. The CSI-RS of the second burst shall have the same antenna port index, OFDM symbol allocations in a slot, same PRB allocation location as the CSI-RS of the first burst. - Where the CSI-RS burst is defined as four CSI-RS resources in two consecutive slots in clause 5.1.6.1.1.1, and m1 and m2 are provided by the MAC-CE and higher layer configuration.

To clarify this ambiguous issue, the following TP is proposed.

----------------------------------------Text Proposal for Section 5.2.1.5.3 in TS 38.214 h10------------------------------------

- Reason for change: There is ambiguous for fast SCell activation because the cell of the reference slot n+k is a PUCCH cell which may be different with the cell of MAC-CE received in slot n. - Summary of change: To correct the cell of the reference slot n+k. - Consequences if not approved: The timeline of the fast cell activation is impacted due to the ambiguous cell of the reference slot.

- 5.2.1.5.3 Aperiodic CSI-RS for tracking for fast SCell activation When the UE receives an Enhanced Scell Activation/Deactivation MAC-CE that triggers one or two CSI-RS bursts for fast SCell activation for a (set of) deactivated SCell(s), - if the MAC-CE indicates that the first CSI-RS burst for SCell activation is present in an SCell, then the UE may assume that the first CSI-RS burst for SCell activation is present in that SCell. The first slot of the first CSI-RS burst starts at the m1th SCell slot after the last SCell slot coinciding with the reference slot n+k, as defined in clause 4.3 of [6, TS38.213], of the cell in which the PUCCH with HARQ-ACK information for the MAC-CE reception was transmitted. - if the MAC-CE indicates that the second CSI-RS burst for SCell activation is present in an SCell, then the UE may assume that the second CSI-RS burst for SCell activation is present in that SCell. The first slot of the second CSI-RS burst starts at the m2th SCell slot after the end of the first CSI-RS burst. The CSI-RS of the second burst shall have the same antenna port index, OFDM symbol allocations in a slot, same PRB allocation location as the CSI-RS of the first burst. - Where the CSI-RS burst is defined as four CSI-RS resources in two consecutive slots in clause 5.1.6.1.1.1, and m1 and m2 are provided by the MAC-CE and higher layer configuration.

Proposal 3: The following TP is preferred for fast SCell activation.

----------------------------------------Text Proposal for Section 5.2.1.5.3 in TS 38.214 h10------------------------------------

- Reason for change: There is ambiguous for fast SCell activation because the cell of the reference slot n+k is a PUCCH cell which may be different with the cell of MAC-CE received in slot n. - Summary of change: To correct the cell of the reference slot n+k. - Consequences if not approved: The timeline of the fast cell activation is impacted due to the ambiguous cell of the reference slot.

- 5.2.1.5.3 Aperiodic CSI-RS for tracking for fast SCell activation When the UE receives an Enhanced Scell Activation/Deactivation MAC-CE that triggers one or two CSI-RS bursts for fast SCell activation for a (set of) deactivated SCell(s), - if the MAC-CE indicates that the first CSI-RS burst for SCell activation is present in an SCell, then the UE may assume that the first CSI-RS burst for SCell activation is present in that SCell. The first slot of the first CSI-RS burst starts at the m1th SCell slot after the last SCell slot coinciding with the reference slot n+k, as defined in clause 4.3 of [6, TS38.213], of the cell in which the PUCCH with HARQ-ACK information for the MAC-CE reception was transmitted. - if the MAC-CE indicates that the second CSI-RS burst for SCell activation is present in an SCell, then the UE may assume that the second CSI-RS burst for SCell activation is present in that SCell. The first slot of the second CSI-RS burst starts at the m2th SCell slot after the end of the first CSI-RS burst. The CSI-RS of the second burst shall have the same antenna port index, OFDM symbol allocations in a slot, same PRB allocation location as the CSI-RS of the first burst. - Where the CSI-RS burst is defined as four CSI-RS resources in two consecutive slots in clause 5.1.6.1.1.1, and m1 and m2 are provided by the MAC-CE and higher layer configuration.

Conclusion

In this contribution, the remaining issues on DSS and MR-DC are provided with the following proposals.

- Proposal 1: If other value of (s1, s2) besides (s1, s2) = (1, 0) can be supported, (s1, s2) = (0.5, 0.5) is slightly preferred as a trade-off.

Proposal 2: Type A UE supports USS sets for DCI formats 0_1,1_1,0_2,1_2 on PCell.

Proposal 3: The following TP is preferred for fast SCell activation.

----------------------------------------Text Proposal for Section 5.2.1.5.3 in TS 38.214 h10------------------------------------

- Reason for change: There is ambiguous for fast SCell activation because the cell of the reference slot n+k is a PUCCH cell which may be different with the cell of MAC-CE received in slot n. - Summary of change: To correct the cell of the reference slot n+k. - Consequences if not approved: The timeline of the fast cell activation is impacted due to the ambiguous cell of the reference slot.

- 5.2.1.5.3 Aperiodic CSI-RS for tracking for fast SCell activation When the UE receives an Enhanced Scell Activation/Deactivation MAC-CE that triggers one or two CSI-RS bursts for fast SCell activation for a (set of) deactivated SCell(s), - if the MAC-CE indicates that the first CSI-RS burst for SCell activation is present in an SCell, then the UE may assume that the first CSI-RS burst for SCell activation is present in that SCell. The first slot of the first CSI-RS burst starts at the m1th SCell slot after the last SCell slot coinciding with the reference slot n+k, as defined in clause 4.3 of [6, TS38.213], of the cell in which the PUCCH with HARQ-ACK information for the MAC-CE reception was transmitted. - if the MAC-CE indicates that the second CSI-RS burst for SCell activation is present in an SCell, then the UE may assume that the second CSI-RS burst for SCell activation is present in that SCell. The first slot of the second CSI-RS burst starts at the m2th SCell slot after the end of the first CSI-RS burst. The CSI-RS of the second burst shall have the same antenna port index, OFDM symbol allocations in a slot, same PRB allocation location as the CSI-RS of the first burst. - Where the CSI-RS burst is defined as four CSI-RS resources in two consecutive slots in clause 5.1.6.1.1.1, and m1 and m2 are provided by the MAC-CE and higher layer configuration.

Reference

RP-211345, Revised WID on NR Dynamic spectrum sharing (DSS), RAN#92e.

RP-201040, Revised WID on Further Multi-RAT Dual-Connectivity enhancements, RAN#88e.

RAN1#103-e meeting, chairman notes.

RAN1#104-e meeting, chairman notes.

RAN1#104b-e meeting, chairman notes.

RAN1#105-e meeting, chairman notes.

RAN1#106-e meeting, chairman notes.

RAN1#106b-e meeting, chairman notes.

RAN1#107-e meeting, chairman notes.

RAN1#108-e meeting, chairman notes.