3GPP TSG RAN WG1 #109-e                                               R1-2203198

e-Meeting, May 9th – May 20th, 2022

Source: 	ZTE

Title: 	Discussion on UE features for NR coverage enhancement

Agenda item:	8.16.8

Document for:	Discussion/Decision

Introduction

In RAN1#108-e meeting, a relatively good progress has been made for NR coverage enhancement [1]. The mainly remaining issues are mainly about the prerequisites and reporting granularity for some UE feature groups. In this contribution, our views on these issues are provided based on the updated RAN1 UE features list in [2].

Discussion

UE FGs for PUSCH repetition type A

As agreed for PUSCH repetition type A in maintenance session, counting based on available slot only applies to repetition number K>1. Therefore the components of FG 30-2 can be updated correspondingly.

Regarding the prerequisite of FG 30-2, we see no much need to add FG 11-6 and 30-1 considering the fundamental FGs {5-14, 5-16, 5-17} have been included. If a UE needs to support available slot together with dynamic indication or increased maximum number of repetitions, it can always report related FGs together.

With above, we have the following proposal.

- Proposal 1: For FG 30-2, adopt the following updates.

| Index | Feature group | Components | Prerequisite feature groups |
| --- | --- | --- | --- |
| 30-2 | PUSCH Type A repetitions based on available slots | Transmission occasions for K>1[K] repetitions for dynamic and configured grant PUSCH are determined on the basis of available slots. | One of {5-14, 5-16, 5-17, [11-6, 30-1]} |

UE FGs for TBoMS

The only remaining issue for FGs for TBoMS, i.e., FG 30-3 and FG 30-3a, is about the prerequisites. For FG 30-3, we don’t think a prerequisite is needed since TBoMS is a totally new functionality and have different UE behavior compared to legacy PUSCH repetition type A. For FG 30-3a, we think it only depends on FG 30-3, and it doesn’t unnecessarily require the UE to support legacy PUSCH repetition. As a result, we have the following proposal.

- Proposal 2: For FGs for TBoMS, no prerequisite is needed for FG 30-3 and the prerequisite of FG 30-3a is FG 30-3.

DM-RS bundling among PUSCH/PUCCH transmissions

- Maximum duration for DMRS bundling (FG 30-4)

- For FG 30-4, a RAN4 LS is received in R1-2203029, with providing the following main information from RAN4:

- RAN4 concluded that for per band UE capability on length of maximum duration

- The set of values of duration lengths for FDD are {4, 8, 16, 32} slots. Meanwhile, RAN4 will not define the requirements with 32 slots for FDD in Rel-17.

- The set of values of duration lengths for TDD should at least include {2, 4, 8} slots, and whether 16 slots can be included in the set of values for UE capability reporting is up to RAN1 to decide, but RAN4 does not intend to define requirements for 16 slots for TDD in Rel-17.

- Regarding above set of values can be reported by UE, it should be added in the note of FG 30-4. For TDD, we can keep value 16 in the set of values similar as value 32 for FDD. As for the prerequisite, we don’t see any dependency on other FGs. Therefore, we have the following proposal.

- Proposal 3: For FG 30-4, no prerequisite is needed, and the following note should be added.

- The set of values of maximum duration lengths for FDD are {4, 8, 16, 32} slots.

- The set of values of maximum duration lengths for TDD should are {2, 4, 8, 16} slots.

- DMRS bundling (FG 30-4x)

- Regarding the prerequisite of FG 30-4x, our views are summarized below. In general, for a given FG, only the fundamental dependent FG needs to be listed. This is similar as the discussion on FG 30-2 above.

  - FG 30-4a

    - FG 30-4 and FG 5-14 or 5-16 or 5-17

  - FG 30-4b

    - FG 30-4 and FG 11-5

  - FG 30-4c

    - FG 30-4 and FG 30-3

  - FG 30-4d

    - FG 30-4 and FG 4-23

  - FG 30-4e

    - FG 30-4a or 30-4b or 30-4c

  - FG 30-4f

    - FG 30-4d, 4-23

  - FG 30-4g

    - FG 30-4

  - FG 30-4h

    - FG 30-4a or 30-4b or 30-4c or 30-4d

- Regarding the UE reporting granularity, we think it’s sufficient for reporting by per UE with no need differentiation for TDD/FDD or FR1/FR2 or mixture of them.

- Proposal 4: For FG 30-4x, the prerequisites are setting as follows, and is reported per UE with no need differentiation for TDD/FDD or FR1/FR2 or mixture of them

  - FG 30-4a

    - FG 30-4 and FG 5-14 or 5-16 or 5-17

  - FG 30-4b

    - FG 30-4 and FG 11-5

  - FG 30-4c

    - FG 30-4 and FG 30-3

  - FG 30-4d

    - FG 30-4 and FG 4-23

  - FG 30-4e

    - FG 30-4a or 30-4b or 30-4c

  - FG 30-4f

    - FG 30-4d, 4-23

  - FG 30-4g

    - FG 30-4

  - FG 30-4h

    - FG 30-4a or 30-4b or 30-4c or 30-4d

Dynamic PUCCH repetitions

The only remaining issue for FG 30-5 is the prerequisite FG. In our view, both legacy FG 4-23 for PUCCH repetition for PUCCH format 1/3/4 or new Rel-17 FG 25-2 for PUCCH repetition for PUCCH format 0/2 are the prerequisite FG for FG 30-5.

Proposal 5: For FG 30-5, the prerequisite FG is 4-23 or 25-2.

Msg3 PUSCH repetition

- For FG 30-6, the only remaining issue is the UE capability reporting granularity. In our view, it’s sufficient for reporting by per UE with no need differentiation for TDD/FDD or FR1/FR2 or mixture of them.

Proposal 6: For FG 30-6, support per UE reporting and no need of differentiation for FDD/TDD, FR1/FR2 or mixture of them.

Conclusion

Based on the discussion, we have the following proposals.

- Proposal 1: For FG 30-2, adopt the following updates.

| Index | Feature group | Components | Prerequisite feature groups |
| --- | --- | --- | --- |
| 30-2 | PUSCH Type A repetitions based on available slots | Transmission occasions for K>1[K] repetitions for dynamic and configured grant PUSCH are determined on the basis of available slots. | One of {5-14, 5-16, 5-17, [11-6, 30-1]} |

- Proposal 2: For FGs for TBoMS, no prerequisite is needed for FG 30-3 and the prerequisite of FG 30-3a is FG 30-3.

- Proposal 3: For FG 30-4, no prerequisite is needed, and the following note should be added.

- The set of values of maximum duration lengths for FDD are {4, 8, 16, 32} slots.

- The set of values of maximum duration lengths for TDD should are {2, 4, 8, 16} slots.

- Proposal 4: For FG 30-4x, the prerequisites are setting as follows, and is reported per UE with no need differentiation for TDD/FDD or FR1/FR2 or mixture of them

  - FG 30-4a

    - FG 30-4 and FG 5-14 or 5-16 or 5-17

  - FG 30-4b

    - FG 30-4 and FG 11-5

  - FG 30-4c

    - FG 30-4 and FG 30-3

  - FG 30-4d

    - FG 30-4 and FG 4-23

  - FG 30-4e

    - FG 30-4a or 30-4b or 30-4c

  - FG 30-4f

    - FG 30-4d, 4-23

  - FG 30-4g

    - FG 30-4

  - FG 30-4h

    - FG 30-4a or 30-4b or 30-4c or 30-4d

Proposal 5: For FG 30-5, the prerequisite FG is 4-23 or 25-2.

Proposal 6: For FG 30-6, support per UE reporting and no need of differentiation for FDD/TDD, FR1/FR2 or mixture of them.

Reference

3GPP RAN1#108-e, R1-2201517, Summary on UE features for NR coverage enhancement, Moderator (NTT DOCOMO, INC.).

3GPP RAN1#108-e, R1-2202928, Updated RAN1 UE features list for Rel-17 NR after RAN1 #108-e, Moderators (AT&T, NTT DOCOMO, INC.).

3GPP, R1-2203029 (R4-2206580), Reply LS on Length of Maximum duration, RAN4.