3GPP TSG RAN WG1 #109-e	R1-2203199

e-Meeting, May 9th – 20th, 2022

Title: 	Discussion on Rel-17 UE features for NR MBS

Source: 	ZTE

Agenda item:	8.16.12

Document for:	Discussion/Decision

# Introduction

After RAN1#108-e meeting, moderators summarized the preliminary RAN1 UE features list for Rel-17 NR [1]. In this contribution, we present our analysis and views on Rel-17 UE features for MBS.

# Discussion

Reporting granularity of FG33-2

Currently, there are different views on the reporting granularity of FG33-2, i.e.., per band or per FSPC. Technically speaking, receiving multicast on which cell doesn’t have much difference in terms of implementation complexity. Meanwhile, in Rel-17, although there is only up to one carrier can be configured to receive multicast. However, in order to have a UE feature design that is future proof, it is more appropriate to make it as per band at least for FG 33-2.

Proposal 1: Make FG33-2 as a per band FG.

Proposal 2: Add FG6-5 (Basic DL NR-NR CA operation) as the prerequisite of FG33-2h.

Separate PUCCH resource configurations from unicast

In FG 33-2a and FG33-4, we have the following two FFS on whether to have separate FG for separate PUCCH resource configuration.

| 33. NR_MBS | 33-2a | Support of ACK/NACK based HARQ-ACK feedback andRRC-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast | 1) Support of ACK/NACK based HARQ-ACK feedback, and support of enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signalling 2) Support of PTM retransmission for multicast 3) support of Type-1 and Type-2 HARQ-ACK CB for multicast feedback only 4) Support of shared PUCCH resource configurations with unicast FFS: Whether to define separate PUCCH resource configurations from unicast as separate FG from FG 33-2a |
| --- | --- | --- | --- |

| 33. NR_MBS | 33-4 | NACK-only based HARQ-ACK feedback for multicast | - [Support NACK-only based HARQ-ACK feedback.] - Support of shared PUCCH resource configurations with unicast FFS: Whether to define separate PUCCH resource configurations from unicast as separate FG from FG 33-4 |
| --- | --- | --- | --- |

Regardless of shared PUCCH resource configuration or separate PUCCH resource configuration, the total number of PUCCH resource is kept unchanged compared with Rel-15. The only difference is where to pick the PUCCH resource. PUCCH resource configuration itself really doesn’t deserve to have a separate FG from our perspective.

Proposal 3: Merge separate PUCCH resource configurations from unicast with FG33-2a and FG33-4.

DCI format 4_2 with G-CS-RNTI

One remaining issue of FG33-5-1 is whether to have a separate FG for support of DCI format 4_2 with CRC scrambled with G-CS-RNTI for multicast SPS scheduling.

| 33. NR_MBS | 33-5-1 | SPS group-common PDSCH for multicast | - Support one SPS group-common PDSCH configuration for multicast - Support {2, 4, 8} times semi-static slot-level repetition for SPS group-common PDSCH FFS whether to separate the capability for support of DCI format 4_2 with CRC scrambled with G-CS-RNTI for multicast SPS scheduling |
| --- | --- | --- | --- |

| 33. NR_MBS | 33-2f | Dynamic multicast with DCI format 4_2 | Support of DCI format 4_2 with CRC scrambled with G-RNTI for multicast |
| --- | --- | --- | --- |

Since we already have a FG 33-2f to indicate dynamic multicast with DCI format 4_2, if UE indicates support of FG 33-2f and FG 33-5-a, then it means UE can support DCI format 4_2 with CRC scrambled with G-CS-RNTI for multicast SPS scheduling.

Proposal 4: No need to have a separate FG for support of DCI format 4_2 with CRC scrambled with G-CS-RNTI for multicast SPS scheduling. If UE indicates support of FG 33-2f and FG 33-5-a, then it means UE can support DCI format 4_2 with CRC scrambled with G-CS-RNTI for multicast SPS scheduling.

FG 33-5-1d

Another left issue is whether to introduce the following FG 33-5-1d.

| 33. NR_MBS | 33-5-1d | PTP retransmission for SPS group-common PDSCH for multicast | Support of PTP retransmission for SPS multicast FFS whether to merge with 33-5-1a | 33-5-1a | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Since we already have FG 33-2d (PTP retransmission for multicast), from our perspective, FG 33-5-1d can be merged with FG 33-2 because the complexity is PTP retransmission itself instead of retransmission for DG PDSCH or SPS.

Proposal 5: Merge FG 33-5-1d with FG 33-2d.

33-5-1a

Another left issue is whether to introduce the following FG 33-5-1a.

| 33. NR_MBS | 33-5-1a | Support of ACK/NACK based HARQ-ACK feedback and RRC-based enabling/disabling ACK/NACK-based feedback for SPS group-common PDSCH for multicast | Support of ACK/NACK based HARQ-ACK feedback, and support of enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signalling for SPS group-common PDSCH without PDCCH scheduling, SPS group-common PDSCH activation, and SPS release PDCCH | 33-5-1 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Since Rel-15, the FG of HARQ-ACK feedback includes both DG PDSCH and SPS. We can follow the same rule, i.e., merging FG 33-5-1a with FG 33-2a.

Proposal 6: Merge FG 33-5-1a with FG 33-2a.

# Conclusion

In this contribution, we discuss and analyse Rel-17 UE features for MBS with the following proposal.

Proposal 1: Make FG33-2 as a per band FG.

Proposal 2: Add FG6-5 (Basic DL NR-NR CA operation) as the prerequisite of FG33-2h.

Proposal 3: Merge separate PUCCH resource configurations from unicast with FG33-2a and FG33-4.

Proposal 4: No need to have a separate FG for support of DCI format 4_2 with CRC scrambled with G-CS-RNTI for multicast SPS scheduling. If UE indicates support of FG 33-2f and FG 33-5-a, then it means UE can support DCI format 4_2 with CRC scrambled with G-CS-RNTI for multicast SPS scheduling.

Proposal 5: Merge FG 33-5-1d with FG 33-2d.

Proposal 6: Merge FG 33-5-1a with FG 33-2a.

# Reference

- R1-2202928, Updated RAN1 UE features list for Rel-17 NR after RAN1 #108-e, RAN1#108-e,	Moderators (AT&T, NTT DOCOMO, INC.)