3GPP TSG RAN WG1 #109-e	R1-2203200

e-Meeting, May 9th – 20th, 2022

Title: 	Discussion on Rel-17 UE features for DSS

Source: 	ZTE

Agenda item:	8.16.13

Document for:	Discussion/Decision

# Introduction

After RAN1#108-e meeting, moderators summarized the preliminary RAN1 UE features list for Rel-17 NR [1]. In this contribution, we present our analysis and views on Rel-17 UE features for DSS and MRDC.

# Discussion

## DSS

The latest feature groups for Rel-17 DSS are as following.

| 34. NR_DSS | 34-1 | Cross-carrier scheduling from SCell to PCell/PSCell with search space restrictions (Type A) | Support of Cross-carrier scheduling from sSCell to PCell/PSCell with search space restrictions (Type A) - Cross-carrier scheduling from sSCell to PCell/PSCell with CIF - Search space restrictions: sSCell USS set(s) (for CCS from sSCell to PCell/PSCell) and at least following search space sets on PCell/PSCell can only be configured such that UE does not monitor them in overlapping slot of PCell/PSCell and sSCell - USS sets for DCI formats 0_1,1_1,0_2,1_2 - USS sets for DCI formats 0_0,1_0 - Type3-CSS set(s) for DCI formats 1_0/0_0 with C-RNTI/CS-RNTI/MCS-C-RNTI - Configuration of scaling factor α for BD and CCE limit handling and PDCCH overbooking handling on P(S)Cell - FFS: # The number of unicast DCI limits for PCell/PSCell scheduling - Processing K1 unicast DCI scheduling DL on PCell/PSCell per PCell/PSCell slot and its aligned N consecutive sSCell slot(s) - Processing K2 unicast DCI scheduling UL on PCell/PSCell per PCell/PSCell slot and its aligned N consecutive sSCell slot(s) - FFS: N is based on pair of (PCell/PSCell SCS, sSCell SCS): N=1 for(15,15), (30,30), (60,60) and N=2 for (15,30), (30,60) and N=4 for (15, 60) - Same numerology between sSCell and P(S)Cell or sSCell SCS is larger than P(S)Cell SCS - USS set(s) for DCI format 0_1,1_1 configured on sSCell for CCS from sSCell to Pcell/PSCell and USS set(s) for DCI format 0_2,1_2 configured on sSCell for CCS from sSCell to PCell/PSCell if UE supports FG 11-1 (dci-Format1-2And0-2-r16) - sSCell USS set(s) (for CCS from sSCell to Pcell/PSCell) and Type0/0A/1/2 CSS sets on Pcell/PSCell can be configured so that the UE monitors them in overlapping slot of Pcell/PSCell and sSCell - no simultaneous monitoring between ‘USS sets (for P(S)Cell scheduling) on sSCell’ and ‘Type 0/0A/1/2/CSS sets on P(S)Cell for DCI formats with CRC scrambled by C-RNTI/MCS-C-RNTI/CS-RNTI’ - simultaneous monitoring of ‘USS sets (for P(S)Cell scheduling) on sSCell’ and ‘Type 0/0A/1/2/CSS sets on P(S)Cell for DCI formats with CRC not scrambled by C-RNTI/MCS-C-RNTI/CS-RNTI’ - FFS: Support of monitoring DCI formats 0_1,1_1,0_2,1_2 on PCell/PSCell USS set(s) - PDCCH monitoring occasion(s) - FFS: frame boundary alignment between PCell/PSCell and sSCell | 6-5 | Yes | N/A | Cross-carrier scheduling from SCell to PCell/PSCell with search space restrictions (Type A) is not supported | Per BC | No | Applicable to FR1 only | No | Candidate value set: One or more of supported SCS combinations ({P(S)Cell SCS in kHz, sSCell SCS in kHz}) from following set are indicated by the UE: {15,15}, {15,30}, (15, 60), [{30,30}, {30,60},{60,60}]) [Candidate value set 2: frequency band pair(s) for {PCell/PSCell, sSCell}] Component 4 candidate values: (K1, K2) = {(1,1) for FDD P(S)Cell; (K1, K2) = (1,2) for TDD P(S)Cell, [(K1, K2) = (2,2) for FDD P(S)Cell; (K1, K2) = (2,4) for TDD P(S)Cell]} Component 9 candidate values: [Value 1: PDCCH monitoring occasion(s) on PCell/PSCell and on sSCell for cross-carrier scheduling to PCell/PSCell is within the first 3 OFDM symbols of a PCell/PSCell slot. Value 2: PDCCH monitoring occasion(s) on PCell/PSCell and on sSCell for cross-carrier scheduling to PCell/PSCell is not restricted to the first 3 OFDM symbols of a PCell/PSCell slot] Note: The CCS from sSCell to PCell is applicable to FR1 only but there can be other SCells in FR2 configured for the UE Note: The SCell configured with Cross-carrier scheduling to PCell/PSCell is referred to as ‘sSCell’ | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| 34. NR_DSS | 34-2 | Cross-carrier scheduling from SCell to PCell/PSCell (Type B) | Support of Cross-carrier scheduling (CCS) from sSCell to PCell/PSCell (Type B) - Cross-carrier scheduling from sSCell to PCell/PSCell with CIF - sSCell USS set(s) (for CCS from sSCell to PCell/PSCell) and search space sets on PCell/PSCell can be configured so that the UE monitors them in overlapping slot of PCell/PSCell and sSCell - Configuration of scaling factor α for BD and CCE limit handling and PDCCH overbooking handling on P(S)Cell - The number of unicast DCI limits for PCell/PSCell scheduling - Processing K1 unicast DCI scheduling DL on PCell/PSCell per PCell/PSCell slot and its aligned N consecutive sSCell slot(s) - Processing K2 unicast DCI scheduling UL on PCell/PSCell per PCell/PSCell slot and its aligned N consecutive sSCell slot(s) - FFS: N is based on pair of (PCell/PSCell SCS, sSCell SCS): N=1 for(15,15), (30,30), (60,60) and N=2 for (15,30), (30,60) and N=4 for (15, 60) - Same numerology between sSCell and P(S)Cell or sSCell SCS is larger than P(S)Cell SCS - USS set(s) for DCI format 0_1,1_1 configured on sSCell for CCS from sSCell to PCell/PSCell and USS set(s) for DCI format 0_2,1_2 configured on sSCell for CCS from sSCell to PCell/PSCell if UE supports FG 11-1 (dci-Format1-2And0-2-r16) - PDCCH monitoring occasion(s) - FFS: frame boundary alignment between PCell/PSCell and sSCell | 6-5 | Yes | N/A | Cross-carrier scheduling from SCell to PCell/PSCell (Type B) is not supported | Per BC | No | Applicable to FR1 only | No | Candidate value set: One or more of supported SCS combinations ({P(S)Cell SCS in kHz, sSCell SCS in kHz}) from following set are indicated by the UE: {15,15}, {15,30}, (15, 60), [{30,30}, {30,60},{60,60})] [Candidate value set 2: frequency band pair(s) for {PCell/PSCell, sSCell}] Component 4 candidate values: (K1, K2) = {(1,1) for FDD P(S)Cell; (K1, K2) = (1,2) for TDD P(S)Cell} Component 7 candidate values: [Value 1: PDCCH monitoring occasion(s) on PCell/PSCell and on sSCell for cross-carrier scheduling to PCell/PSCell is within the first 3 OFDM symbols of a PCell/PSCell slot. Value 2: PDCCH monitoring occasion(s) on PCell/PSCell and on sSCell for cross-carrier scheduling to PCell/PSCell is not restricted to the first 3 OFDM symbols of a PCell/PSCell slot] Note: The CCS from sSCell to Pcell is applicable to FR1 only but there can be other Scells in FR2 configured for the UE Note: The SCell configured with Cross-carrier scheduling to PCell/PSCell is referred to as ‘sSCell’ | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

There are mainly the following issues left.

Issue#1: FG34-1, 8)	FFS: Support of monitoring DCI formats 0_1,1_1,0_2,1_2 on PCell/PSCell USS set(s)

Based on the discussion, the main complexity from UE implementation on SCell scheduling PCell is the simultaneous monitoring of PDCCH on both Cells instead of the DCI formats. Meanwhile, it would be too restrictive if only fallback DCI formats can be used in PCell scheduling if sSCell experiences some heavy channel fade.

Proposal 1: Include “Support of monitoring DCI formats 0_1,1_1,0_2,1_2 on PCell/PSCell USS set(s)” as a component for FG 34-1.

Issue#2: FG34-1 and FG34-2, supported SCS combinations ({P(S)Cell SCS in kHz, sSCell SCS in kHz})

The controversial part is whether to include “[{30,30}, {30,60},{60,60})]”. From our perspective, since it is anyway optional UE features and UE is allowed to indicate support or not for each SCS combination, they should not be excluded in the specification/feature group.

Proposal 2: Include “{30,30}, {30,60},{60,60}” in the supported SCS combinations for FG34-1 and FG34-2.

Issue#3: FG34-1 and FG34-2, [Candidate value set 2: frequency band pair(s) for {PCell/PSCell, sSCell}]

Since we already have the component to allow UE to report the supported SCS combination, we don’t need this component to indicate the band pair. If this component is introduced, it basically means UE can indicate which Cell is allowed to be PCell and which is allowed to be SCell. This kind of reporting should be avoided.

Proposal 3: Remove “Candidate value set 2: frequency band pair(s) for {PCell/PSCell, sSCell}” fom FG34-1 and FG34-2.

Issue#4: FG34-1 and FG34-2, PDCCH monitoring occasion(s)

Currently, the following two values are listed as potential values for PDCCH monitoring occasion(s) for FG34-1 and FG34-2. Our preference is not to have this component at all. However, if UE vendors really see the need of it, we can live with it.

Proposal 4: further discuss whether to introduce the following values for PDCCH monitoring occasion(s) for FG34-1 and FG34-2.

Value 1: PDCCH monitoring occasion(s) on PCell/PSCell and on sSCell for cross-carrier scheduling to PCell/PSCell is within the first 3 OFDM symbols of a PCell/PSCell slot.

Value 2: PDCCH monitoring occasion(s) on PCell/PSCell and on sSCell for cross-carrier scheduling to PCell/PSCell is not restricted to the first 3 OFDM symbols of a PCell/PSCell slot

Issue#5: FG34-1 and FG34-2, FFS: frame boundary alignment between PCell/PSCell and sSCell

During the discussion, companies didn’t figure out any spec impact to support sSCell scheduling PCell and unaligned frame boundary CA together. Since we already have one FG from Rel-16, we don’t think we need separate FG for sSCell scheduling PCell in case of unaligned frame boundary CA.

Proposal 5: Remove “frame boundary alignment between PCell/PSCell and sSCell” for FG34-1 and FG34-2.

## MRDC

The latest feature group for Rel-17 MRDC is as following.

| 35. LTE_NR_DC_enh2 | 35-1 | Aperiodic CSI-RS for tracking for fast SCell activation | - Aperiodic CSI-RS for tracking for fast SCell activation is triggered by enhanced SCell activation/deactivation MAC CE - Aperiodic CSI-RS for tracking for fast SCell activation is triggered within the BWP indicated by firstActiveDownlinkBWP-Id for the SCell - Maximum number of aperiodic CSI-RS resource set configurations for tracking for fast SCell activation that can be configured to UE per CC in a reported band - Maximum number of aperiodic CSI-RS resource set configurations for tracking for fast SCell activation that can be configured to UE across CCs in a reported band | 6-5 | Yes | N/A | Aperiodic CSI-RS for tracking for fast SCell activation is not supported | Per band | N/A | N/A | N/A | Component 3 candidate values: {[1,] 8,16,32,48,64,128,255} Component 4 candidate values: {[1,] 8,16,32,64,128,256,512,1024} Note: In component 3, the candidate component values {FFS} do not apply to FR2 Note: In component 4, the candidate component values {FFS} do not apply to FR2 Note: component 3 and 4 candidate values refer to the number of RS configurations for fast SCell activation that can be indicated by the MAC CE The NZP-CSI-RS configured as RS for tracking for fast SCell activation are not considered when counting the maximum NZP-CSI-RS configurations of FG2-33 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

One left issue is whether to have value “1” as the candidate value for component 3 and component 4. From network perspective, value “1” is too limited especially for TDD case, network may need different slot offset for TRS.

Proposal 6: Remove value “1” from candidate value of component 3 and component 4 for FG35-1.

# Conclusion

In this contribution, we discuss and analyse Rel-17 UE features for DSS and MRDC with the following proposal.

DSS

Proposal 1: Include “Support of monitoring DCI formats 0_1,1_1,0_2,1_2 on PCell/PSCell USS set(s)” as a component for FG 34-1.

Proposal 2: Include “{30,30}, {30,60},{60,60}” in the supported SCS combinations for FG34-1 and FG34-2.

Proposal 3: Remove “Candidate value set 2: frequency band pair(s) for {PCell/PSCell, sSCell}” fom FG34-1 and FG34-2.

Proposal 4: further discuss whether to introduce the following values for PDCCH monitoring occasion(s) for FG34-1 and FG34-2.

Value 1: PDCCH monitoring occasion(s) on PCell/PSCell and on sSCell for cross-carrier scheduling to PCell/PSCell is within the first 3 OFDM symbols of a PCell/PSCell slot.

Value 2: PDCCH monitoring occasion(s) on PCell/PSCell and on sSCell for cross-carrier scheduling to PCell/PSCell is not restricted to the first 3 OFDM symbols of a PCell/PSCell slot

Proposal 5: Remove “frame boundary alignment between PCell/PSCell and sSCell” for FG34-1 and FG34-2.

MRDC

Proposal 6: Remove value “1” from candidate value of component 3 and component 4 for FG35-1.

# Reference

- R1-2202928, Updated RAN1 UE features list for Rel-17 NR after RAN1 #108-e, RAN1#108-e,	Moderators (AT&T, NTT DOCOMO, INC.)