3GPP TSG RAN WG1 #109-e	R1-2203201

e-Meeting, May 9th – 20th, 2022

Title: 	Discussion on Rel-17 UE features for LTE based 5G terrestrial broadcast

Source: 	ZTE

Agenda item:	8.16.15

Document for:	Discussion/Decision

# Introduction

After RAN1#108-e meeting, moderators summarized the preliminary RAN1 UE features list for Rel-17 LTE [1]. In this contribution, we present our analysis and views on Rel-17 UE features for LTE based 5G terrestrial broadcast.

# Discussion

RAN1 has agreed to define separate UE capabilities for 6MHz/7MHz/8MHz channel bandwidth for PMCH.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the eNB to know if the feature is supported | [Need for the UE to know if the feature is supported (only for V2X WI, where the PC5-RRC capability signalling is delivered between the UEs)] | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Capability interpretation for mixture of FDD/TDD | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3. LTE_terr_bcast_bands_part1 | 3-1a | Support of 6MHz channel bandwidth for PMCH | Support of 6MHz channel bandwidth for PMCH | Support of dedicated MBMS cells | Yes |  | UE cannot receive MBMS in the corresponding MBSFN area | Per band | N/A | N/A | NOTE: This capability is sent to a unicast eNB, and indicates whether the UE in RRC_CONNECTED supports MBMS reception via MBSFN from MBMS-dedicated cells in an MBSFN area with 6MHz. |
| 3. LTE_terr_bcast_bands_part1 | 3-1b | Support of 7MHz channel bandwidth for PMCH | Support of 7MHz channel bandwidth for PMCH | Support of dedicated MBMS cells | Yes |  | UE cannot receive MBMS in the corresponding MBSFN area | Per band | N/A | N/A | NOTE: This capability is sent to a unicast eNB, and indicates whether the UE in RRC_CONNECTED supports MBMS reception via MBSFN from MBMS-dedicated cells in an MBSFN area with 7MHz. |
| 3. LTE_terr_bcast_bands_part1 | 3-1c | Support of 8MHz channel bandwidth for PMCH | Support of 8MHz channel bandwidth for PMCH | Support of dedicated MBMS cells | Yes |  | UE cannot receive MBMS in the corresponding MBSFN area | Per band | N/A | N/A | NOTE: This capability is sent to a unicast eNB, and indicates whether the UE in RRC_CONNECTED supports MBMS reception via MBSFN from MBMS-dedicated cells in an MBSFN area with 8MHz. |

However, the “Consequence if the feature is not supported by the UE” is worded as “UE cannot receive MBMS in the corresponding MBSFN area”. This needs to be updated to reflect that UE cannot receive MBMS in the corresponding MBSFN area for the corresponding PMCH channel bandwidth.

Proposal 1: Update the “Consequence if the feature is not supported by the UE” for FG 3-1a, 3-1b and 3-1c as following.

FG 3-1a: UE cannot receive MBMS in the corresponding MBSFN area with 6MHz

FG 3-1b: UE cannot receive MBMS in the corresponding MBSFN area with 7MHz

FG 3-1c: UE cannot receive MBMS in the corresponding MBSFN area with 8MHz

# Conclusion

In this contribution, we discuss and analyse Rel-17 UE features for LTE based 5G terrestrial broadcast with the following proposal.

Proposal 1: Update the “Consequence if the feature is not supported by the UE” for FG 3-1a, 3-1b and 3-1c as following.

FG 3-1a: UE cannot receive MBMS in the corresponding MBSFN area with 6MHz

FG 3-1b: UE cannot receive MBMS in the corresponding MBSFN area with 7MHz

FG 3-1c: UE cannot receive MBMS in the corresponding MBSFN area with 8MHz

# Reference

- R1-2202925, Updated RAN1 UE features list for Rel-17 LTE after RAN1 #108-e, RAN1#108-e,	Moderators (AT&T, NTT DOCOMO, INC.)