3GPP TSG RAN WG1 #109-e	R1-2203202

e-Meeting, May 9th – 20th, 2022

Title: 	Discussion on other Rel-17 UE features

Source: 	ZTE

Agenda item:	8.16.17

Document for:	Discussion/Decision

# Introduction

After RAN1#108-e meeting, moderators summarized the preliminary RAN1 UE features list for Rel-17 NR [1]. In this contribution, we present our analysis and views on other Rel-17 UE features.

# Discussion

FG for Rel-17 UL Tx switching

During RAN1#108-e meeting, companies discussed the following proposal and it was almost agreeable to companies.

Final proposal

- Specify a new per-BC FG for Rel-17 2Tx-2Tx UL Tx switching CA Option 2

  - UE supports this new FG needs to support Rel-16 CA Option 2

- UE indicating [uplinkTxSwitchingPeriod2T2T-r17 or supportedBandPairListNR-v17xx] and indicating “option 1” or “both option 1 and option 2” support for Rel-16 1Tx-2Tx UL Tx switching via FG 22-1 supports Rel-17 2Tx-2Tx UL Tx switching Option 1 as well.

- UE indicating [uplinkTxSwitchingPeriod2T2T-r17 or supportedBandPairListNR-v17xx] supports Rel-17 2Tx-2Tx UL Tx switching Option 2, if the UE indicates either one of the following,

- Indicating “option 2” via the new per-BC FG

- Indicating “both option 1 and option 2” via FG 22-1 and the new per-BC FG (i.e. support “option 2” for Rel-17 2Tx-2Tx UL Tx switching when “both option 1 and option 2” is indicated via FG 22-1)

From our perspective, the above can be used as the baseline. Regarding whether to use “uplinkTxSwitchingPeriod2T2T-r17” or “supportedBandPairListNR-v17xx”, our understanding is that “uplinkTxSwitchingPeriod2T2T-r17” is the correct IE since RAN2 also uses this IE to indicate whether UE supports Rel-17 UL Tx switching.

Thus the proposal can be updated as following.

Proposal 1: Endorse the following

- Specify a new per-BC FG for Rel-17 2Tx-2Tx UL Tx switching CA Option 2

  - UE supports this new FG needs to support Rel-16 CA Option 2

- UE indicating uplinkTxSwitchingPeriod2T2T-r17 and indicating “option 1” or “both option 1 and option 2” support for Rel-16 1Tx-2Tx UL Tx switching via FG 22-1 supports Rel-17 2Tx-2Tx UL Tx switching Option 1 as well.

- UE indicating uplinkTxSwitchingPeriod2T2T-r17 supports Rel-17 2Tx-2Tx UL Tx switching Option 2, if the UE indicates either one of the following,

- Indicating “option 2” via the new per-BC FG

- Indicating “both option 1 and option 2” via FG 22-1 and the new per-BC FG (i.e. support “option 2” for Rel-17 2Tx-2Tx UL Tx switching when “both option 1 and option 2” is indicated via FG 22-1)

# Conclusion

In this contribution, we discuss and analyse other Rel-17 UE features with the following proposal.

Proposal 1: Endorse the following

- Specify a new per-BC FG for Rel-17 2Tx-2Tx UL Tx switching CA Option 2

  - UE supports this new FG needs to support Rel-16 CA Option 2

- UE indicating uplinkTxSwitchingPeriod2T2T-r17 and indicating “option 1” or “both option 1 and option 2” support for Rel-16 1Tx-2Tx UL Tx switching via FG 22-1 supports Rel-17 2Tx-2Tx UL Tx switching Option 1 as well.

- UE indicating uplinkTxSwitchingPeriod2T2T-r17 supports Rel-17 2Tx-2Tx UL Tx switching Option 2, if the UE indicates either one of the following,

- Indicating “option 2” via the new per-BC FG

- Indicating “both option 1 and option 2” via FG 22-1 and the new per-BC FG (i.e. support “option 2” for Rel-17 2Tx-2Tx UL Tx switching when “both option 1 and option 2” is indicated via FG 22-1)

# Reference

- R1-2202928, Updated RAN1 UE features list for Rel-17 NR after RAN1 #108-e, RAN1#108-e,	Moderators (AT&T, NTT DOCOMO, INC.)