3GPP TSG RAN WG1 #109-e	R1-2203206

e-Meeting, May 9th – 20th, 2022

Title: 	Discussion of preliminary simulation results for NR duplex evolution

Source: 	ZTE

Agenda item:	9.3.4

Document for:	Discussion/Decision

Introduction

In RAN#95 meeting, a revised SID on NR duplex evolution has been endorsed with the following objectives [1].

The objective of this study is to identify and evaluate the potential enhancements to support duplex evolution for NR TDD in unpaired spectrum. In this study, the followings are assumed: - Duplex enhancement at the gNB side - Half duplex operation at the UE side - No restriction on frequency ranges The detailed objectives are as follows: - Identify applicable and relevant deployment scenarios (RAN1). - Develop evaluation methodology for duplex enhancement (RAN1). - Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4). - Identify possible schemes and evaluate their feasibility and performances (RAN1). - Study inter-gNB and inter-UE CLI handling and identify solutions to manage them (RAN1). - Consider intra-subband CLI and inter-subband CLI in case of the subband non-overlapping full duplex. - Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1). - Study the feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4). - Study the feasibility of and impact on RF requirements considering the self-interference, the inter-subband CLI, and the inter-operator CLI at gNB and the inter-subband CLI and inter-operator CLI at UE (RAN4). - Note: RAN4 should be involved early to provide necessary information to RAN1 as needed and to study the feasibility aspects due to high impact in antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression. - Summarize the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4). Note: For potential enhancements on dynamic/flexible TDD, utilize the outcome of discussion in Rel-15 and Rel-16 while avoiding the repetition of the same discussion.

In this contribution, we provide some preliminary geometry results for subband full duplex.

Discussion

Simulation parameters

The simulation parameters are summarized in the following table. And the detail simulation assumptions can be found in [2].

Table.1 simulation parameter

| Parameters | Value |
| --- | --- |
| Scenario | Urban Macro |
| Layout | Single layer - Macro layer: Hex. Grid 7 sites, 21cells |
| Inter-BS distance | 500m |
| Carrier frequency | 4GHz |
| Simulation bandwidth | 100 MHz |
| BS Tx power | 33dBm/MHz based on TR 38.830 |
| UE Tx power | 23dBm |
| BS antenna configuration | 64 Tx/64 Rx antenna ports (M, N, P, Mg, Ng; Mp, Np) = (4, 8, 2, 1, 1; 4, 8) ; (dH, dV) = (0.5, 0.8)λ |
| BS antenna height | 25m |
| BS receiver noise figure | 5dB for 4GHz |
| UE antenna configuration | For 4GHz: 2 Tx/4 Rx antenna ports Panel model 1: Mg = 1, Ng = 1, P = 2, dH = 0.5 (M, N, P, Mg, Ng; Mp, Np) = (1, 2, 2, 1, 1; 1, 2) for 4 Rx; (M, N, P, Mg, Ng; Mp, Np) = (1, 1, 2, 1, 1; 1, 1) for 2 Tx; |
| UE receiver noise figure | 9 dB for 4GHz |
| UE power control | P0 = -80, alpha =0.8 |
| Minimum BS-UE (2D) distance | Macro-to-UE: 35m [TR36.897] |
| Minimum UE-UE (2D) distance | 3m (TR36.843) |
| ACIR | BS-BS: 43 dB UE-UE: 28 dB |
| self-interference cancellation | 130 dB |

Preliminary geometry

The preliminary geometry SINR of Urban Macro can be found in the following figure.

Figure.1 Geometry SINR comparison.

The legend is summarized as following.

- Normal SINR: The legacy scenario, where only legacy interference is considered. The DL: UL ratio for the allocated slot is fixed and the same DL: UL ratio is used by all nodes in the network.

- SINR of Dynamic TDD: Dynamic TDD case. In addition to the legacy interference, intra-subband interference is also considered. It is assumed that the direction of neighbor gNB transmission can be changed dynamically between DL and UL. Network randomizes the DL and UL pattern for neighbor gNB in the calibration.

- SINR of Subband Duplex: Subband full duplex case. In addition to the legacy interference, inter-subband interference with ACIR and self-interference cancellation also needs to be considered. It is assumed that all gNBs have the same DL-UL frequency&time domain pattern.

Observation 1: Regarding the geometry

- For DL SINR, there is about 5dB gap between subband full duplex and legacy TDD and about 10dB gap between dynamic TDD and subband full duplex.

- For UL SINR, there is about 15dB gap between subband full duplex and legacy TDD and about 5dB gap between dynamic TDD and subband full duplex.

Conclusion

In this contribution, we provide some preliminary geometry results for subband full duplex. Note that this is just the preliminary geometry results that haven’t been calibrated yet.

Observation 1: Regarding the geometry

- For DL SINR, there is about 5dB gap between subband full duplex and legacy TDD and about 10dB gap between dynamic TDD and subband full duplex.

- For UL SINR, there is about 15dB gap between subband full duplex and legacy TDD and about 5dB gap between dynamic TDD and subband full duplex.

Reference

- RP-213591, “New SI: Study on evolution of NR duplex operation”, CMCC (Moderator)

- R1-2203203, “Discussion of evaluation on NR duplex evolution”, ZTE