3GPP TSG RAN WG1 Meeting #109-e	R1-2203208

e-Meeting, May 9th – 20th, 2022

Source: 	ZTE

Title: 	Discussion on Multi-carrier UL Tx switching scheme

Agenda item:	9.10.2

Document for:	Discussion and Decision

Introduction

In RAN#94-e meeting, a new work item on Multi-carrier enhancements was approved for Rel-18 and revised in RAN#95-e meeting [1]. One of the objectives of this WI is to study and if necessary support UL Tx switching schemes across up to 3 or 4 bands with restriction of up to 2 Tx simultaneous transmission for FR1 UEs. The details of the objective are shown below.

2. Study and if necessary specify following enhancements for multi-carrier UL operation [RAN1, RAN2, RAN4] - UL Tx switching schemes across up to 3 or 4 bands with restriction of up to 2 Tx simultaneous transmission for FR1 UEs, including mechanisms to enable more configured UL bands than its simultaneous transmission capability and to support dynamic Tx carrier switching across the configured bands for both single TAG and multiple TAGs configurations (RAN1, RAN4) - UE capability and RRC configuration related signalling (RAN2) - Note: strive for RAN1/2 design agnostic with the number of bands, i.e., common design between 3 and 4 bands - Note: no additional TAG is introduced for UL transmission on a carrier without corresponding DL carrier - Note: this objective does not target to extend the SUL framework to support more than 1 SUL for 1 NUL - Switching time and other RF aspects, and RRM requirements for above UL Tx switching schemes across up to 3 or 4 bands (RAN4) - Note: Prioritize UL Tx switching across up to 3 bands is to be addressed first and then that for up to 4 bands can also be addressed

In this contribution, our analyses and views on the Rel-18 UL Tx switching are provided, including clarification on scenarios, the framework for Rel-18 UL TX switching and multiple TAGs.

Overviews

Clarification on the scenarios

In Rel-16, UL Tx switching was specified for CA, standalone SUL and EN-DC with two inter-band uplink carriers. In Rel-17, UL Tx switching was enhanced to support the following new features.

- 2T-2T UL Tx switching for inter-band CA with one carrier on each of the two bands;

- 1T-2T UL Tx switching for inter-band CA with two intra-band contiguous carriers on one of the two bands;

- 2T-2T UL Tx switching for inter-band CA with two intra-band contiguous carriers on one of the two bands;

- 2T-2T UL Tx switching for SUL with one carrier on each of the two bands, where one carrier in one band is NUL and another carrier in the other band is SUL;

- 1T-2T UL Tx switching for SUL with two intra-band contiguous carriers on the band containing NUL;

- 2T-2T UL Tx switching for SUL with two intra-band contiguous carriers on the band containing NUL;

Unlike Rel-16/Rel-17 UL Tx switching, the scenarios of Rel-18 multi-carrier UL operation is not clearly defined in the WID. To avoid further confusion, it is beneficial to clarify the target scenarios first.

CA (including both intra-band CA and inter-band CA) has been commonly deployed in 5G, especially for operators who own more than one band. For UEs supporting 3 or 4 bands, to fully enjoy the benefits of configuring more UL bands than its simultaneous transmission capability, it is necessary to support 2 Tx simultaneous transmission on two bands instead of limiting to 1Tx simultaneous transmission on one band. In this sense, it is straightforward that UE supporting UL Tx switching schemes across up to 3 or 4 bands requires legacy CA as the prerequisite capability. In Rel-16/Rel-17, both Option1 (switched UL) and Option2 (dual UL) has been specified for UL Tx switching for CA. Among Option1 and Option2, Option2 should be prioritized to fully enjoy the benefits of configuring more UL bands than its simultaneous transmission capability.

Observation 1: UE supporting Rel-18 UL Tx switching schemes across up to 3 or 4 bands requires legacy CA as the prerequisite capability.

Proposal 1: In order to fully enjoy the benefits of configuring more UL bands than its simultaneous transmission capability, prioritize CA (especially CA Option2) for Rel-18 UL Tx switching across up to 3 or 4 bands.

If CA is the prerequisite capability for UL Tx switching schemes across up to 3 or 4 bands, then the motivation and necessity to further support SUL is not clear from our perspective. As shown in Figure 1, ‘pure CA’ means all the uplink carriers are aggregated via CA.  ‘CA+1SUL’ means only one of the aggregated cells contains one SUL. From our perspective, ‘pure CA’ case should be prioritized and more justification is needed for ‘CA+1SUL’ considering that CA is the prerequisite capability for UL Tx switching schemes across up to 3 or 4 bands.

Regarding the ‘CA+2SUL’, it only exists in the 4 bands case. In this case, there are 2 cells and each of the cell contains one SUL. In total, there are two SUL carriers. If UE can already support CA since CA is the prerequisite capability for UL Tx switching schemes across up to 3 or 4 bands, the benefits or motivations to support ‘CA+2SUL’ is not clear to us. Most justification is needed. In addition, RAN1 has never discussed this 2 SUL case before. From our perspective, this is trying to extend the SUL framework. Furthermore, this case only exists for the 4 bands case, which is conflicting with the requirement of defining a common solution for 3 bands case and 4 bands case as noted in the WID. Thus, from our perspective, RAN1 should focus on the “Pure CA” and “CA+1SUL” case in this WI.

Observation 2: Compared with CA, the benefits or motivations to support ‘CA+2SUL’ is not clear especially considering that ‘CA+2SUL’ requires CA as the prerequisite.

Figure 1: Potential candidate scenarios for Rel-18 multi-carrier operation.

Clarification on a carrier without corresponding DL carrier

In the WID, a term “a carrier without corresponding DL carrier” is mentioned. Also, companies discussed this issue during RAN#94-e meeting [2]. It is beneficial to clarify the detailed meaning of “a carrier without corresponding DL carrier” before jumping into the detailed solution discussion.

There are at least the following interpretations of the term “a carrier without corresponding DL carrier”.

- Alt.1: The legacy Rel-15/16/17 SUL operation with no further enhancement.

- Alt.2: New interpretations as discussed during RAN#94-e meeting.

Alt.2-1: Multi-band serving cell, where one cell containing more than one uplink carriers without corresponding DL carrier.

Alt.2-2: One Cell only contains uplink carrier but without DL carrier (i.e., UL-only cell). The scheduling of UL-only cell can be done via another cell containing both DL carrier and uplink carrier.

Regarding Alt.2-1, since the WID has clearly noted that this objective does not target to extend the SUL framework to support more than 1 SUL for 1 NUL, it seems we can exclude Alt.2-1 first.

Regarding Alt.2-2, basically, it is based on CA framework to support“a carrier without corresponding DL carrier”. The main motivations to support UL-only cell are as following.

- Support asymmetric traffic where UEs have uplink-heavy traffic but with light downlink traffic. In this case, network can configure UL-only cell for these UEs.

- Leverage the band where only uplink transmission is allowed and downlink transmission is restricted e.g. due to regulation.

The spec impacts for Alt.2-2 can be minimal since the CA framework can be largely reused and cross-carrier scheduling mechanism can be reused for scheduling of UL-only cell. Take Figure 2 as an example, Cell A @ 2.5GHz contains both DL carrier and UL carrier, Cell B @ 2.3GHz only contains UL carrier (i.e., uplink carrier2). The downlink carrier1 of Cell A is used for scheduling uplink on uplink carrier2.

Proposal 2: Support UL-only cell (a cell only contains UL carrier but without DL carrier) for multi-carrier UL operation.

UL-only cell is also related to the discussion of SSB-less cell in case of inter-band CA. If SSB is not mandatory for a SCell in case of inter-band CA, then this SCell can be an UL-only cell since all the other DL signals/channels are optional.

Observation 3: If SSB is not mandatory for a SCell in case of inter-band CA, then this SCell can be an UL-only cell since all the other DL signals/channels are optional.

Figure 2: An example of UL-only cell.

Framework for Rel-18 UL Tx switching

For the Rel-18 UL Tx switching, there could be two potential frameworks to achieve the objective.

- Framework#1: Network can indicate any UL carrier among the 3/4 bands for UL transmission

- Framework#2: Network indicates 2 out of 3/4 bands for the subsequent UL transmission

Framework#1 can be regarded as the similar scheme used in Rel-16/17 UL Tx switching, where new tables for different cases for 3 or 4 bands are first defined respectively and then identify each switching case and potential ambiguity issues respectively. Framework#2 is designed with the intention of reusing Rel-16/17 UL Tx switching as much as possible. With framework#2, gNB first indicates the cells within up to 2 bands for subsequent UL transmission, and then Rel-16/17 UL Tx switching can be performed within the 2 bands until next gNB indication. Each framework will be discussed in the section 3.1 and 3.2 respectively, and with a comparison in section 3.3.

Framework#1: Indicating any UL carrier for UL transmission

UL Tx Switching for 3 bands

Similar as Rel-16/17, under framework #1, we should first identify the scenarios that should be supported in Rel-18 for UL Tx switching for up to 4 bands. Take 3 bands as an example, the following cases listed in Table 1 should be considered.

Table 1: Cases for UL Tx switching with 3 bands

|  | Number of Tx Chains (Band A + Band B + Band C) |
| --- | --- |
| Case 1-1 | 1T+1T+0T |
| Case 1-2 | 0T+1T+1T |
| Case 1-3 | 1T+0T+1T |
| Case 2-1 | 2T+0T+0T |
| Case 2-2 | 0T+2T+0T |
| Case 2-3 | 0T+0T+2T |

Following the design principle of Rel-17, many scenarios should be considered under different combinations of supported (number of Tx per band, number of carriers per band). Figure 3 illustrates the possible scenarios when taking different combinations into account. We propose that we should first identify which scenarios are to be supported in Rel-18 with higher priority.

Figure 3: Possible scenarios for UL Tx Switching with 3 bands

For different scenarios, the mapping of Tx chains and antenna ports may be different. We may need to define the mapping case by case. Take CA with Option 2 under 3 bands with 2Tx-2Tx-2Tx chain configuration with one carrier per band as an example, the mapping of Tx chains and antenna ports may be defined as Table 2.

Table 2: Mapping of Tx chains and antenna ports for UL Tx Switching with 3 bands for CA with Option 2

|  | Number of Tx Chains (Band A + Band B + Band C) | Number of antenna ports for UL transmission Band A(Carrier 1)+Band B(Carrier 2)+Band C(Carrier 3) |
| --- | --- | --- |
| Case 1-1 | 1T+1T+0T | 1P+0P+0P, 1P+1P+0P, 0P+1P+0P |
| Case 1-2 | 0T+1T+1T | 0P+1P+0P, 0P+1P+1P, 0P+0P+1P |
| Case 1-3 | 1T+0T+1T | 1P+0P+0P, 1P+0P+1P, 0P+0P+1P |
| Case 2-1 | 2T+0T+0T | 2P+0P+0P, 1P+0P+0P |
| Case 2-2 | 0T+2T+0T | 0P+2P+0P, 0P+1P+0P |
| Case 2-3 | 0T+0T+2T | 0P+0P+2P, 0P+0P+1P |

Although the mapping of Tx chains and antenna ports should be defined case by case, the UE behavior defined in Rel-16/17 can be reused for most of the switching cases, except the switching among Case 1-x (x=1,2,3). More specifically, a new switching case should be defined for Rel-18.

- For the UE configured with uplinkTxSwitchingOption set to 'dualUL', when the UE is to transmit a 1-port transmission on one uplink carrier on one band and if the preceding uplink transmission was a 1-port transmission on at least one of the other two carriers on the corresponding other two bands and the UE is under the operation state in which 2-port transmission cannot be supported on the same uplink carrier, then a switching gap is required.

Another issue is the potential ambiguity issue. Compared to Rel-17, the ambiguity issue in Rel-18 is more complex. As highlighted in Table 2, if UL Tx switching is triggered for 1-port transmission on Carrier 1 on Band A, (i.e. 1P+0P+0P in Table 2), the state of Tx chain to support such transmission can be in Case 1-1, Case 1-3 or Case 2-1. And the preceding transmission for such switching also includes several cases. Figure 4 shows the possible transitions for different preceding transmission. The situation may be more complex for 4 bands.

Figure 4: Possible UL Tx switching for 1-port transmission on Carrier 1 on Band A.

UL Tx Switching for 4 bands

Similar to the analysis for 3 bands, the situation for 4 bands will be more complex. First of all, the following cases listed in Table 3 should be considered.

Table 3: Cases for UL Tx switching with 4 bands

|  | Number of Tx Chains (Band A + Band B + Band C+Band D) |
| --- | --- |
| Case 1-1 | 1T+1T+0T+0T |
| Case 1-2 | 0T+1T+1T+0T |
| Case 1-3 | 0T+0T+1T+1T |
| Case 1-4 | 1T+0T+0T+1T |
| Case 1-5 | 1T+0T+1T+0T |
| Case 1-6 | 0T+1T+0T+1T |
| Case 2-1 | 2T+0T+0T+0T |
| Case 2-2 | 0T+2T+0T+0T |
| Case 2-3 | 0T+0T+2T+0T |
| Case 2-4 | 0T+0T+0T+2T |

And some scenarios as shown in Figure 5 may need to be considered for UL Tx switching for 4 bands. Note that not all possible scenarios are listed in Figure 5.

Figure 5: Possible scenarios for UL Tx Switching with 4 bands

For different scenarios, the mapping of Tx chains and antenna ports may be different. We may need to define the mapping case by case. Take CA with Option 2 under 4 bands with 2Tx-2Tx-2Tx-2Tx chain configuration with one carrier per band as an example, the mapping of Tx chains and antenna ports may be defined as Table 4.

Table 4: Mapping of Tx chains and antenna ports for UL Tx Switching with 4 bands for CA with Option 2

|  | Number of Tx Chains (Band A + Band B + Band C+Band D) | Number of antenna ports for UL transmission Band A(Carrier 1)+Band B(Carrier 2)+Band C(Carrier 3) +Band D (Carrier 4) |
| --- | --- | --- |
| Case 1-1 | 1T+1T+0T+0T | 1P+0P+0P+0P, 1P+1P+0P+0P, 0P+1P+0P+0P |
| Case 1-2 | 0T+1T+1T+0T | 0P+1P+0P+0P, 0P+1P+1P+0P, 0P+0P+1P+0P |
| Case 1-3 | 0T+0T+1T+1T | 0P+0P+1P+0P, 0P+0P+1P+1P, 0P+0P+0P+1P |
| Case 1-4 | 1T+0T+0T+1T | 1P+0P+0P+0P, 1P+0P+0P+1P, 0P+0P+0P+1P |
| Case 1-5 | 1T+0T+1T+0T | 1P+0P+0P+0P, 1P+0P+1P+0P, 0P+0P+1P+0P |
| Case 1-6 | 0T+1T+0T+1T | 0P+1P+0P+0P, 0P+1P+0P+1P, 0P+0P+0P+1P |
| Case 2-1 | 2T+0T+0T+0T | 2P+0P+0P+0P, 1P+0P+0P+0P |
| Case 2-2 | 0T+2T+0T+0T | 0P+2P+0P+0P, 0P+1P+0P+0P |
| Case 2-3 | 0T+0T+2T+0T | 0P+0P+2P+0P, 0P+0P+1P+0P |
| Case 2-4 | 0T+0T+0T+2T | 0P+0P+0P+2P, 0P+0P+0P+1P |

Similar to 3 bands, new switching case should also be introduced for 4 bands when 1-port transmission is triggered on a carrier on a band and the preceding transmission was 1-port transmission on at least one of the other two carriers on the corresponding other two bands and the UE was under the operation state in which 2-port transmission cannot be supported on the same uplink carrier, i.e., the switching among Case 1-x, where x = 1,2,3,4,5,6. An example is when the UE is triggered for a 1-port transmission on Carrier 4 on Band D and the UE was under the operation state of Case 1-1.

And the ambiguity issue is more complex for 4 bands. As highlighted in Table-4, if UL Tx switching is triggered for 1-port transmission on Carrier 1 on Band A, (i.e. 1P+0P+0P+0P in Table-4), the state of Tx chain to support such transmission can be in Case 1-1, Case 1-4, Case 1-5 or Case 2-1. And the preceding transmission for such switching also includes several cases, such as from Case 1-2, Case 1-3, Case 1-6 or Case 2-2, Case 2-3, Case 2-4. As can be seen, the ambiguity issues may become very complex.

In short, as discussed above, the potential work RAN1 needed to be solved under potential framework#1 to support UL Tx switching for up to 4 bands may include:

- Identify the scenarios that should be supported in Rel-18 for up to 4 bands.

- Define the mapping of Tx chains and antenna ports for the identified scenarios case by case.

- Introduce new switching case(s) that should be supported in Rel-18 for up to 4 bands.

- Resolve the potential ambiguity issues.

Observation 4: Based on potential framework#1, network can indicate any UL carrier within up to 2 bands for UL transmission. The following issues should be discussed for 3 bands and 4 bands respectively.

- Identify the supported scenarios, including the number of Tx antennas for each band, number of continuous UL carriers and number of bands.

- Define the mapping of Tx chains and antenna ports for the identified scenarios case by case.

- Introduce new switching case(s) for Rel-18 for up to 3 or 4 bands.

- Resolve the potential ambiguity issues.

Overall, from our perspective, if framework#1 is adopted, all the following 32 combinations need to be considered as shown in Figure 6.

The 12 combinations for 3-band case are listed as below: {A3-1, B3-1}, {A3-2, B3-1}, {A3-3, B3-1}, {A3-4, B3-1}, {A3-1, B3-2}, {A3-2, B3-2}, {A3-3, B3-2}, {A3-4, B3-2}, {A3-1, B3-3}, {A3-2, B3-3}, {A3-3, B3-3}, {A3-4, B3-3}.

The 20 combinations for 4-band case are listed as below: {A4-1, B4-1}, {A4-2, B4-1}, {A4-3, B4-1}, {A4-4, B4-1}, {A4-5, B4-1},{A4-1, B4-2}, {A4-2, B4-2}, {A4-3, B4-2}, {A4-4, B4-2}, {A4-5, B4-2}, {A4-1, B4-3}, {A4-2, B4-3}, {A4-3, B4-3}, {A4-4, B4-3}, {A4-5, B4-3},{A4-1, B4-4}, {A4-2, B4-4}, {A4-3, B4-4}, {A4-4, B4-4}, {A4-5, B4-4}.

Take combination {A4-1, B4-1} as an example, it means UE supports UL Tx switching across 4 bands and each of the 4 bands has only one carrier and all the 4 bands support up to 2Tx.

Observation 5: If framework#1 is adopted, all the 32 different combinations should be considered for Rel-18 UL Tx switching across 3/4 bands.

- The 12 combinations for 3-band case: {A3-1, B3-1}, {A3-2, B3-1}, {A3-3, B3-1}, {A3-4, B3-1}, {A3-1, B3-2}, {A3-2, B3-2}, {A3-3, B3-2}, {A3-4, B3-2}, {A3-1, B3-3}, {A3-2, B3-3}, {A3-3, B3-3}, {A3-4, B3-3}.

- The 20 combinations for 4-band case: {A4-1, B4-1}, {A4-2, B4-1}, {A4-3, B4-1}, {A4-4, B4-1}, {A4-5, B4-1},{A4-1, B4-2}, {A4-2, B4-2}, {A4-3, B4-2}, {A4-4, B4-2}, {A4-5, B4-2}, {A4-1, B4-3}, {A4-2, B4-3}, {A4-3, B4-3}, {A4-4, B4-3}, {A4-5, B4-3},{A4-1, B4-4}, {A4-2, B4-4}, {A4-3, B4-4}, {A4-4, B4-4}, {A4-5, B4-4}.

Figure 6 Scenarios of different Combinations (number of Tx per band, number of carriers per band)

Framework#2: Indicating two bands for subsequent UL transmission

Based on the analysis in section 3.1, it can be found that the new tables for different cases among 3 bands and corresponding ambiguity issues are more complex than the traditional 2 bands cases. For the 4 bands cases, the situation is even worse.

Framework#2 is proposed with the intention to reuse Rel-16/17 UL Tx switching as much as possible and avoid the discussion on the new switching cases based on new tables and the corresponding ambiguity issues when considering a new number of bands larger than 2. As shown in Figure 7, gNB will first indicate the cells within up to 2 bands for a UE by the switching signaling, then Rel-16/17 UL Tx switching can be performed within the 2 bands until next gNB indication.

Figure 7: An example for 4 bands in Framework#2

Base on the framework#2, UE will perform UL Tx switching between the 2 indicated bands until the next indication received. The legacy Rel-16/17 UL Tx switching can be reused including the following.

- Legacy tables can be reused between the 2 indicated bands

- The legacy switching cases can be reused

- The legacy solution for ambiguity issues can be reused

The only new issues for framework#2 are the switching signaling design and the initial state after receiving the switching signaling.

For the first issue, MAC CE or DCI can be used as the switching signaling, both can be regarded as dynamic bands switching compared with legacy RRC reconfiguration.

For the second issue, one scheme is not to address the initial state similar as Rel-16/17 if up to 2 bands in Rel-18 is not changed frequently. Otherwise, the initial state after receiving the switching signaling should be determined by gNB indication or RRC configuration.

The switching signaling of framework#2 can be based on a DCI or MAC-CE. If MAC-CE is adopted, additional signaling may be introduced for band combination switching. However, if DCI is adopted, the UL grant can be used to indicate the two-band switching. Then, there is not additional signaling.

It is commented that additional delay may be introduced for MAC-CE based switching in framework#2 compared with framework#1. This issue can be discussed and clarified whether additional delay of the dynamic switching in framework#1 is existed or not, especially whether the band combination within 2 bands can be switched smoothly among 3 or 4 bands from the perspective of UE. Compared with current semi-static band combination switching based on RRC reconfiguration, the framework#2 is a trade-off between fast band combination switching and complexity of specification.

Above all, framework#2 is proposed based on the intention to reuse Rel-16/17 UL Tx switching as much as possible, which is much simpler than framework#1. Considering common design between 3 and 4 bands, and potential more bands in future is a target in the WID, a common design agnostic with the number of bands is preferred.

Observation 6: Based on framework#2, network can indicate the cells within two bands for subsequent transmission via MAC-CE or DCI. UL Tx switching is performed within these two bands until new indication is received.

- The legacy UL Tx switching mechanism can be fully reused.

- It is a design agnostic with the number of bands, i.e., a common design between 3 and 4 bands.

Comparison for the two frameworks

From the above analysis on the two potential frameworks, it can be summarized briefly in Table 5. For framework#1, new tables should be defined and lots of ambiguity issues need to be addressed. In addition, since different tables have to be defined for 3 bands and 4 bands, it may not be a common design for 3 bands and 4 bands. It also lacks of extensibility if more bands are to be supported for UL Tx switching in the future.

For framework#2, the legacy UL Tx switching mechanism for CA and SUL can be fully reused. Thus, there is no need to define new tables or address the ambiguity. Furthermore, it have better extensibility for even more bands in the future. The only thing needs to be specified in Rel-18 is how to indicate the cells (or bands) for subsequent transmission.

Furthermore, framework#2 can be easily extended to support mixture of CA option1 (switched UL) and option2 (dual UL). For example, UE may report band1+band2 as CA option1 and band3+band4 as CA option2. Then, if band1+band2 is indicated for subsequent transmission, UE will operate in switched UL; if band3+band4 is indicated for subsequent transmission, UE will operate in dual UL. However, if framework#1 is adopted, new switching behaviors have to be specified to support this kind of mixture of CA option1 and option2.

Proposal 3: To strive for a common design for 3 bands and 4 bands and strive for an extensible solution for UL Tx switching, framework#2 is supported for Rel-18 UL Tx switching, where network can indicate the cells within two bands (or indicate the two bands) for subsequent transmission via MAC-CE or DCI.

Table 5: Comparison for the two frameworks

|  | Framework#1 | Framework#2 |
| --- | --- | --- |
| New tables for more than 2 bands | Yes | No |
| New switching cases for more than 2 bands | Yes | No |
| New ambiguity issues | Yes | No |
| {number of carriers, number of Tx} combination | 32 different combinations | Follow legacy |
| Mixture of CA option1 and option2 | New behaviors | Follow legacy |
| Common design between 3 and 4 bands | No | Yes |
| Additional signaling | No | May or may not |

Other potential issues

Multiple TAGs

Currently, UL Tx switching is only performed between the UL carriers within the same TAG. If the UL carriers belong to the different TAGs, the uplink frame or UL transmission timing may not be aligned between the UL carriers. Therefore, when the UL Tx switching is performed from one carrier to another carrier, the UL transmission timing adjustment may be needed after switching. It may lead to a longer switching period. The switching period should be defined by RAN4, if needed. Therefore, we think this issue should be discussed in RAN4.

Proposal 4: The issue related to UL Tx switching with multiple TAGs should be discussed in RAN4.

Conclusion

According to the discussion above, we have the following observations and proposals.

Overviews

Observation 1: UE supporting Rel-18 UL Tx switching schemes across up to 3 or 4 bands requires legacy CA as the prerequisite capability.

Proposal 1: In order to fully enjoy the benefits of configuring more UL bands than its simultaneous transmission capability, prioritize CA (especially CA Option2) for Rel-18 UL Tx switching across up to 3 or 4 bands.

Observation 2: Compared with CA, the benefits or motivations to support ‘CA+2SUL’ is not clear especially considering that ‘CA+2SUL’ requires CA as the prerequisite.

Proposal 2: Support UL-only cell (a cell only contains UL carrier but without DL carrier) for multi-carrier UL operation.

Observation 3: If SSB is not mandatory for a SCell in case of inter-band CA, then this SCell can be an UL-only cell since all the other DL signals/channels are optional.

Framework for Rel-18 UL Tx switching

Observation 4: Based on potential framework#1, network can indicate any UL carrier within up to 2 bands for UL transmission. The following issues should be discussed for 3 bands and 4 bands respectively.

- Identify the supported scenarios, including the number of Tx antennas for each band, number of continuous UL carriers and number of bands.

- Define the mapping of Tx chains and antenna ports for the identified scenarios case by case.

- Introduce new switching case(s) for Rel-18 for up to 3 or 4 bands.

- Resolve the potential ambiguity issues.

Observation 5: If framework#1 is adopted, all the 32 different combinations should be considered for Rel-18 UL Tx switching across 3/4 bands.

- The 12 combinations for 3-band case: {A3-1, B3-1}, {A3-2, B3-1}, {A3-3, B3-1}, {A3-4, B3-1}, {A3-1, B3-2}, {A3-2, B3-2}, {A3-3, B3-2}, {A3-4, B3-2}, {A3-1, B3-3}, {A3-2, B3-3}, {A3-3, B3-3}, {A3-4, B3-3}.

- The 20 combinations for 4-band case: {A4-1, B4-1}, {A4-2, B4-1}, {A4-3, B4-1}, {A4-4, B4-1}, {A4-5, B4-1},{A4-1, B4-2}, {A4-2, B4-2}, {A4-3, B4-2}, {A4-4, B4-2}, {A4-5, B4-2}, {A4-1, B4-3}, {A4-2, B4-3}, {A4-3, B4-3}, {A4-4, B4-3}, {A4-5, B4-3},{A4-1, B4-4}, {A4-2, B4-4}, {A4-3, B4-4}, {A4-4, B4-4}, {A4-5, B4-4}.

Observation 6: Based on framework#2, network can indicate the cells within two bands for subsequent transmission via MAC-CE or DCI. UL Tx switching is performed within these two bands until new indication is received.

- The legacy UL Tx switching mechanism can be fully reused.

- It is a design agnostic with the number of bands, i.e., a common design between 3 and 4 bands.

Proposal 3: To strive for a common design for 3 bands and 4 bands and strive for an extensible solution for UL Tx switching, framework#2 is supported for Rel-18 UL Tx switching, where network can indicate the cells within two bands (or indicate the two bands) for subsequent transmission via MAC-CE or DCI.

Other potential issues

Proposal 4: The issue related to UL Tx switching with multiple TAGs should be discussed in RAN4.

Reference

RP-220834, 3GPP RAN#95-e, Revised WID on Multi-carrier enhancements.

RP-213578, [94e-13-R18-CAEnh] - Version 0.0.8, NTT DOCOMO, INC.