3GPP TSG RAN WG1 Meeting #109-e	R1-2203209

e-Meeting, May 9th – 20th, 2022

Source: 	ZTE

Title: 	Simulation evaluation on Rel-18 Multi-carrier enhancement

Agenda item:	9.10.3

Document for:	Discussion and Decision

Introduction

In RAN#94-e meeting, a new work item on Multi-carrier enhancements was approved for Rel-18 and revised in RAN#95-e meeting [1]. One objective is to specify a multi-cell scheduling DCI to reduce the PDCCH overhead and increase the spectral efficiency in CA operation.

In this contribution, we provide our analyses and views for multi-cell scheduling with more detailed simulation results.

Simulation for multi-cell scheduling by a single DCI

The motivation of the simulation is to identify the maximum number of the cells that can be scheduled by a single DCI. To evaluate the relation between the gain and the number of scheduled cells in multi-cell scheduling, the simulation is provided in this section. Based on the WID in [1], the scenario of both intra-band CA and inter-band CA are considered. For intra-band CA scenario, same SCS for the serving cells is a typical case because of the similar carrier frequency. Similarly, different SCS for serving cells in different bands may be a typical case for the inter-band CA scenario. The simulation assumptions and theoretical analysis are made based on the assumption above.

Fig. 1 Different scenarios

With common understanding, the size of the single multi-cell scheduling DCI will increase significantly when it schedules PUSCH/PDSCH on multiple cells, which requires a higher aggregation level (AL). It may increase PDCCH blocking rate. However, as the number of scheduled cell increases, more CCE resources can be saved compared with multiple single-cell scheduling DCIs, which has a greater and more positive impact on the PDCCH blocking rate gain.

Simulation metric and methodology

In Rel-17, PDCCH blocking rate and the cell throughput are used to compare the performance between single-cell scheduling DCI and two-cell scheduling DCI. Similarly, the PDCCH blocking rate and the cell throughput can be used as the simulation metric in Rel-18 for defining the maximum number of scheduled cells.

PDCCH Blocking Rate

The following steps are adopted to perform the simulation of PDCCH blocking rate comparison.

Step1: Perform system level simulation to get geometry results;

Step2: Perform link level simulation to get the BLER-SNR results for different size of DCI;

Step3: Calculate the probability of each aggregation level targeting 10^-2 BLER based on the simulation results in Step1 and Step2;

Step4: Simulate the PDCCH blocking rate for different cases (i.e., different DCI size corresponding to different cell numbers) based on the probability of each aggregation level in Step3 for both single-cell scheduling and multi-cell scheduling;

Step5: Calculate the potential gain of PDCCH blocking rate for different cases for multi-cell scheduling

Cell throughput

The following options can be adopted to evaluate the PUSCH/PDSCH throughput in multi-cell scheduling.

Option 1:

Step1: Calculate the potential gain of PDCCH blocking rate for different cases for multi-cell scheduling in accordance with Step 1-4 of the simulation method for PDCCH blocking rate.

Step 2: Throughput gain = PDCCH blocking rate gain.

Option 2:

Step 1: Calculate the average CCE utilization for both baseline and multi-cell scheduling in accordance with Step3 of the simulation method for PDCCH blocking rate.

Step 2: Derive the CCE saving accordance with Step1.

Step 3: Throughput gain = number of RBs saved derived by converting the result of Step2 / total number of originally available RBs.

For the first option, assume the system bottleneck is the PDCCH. If the PDCCH blocking rate decreases, then the system throughput increases since the number of transmitted PDCCH increases, so that throughput gain is about equal to PDCCH blocking rate gain. For the second option, assume that all the RB resources saved by a multi-cell scheduling DCI can be used for PUSCH/PDSCH transmission, and loss caused by DCI filed sharing is not taken into account. In this case, the resource saving rate is about equal to the gain of PUSCH/PDSCH throughput. This is the maximum throughput gain under this assumption.

Simulation case

Two cases are simulated, one is the single-cell scheduling case (baseline) and the other is the multi-cell scheduling case. For the baseline, 2~8 DCIs with normal size are used to schedule PUSCH/PDSCH on 2~8 cells respectively. For the multi-cell scheduling case, one bigger size DCI is used to schedule PUSCH/PDSCH on 2~8 cells. For the baseline, DCI size (excluding CRC) is assumed to be 60 bits for all UEs. DCI size of 72, 84, 96 ,108, 120, 132 and 144 bits are used for multi-cell scheduling. As shown in Fig.1, the SCS of PCell in intra-band CA scenario is the same with that in SCell due to similar carrier frequencies, and it is different for inter-band CA scenario.

For multiple cells scheduled via a single DCI, PDCCH transmitted on a first cell schedules one PUSCH/PDSCH on the first cell and the other PUSCH(s)/PDSCH(s) on the other cell(s). For single-cell scheduling (baseline), one PDCCH transmitted on a first cell schedules one PUSCH/PDSCH on the first cell via self-scheduling and the other PDCCH(s) also transmitted on the first cell schedule the other PUSCH(s)/PDSCH(s) on the other cell(s) via cross-carrier scheduling.

The remaining details of simulation assumptions (Table A-1) are listed in the appendix.

Evaluation result and analysis

Intra-band CA scenario (2GHz)

The simulation results of the gain of PDCCH blocking rate for intra-band CA scenario are shown in Fig.2 and the calculation results of the gain of CCE saving are shown in Fig.3.

Fig.2 Gain of PDCCH blocking rate for intra-band CA             Fig.3 Gain of CCE saving for intra-band CA

As shown in Fig.2, the gain of the PDCCH blocking rate increases gradually. The average gain of PDCCH blocking rate is more than 40% when the number of scheduled cells is set to 2~8. With the increase of the number of scheduled cell, the number of DCIs will increase for baseline, which will lead to PDCCH blocking rate increase significantly. For multi-cell scheduling, compared with the number of DCIs, the DCI size has a smaller impact on the PDCCH blocking rate. That is the reason why the gain gets larger with the increase of the number of scheduled cells.

For the first option, according to our previous analysis, the throughput gain is same as the PDCCH blocking rate gain, that is, the average gain is above 40%. For the second option, the throughput gain is same as the gain of CCE saving. As shown in Fig.3, when the scheduled cell is configured as 2, 3, 4, 5, 6, 7 and 8, the gain is greater than 40% in each configuration. In multi-cell scheduling, a single DCI can schedule multiple cells, which greatly saves the number of DCI. The size of the single DCI is much lower than that of multiple DCIs, and the saved CCE resources increases with the increase of the number of scheduled cell.

Based on the analysis above, we have the following observation.

Observation 1: For intra-band (2GHz) CA case, the gain of PDCCH blocking rate for number of scheduled cell 2, 3, 4, 5, 6, 7 and 8 is about 10.9%, 33.0%, 44.2%, 49.7%, 51.9%, 53.0% and 54.1%, respectively.

Observation 2: For intra-band (2GHz) CA case,

- According to the first option, the gain of throughput gain for number of scheduled cell 2, 3, 4, 5, 6, 7 and 8 is about 10.9%, 33.0%, 44.2%, 49.7%, 51.9%, 53.0% and 54.1%, respectively.

- According to the second option, the gain of throughput gain for number of scheduled cell 2, 3, 4, 5, 6, 7 and 8 is about 43.0%, 57.3%, 62.2%, 68.7%, 70.8% , 73.4% and 75.1%, respectively.

Inter-band CA scenario (700MHz+4GHz)

The simulation results of the gain of PDCCH blocking rate for inter-band CA scenario are shown in Fig.4 and the calculation results of the gain of CCE saving are shown in Fig.5.

Fig.4 Gain of PDCCH blocking rate for inter-band CA          Fig.5 Gain of CCE saving for inter-band CA

As shown in Fig.4, although the average gain of the PDCCH blocking rate of inter-band CA is smaller than that of intra-band scenario, the average gain of PDCCH blocking rate is still more than 14%. In the inter-band scenario, the multi-cell scheduling can also reduce the number of DCIs, which ensures the gain of PDCCH blocking rate, especially in the case of a large number of scheduled cell.

In the inter-band scenario, CCE saving gain or the PDCCH blocking rate are calculated according to the geometry and BLER-SNR of the PCell with 700MHz. For the first option, the average gain is above 14% which is same as the gain of PDCCH blocking rate. For the second option, as shown in Fig.5, when the scheduled cell is configured as 2, 3 and 4, the gain is greater than 20% in each configuration.

Based on the analysis above, we have the following observation.

Observation 3: For inter-band (700MHz+4GHz) CA case, the gain of PDCCH blocking rate for number of scheduled cell 2, 3 and 4 is about 1.6%, 17.8% and 24.2%, respectively.

Observation 4: For inter-band (700MHz+4GHz) CA case,

- According to the first option, the gain of throughput gain for number of scheduled cell 2, 3 and 4 is about 1.6%, 17.8% and 24.2%, respectively.

- According to the second option, the gain of throughput gain for number of scheduled cell 2, 3, and 4 is about  23.7%, 44.1% and 54.2%, respectively.

Conclusion

According to the discussion above, we have the following observations.

Observation 1: For intra-band (2GHz) CA case, the gain of PDCCH blocking rate for number of scheduled cell 2, 3, 4, 5, 6, 7 and 8 is about 10.9%, 33.0%, 44.2%, 49.7%, 51.9%, 53.0% and 54.1%, respectively.

Observation 2: For intra-band (2GHz) CA case,

- According to the first option, the gain of throughput gain for number of scheduled cell 2, 3, 4, 5, 6, 7 and 8 is about 10.9%, 33.0%, 44.2%, 49.7%, 51.9%, 53.0% and 54.1%, respectively.

- According to the second option, the gain of throughput gain for number of scheduled cell 2, 3, 4, 5, 6, 7 and 8 is about 43.0%, 57.3%, 62.2%, 68.7%, 70.8% , 73.4% and 75.1%, respectively.

Observation 3: For inter-band (700MHz+4GHz) CA case, the gain of PDCCH blocking rate for number of scheduled cell 2, 3 and 4 is about 1.6%, 17.8% and 24.2%, respectively.

Observation 4: For inter-band (700MHz+4GHz) CA case,

- According to the first option, the gain of throughput gain for number of scheduled cell 2, 3 and 4 is about 1.6%, 17.8% and 24.2%, respectively.

- According to the second option, the gain of throughput gain for number of scheduled cell 2, 3, and 4 is about  23.7%, 44.1% and 54.2%, respectively.

Reference

RP-220834, Revised WID on Multi-carrier enhancements, RAN#95-e.

- Appendix Simulation assumptions for multi-cell scheduling by a single DCI

LLS and SLS assumption

Table A-1 PDCCH blocking rate simulation assumptions

| Parameters | Value |
| --- | --- |
| DCI payload (excluding 24 bits CRC) | Legacy DCI: 60 bits, Multi-cell scheduling DCI (intra-band CA): 72, 84, 96 and 108,120,132,144 bits Multi-cell scheduling DCI (inter-band CA): 84, 108 and 132 bits |
| System bandwidth | PCell 10/20MHz + SCell(s) 20/100MHz |
| Carrier Frequency | Inter-band CA (700MHz + 4GHz) Intra-band CA (2GHz) |
| SCS | 15 kHz for 700MHz/2GHz 30 kHz for 4GHz |
| UE speed | 3 km/h |
| Number of UE antennas | 2 Rx |
| Number of symbols for CORESET | 2 for 2GHz, 3 for 700MHz |
| Transmission type | Interleaved(R=3 for 3OS,others,R=2) |
| REG bundling size | 6 |
| Modulation | QPSK |
| Channel coding | Polar code (DCI) |
| Transmission scheme | 1-port precoder cycling |
| Channel model | TDL-C (delay spread: 300ns) |
| Channel estimation | Practical |
| Receiver type | MMSE |
| Note: CORESET size is 24 CCEs for 700M and 32 CCEs for 2G, and number of candidates for AL=1, 2, 4, 8, 16 CCEs are configured as 6, 4, 2, 2, 1. Note: For multi-cell scheduling via a single DCI, PDCCH transmitted on a first cell schedules one PUSCH/PDSCH on the first cell and the other PUSCH/PDSCH(s) on the other cell(s). For single-cell scheduling (baseline), one PDCCH transmitted on a first cell schedules one PUSCH/PDSCH on the first cell via self-scheduling and the other PDCCH(s) transmitted on the first cell schedule(s) the other PUSCH/PDSCH(s) on the other cell(s) via cross-carrier scheduling. |  |

Table A-2 System level simulation assumptions

| Parameters | Value |  |
| --- | --- | --- |
| Carrier frequency | 700MHz | 2 GHz |
| BS antenna configurations | 2 Tx antenna ports: (M, N, P, Mg, Ng; Mp, Np) = (1, 2, 2, 1, 1; 1, 1) | 2 Tx antenna ports: (M, N, P, Mg, Ng; Mp, Np) = (2, 8, 2, 1, 1; 1, 1) |
| UE antenna configuration | 2 Rx antenna ports: (M, N, P, Mg, Ng; Mp, Np) = (1, 1, 2, 1, 1; 1, 1) | 2 Rx antenna ports: (M, N, P, Mg, Ng; Mp, Np) = (1, 1, 2, 1, 1; 1, 1) |
| Simulation bandwidth | 20 MHz | 20 MHz |
| Inter-BS distance | 500m |  |
| UE Tx power | 23dBm |  |
| BS antenna element gain + connector loss | 8 dBi |  |
| BS receiver noise figure | 5dB |  |
| BS antenna height | 25 m |  |
| UE antenna height | 1.5m |  |
| UE antenna gain | 0dBi |  |
| BS Tx power | 46 dBm per 10 MHz |  |
| BS receiver | MMSE-IRC |  |
| UE receiver noise figure | 9 dB |  |
| SCS | 15 kHz for 700MHz/2GHz 30 kHz for 4GHz |  |
| Channel model | UMa in TR 38.901 |  |
| UE distribution | 80% of users are indoor: 3 km/h |  |
| HARQ | Max number of transmissions = 4 with target BLER = 0.1 |  |
| Channel estimation | Ideal |  |
| CSI feedback Period and scheduling delay | 5 , 2 |  |
| CSI feedback mode | Sub-band feedback |  |

Geometry

- Fig.A-1. CDF of SNR for PDCCH in Urban scenario (700MHz)

- Fig.A-2. CDF of SNR for PDCCH in Urban scenario (2GHz)

PDCCH LLS results

- Fig.A-3. Link level simulation results (700MHz)

- Fig.A-4. Link level simulation results (2GHz)