3GPP TSG RAN WG1 #109-e                                            R1-2203213

e-Meeting, May 9th –20th, 2022

Source:	New H3C

Title:	Remaining issue on intra-UE multiplexing and prioritization

Agenda Item:	8.3.2

Document for:	Discussion and Decision

- Introduction

In the RAN1 #108-e meeting, intra-UE multiplexing and prioritization in Rel.17 IIOT/URLLC were discussed. Some agreements were made as below [1].

Agreement

When a PUCCH carrying HP SR with PF0/1 overlaps with a PUCCH carrying LP HARQ-ACK with PF0/1, the LP HARQ ACK is dropped when colliding with positive SR

Agreement

The values for beta-offset values <1 are:

βoffsetHARQ-ACK

0.6

0.4

0.2

0.1

0.05

  - They are mapped to indices 16-20 in the table.

  - These values are used in addition to the legacy values in indices 0-15.

Agreement

If LP HARQ-ACK without HP HARQ-ACK would be transmitted on HP PUSCH,

- Option 2: UE follows the same behaviour as that in case of PUSCH with HP HARQ-ACK assuming two bits

  - FFS for CG-UCI PUSCH.

In this contribution, we further provide our considerations on intra-UE multiplexing and prioritization.

- Discussion

  - Multiplexing UCIs of different priorities in a PUCCH

Based on the below agreement in RAN1#108-e meeting, the below agreement is achieved and for LP HARQ-ACK with CG-UCI need be further studied.

Agreement If LP HARQ-ACK without HP HARQ-ACK would be transmitted on HP PUSCH, - Option 2: UE follows the same behaviour as that in case of PUSCH with HP HARQ-ACK assuming two bits - FFS for CG-UCI PUSCH.

From our perspective, in order to avoid LP HARQ-ACK dropping, CG-UCI and LP HARQ-ACK should be multiplexed together based on current multiplexing scheme of Rel-16 NR-U[2] as follows

### 6.3.2 Uplink control information on PUSCH The following clauses 6.3.2.2, 6.3.2.3, and 6.3.2.5 apply regardless of whether the higher layer parameter UCI-MuxWithDifferentPriority is configured or not. The following clauses 6.3.2.1, 6.3.2.4, and 6.3.2.6 apply by assuming UCI-MuxWithDifferentPriority is not configured, or UCI-MuxWithDifferentPriority is configured and the UCIs for transmission on a PUSCH are of the same priority index, unless stated otherwise. In addition, clauses 6.3.2.1.4, 6.3.2.4.1.5, 6.3.2.4.2.5 and 6.3.2.6 also apply if UCI-MuxWithDifferentPriority is configured and CG-UCI is of a different priority index with HARQ-ACK.

Proposal 1: If LP HARQ-ACK with CG-UCI would be transmitted on HP PUSCH, CG-UCI and LP HARQ-ACK should be multiplexed together based on current legacy scheme as mentioned in section 6.3.2 of TS 38.212.

- Conclusion

In this contribution, we discuss about the intra-UE multiplexing and prioritization with the following proposal.

Proposal 1: If LP HARQ-ACK with CG-UCI would be transmitted on HP PUSCH, CG-UCI and LP HARQ-ACK should be multiplexed together based on current legacy scheme as mentioned in section 6.3.2 of TS 38.212.

- Reference

- Chairman’s Notes, RAN1#108-e, final, February 21st – March 3rd, 2022

- TS38.212 V17.1.0, 3rd Generation Partnership Project, Technical Specification Group Radio Access Network, Multiplexing and channel coding, (Release 17),