3GPP TSG RAN WG1 #109-e                                            R1-2203214

e-Meeting, May 9th – 20th , 2022

Source:	New H3C

Title:	Discussion for Evaluation on NR duplex evolution

Agenda Item:	9.3.1

Document for:	Discussion and Decision

- Introduction

In the RAN#95-e meeting, WID on Evolution of NR Duplex Operation was discussed and defined as below [1].

1.1 Justification

TDD is widely used in commercial NR deployments. In TDD, the time domain resource is split between downlink and uplink. Allocation of a limited time duration for the uplink in TDD would result in reduced coverage, increased latency and reduced capacity. As a possible enhancement on this limitation of the conventional TDD operation, it would be worth studying the feasibility of allowing the simultaneous existence of downlink and uplink, a.k.a. full duplex, or more specifically, subband non-overlapping full duplex at the gNB side within a conventional TDD band.

The NR TDD specifications allow the dynamic/flexible allocation of downlink and uplink in time and CLI handling and RIM for NR were introduced in Rel-16. Nevertheless, further study may be required for CLI handling between the gNBs of the same or different operators to enable the dynamic/flexible TDD in commercial networks. The inter-gNB CLI may be due to either adjacent-channel CLI or co-channel-CLI, or both, depending on the deployment scenario. One of the problems not addressed in the previous releases is gNB-to-gNB CLI.

This study aims to identify the feasibility and solutions of duplex evolution in the areas outlined above to provide enhanced UL coverage, reduced latency, improved system capacity, and improved configuration flexibility for NR TDD operations in unpaired spectrum. In addition, the regulatory aspects need to be examined for deploying identified duplex enhancements in TDD unpaired spectrum considering potential constraints.

1.2 Objective

1.2.1	Objective of SI

The objective of this study is to identify and evaluate the potential enhancements to support duplex evolution for NR TDD in unpaired spectrum.

In this study, the followings are assumed:

- Duplex enhancement at the gNB side

- Half duplex operation at the UE side

- No restriction on frequency ranges

The detailed objectives are as follows:

- Identify applicable and relevant deployment scenarios (RAN1).

- Develop evaluation methodology for duplex enhancement (RAN1).

- Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4).

- Identify possible schemes and evaluate their feasibility and performances (RAN1).

- Study inter-gNB and inter-UE CLI handling and identify solutions to manage them (RAN1).

  - Consider intra-subband CLI and inter-subband CLI in case of the subband non-overlapping full duplex.

- Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1).

- Study the feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4).

- Study the feasibility of and impact on RF requirements considering the self-interference, the inter-subband CLI, and the inter-operator CLI at gNB and the inter-subband CLI and inter-operator CLI at UE (RAN4).

- Note: RAN4 should be involved early to provide necessary information to RAN1 as needed and to study the feasibility aspects due to high impact in antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression.

- Summarize the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4).

Note: For potential enhancements on dynamic/flexible TDD, utilize the outcome of discussion in Rel-15 and Rel-16 while avoiding the repetition of the same discussion.

In this contribution, we provide our considerations on evaluation on NR duplex evolution.

- Discussion

  - Simulation assumptions

Based on the WID [1], we need develop evaluation methodology for duplex enhancement and study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels. The system level simulations is a choice to evaluate intra-subband CLI and inter-subband CLI for inter-gNB and inter-UE in case of the subband non-overlapping full duplex according to the agreed evaluation assumptions for the NR SI as described in [2]. The deployment scenario considered is the dense urban scenario, urban macro and indoor hotspot. Detail evaluation parameters are show in table 1 [2]. Penetration loss for UE-to-UE link are shown in table 2 and table 3[2].

The following duplex mode options are considered for the system level evaluations:

•	Case 1: Dynamic duplex mode

Any resource in duplex manner can be changed dynamically between DL and UL. In the evaluation, depending on the incoming traffic and the scheduler decision, any suband in the slots/symbols for duplex mode can carry DL or UL traffic.

Operation based on dynamic TDD is expected to cause so-called cross-link interference where the dominant interference for a transmission in one direction (e.g., downlink) is caused by another transmission in the other direction (e.g., uplink).

•	Case 2: Static duplex mode

A coordinated scheme where the DL:UL ratio for the allocated duplex mode slots/symbols is fixed for some period of time and the same DL:UL ratio is used by all nodes in the network. This scheme for the allocated duplex mode slots/symbols is equivalent to the traditional legacy FDD. In other words, the DL:UL ratio for the allocated duplex mode slots/symbols is the same and synchronous across all the nodes in the network.

Operation based on static duplex mode is immune to so-called cross-link interference while the DL to UL ratio for the allocated duplex mode slots follows a static or semi-static structure that is matched to the long-term statistics of the incoming DL to UL traffic ratio.

Table 1: Evaluation parameters specific to NR duplex evolution

| Parameters | Dense urban | Urban macro |  | Indoor hotspot |
| --- | --- | --- | --- | --- |
| Inter-BS distance | Macro-to-macro: 200m Macro-to-micro: 105m [TR36.897] Micro-to-micro: 57.9m | 500m |  | 20m |
| Minimum BS-UE (2D) distance | Macro-to-UE: 35m [TR36.897] Micro-to-UE: 10m [TR36.897] | 35m [TR36.897] |  | 0m [TR 38.901 [15]] |
| Minimum UE-UE (2D) distance | 3m [TR36.843] |  |  |  |
| Carrier frequency | Macro layer: 4 GHz, 30 GHz [TR38.913] Micro layer: 4 GHz, 30 GHz | 2GHz [TR38.913], 4 GHz, 30 GHz |  | 4 GHz, 30 GHz |
|  | For 30GHz: Un-paired spectrum is preferred. For 2GHz: paired spectrum is preferred. For 4GHz: both paired and un-paired spectrum can be considered. |  |  |  |
| Aggregated system bandwidth | 4GHz: Up to 200MHz (DL+UL) 30GHz: Up to1GHz (DL+UL) | 2GHz: Up to 40 MHz (DL+UL) 4GHz: Up to 200 MHz (DL+UL) 30GHz: Up to 1GHz (DL+UL) |  | 4GHz: Up to 200MHz (DL+UL) 30GHz: Up to 1GHz (DL+UL) |
| Simulation bandwidth | 20MHz per CC below 6GHz and 80 MHz per CC above 6GHz Note: For FDD, simulation BW is split equally between UL and DL Note: UE TX power scaling will impact final results |  |  |  |
| Large-scale channel parameters(*) | Below 6GHz: - Macro-to-UE: 3D UMa - Micro-to-UE: 3D UMi - Macro-to-Macro: 3D UMa (hUE =25m) - Macro-to-Micro: 3D UMa (hUE =10m) - Micro-to-Micro: 3D UMi (hUE =10m) - UE-to-UE: A.2.1.2 in TR36.843(***), penetration loss between UEs follows Table A.2.1-13 Above 6GHz: - Macro-to-UE: 5GCM UMa - Micro-to-UE: UMi-Street canyon - Macro-to-Macro: 5GCM UMa (hUE =25m) - Macro-to-Micro: 5GCM UMa (hUE =10m) - Micro-to-Micro: UMi-Street canyon (hUE =10m) - UE-to-UE: UMi-Street canyon (hBS =1.5m ~ 22.5m), penetration loss between UEs follows Table A.2.1-12 |  | Below 6GHz: - TRP-to-UE: ITU InH - TRP-to-TRP: ITU InH (hUE =3m) - UE-to-UE: A.2.1.2 in TR36.843 (***) Above 6GHz: - TRP-to-UE: 5GCM Indoor-office - TRP-to-TRP: 5GCM Indoor-office (hUE =3m) - UE-to-UE: 5GCM Indoor-office (hBS =1.5m) |  |
| Fast fading parameters(*) | Below 6GHz: - Macro-to-UE: 3D UMa - Micro-to-UE: 3D UMi - Macro to Macro: 3D UMa O-to-O (hUE =25m); ASA and ZSA statistics(**) updated to be the same as ASD and ZSD; ZoD offset = 0 - Macro to Micro: 3D UMa O-to-O - Micro to Micro: 3D UMi O-to-O (hUE =10m); ASA and ZSA statistics updated to be the same as ASD and ZSD; ZoD offset = 0 - UE to UE: InH for indoor to indoor, and 3D UMi for other cases. ASD and ZSD statistics updated to be the same as ASA and ZSA. Dual mobility support. Above 6GHz: - Macro-to-UE: 5GCM UMa - Micro-to-UE: UMi-Street canyon - Macro to macro: 5GCM UMa O-to-O (hUE =25m); ASA and ZSA statistics updated to be the same as ASD and ZSD; ZoD offset = 0 - Macro to micro: 5GCM UMa O-to-O - Micro to Micro: UMi-Street canyon O-to-O (hUE =10m); ASA and ZSA statistics updated to be the same as ASD and ZSD; ZoD offset = 0 - UE to UE: UMi-Street canyon; ASD and ZSD statistics updated to be the same as ASA and ZSA. Dual mobility support. |  | Below 6GHz: - TRP-to-UE: ITU InH - TRP-to-TRP: ITU InH (hUE =3m), ASA statistics updated to be the same as ASD - UE-to-UE: A.2.1.2 in TR36.843 (ITU InH), ASD statistics updated to be the same as ASA. Above 6GHz: - TRP-to-UE: 5GCM Indoor-office - TRP-to-TRP: 5GCM Indoor-office (hUE =3m), ASA and ZSA statistics(**) updated to be the same as ASD and ZSD - UE-to-UE: 5GCM Indoor-office (hBS =1.5m), ASD and ZSD statistics updated to be the same as ASA and ZSA |  |
| Traffic model | Baseline: - FTP traffic model 3 with packet size 0.1, 0.5 and 2.0Mbytes - Ratio of DL/UL traffic = {2:1}, {4:1} and {1: 1} for optional [TR36.828] |  |  |  |
| UE distribution | For FTP traffic model 3: 2/3 users randomly and uniformly dropped within the clusters, 1/3 users randomly and uniformly dropped throughout the macro geographical area, and 60 users per macro geographical area | For FTP traffic model 3: 10 users per macro TRP | For FTP traffic model 3: 10 users per TRP |  |
| Delay assumption | Companies to report the delay assumption used for coordination schemes if used in the simulations (delay assumption depends on RAN3 architecture discussion) |  |  |  |
| (*): The assumption is used as starting point for flexible duplex evaluation, and further update might be made. (**): Statistics of ASA/ASD and ZSA/ZSD include its mean, standard variance, correlation distance in the horizontal plane, and in-cluster angular spread (e.g., cluster ASA/ASD). (***): For outdoor to indoor case, and indoor to indoor case, use “Remaining Layout Options” in A.2.1.2 of TR36.843 for pathloss calculation, and “ITU-R IMT UMi” for LOS Probability derivation. For outdoor to indoor case, the penetration loss term “20.0+0.5* din” is excluded in pathloss formula given in A.2.1.2 of TR36.843, and the penetration loss is derived according to Table A.2.1-13. |  |  |  |  |

Table 2: Penetration loss for UE-to-UE link for 30GHz specific to NR duplex evolution

| Location of UE_x | Location of UE_y | Sub-scenario | Penetration loss (for around 30GHz) |
| --- | --- | --- | --- |
| Indoor | Indoor | In different building (if inter-user 2D distance > 50m) | is the building penetration loss as given by subclause 7.4.3 in TR 38.901 [15]. i=x, y |
|  |  | In the same building (if inter-user 2D distance ≤ 50m) | where Lconcrete is given by Table 7.4.3-1 in TR 38.901 [15], and ni is the floor number for UE_i, i=x, y. |
| Indoor | Outdoor | N.A. | is the building penetration loss as given by subclause 7.4.3 in TR 38.901 [15]. is the car penetration loss as given by subclause 7.4.3 in TR 38.901 [15]. |
| Outdoor | Indoor | N.A. | is the car penetration loss as given by subclause 7.4.3 in TR 38.901 [15]. is the building penetration loss as given by subclause 7.4.3 in TR 38.901 [15]. |
| Outdoor | Outdoor | N.A. | is the car penetration loss as given by subclause 7.4.3 in TR 38.901 [15]. i=x, y |

Table 3: Penetration loss for UE-to-UE link for 4GHz and 2GHz specific to NR duplex evolution.

| Location of UE_x | Location of UE_y | Sub-scenario | Penetration loss (for around 4GHz and 2GHz) |
| --- | --- | --- | --- |
| Indoor | Indoor | In different building (if inter-user 2D distance > 50m) | where PLtw=20dB, and in meter TR 36.814 is the distance from user to internal wall, i=x, y, and U(a,b) indicates uniform distribution. |
|  |  | In the same building (if inter-user 2D distance ≤ 50m) | for UEs on different floors TR 36.872; otherwise 0dB. |
| Indoor | Outdoor | N.A. | with PLtw=20dB and in meter is the building penetration loss as given by TR 36.814. is the car penetration loss as given by subclause 7.4.3 in TR 38.901 [15]. |
| Outdoor | Indoor | N.A. | is the car penetration loss as given by subclause 7.4.3 in TR 38.901 [15]. with PLtw=20dB and in meter is the building penetration loss as given by TR 36.814. |
| Outdoor | Outdoor | N.A. | is the car penetration loss as given by subclause 7.4.3 in TR 38.901 [15]. i=x, y |

Proposal 1: The system level simulation is used for evaluation on NR duplex evolution.

Proposal 2: Simulation assumption on flexible duplex in TR38.802 is used as starting point for evaluation on NR duplex evolution.

Proposal 3: Above-mentioned table 1, table 2 and table 2 related Simulation parameters are made as baseline for evaluation on NR duplex evolution.

Proposal 4: The following duplex mode options are considered for on NR duplex evolution.

• Case 1: Dynamic duplex mode

Any resource in duplex manner can be changed dynamically between DL and UL. In the evaluation, depending on the incoming traffic and the scheduler decision, any suband in the slots/symbols for duplex mode can carry DL or UL traffic.

Operation based on dynamic TDD is expected to cause so-called cross-link interference where the dominant interference for a transmission in one direction (e.g., downlink) is caused by another transmission in the other direction (e.g., uplink).

•Case 2: Static duplex mode

A coordinated scheme where the DL:UL ratio for the allocated duplex mode slots/symbols is fixed for some period of time and the same DL:UL ratio is used by all nodes in the network. This scheme for the allocated duplex mode slots/symbols is equivalent to the traditional legacy FDD. In other words, the DL:UL ratio for the allocated duplex mode slots/symbols is the same and synchronous across all the nodes in the network.

Operation based on static duplex mode is immune to so-called cross-link interference while the DL to UL ratio for the allocated duplex mode slots follows a static or semi-static structure that is matched to the long-term statistics of the incoming DL to UL traffic ratio.

Conclusion

In this contribution, we discuss about evaluation on NR duplex evolution with the following proposal.

Proposal 1: The system level simulation is used for evaluation on NR duplex evolution.

Proposal 2: Simulation assumption on flexible duplex in TR38.802 is used as starting point for evaluation on NR duplex evolution.

Proposal 3: Above-mentioned table 1, table 2 and table 2 related Simulation parameters are made as baseline for evaluation on NR duplex evolution.

Proposal 4: The following duplex mode options are considered for on NR duplex evolution.

• Case 1: Dynamic duplex mode

Any resource in duplex manner can be changed dynamically between DL and UL. In the evaluation, depending on the incoming traffic and the scheduler decision, any suband in the slots/symbols for duplex mode can carry DL or UL traffic.

Operation based on dynamic TDD is expected to cause so-called cross-link interference where the dominant interference for a transmission in one direction (e.g., downlink) is caused by another transmission in the other direction (e.g., uplink).

•Case 2: Static duplex mode

A coordinated scheme where the DL:UL ratio for the allocated duplex mode slots/symbols is fixed for some period of time and the same DL:UL ratio is used by all nodes in the network. This scheme for the allocated duplex mode slots/symbols is equivalent to the traditional legacy FDD. In other words, the DL:UL ratio

- Reference

- RP-213591, Study on Evolution of NR Duplex Operation, Moderator（CMCC）

- TR38.802, Study on New Radio Access Technology, Physical Layer Aspects