3GPP TSG RAN WG1 #109-e                                            R1-2203215

e-Meeting, May 9th – 20th , 2022

Source:	New H3C

Title:	Discussion for subband non-overlapping full duplex

Agenda Item:	9.3.2

Document for:	Discussion and Decision

- Introduction

In the RAN#95-e meeting, WID on Evolution of NR Duplex Operation was discussed and defined as below [1].

1.1 Justification

TDD is widely used in commercial NR deployments. In TDD, the time domain resource is split between downlink and uplink. Allocation of a limited time duration for the uplink in TDD would result in reduced coverage, increased latency and reduced capacity. As a possible enhancement on this limitation of the conventional TDD operation, it would be worth studying the feasibility of allowing the simultaneous existence of downlink and uplink, a.k.a. full duplex, or more specifically, subband non-overlapping full duplex at the gNB side within a conventional TDD band.

The NR TDD specifications allow the dynamic/flexible allocation of downlink and uplink in time and CLI handling and RIM for NR were introduced in Rel-16. Nevertheless, further study may be required for CLI handling between the gNBs of the same or different operators to enable the dynamic/flexible TDD in commercial networks. The inter-gNB CLI may be due to either adjacent-channel CLI or co-channel-CLI, or both, depending on the deployment scenario. One of the problems not addressed in the previous releases is gNB-to-gNB CLI.

This study aims to identify the feasibility and solutions of duplex evolution in the areas outlined above to provide enhanced UL coverage, reduced latency, improved system capacity, and improved configuration flexibility for NR TDD operations in unpaired spectrum. In addition, the regulatory aspects need to be examined for deploying identified duplex enhancements in TDD unpaired spectrum considering potential constraints.

1.2 Objective

1.2.1	Objective of SI

The objective of this study is to identify and evaluate the potential enhancements to support duplex evolution for NR TDD in unpaired spectrum.

In this study, the followings are assumed:

- Duplex enhancement at the gNB side

- Half duplex operation at the UE side

- No restriction on frequency ranges

The detailed objectives are as follows:

- Identify applicable and relevant deployment scenarios (RAN1).

- Develop evaluation methodology for duplex enhancement (RAN1).

- Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4).

- Identify possible schemes and evaluate their feasibility and performances (RAN1).

- Study inter-gNB and inter-UE CLI handling and identify solutions to manage them (RAN1).

  - Consider intra-subband CLI and inter-subband CLI in case of the subband non-overlapping full duplex.

- Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1).

- Study the feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4).

- Study the feasibility of and impact on RF requirements considering the self-interference, the inter-subband CLI, and the inter-operator CLI at gNB and the inter-subband CLI and inter-operator CLI at UE (RAN4).

- Note: RAN4 should be involved early to provide necessary information to RAN1 as needed and to study the feasibility aspects due to high impact in antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression.

- Summarize the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4).

Note: For potential enhancements on dynamic/flexible TDD, utilize the outcome of discussion in Rel-15 and Rel-16 while avoiding the repetition of the same discussion.

In this contribution, we provide our considerations on subband non-overlapping full duplex.

- Discussion

This contribution introduce two kinds of subband resource allocation including semi-static resource allocation and dynamic resource allocation.

  - Semi-static resource allocation

Based on the WID [1], in order to reduce inter-gNB and inter-UE CLI (Cross-link interference), we propose the method on semi-static subband resource allocation. Resource allocation across cells is easily configured consistently by semi-static subband resource allocation method. This can efficiently reduce co-frequency interference between cells. Semi-static subband resource allocation method mainly uses semi-static frequency division to distinguish DL and UL resources by sub-band (including at least 1 RB/1RBG) within one TDD time slot. DL and UL time domain resources within one TDD time slot can be configured by SLIV. Semi-static subband resource allocation between multiple cells is shown in Figures 1. As shown in Figure 1, Cell1 and Cell2 use the same frame structure and the same time/frequency resource allocation for full duplex time slots. DL and UL time domain resource allocations for cell 1 and cell2 are the same as the length of one time slot.

| Slot# | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Cell 1 | DL | DL | UL | DL Subband1 | DL Subband1 | DL | DL | UL | DL Subband1 | DL Subband1 |
|  |  |  |  | UL Subband2 | UL Subband2 |  |  |  | UL Subband2 | UL Subband2 |
| Cell 2 | DL | DL | UL | DL Subband1 | DL Subband1 | DL | DL | UL | DL Subband1 | DL Subband1 |
|  |  |  |  | UL Subband2 | UL Subband2 |  |  |  | UL Subband2 | UL Subband2 |

Figure 1 Semi-static subband resource allocation between multiple cells

Proposal 1: Support semi-static resource allocation for evolution of NR duplex operation by broadcast signalling.

  - Dynamic resource allocation

If gNB does not enable semi-static resources for duplex operation,  gNB could use DCI (downlink control indication) to dynamically schedule UE to send or receive DL/UL data within one TDD full duplex mode slot. As show in Figure 9, no overlapping the DL/UL resources are allocated for multiple UEs and coordination between cells is required to reduce co-frequency intra-subband CLI and inter-subband CLI in case of the subband non-overlapping full duplex among the cells.

Figure 2 Dynamic subband resource allocation

Proposal 2: Support dynamic resource allocation and coordination between cells for evolution of NR duplex operation.

Conclusion

In this contribution, we discuss about subband non-overlapping full duplex with the following proposal.

Proposal 1: Support semi-static resource allocation for evolution of NR duplex operation by broadcast signalling.

Proposal 2: Support dynamic resource allocation and coordination between cells for evolution of NR duplex operation.

- Reference

- RP-213591, Study on Evolution of NR Duplex Operation, Moderator（CMCC）