3GPP TSG RAN WG1 #109-e                                                                                R1-2203216

e-Meeting, May 9th – 20th , 2022

Source:	New H3C

Title:	Discussion for potential enhancements on dynamic/flexible TDD

Agenda Item:	9.3.2

Document for:	Discussion and Decision

- Introduction

In the RAN#95-e meeting, WID on Evolution of NR Duplex Operation was discussed and defined as below [1].

  - Justification

TDD is widely used in commercial NR deployments. In TDD, the time domain resource is split between downlink and uplink. Allocation of a limited time duration for the uplink in TDD would result in reduced coverage, increased latency and reduced capacity. As a possible enhancement on this limitation of the conventional TDD operation, it would be worth studying the feasibility of allowing the simultaneous existence of downlink and uplink, a.k.a. full duplex, or more specifically, subband non-overlapping full duplex at the gNB side within a conventional TDD band.

The NR TDD specifications allow the dynamic/flexible allocation of downlink and uplink in time and CLI handling and RIM for NR were introduced in Rel-16. Nevertheless, further study may be required for CLI handling between the gNBs of the same or different operators to enable the dynamic/flexible TDD in commercial networks. The inter-gNB CLI may be due to either adjacent-channel CLI or co-channel-CLI, or both, depending on the deployment scenario. One of the problems not addressed in the previous releases is gNB-to-gNB CLI.

This study aims to identify the feasibility and solutions of duplex evolution in the areas outlined above to provide enhanced UL coverage, reduced latency, improved system capacity, and improved configuration flexibility for NR TDD operations in unpaired spectrum. In addition, the regulatory aspects need to be examined for deploying identified duplex enhancements in TDD unpaired spectrum considering potential constraints.

1.2 Objective

Objective of SI

The objective of this study is to identify and evaluate the potential enhancements to support duplex evolution for NR TDD in unpaired spectrum.

In this study, the followings are assumed:

- Duplex enhancement at the gNB side

- Half duplex operation at the UE side

- No restriction on frequency ranges

The detailed objectives are as follows:

- Identify applicable and relevant deployment scenarios (RAN1).

- Develop evaluation methodology for duplex enhancement (RAN1).

- Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4).

- Identify possible schemes and evaluate their feasibility and performances (RAN1).

- Study inter-gNB and inter-UE CLI handling and identify solutions to manage them (RAN1).

  - Consider intra-subband CLI and inter-subband CLI in case of the subband non-overlapping full duplex.

- Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1).

- Study the feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4).

- Study the feasibility of and impact on RF requirements considering the self-interference, the inter-subband CLI, and the inter-operator CLI at gNB and the inter-subband CLI and inter-operator CLI at UE (RAN4).

- Note: RAN4 should be involved early to provide necessary information to RAN1 as needed and to study the feasibility aspects due to high impact in antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression.

- Summarize the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4).

Note: For potential enhancements on dynamic/flexible TDD, utilize the outcome of discussion in Rel-15 and Rel-16 while avoiding the repetition of the same discussion.

In this contribution, we further provide our considerations on potential enhancements on dynamic/flexible TDD.

- Discussion

  - Guard period

Guard period is used in the legacy TDD for the UL-DL switching and compensating for propagation delays. In the WID of FD[1] , the gNB is assumed worked at the FD mode, while the UE is assumed worked as the HD mode. gNB does not need GP anymore because the gNB should have two chains for handling both DL and UL simultaneously. However, the UE can only handle DL or UL at the same time. Therefore, the GP shall be reserved for HD UE.

Observation 1: GP shall be reserved for the purpose of HF UE even the gNB works in the FD mode.

  - Frame structure configurations in FD mode

This contribution introduces the frame structure configurations in the full duplex (FD) mode, where the FD symbol can be configured implicitly or explicitly. For the former one, the flexible symbol is used as FD symbol, for the latter one, a dedicated consecutive symbols are explicitly configured as FD symbol.

Flexible symbol as FD symbol

In the legacy TDD, the symbol can be configured to Flexible, and the flexible symbol can be used as DL, UL or GP. For backward compatibility with the legacy TDD, the functionality of the flexible symbol can be extended, which means the flexible symbol can be used as DL, UL, GP and FD symbol. From gNB perspective, some sub-bands in the F symbol can be used for DL, the other sub-bands can be used for UL. For UE, the whole F symbol is only for one direction. Figure 1 shows the frame structure in the FD mode, where the flexible symbols are used as FD symbols.

Figure 1. Frame structure in FD mode

As by WID in [1], for potential enhancements on dynamic/flexible TDD, utilize the outcome of discussion in Rel-15 and Rel-16 while avoiding the repetition of the same discussion. For the frame structure configuration in FD mode, the semi-static configuration and dynamic indication by DCI format 2_0  in the legacy TDD mode can be reused.

Proposal 1:  Extend the functionality of flexible symbol for supporting FD, and the frame structure configuration mechanism in legacy TDD can be reused.

Dedicated FD symbol

Too much flexible symbols in the frame structure will introduce the scheduling complexity in the FD mode. A number of dedicated symbols can be configured as the FD symbols in legacy TDD frame structure. As shown in Figure2, symbols #10-#13 in slot1 and symbols #0-#4 in slot2 are configured as FD symbol.

| Slot 0 |  |  |  |  |  |  |  |  |  |  |  |  |  | Slot 1 |  |  |  |  |  |  |  |  |  |  |  |  |  | Slot 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 |
| D | D | D | D | D | D | D | D | D | D | D | D | D | D | F | F | U | U | U | U | U | U | U | U | FD | FD | FD | FD | FD | FD | FD | FD | FD | D | D | D | D | D | D | D | D | D |

Figure 2. Dedicated FD symbol configuration.

In the FD mode, the UE still works in the FD mode, so the GP shall be reserved in the UL-DL switching. Too many GP symbols will reduce the resource efficiency, so the dedicated FD symbol should be configured consecutively in a period of frame structure. For one UE, the transmission should be scheduled before the reception in the configured FD symbols in order to avoid additional GP configurations. Let’s take figure 2 as an example, UE can be scheduled to transmit PUSCH in symbols #0 - #3 of slot 2, then be scheduled to receive PDSCH in slot symbol #3-#13 of slot 2.

Proposal 2: support the configurations of a number of dedicated symbols as FD symbols, the dedicated FD symbols should be consecutive in a period of the frame structure.

Conclusion

In this contribution, we discuss about potential enhancements on dynamic/flexible TDD with the following proposal.

Observation 1: Observation 1: GP shall be reserved for the purpose of HF UE even the gNB works in the FD mode.

Proposal 1:  Extend the functionality of flexible symbol for supporting FD, and the frame structure configuration mechanism in legacy TDD can be reused.

Proposal 2: support the configurations of a number of dedicated symbols as FD symbols, the dedicated FD symbols should be consecutive in a period of the frame structure.

- Reference

- RP-213591, Study on Evolution of NR Duplex Operation, Moderator（CMCC）