3GPP TSG RAN WG1 Meeting # 109-e	R1-2203222

E-Meeting, May 9th – May 20th, 2022

Agenda Item:	9.3.4

Source:	TCL

Title:	Discussion on Signaling Overhead Reduction for Inter gNB Information Exchange

Document for:	Discussion and Decision

# Introduction

In RANP#94-e meeting, a new study item (SI), study on evolution of NR duplex operation was approved  [1], which specified the study of sub-band non overlapping full duplex, feasibility and enhancement of  dynamic/flexible TDD using conventional TDD bands, and identification of possible solutions for CLI handling in dynamic TDD. Some of the objectives are given below:

- Identify applicable and relevant deployment scenarios (RAN1). - Develop evaluation methodology for duplex enhancement (RAN1). - Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4). - Identify possible schemes and evaluate their feasibility and performances (RAN1). - Study inter-gNB and inter-UE CLI handling and identify solutions to manage them (RAN1).

In this contribution we provide our views on the issues related to the information exchange among gNB, to support CLI mitigation.

# Inter-gNB Information Exchange signaling reductions

During Rel16 NR WIs related to the evolution of duplex operation, inter gNB information exchange including the intended DL/UL configuration were discussed, to support CLI mitigation in dynamic TDD. However, information exchange among gNB may increase the backhaul or OTA signaling especially in a dense deployment scenario. In addition, there was no discussion in Rel-16 which targets the backhaul or OTA signaling reduction for inter gNB information exchange, as the WI were deprioritized. Therefore, in this contribution we discuss several optimized methods of information exchange among gNBs with reduced number of backhaul or OTA signaling.

## Information exchange via Master/Slave gNB model

Master-slave gNBs model may reduce the backhaul or OTA signaling which are used to exchange the assistance information (i.e. the intended DL/UL configuration, traffic information, or CLI/SRS report) among gNBs to support CLI mitigation. In Master-slave gNB model, a cluster or group of neighbor’s gNBs may comprise of a master gNB and two or more than two slaves gNBs. The slave gNBs may exchange information with the master gNB, and the master gNB may calculate and tabulate the assistance information and share with the entire group or cluster of gNBs, as illustrated in Figure 1. The advantage of information exchange through Master slave gNB model is to minimize the repetitive information exchange among gNBs and thus reduces the backhaul or OTA signaling.

On the other hand, as illustrated in Figure 2, the assistance information are exchanged in a legacy way, such that, each gNB exchange information with each other gNB in a cluster or groups of gNBs and using more backhaul or OTA signaling as compared to the Master-slave gNB model.

Figure 1 Master/Slave model for assistance information exchange for CLI mitigation

Figure 2 Legacy way of assistance Information exchange for CLI mitigation

Observation 1: Information exchange among gNB through a master slave gNB  model reduces the backhaul or OTA signaling significantly as compared to the information exchange through legacy way.

Proposal 1: Study master slave gNB model for the assistance information exchange among gNB to reduce the backhaul or OTA signaling among gNB.

## Information exchange above a threshold value

Information exchange among gNB to support CLI mitigation above a certain threshold value may also reduce the backhaul or OTA signaling which are used for information exchange among gNBs. For instance, when a DL or UL traffic of a UE in a cell is above a threshold value, the gNB may exchange information among teach others. Similarly, when the DL or UL traffic of a UE in a cell is below the threshold value, the information may not be exchange among gNBs. The value of threshold may depends on DL or UL traffic and it can be define in terms of slots or sub-frames. This method is beneficial in targeting those scenarios where the DL or UL traffic of UEs are very small. For instance, the traffic of the industrial wireless sensors, or the traffic of the industrial actuators with small transport block may accommodate in a single slot/sub-frame and it may create negligible CLI for very short interval in the neighbor cells. Therefore, establishing a certain threshold for the assistance information exchange may reduce the backhaul or OTA signaling among gNB.

Observation 2: Information exchange among gNB above a certain threshold value of DL or UL slots may reduce the backhaul or OTA signaling.

Proposal 2: Study the information exchange among gNBs above a certain threshold value of DL or UL traffic.

## Offline Information exchange

Offline information exchange among neighbor’s gNBs or a cluster of gNBs, may help to reduce the real time online backhaul or OTA signaling. Offline information exchange can target those scenarios where the UEs are static, and transmit or receive a DL dominant or UL dominant data traffic in a certain time. For instance, video surveillance traffic is UL dominant and transmitted in a certain time. Similarly, the industrial wireless sensors/actuators are DL dominant and transmitted in a certain time. Each serving gNB can share its static UEs UL or DL traffic information with its neighbor gNB or master gNB within a cluster or group of neighbor gNB offline to reduce the real time online signaling.

Observation 3: offline Information exchange may reduce the real time backhaul or OTA signaling among gNBs.

Proposal 3: Study the offline Information exchange among gNBs to reduce the real time online backhaul or OTA signaling for the assistance information exchange among gNB.

# Conclusion

In this contribution, we discussed our view on backhal or OTA signalling reduction methods for inter –gNB assistance information exchange to support CLI mitigation, and made the following observations and proposals.

Observation 1: Information exchange among gNB through a master slave gNB  model reduces the backhaul or OTA signaling significantly as compared to the information exchange through legacy way.

Observation 2: Information exchange among gNB above a certain threshold value of DL or UL slots may reduce the backhaul or OTA signaling.

Observation 3: offline Information exchange may reduce the real time backhaul or OTA signaling among gNBs.

Proposal 1: Study master slave gNB model for the assistance information exchange among gNB to reduce the backhaul or OTA signaling among gNB.

Proposal 2: Study the information exchange among gNBs above a certain threshold value of DL or UL traffic.

Proposal 3: Study the offline Information exchange among gNBs to reduce the real time online backhaul or OTA signaling for the assistance information exchange among gNB.

# References

RP-213591, “study item on evolution of NR duplex operation”, 3GPP TSG RAN#94-e, Dec 6th – 17th, 2021.